Table 7. 	Annual average load estimates for nutrients and trace metals discharged to Mamala Bay and adjacent waters

from nonpoint source discharges. Nutrient loading data given as metric tonnes (T)per year. Metal loading

given as kg per year. From data in Stevenson et al. (1995)

| Water Body Receiving Nonpoint Source Discharge | Flow (m 3 x 10 6) | Nitrate/Nitrite (T) | Total N (T) | Total P (T) | Cd (kg) | Cu (kg) | Pb (kg) | Zn (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ala Wai Canal | 20.8 | 19 | 61 | 7.8 | 2.18 | 590 | 218 | 2,449 |
| Kewalo Basin | 4.6 | 2.6 | 8 | 0.99 | 0.73 | 136 | 72 | 1,179 |
| Keehi Lagoon | 62.4 | 41 | 125 | 15 | 56.3 | 1,361 | 562 | 7,802 |
| Pearl Harbor (E. Loch) | 47.3 | 40 | 101 | 11 | 18.1 | 671 | 181 | 2,540 |
| Pearl Harbor (M. Loch) | 45.6 | 39 | 113 | 13 | 12.7 | 508 | 127 | 1,996 |
| Pearl Harbor (W. Loch) | 100.3 | 118 | 303 | 34 | 58.1 | 1,270 | 572 | 8,981 |
| Ewa Plain | 5.5 | 9.1 | 29 | 3.4 | 4.54 | 82 | 40 | 526 |