Table 6. 	Annual average load estimates for conventional pollutants and microbiological pollutants discharged

to Mamala Bay and adjacent waters from nonpoint source discharges. All loading data for TSS, BOD,

COD and Oil and Grease given as metric tons (Tonnes) per year. Loading data for microbiological pollutants given

as CFU X 1015 per year. See Stevenson et al. (1995) for description of drainage basins for each

waterbody.

| Water Body Receiving Nonpoint Source Discharge | Flow (m 3 x 10 6) | TSS | BOD | COD | Clostridium perfringens | Enterococci | Fecal Coliform |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ala Wai Canal | 20.8 | 9,389 | 232 | 1,438 | 0.052 | 3.9 | 3.2 |
| Kewalo Basin | 4.6 | 576 | 39.9 | 232 | 0.016 | 1.2 | 0.9 |
| Keehi Lagoon | 62.4 | 19,777 | 518 | 3,195 | 0.13 | 9.6 | 7.9 |
| Pearl Harbor (E. Loch) | 47.3 | 20,593 | 366 | 2,193 | 0.043 | 3.2 | 23.9 |
| Pearl Harbor (M. Loch) | 45.6 | 20,639 | 351 | 2,105 | 0.031 | 2.3 | 2.2 |
| Pearl Harbor (W. Loch) | 100.3 | 27,443 | 746 | 4,125 | 0.13 | 10 | 8.5 |
| Ewa Plain | 5.5 | 2,694 | 78.9 | 406 | 0.0098 | 0.73 | 0.66 |