Table 3. 	Percentage of various pollutants entering Mamala Bay from four sub-embayments (retention basins) based upon proportion of the pollutant in the suspended fraction and percent solids retained in each basin. From Stevenson et al. (1995)

| Pollutant | % Suspended | Ala Wai Canal (50% Retained) | Pearl Harbor (95% Retention) | Kewalo Basin (95% Retention) | Keehi Lagoon (95% Retention |
| --- | --- | --- | --- | --- | --- |
| BOD | 0 | 100 | 100 | 100 | 100 |
| COD | 0 | 100 | 100 | 100 | 100 |
| TSS | 100 | 50 | 5 | 5 | 5 |
| TDS | 0 | 100 | 100 | 100 | 100 |
| TP | 40 | 80 | 62 | 62 | 62 |
| DP | 0 | 100 | 100 | 100 | 100 |
| TKN | 20 | 90 | 81 | 81 | 81 |
| NO2/NO3 | 50 | 75 | 52 | 52 | 52 |
| Lead | 90 | 15 | 10 | 10 | 10 |
| Copper | 60 | 70 | 43 | 43 | 43 |
| Zinc | 40 | 80 | 62 | 62 | 62 |
| Cadmium | 70 | 65 | 34 | 34 | 34 |

BOD = Biochemical Oxygen Demand; COD = Chemical Oxygen Demand; TSS = Total Suspended Solids; TDS = Total Dissolved Solids; TP = Total Phosphorus; DP = Dissolved Phosphorus; TKN = Total Kjeldahl Nitrogen.