Table 6.  	Comparison of chemical stressor concentrations in WWTP effluents discharging to Mamala Bay with Hawaii Water Quality Standards. BOD, Oil and Grease, and TSS data are in mg/L. All other data are in g/L. Numbers in parentheses are the number of analyses for which the stressor concentration was above the detection limit. Analytes in bold type have effluent concentrations in the same order of magnitude as the Hawaii WQS.

| Pollutant | Hawaii Acute Saltwater Standard | Hawaii Chronic Saltwater Standard | Sand Island Effluent | Honouliuli Effluent | Fort Kam. Effluent |
| --- | --- | --- | --- | --- | --- |
| Ammonia | 3.5 | ns1 | nm2 | nm | 1.9 (31) |
| Benzene | 1,700 | ns | 9.9 (7) | nd | nd |
| BOD (mg/L) | ns | ns | 112.5 (96) | 112.2 (60) | 7.8 (48) |
| Cadmium | 43 | 9.3 | 2.6 (1) | nd | nd |
| Chloride ion | 13 | 7.5 | na3 | na | 2.6 (48) |
| Chromium | 1,100 | 50 | 22.0 (1) | nd | nd |
| Copper4 | ns | ns | 17.9 (5) | 32.6 (4) | 18.5 (4) |
| Diethylphthalate | ns | ns | nd5 | 21.2 (1) | nd |
| Lead | 140 | ns | 39.7 (2) | nd | 24.5 (1) |
| Methylchloride | ns | ns | 6.8 (4) | nd | nd |
| Oil & Grease (mg/L) | ns | ns | 26.2 (44) | 31.6 (21) | 3.5 (32) |
| Phenol | 170 | ns | 15.3 (1) | nd | nd |
| Tetrachloroethylene | 3,400 | 145 | 4.8 (1) | nd | nd |
| Toluene | 2,100 | ns | 6.0 (3) | nd | nd |
| TSS (mg/L) | ns6 | ns6 | 36.9 (96) | 54.6 (60) | 20.5 (48) |
| Zinc | 95 | 86 | 61.0 (8) | 79.5 (5) | 104.8 (8) |

1.	“ns” indicates no standard has been established.

2.	“nm” indicates not measured at the WWTP for which data are available, or that measurements were so few in number (one or two) that a long-term average was not considered practical.

3.	“na” indicates not applicable; chlorination was performed only at Fort Kamehameha WWTP.

4.	While no Hawaii water quality standards exist for copper, and while effluent concentrations are not limited for the plants, the high toxicity of copper and the effluent limits applied in other locations (e.g., San Francisco, effluent limitation of 79 ug/L [US EPA Region IX, 1990]), we include copper as a chemical stressor of concern in this document.

5.	“nd” indicates not detected; the pollutant was not measured in any samples

6.	Standards exist for light extinction and turbidity, but not TSS.