TABLE OF CONTENTS

EXECUTIVE SUMMARY	  i

1.0	INTRODUCTION	1-1

1.1	Scope of Work	1-2

1.2	Objectives	1-3

1.3	Project Organization	1-3

1.4	Background for Modeling Approach	1-3

1.5	Report Organization	1-6

Chapter 1.0 - References

2.0	METHODS	2-1

2.1	Conceptual Description of Model	2-1

2.2	Literature Search for Selected Pathogens	2-4

2.3	Analysis and Simulation Approach	2-6

2.3.1	Giardia and Ala Moana Beach	2-6

2.3.2	Other Microorganism and Beach Combinations	2-11

Chapter 2.0 - References

3.0	RESULTS	3-1

3.1	Giardia and Ala Moana Beach	3-1

3.2	Other Pathogen and Beach Combinations	3-4

4.0	CONCLUSIONS	4-1

List of Figures

2.1	Model Structure

2.2	Beach and Lifeguard Station Locations

3.1a	Effect of Increasing Water Flow Rate Out of Swimming/Surfing Area (FD) on Prevalence

3.1b	Effect of Increasing Water Flow Rate Out of Swimming/Surfing Area (FD) on Pathogen Concentration

3.2	Average Daily Prevalence for Ala Moana Beach

3.3	Average Daily Prevalence for Waikiki Beach

3.4	Pathogen Concentrations for Ala Moana Beach

3.5	Pathogen Concentrations for Waikiki Beach

A.1	Ala Moana -- Giardia, Rate of Shedding vs Pathogen Concentration

A.2	Ala Moana -- Cryptosporidium, Rate of Shedding vs Pathogen Concentration

A.3	Ala Moana -- Salmonella, Rate of Shedding vs Pathogen Concentration

A.4	Ala Moana -- Enteroviruses, Rate of Shedding vs Pathogen Concentration

A.5	Sampling Range for BM  The Fraction of the Population that Visits the Beach Each Day During a Given Month of the Year  Ala Moana Beach

A.6	Sampling Range for BM  The Fraction of the Population that Visits the Beach Each Day During a Given Month of the Year  Waikiki Beach

A.7	Sampling Range for SM  The Fraction of the Beachgoers that Swim or Surf Each Day During a Given Month of the Year  Ala Moana Beach

A.8	Sampling Range for SM  The Fraction of the Beachgoers that Swim or Surf Each Day During a Given Month of the Year  Waikiki Beach

A.9	WNS  Average Daily Giardia Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.10	WNS  Average Daily Cryptosporidium Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.11	WNS  Average Daily Salmonella Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.12	WNS  Average Daily Enteroviruses Concentration of Pathogen from Sources Other than Shedding at Ala Moana Beach

A.13	WNS  Average Daily Giardia Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.14	WNS  Average Daily Cryptosporidium Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.15	WNS  Average Daily Salmonella Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.16	WNS  Average Daily Enteroviruses Concentration of Pathogen from Sources Other than Shedding at Waikiki Beach

A.17	FD  Average Daily Flow Rate Out  Ala Moana Beach

A.18	FD  Average Daily Flow Rate Out  Waikiki Beach

List of Figures (continued)

A.19	VD  Average Daily Volume  Ala Moana Beach

A.20	VD  Average Daily Volume  Waikiki Beach

List of Tables

1.1	Project Organization

2.1	Equations, Variables and Parameters

2.2	Parameterization for Giardia lamblia and Ala Moana Beach

2.3	List of Dependent Parameters and Functional Dependence on the Sampled Parameters

2.4	Microorganism-Dependent Parameters for Giardia lamblia

2.5	Microorganism-Dependent Parameters for Cryptosporidium spp.

2.6	Microorganism-Dependent Parameters for Salmonella spp.

2.7	Microorganism-Dependent Parameters for Enteroviruses

3.1	Average Daily Prevalence per 100,000 Statistics for Six Scenarios for Giardia and Ala Moana Beach

3.2	Average Daily Prevalence per 100,000 Statistics for Three Scenarios, Four Pathogens and Two Beaches

A.1	Dose-Response for Giardia lamblia

A.2	Dose-Response for Cryptosporidium

A.3	Dose-Response for Salmonella typhi

A.4	Dose-Response for Rotavirus

A.5	Beaches and Corresponding Lifeguard Stations

List of Appendices

A	Model Parameterization

B	Pathogen Monitoring Data

C	BM - Fraction of Population that Visits the Beach Each Day During a Given Month of the Year

D	SM - Fraction of Beachgoers that Swim or Surf Each Day During a Given Month of the Year

E	Ala Moana Beach - Time-Varying Parameters from HydroQual Data

F	Waikiki Beach - Time-Varying Parameters from HydroQual Data

EXECUTIVE SUMMARY

The Mamala Bay Study Commission is conducting a comprehensive study of the sources and effects of point and non-point pollution in Mamala Bay. The study will result in recommendations for strategies to reduce pollution levels in Mamala Bay to protect human health and the marine environment. EOA, Inc. (EOA) was retained by PRC Environmental Management, Inc. (PRC) to perform an assessment of the public health risk associated with accidental exposure to microbial pathogens during recreational use of Mamala Bay waters.

The primary objectives of this project were to: 1) apply an existing quantitative microbial risk assessment model to estimate the level of microbial risk associated with recreational exposure to Mamala Bay waters; 2) evaluate how public health risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers; 3) identify important parameters that impact the risk assessment results.

The risk assessment model used for this project is based closely on models used in infectious disease epidemiology. Advantages of this type of model include that it can be used to integrate and organize diverse data bearing on disease risk, account for immunity to disease, model aspects of the transmission dynamics of the agent in the environment and explicitly acknowledge the uncertainty and variability in the many parameter values characteristic of comprehensive models.

Four microbial pathogens (Giardia lamblia, Cryptosporidium spp., Salmonella spp. and enteroviruses) were selected for inclusion in the risk assessment. A literature search was performed on the pathogens to help parameterize the model. For use as input to the risk assessment model, modeled pathogen concentrations in the recreational use water were provided by the Mamala Bay Study group responsible for fate and transport modeling, HydroQual, Inc. Data from Mamala Bay Study monitoring groups were used to define the rate of direct pathogen shedding by swimmers/surfers. Two Mamala Bay beaches (Ala Moana and Waikiki) were included in the risk assessment. Data on attendance and the numbers of swimmers and surfers using these two beaches were provided to EOA by PRC and used for model parameterization.

Average daily prevalences modeled for disease associated with exposure to the selected pathogens at the two Mamala Bay beaches did not vary significantly from the modeled background prevalences in the population. (Background prevalence is the expected prevalence in the population when exposure to recreational water is not the vehicle of disease transmission).

Increasing the concentration of pathogen from sources other than direct shedding by swimmers and surfers by an order of magnitude did not significantly increase the prevalence of disease in the population above background. The risk assessment modeling results, based on available water quality pathogen monitoring and modeling data, therefore suggest that water quality management strategies designed to prevent additional pathogens from point and non-point sources from reaching the beach would not appear to affect the disease prevalences associated with the selected pathogens. It should be noted that this conclusion is based on the assumption that the uncertainty in the water quality modeling results used as input to the risk assessment model is less than approximately an order of magnitude.

1.0	INTRODUCTION

The Mamala Bay Study Commission is conducting a comprehensive study of the sources and effects of point and non-point pollution in Mamala Bay. The study will result in recommendations for strategies to reduce pollution levels in Mamala Bay to protect human health and the marine environment. It consists of ten individual projects referred to as MB-1 through MB-10. This study is part of project MB-10. MB-10 involves reviewing existing data and new data collected by other Mamala Bay Study project teams, conducting ecological and human health risk assessments and identifying and ranking alternative water quality management strategies based on the risks to the ecosystem and human health.  One principal concern of project MB-10 is the public health risk associated with accidental exposure to microbial pathogens during recreational use of Mamala Bay waters. EOA, Inc. (EOA) was retained by PRC Environmental Management, Inc. (PRC) to address this concern by performing a microbial risk assessment for Mamala Bay. This report documents the results of the microbial risk assessment.

The work performed for this study is closely tied to work performed by other Mamala Bay Study groups. Part of the input to the microbial risk assessment model was provided by the group responsible for fate and transport modeling, HydroQual, Inc. (HydroQual) of Mamala Bay Study group MB-5. HydroQual provided predicted pathogen concentrations at the beaches over a nine-month modeling simulation period. These estimated concentrations were for pathogens from sources other than shedding by swimmers and surfers in the waters used for recreation. In addition, data from Mamala Bay Study monitoring groups (MB-7) were used to define the rate of direct pathogen shedding by swimmers/surfers. It should also be noted that the microbial risk assessment results will be used in conjunction with the results of the chemical risk assessment performed by PRC as part of project MB-10 to estimate the overall public health risk associated with recreational exposure to Mamala Bay waters and, if necessary, to help develop water quality management strategies.

1.1	Scope of Work

EOA's scope of work was comprised of the five major tasks described below.

Task 1:	Literature Search

EOA conducted a literature search for selected pathogens (Giardia lamblia, Cryptosporidium spp., Salmonella spp. and enteroviruses)

Task 2:	Assessment of How to Apply Available Data to Microbial Risk Assessment Model

EOA assessed how to apply data provided by other Mamala Bay study groups as input to a risk assessment model. This included human exposure data from local lifeguard beach surveys and water quality and hydraulic modeling data.

Task 3:	Apply Model

EOA estimated the public health risk for the selected microorganisms and beaches and evaluated how risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers.

Task 4:	Uncertainty Analysis

EOA performed an uncertainty analysis. Parameters that impact the risk assessment results were identified.

Task 5:	Report

EOA prepared this report which documents the work performed and presents our conclusions and recommendations.

1.2	Objectives

The primary objectives of this project were to: 1) apply an existing quantitative microbial risk assessment model to estimate the level of microbial risk associated with recreational exposure to Mamala Bay waters; 2) evaluate how public health risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers; 3) identify parameters that impact the risk assessment results.

1.3	Project Organization

Table 1.1 presents the members of the project team and their affiliations and briefly describes each member's role in the project.

1.4	Background for Modeling Approach

To perform this study, it was necessary to select a methodology for estimating the risk of waterborne disease transmission. As current exposures to environmental pathogens are generally quite low in industrialized countries, field epidemiology may not produce sufficiently sensitive information for assessing risks associated with exposure to pathogens during recreational use of water.  An alternative to the epidemiological approach is the quantitative estimation of the intensity of human exposure and the probability of human response from this exposure.  This approach is highly developed in assessing cancer risks arising from environmental exposures to chemical agents, and has resulted in a field of study called quantitative risk assessment.

Because environmental risk assessment is subject to a variety of uncertainties, the process is often cast in probabilistic terms.  Moreover, field data are frequently unavailable to quantify some elements of the process, and mathematical modeling is used to bridge these data gaps.  The principal advantage of mathematical modeling in risk assessment applications is that it makes assumptions explicit, including structural mechanisms relating human exposure to pathogens and the public health outcome and quantitative assumptions such as the dose-response relationship.  A mathematical model organizes data and assumptions in a framework leading to quantitative predictions and can be an indispensable tool for decision making.  However, the model itself brings no new data or information to the process.  Thus the biological significance of a model's output is completely dependent on the appropriateness and accuracy of the assumptions used to build the model.

Past attempts to provide a quantitative framework for the assessment of human health risks associated with the ingestion of waterborne pathogens have generally focussed on the probability of individual infection or disease as a result of a single exposure event.  Most models described in the literature are of the same generic form.1-4  They give a point estimate of the probability of a particular exposure leading to infection or disease in a single individual and, except for Dudley's work1, carry little or no information about the uncertainty or variability in this estimate. Much of quantitative risk assessment, in particular, focuses on a point estimate of the response probability, often using worst-case assumptions for exposure and other parameters.  From a public health perspective, the probable number of people infected in an exposed population is more meaningful than the probability of individual infection.  In the past, the probability of individual infection (using worst-case assumptions) has sometimes been multiplied by the population number in an attempt to predict the disease incidence in the population. This may lead to unrealistically high risk forecasts.

The project team took a somewhat different point of view in a risk analysis of waterborne disease carried out for the U.S. Army.5-7  In this work a population perspective was taken and the analysis was carried beyond the risk of infection to an individual by estimating the probability distribution of the number of infected/diseased people in the exposed population.  One feature of the Army model was its probabilistic treatment of dose-response data (i.e., data which provide a quantitative linkage between the number of organisms ingested and the probability of infection or overt disease).  From this model's population perspective, each member of the population received a different dose and also had a different probability of responding to this dose.  The combination of these two factors resulted in each member of the population carrying a different probability of becoming infected or diseased.

In general, the above models assume that the populations are homogeneous and the disease transmission processes static. The risk assessment model used for this project takes advantage of a large literature describing the use of deterministic and stochastic dynamic population models in the study of epidemics.8  These epidemiological models emphasize the importance of the changing immune status of a population over time and are therefore dynamic, requiring a subdivision of the population by susceptibility status.  Thus an epidemiological risk assessment model that accounts for immunity and the transmission dynamics of the system was used for this project.

One central issue in biological risk assessment is how to extract information from biological data, which tends to be highly uncertain and variable.  In particular, the uncertainty and variability of factors affecting infectious disease transmission limit the usefulness of traditional curve-fitting techniques.  An alternative goodness-of-fit procedure that explicitly acknowledged these uncertainties and variabilities was therefore used for this project.  The approach consisted of assigning probability distributions to each parameter, sampling these distributions during Monte Carlo simulations, and using a binary classification to assess the output of each simulation.

1.5	Report Organization

Chapter 2.0 presents a conceptual description of the risk assessment model and the methods used to apply the model to examine the risk associated with ingestion of waterborne pathogens during recreational use of Mamala Bay.  The methodology used for addressing the uncertainties in the process is given.  Chapter 3.0 presents the results of the study. The estimated health risks associated with four microorganisms and two beaches are compared. An evaluation of how health risk could change with order of magnitude variations in contribution of pathogen to the swimming/surfing area from sources other than shedding by swimmers/surfers is presented. Using the results of the uncertainty analysis, important parameters that impact the risk assessment results are identified. Chapter 4.0 presents our conclusions.

Chapter 1.0 - References

1.	Dudely, R.H., K.K. Hekimain, and B.J. Mechalas, "A Scientific Basis for Determining Recreational Water Quality Criteria," Journal of the Water Pollution Control Federation, 48, 2761-2777 (1976).

2.	Fuhs, G.W., "A Probabilistic Model of Bathing Beach Safety," The Science of the Total Environment, 4, 165-175 (1975).

3.	Hass, C.N., "Estimation of Risk Due to Low Doses of Microorganisms: A Comparison of Alternative Methodologies," American Journal of Epidemiology, 55, 573-82 (1983).

4.	Regli, S., J.B. Rose, C.N. Haas, and C.P. Gerba, "Modeling the Risk from Giardia and Viruses in Drinking Water," Journal of the American Water Works Association, 83(11), 76-84 (1991).

5.	Cooper, R.C., A.W. Olivieri, R.E. Danielson, P.G. Badger, R.C. Spear, and S. Selvin, Evaluation of Military Field-Water Quality, Volume 5: Infectious Organisms of Military Concern Associated With Consumption: Assessment of Health Risks and Recommendations for Establishing Related Standards, (Lawrence Livermore National Laboratory, 1986).

6.	Olivieri, A.W. et al., "Risk Assessment of Waterborne Infectious Agents," Proceedings of the International Conference on Development and Application of Computer Techniques to Environmental Studies, Los Angeles (1986).

7.	Olivieri, A.W. et al., "Risk of Waterborne Infectious Illness Associated with Diving in the Point Loma Kelp Beds, San Diego, CA," Proceedings of the ASCE 1989 Specialty Conference on Environmental Engineering, Austin, Texas (1989).

8.	Anderson, R.M., and R. May, Infectious Diseases of Human Dynamics and Control, (Oxford University Press, New York, 1991).

2.0	METHODS

This chapter describes the methods used to perform the risk assessment.

2.1	Conceptual Description of Model

The structure of the risk assessment model is illustrated in Figure 2.1.  The model is composed of five state variables, one output variable and 15 parameters, as summarized in Table 2.1.  Four of the state variables represent the human population, which is divided into four epidemiological groups:

X - susceptible individuals

Y - infectious/asymptomatic individuals

Z - non-infectious/asymptomatic individuals

D - infectious/symptomatic individuals

Individuals in state X are susceptible to infection. For the remaining groups, the terms infectious or non-infectious define whether or not individuals sheds pathogen in their stool, and the terms symptomatic and asymptomatic define whether or not an individual exhibits symptoms of disease. The state variables X, Y, Z and D keep track of the population levels in each group.  The remaining state variable, WS, keeps track of the concentration of pathogen in the water to which the population is exposed associated with direct shedding of pathogen by swimmers and surfers. The movement of individuals from one state to another and the concentration of pathogen are governed by the set of five differential equations shown in Table 2.1.

The rate at which members of the population move from state X to state Y is governed by two factors.  One is the background rate of infection, which accounts for non-outbreak transmission due to exposure routes other than ingestion of water during recreational use.  The second is a dose-response term specific to the scenario under evaluation, which is dependent on the pathogen concentration in recreational water and the amount of water ingested.

Once in state Y, an individual can move in any given time step to either state D or state Z. The rates of these two transitions, represented respectively by the parameters  and , are dependent on each other, i.e., at any given time step an individual in state Y will, with probability of 1, either stay in this state, move to state D, or move to state Z.

Individuals in state D, who show symptoms of disease and shed pathogen, move to state Z at a rate of . Individuals in state Z are asymptomatic and do not shed pathogen. The parameter  is defined as the rate at which symptoms of disease disappear as an individual recovers, i.e., the reciprocal of the duration of symptoms. This definition was chosen because state D is used to calculate average daily prevalence in the population, which is the model output used to assess risk. To minimize the number of state variables, it is assumed that an infectious/symptomatic individual will transition directly to the non-infectious/asymptomatic state.

Individuals in state Z revert back to state X at a rate of . By definition,  is the reciprocal of the period of time for an immune individual to become susceptible, i.e., the rate of immunity loss. Thus it is assumed that non-infectious/asymptomatic individuals in state Z are immune.

In addition to movement of individuals among the epidemiological states, the model also describes the concentration of the waterborne pathogen at the exposure site.  The pathogen may arrive at the exposure site in two ways.  First, individuals in state Y directly shed pathogen into the water used for swimming and surfing at a rate of . Second, pathogen from ocean wastewater outfalls and non-point sources such as the Ala Wai canal may migrate to the recreational waters. The concentration of pathogen in the recreational waters from these sources is given by the parameter WNS and was estimated using water quality modeling data provided by HydroQual.

Assumptions made by the model include the following:

	The period of time that an individual is asymptomatic and infectious is short relative to the duration of the symptomatic and infectious period.

	Background disease transmission occurs independently of the water recreation scenario under study.

	Exposure to pathogen occurs via ingestion of recreational water containing pathogen.

	The population is homogeneous with respect to susceptibility to disease.

To describe the 15 model parameters, 20 pieces of data were required.  Therefore 20 sampling parameters were established, five of which varied with time.  With the exception of the three time-varying sampling parameters that were inputted for each day of the model simulation (WNS, FD and VD), lower and upper bounds were selected for each parameter to account for the variability of the data used to parameterize the model. The ranges for the two other time-varying sampling parameters (BM and SM) were varied for each month of the simulation.

The 20 parameters were sampled from uniform distributions, except for values that spanned three or more orders of magnitude, in which case log uniform sampling was used.  Tables 2.2 lists the 20 sampling parameters and classifies the parameters as biological-, community- or water quality/flow-based parameters.  Four of the 15 model parameters are dependent on sampling parameters. Table 2.3 shows the relationship between the four dependent parameters and the appropriate sampling parameters. Appendix A describes the model parameterization in detail. Appendix B contains pathogen monitoring data provided by Mamala Bay Study group MB-7 used to define the rate of shedding of pathogen by swimmers and surfers (see Appendix A). Appendices C and D give the respective ranges that BM and SM were sampled from for each month of the simulation. Appendices E and F give the values used for WNS, FD and VD for each month of the simulation for each of the two beaches included in the risk assessment.

2.2	Literature Search for Selected Pathogens

Four waterborne pathogens were selected to be included in the risk assessment. Giardia lamblia  and Cryptosporidium spp. were selected to represent the protozoan pathogens, Salmonella spp. was selected to represent the bacterial pathogens and enteroviruses were selected to represent the viral pathogens. A literature review was performed on the selected microorganisms to establish ranges of values for appropriate model parameters.  The first step in the search was to review a literature search performed previously as part of the microbial risk assessment for the U.S. Army described in Chapter 1.0. The Army review was then updated by collecting new relevant data.  The emphasis was on recent literature (1980 to the present).  An information retrieval service was used to access four literature data bases: Medline, Water Resources Abstracts, SciSearch and Ei Compendex.  These data bases were selected following a review of readily accessible data bases and the relevant journals which they include.  Medline was the primary data base used; the other three data bases were used to search for articles in two journals not included in Medline but deemed important to this study (Journal of the Water Pollution Control Federation and the American Water Works Association Journal).  Selected relevant key words and the microorganism's name were searched for in titles and author's key word lists, and a list of titles was generated.  The selected key words were:

| virulence | dose | outbreak | mortality |
| --- | --- | --- | --- |
| persistence | pathogenicity | latency | review |
| immune response | indicator organism | morbidity | prevalence |
| shedding | infectivity | epidemiology | vaccine |
| infection | occurrence | incubation | risk assessment |

Relevant abstracts were selected for review from the titles.  The selected abstracts were reviewed and selected articles were then obtained.  The articles were read and data relevant to performing the risk assessment were recorded and summarized.

Ranges for appropriate model parameters were selected for each microorganism using the available data found during the literature review and, when the data were unavailable or incomplete, professional judgement.  The selected ranges are presented in Tables 2.4 through 2.7

Data on the background incidence of disease due to the enteroviruses were not found during the literature search. However, a recent edition of the Federal Register1 gives estimated numbers of cases of disease from foodborne pathogens in the United States during 1992. Most of the cases are associated with foodborne transmission. This report estimates that approximately 4,000,000 cases of bacterial gastrointestinal disease occurred in the United States in 1992. Using 250,000,000 for the population of the United States, this equates to a annual incidence of 1,600 per 100,000. Since Shigella and Yersinia are not included in the Federal Register estimate, we assumed that the total annual incidence of bacterial gastrointestinal disease would be about twice as high, or 3,200 per 100,000. Britton2 states that fifty percent of gastrointestinal disease is due to viruses. We therefore assumed that the annual incidence of viral gastrointestinal disease would also be approximately 3,200 per 100,000. A range of 2,000 to 4,000 was selected around this estimate and used as the background incidence for disease associated with enteroviruses (Table 2.7).

Data on the background incidence of cryptosporidiasis were also not found during the literature search. It was assumed to be the same as the range used for giardiasis, the other disease included in this study caused by a protozoan parasite.

2.3	Analysis and Simulation Approach

This study included the four selected pathogens described earlier (Giardia lamblia, Cryptosporidium spp., Salmonella spp. and enteroviruses). Two Mamala Bay beaches were selected for inclusion in the risk assessment: Ala Moana and Waikiki. These were the beaches for which water quality modeling, exposure and pathogen modeling data were available from other Mamala Bay study groups. Figure 2.2 shows the approximate locations provided by HydroQual of the water quality modeling segments which were assumed to comprise the swimming surfing area for their adjacent beaches. Figure 2.2 also shows the approximate locations provided by PRC of lifeguard stations located at the two beaches. Data from these stations were used to estimate the numbers of swimmers and surfers using the beaches for recreation (see Appendix A).

2.3.1	Giardia and Ala Moana Beach

One pathogen and beach combination, Giardia and Ala Moana beach, was selected for in-depth exploration. This section describes the analysis and simulation approach used for this combination.

The approach was designed to address the uncertainty and variability in the data used to parameterize the model. In general, biological systems have large variability due to both genetic differences among individuals and environmental factors that are not explicitly modeled.  Standard analytical tools, such as curve-fitting techniques and sensitivity analysis, become less useful when data such as that produced from surveillance of infectious diseases are so variable.  Traditionally, a sensitivity analysis procedure involves selecting a point in the parameter space and perturbing the parameter values about this point.  Unfortunately, in many biological models there is sufficient uncertainty in parameter values to make the selection of any particular parameter set about which to conduct the sensitivity analysis a questionable procedure.  This is particularly true with infectious disease data, which are often hard to quantify. To address this problem, a technique termed Regional Sensitivity Analysis (RSA) was used for this project.

RSA involves describing, a priori, the uncertainty and variability in each model parameter by a probability distribution function.  Multiple simulations called Monte Carlo simulations are run and for each simulation a different set of parameter values is used.  The parameter values are chosen by randomly sampling each parameter from its distribution.  Assigning a bounded uniform distribution to each parameter allowed us to take into account data from various literature sources without bias toward one value or another.

A binary classification algorithm was then applied to each simulation output, in which the simulation output either passes or fails a set of criteria.  The multivariate parameter distribution associated with a pass classification can be analyzed through a variety of statistical procedures to assess parameter sensitivity.  The binary classification is basically a goodness-of-fit criterion based on whether or not the output is representative of the data.  The strength of this approach is that it acknowledges both the uncertainty and variability in parameter values in a structured fashion.  The RSA procedure has now been applied to a variety of problems.3-6

Due to the nature of this study, the approach used was slightly different from previous applications of RSA.  The simulation approach consisted of a six-scenario comparative study, in which the first scenario (the background scenario) used the same binary classification scheme as RSA.  The remaining scenarios then used the calibrated parameter sets obtained from the first scenario to generate a distribution of prevalence levels. The six scenarios will be described later in this section.

Specifically, a classification scheme was used to identify the ten parameter values that describe the background scenario of the model, in which exposure to recreational water is not the vehicle of disease transmission.  Each simulation was classified as acceptable if its output was consistent with available disease incidence data for non-outbreak conditions.  For other scenarios, which will be discussed later in this section, the appropriate remaining parameters that describe human exposure to recreational water were sampled, combined with the valid parameter sets from the background scenario, and used as a model representative of a community exposed to pathogens via recreational water use.  The outputs generated by running the model with this complete parameter set were statistically analyzed to identify parameters whose values strongly influence the magnitude of risk.

For Giardia transmission, surveillance data from non-outbreak conditions in Vermont7 were used to obtain baseline values for the ten of the parameters not associated with recreational water transmission.  The Vermont study found that between 1983 and 1986 the annual incidence rate was 45 cases per 100,000 per year.  Selecting a range around this value, the incidence rate criterion was set at 20 - 60 cases per 100,000 per year.  To calculate the incidence rate from the simulation runs, the following equation was used:

I =   Y365  365 / N

where I is the annual incidence rate,  is the fraction of individuals in state Y who move to state D per day, Y365 is the number of individuals in state Y at day 365, and N is the total population.  This equation assumes that the system is at steady state, a good approximation for these non-outbreak simulations.

Scenario 1:	Background

Using the above criterion, Scenario 1 simulations were performed until 1,000 sets of parameter values were produced consistent with non-outbreak conditions in Vermont. The number 1,000 was selected to produce a body of data sufficiently large for meaningful statistical analysis without making the simulation process unreasonably time-consuming. Since none of the parameters related to exposure to recreational water was required for this scenario, these simulations used only ten of the 20 sampling parameters (X0, T, P, Rand, a, , , , 0 and ). Four of the remaining 11 sampling parameters (WNS, SM, BM and F) were set to zero, which removes their effect on the output of the model and results in the rest of these 11 sampling parameters being mathematically canceled.

Once established, the parameter sets for which a Scenario 1 simulation resulted in an annual incidence of 20-60 were used as a basis to run Scenarios 2 through 6.  Therefore, in the remaining scenarios the ten parameter values were predetermined while the remaining 11 parameters values were obtained by randomly sampling the parameter distributions. Scenarios 2 through 6 are described below and the model parameterization is described in detail in Appendix A.

Scenario 2:	Shedding of Pathogen by Swimmers/Surfers, No Pathogen from Non-Shedding Sources

For this scenario, the only source of pathogen in the surfing/swimming area was direct shedding by swimmers and surfers. WNS, the concentration of pathogen in the swimming/surfing water from sources other than shedding by swimmers and surfers, was therefore set to zero.

Scenario 3:	No Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is only from sources other than shedding by swimmers and surfers. The parameter F, the rate of pathogen shedding per infectious swimmer, was therefore set to zero.

Scenario 4:	Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is from shedding by swimmers and surfers and non-shedding sources. All the model parameters were therefore sampled.

Scenario 5:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Increase in Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is from shedding by swimmers and surfers and non-shedding sources, with an order of magnitude increase in pathogen from non-shedding sources. All values of the input parameter WNS, the concentration of pathogen in the swimming/surfing water from sources other than shedding by swimmers and surfers, were therefore multiplied by a factor of ten.

Scenario 6:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Decrease in Pathogen from Non-Shedding Sources

For this scenario, pathogen in the swimming/surfing water is from shedding by swimmers and surfers and non-shedding sources, with an order of magnitude decrease in pathogen from non-shedding sources. All values of the input parameter WNS, the concentration of pathogen in the swimming/surfing water from sources other than shedding by swimmers and surfers, were therefore divided by ten.

Nine-month periods were simulated on a Sun Sparc Station using the MCSim simulation software package.8  The output variable used in the analysis was average daily prevalence, which was defined as the proportion of population that was symptomatic (in state D) calculated for each day of the simulation averaged over the nine-month simulation period. Average prevalence incorporates both the number of cases and the duration of the disease, resulting in a measure of disease intensity, whereas incidence accounts for the number of cases but not the duration of disease.  Average prevalence can be compared with incidence by the following approximation:

P  I  d

where I is the incidence and d is the duration of the disease.

2.3.2	Other Microorganism and Beach Combinations

The four selected pathogens and two beaches included in the risk assessment result in eight pathogen/beach combinations. In addition to Giardia and Ala Moana beach simulation approach described above, simulations were performed for the seven other microorganism and beach combinations. The simulation approach was the same as that described above, except that only three of the previously described six scenarios were performed. The three scenarios were Scenario 1 (the background scenario in which recreational use of Mamala Bay waters was not the exposure vehicle), Scenario 4 (pathogen from shedding by swimmers and surfers and from non-shedding sources) and Scenario 5 (shedding of pathogen by swimmers/surfers, order of magnitude increase in pathogen from non-shedding sources).

Microorganism-dependent parameters used in the simulations are given in Tables 2.4 through 2.7. For each of the two beaches, time-varying parameters related to human exposure to the recreational water and water quality associated with non-shedding sources are described in Appendix A.

Chapter 2.0 - References

1.	Federal Register, Vol. 59, No. 59 / Rules and Regulations, March 28, 1994.

2.	Introduction to Environmental Virology, Britton, G. (John Wiley and Sons, 1980).

3.	Auslander, D.A., "Spatial Effects on the Stability of a Food Limited Moth Population," Journal of the Franklin Institute, 314, 347-365 (1982).

4.	Bois, F.Y., M.T. Smith, and R.C. Spear, "Mechanisms of Benzene Carcinogenesis: Application of a Physiological Model of Benzene Pharmacokinetics and Metabolism," Toxicology Letters, 56 (1991).

5.	Spear, R.C., "Control of DO Level in a River Under Uncertainty." Water Resources Research, 19, 1266-1270 (1983).

6.	Tsai, K.C., and D.M. Auslander, "A Statistical Methodology for the Design of Robust Process Controllers," Transactions of the ASME, Journal of Dynamic Systems, Measurements, and Control, 110, (1988).

7.	Birkhead, G., and R.L. Vogt, "Epidemiologic Surveillance for Endemic Giardia lamblia Infection in Vermont," American Journal of Epidemiology, 129:4, 762-768 (1989).

8.	Maszle, D.R. and F.Y. Bois, "MCSIM: A Monte Carlo Simulation Program - User's Guide" (1993).

9.	Risk Assessment Guidance for Superfund, Volume 1, Human Health Evaluation Manual (Part A), EPA/540/1-89/002, (Office of Emergency and Remedial Response, US Environmental Protection Agency, Washington, DC, 1989).

10.	Superfund Exposure Assessment Manual, EPA/540/1-88/001 (Office of Remedial Response, US Environmental Protection Agency, Washington, DC, 1988).

11.	Jokipii, L., and A.M. Jokipii, "Giardiasis in Travelers: A Prospective Study," Journal of Infectious Disease, 130, 295-299, (1974).

12.	Nash T.E., D.A. Herrington, G.A. Losonsky, and M.M. Levine, "Experimental Human Infections With Giardia lamblia," Journal of Infectious Disease, 156:(6), 974-84 (1987).

13.	Dupont H.L., and P.S. Sullivan, "Giardiasis: The Clinical Spectrum, Diagnosis and Therapy," Pediatric Infectious Disease Journal,  5:1, S31-8 (1986).

14.	Craun, G.F., "Waterborne Outbreaks of Giardiasis" in Waterborne Transmission of Giardiasis, W. Jakubowski and J. C. Hoff, Eds., (U.S. Environmental Protection Agency, Environmental Research Center, Cincinnati, OH, 1978).

15.	Jokipii, A.M., and L. Jokipii, "Prepatency of Giardiasis," Lancet, 1(8021), 1095-1096 (1977).

16.	Wolfe, M.S., "Giardiasis," Clinical Microbiology Review, 5(1), 93-100 (1992).

17.	Greensmith C.T., R.S. Stanwick, B.E. Elliot, M.V. Fast, "Giardiasis Associated With the Use of a Water Slide," Pediatric Infectious Disease Journal, 7(2), 91-4 (1988).

18.	Flanagan, P.A., "Giardia--Diagnosis, Clinical Course and Epidemiology: A Review," Epidemiology of Infection, 109, 1-22 (1992).

19.	Lopez, C.E., A.C. Dykes, D.D. Juranek, S.P. Sinclair, J.M. Conn, R.W. Christie, E.C. Lippy, M.G. Schultz, and M.H. Mires, "Waterborne Giardiasis: A Community-Wide Outbreak of Disease and a High Rate of Asymptomatic Infection," American Journal of Epidemiology, 112(4), 495-507 (1980).

20.	Boreham, P.F., J.A. Upcroft, and P. Upcroft, "Changing Approaches to the Study of Giardia Epidemiology: 1681-2000," International Journal for Parasitology, 20(4), 479-87 (1990).

21.	Ruach, A.M. et al., "Longitudinal Study of Giardia lamblia Infection In a Day Care Center Population,"  Pediatric Infectious Disease Journal, 9(3), 186-189 (1990).

22.	Rose, J.B., C.N. Haas, and S. Regli, "Risk Assessment and Control of Waterborne Giardiasis," American Journal of Public Health, 81(6), 709-713 (1991).

23.	Basic Clinical Parasitology, H.W. Brown and F.A. Neva (Appleton-Century-Crofts, Norwalk, CT, 1983), 5th ed.

24.	Barbout, A.G., C.R. Nichols and T. Fukushima, "An Outbreak of Giardiasis in a Group of Campers," American Journal of Tropical Medical Hygiene, 25, 384-389 (1976).

25.	Adam, R.D., "The Biology of Giardia spp.," Microbiological Reviews, 55:4, 706-32 (1991).

26.	Moore, G.T., W.M. Cross, D. McGuire, C.S. Mollohan, N.N. Gleason, G.R. Healy, and L.H. Newton, "Epidemic Giardiasis at a Ski Resort," New England Journal of Medicine, 298, 402-407 (1969).

27.	Rendtorff, R.C., "The Experimental Transmission of Giardia lamblia among Volunteer Subjects," Waterborne Transmission of Giardiasis, W. Jakubowski and J.C. Hoff, Eds. (U.S. Environmental Protection Agency, Environmental Research Center, Cincinnati, OH, 1978).

28.	Taylor G.D., and W.M. Wenman, "Human Immune Response to Giardia lamblia Infection," Journal of Infectious Disease, 155:1, 137-40 (1987).

29.	Control of Communicable Diseases in Man, A.S. Benenson, Ed. (American Public Health Association, Washington, D.C., 1990), 15th ed.

30.	Jarroll, E.L., A.K. Bingham, and E.A. Meyer, "Effect of Chlorine on Giardia lamblia Cyst Viability," Applied Environmental Microbiology, 41, 483-487 (1981).

31.	Rendtorff, R.C., and C.J. Holt, "The Experimental Transmission of Human Intestinal Protozoan Parasites, IV. Attempts to Transmit Entamoeba Coli and Giardia lamblia Cysts by Water," American Journal of Hygiene, 60, 327-338 (1954).

32.	Preliminary Risk Assessment for Parasites in Municipal Sewage Sludge Applied to Land, Hadden, C.T. et al., (U.S. Environmental Protection Agency, Environmental Criteria and Assessment Office, Cincinnati, OH, 1991).

33.	Regli, S., J.B. Rose, C.N. Haas, and C.P. Gerba, "Modeling the Risk from Giardia and Viruses in Drinking Water," Journal of the American Water Works Association, 83(11), 76-84 (1991).

34.	Rendtorff, R.C., "The Experimental Transmission of Human Intestinal Protozoan Parasites, II. Giardia lamblia Cysts Given in Capsules," American Journal of Hygiene, 59, 209-220 (1954).

35.	Zu, S.X., and R.L. Guerrant, "Cryptosporidiosis," Journal of Tropical Pediatrics, 39:3, 132-6 (1993).

36.	Robertson, L.J., and H.V. Smith, "Cryptosporidium and Cryptosporidiasis, Part I: Current Perspective and Present Technologies," European Microbiology, 20-29 (1992).

37.	Flanigan, T.P., and R. Soave, "Cryptosporidiosis," Progress in Clinical Parasitology, 3, 1-20 (1993).

38.	Lacroix, C. et al., "Cryptosporidium Oocysts in Immunocompetent Children: Epidemiologic Investigations in the Day-Care Centers of Poitiers, France," European Journal of Epidemiology, 3:4, 381-5 (1987).

39.	Crawford, F.G. et al., "Asymptomatic Cryptosporidiosis in a New York City Day Care Center," Pediatric Infectious Disease Journal, 7:11, 806-7 (1988).

40.	Diers, J. and G.L. McCallister, "Occurrence of Cryptosporidium in Home Daycare Centers in West-Central Colorado," Journal of Parasitology, 75:4, 637-8 (1989).

41.	Stehr-Green, J.K. et al., "Shedding of Oocysts in Immunocompetent Individuals Infected With Cryptosporidium," American Journal of Tropical Medicine and Hygiene, 36:2, 338-42 (1987).

42.	Current, W.L., and L.S. Garcia, "Cryptosporidiosis," Clinics in Laboratory Medicine, 11:4, 873-97 (1991).

43.	Dupont, Herbert L. et al, "The Infectivity of Cryptosporidium parvum in Healthy Volunteers," New England Journal of Medicine, 332(13), 855-9 (1995).

44.	Sprinz, H., E.Z. Gangarosa, M. Williams, R.B. Hornick, and T.E. Woodward, "Histopathology of the Upper Small Intestines in Typhoid Fever," American Journal of Digestive Disease, 11, 615-624 (1966).

45.	Blaser, M.J., and L.S. Newman, "A Review of Human Salmonellosis: I. Infective Dose," Review of Infectious Disease, 4, 1096-1106 (1982).

46.	Naylor, G.R.E., "Incubation Period and Other Features of Foodborne and Waterborne Outbreaks of Typhoid Fever in Relation to Pathogenesis and Genetics of Resistance," Lancet, 1:8329, 864-866 (1983).

47.	Chalker, R.B. and M.J. Blaser, "A Review of Human Salmonellosis, III: Magnitude of Salmonella Infection in the United States," Review of Infectious Diseases, 10, 111-124 (1988).

48.	Kantele, A., J.M. Kantele, H. Arvilommi and P.H. Makela, "Active Immunity is Seen as a Reduction in the Cell Response to Oral Live Vaccine," Vaccine, 9, 428-431 (1991).

49.	Sanitation and Disease: Health Aspects of Excreta and Wastewater Management, Feacham, R.G. et al. (John Wiley and Sons, New York, N.Y., 1983).

50.	A Collaborative Report, "A Waterborne Epidemic of Salmonellosis in Riverside, California, 1965, Epidemiologic Aspects," American Journal of Epidemiology, 93, 33-48 (1971).

51.	Feldman, R.E., W.B. Baine, J.L. Nitzkin, M.S. Saslaw, and R.A. Pollard, "Epidemiology of Salmonella typhi Infection in a Migrant Labor Camp in Dade County, Florida," Journal of Infectious Disease, 130, 334-342 (1974).

52.	Gamble, D.R., "Viruses in Drinking Water: Reconsideration of Evidence for Postulated Health Hazard and Proposals for Virological Standards of Purity," Lancet, 8113, 425-428 (1979).

53.	Lo, S., J. Gilbert, and F. Hetriclc, "Stability of Human Enteroviruses in Estuarine and Marine Waters," Applied Environmental Microbiology, 32, 245-249 (1976).

54.	Katzenelson, E., "Survival of Viruses," in Indicators of Viruses in Water and Food, G. Berg, Ed. (Ann Arbor Publishers, Inc., Ann Arbor, MI, 1978).

55.	Fujioka, R.S., P.C. Loh and L.S. Lau, "Survival of Human Enteroviruses in the Hawaiian Ocean Environment: Evidence for Virus-Inactivating Microorganisms," Applied and Environmental Microbiology, 39:6, 1105-1110 (1980).

56.	Ward, R.L., D.I. Bernstein, E.C. Young, J.R. Sherwood, D.R. Knowlton and G.M. Schiff, "Human Rotavirus Studies in Volunteers: Determination of Infectious Dose and Serological Response to Infection," Journal of Infectious Disease, 154, 871-880 (1986).

3.0	RESULTS

A simulation study in which a risk assessment model (used as a comparative analysis tool) was designed to estimate the level of public health risk associated with recreational exposure to microbial agents in Mamala Bay waters. This chapter presents the results of the study.

3.1	Giardia and Ala Moana Beach

One microorganism and beach combination (Giardia and Ala Moana beach) was selected for in-depth exploration. For this combination six transmission scenarios were compared to analyze the relative risk of contracting waterborne disease. The first scenario described the background in which recreational use of Mamala Bay waters was not the exposure vehicle.  The results were used to establish a baseline prevalence with which to compare the effects of the next five scenarios. The six scenarios were described in Section 2.3.1, and are summarized below:

Scenario 1:	Background - Exposure to Recreational Water Is Not the Vehicle of Disease Transmission

Scenario 2:	Shedding of Pathogen by Swimmers/Surfers, No Pathogen from Non-Shedding Sources

Scenario 3:	No Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

Scenario 4:	Shedding of Pathogen by Swimmers/Surfers, Pathogen from Non-Shedding Sources

Scenario 5:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Increase in Pathogen from Non-Shedding Sources

Scenario 6:	Shedding of Pathogen by Swimmers/Surfers, Order of Magnitude Decrease in Pathogen from Non-Shedding Sources

1,000 nine-month simulations were performed for each scenario. Table 3.1 presents the results. The mean, variance, minimum and maximum values for average daily prevalence modeled for giardiasis at Ala Moana beach for Scenarios 2 through 5 did not vary significantly from Scenario 1, the modeled background prevalence. (Background prevalence is the expected prevalence in the population when exposure to recreational water is not the vehicle of disease transmission).

To determine which parameters played the most important role in determining the prevalence output, a multiple linear regression analysis was performed for Scenarios 1 and 4.  The dependent variable in these regressions was the natural log of the average daily prevalence.

The linear regression for Scenario 1 (background - exposure to recreational water not the vehicle of disease transmission) with all nine parameters included provided a good fit (R2 = 0.82).  The most important determinants of the level of disease prevalence in order of importance were as follows:

	the fraction of individuals in state D who move to state Z per day

0	the background transmission rate

the remaining parameters were much less important in determining the prevalence.  A linear regression with only  and 0 still produced a good fit (R2 = 0.77).

The linear regression for Scenario 4 (shedding of pathogen by swimmers/surfers, pathogen from non-shedding sources) with all thirty-two parameters included provided a good fit (R2 = 0.82).  The most important determinants of prevalence were found to be the same as those in Scenario 1 ( and 0).  A linear regression with only  and 0 still produced a good fit (R2 = 0.77).

The above results are consistent with the fact that the mean, variance, minimum and maximum values for average daily prevalence modeled for Scenario 4 did not vary significantly from Scenario 1, the modeled background prevalence.

Analysis of FD

The parameter FD, the water flow rate out of the swimming/surfing area, is based on modeling and therefore subject to uncertainty and variability.  This motivated a preliminary analysis of the importance of FD in removing pathogen shed by swimmers and surfers and affecting the model output.

To perform the analysis, the parameter sets for Scenario 4 were reused, except for the time-varying parameter FD, which was multiplied by a factor sampled between 0.01 and 100 for each simulation. Figure 3.1a shows the relationship between average daily prevalence and the factor by which FD is multiplied. Figure 3.1b shows the relationship between average daily pathogen concentration in the recreational water and the factor by which FD is multiplied. Examination of these figures reveals that despite a decrease in pathogen concentration with increasing flow rate out, average daily prevalences did not vary and remained at or below the background level of approximately 10 per 100,000.

It should be noted that this analysis was not a sensitivity analysis; rather, it was a simple preliminary analysis to generally illustrate the effect of increasing water flow rate out of the swimming area on the model output (i.e., disease prevalence).

3.2	Other Pathogen and Beach Combinations

The four selected pathogens and two beaches included in the risk assessment result in a total of eight pathogen/beach combinations. In addition to the Giardia and Ala Moana beach simulations described above, simulations were performed for the seven other microorganism and beach combinations. For these simulations three of the previously described six scenarios were performed to analyze the relative risk of contracting waterborne disease. The three scenarios were Scenario 1 (the background scenario in which recreational use of Mamala Bay waters was not the exposure vehicle), Scenario 4 (pathogen from shedding by swimmers and surfers and from non-shedding sources) and Scenario 5 (shedding of pathogen by swimmers/surfers, order of magnitude increase in pathogen from non-shedding sources). Table 3.2 and Figures 3.2 and 3.3 present the results for the eight pathogen and beach combinations.

Examination of Table 3.2 and Figures 3.2 and 3.3 reveals that the mean, variance, minimum and maximum values for average daily prevalences modeled for Scenarios 4 and 5 did not vary significantly from Scenario 1, the modeled background prevalences. Since maximum average daily prevalences never significantly exceeded background maximum prevalences, none of the simulations for Scenarios 4 and 5 would be considered an outbreak.

Figures 3.4 and 3.5 present comparisons of the following:

	The maximum pathogen concentration values of the beach monitoring data provided by Mamala Bay Study group MB-7 that were used to define the rate of shedding of pathogen by swimmers and surfers (Appendix B). Detection limits were used as maximum values for Salmonella at both beaches and for enteroviruses at Waikiki, since these organisms were not detected at these locations.

	The daily concentrations of pathogens at the beach due to shedding and non-shedding sources generated by the risk assessment model.

Examination of these figures reveals that the maximum beach concentrations generated by the risk assessment model always exceeded the maximum for the water quality monitoring data. The pathogen concentrations in the swimming area therefore did not appear to be underestimated by the risk model.

4.0	CONCLUSIONS

Based on the results of the public health risk assessment modeling we reached the following conclusions:

	Average daily prevalences modeled for disease associated with exposure to selected pathogens (Giardia, Cryptosporidium, Salmonella and enteroviruses) during recreational use of two Mamala Bay beaches did not vary significantly from modeled background prevalences in the population. (Background prevalence is the expected prevalence in the population when exposure to recreational water is not the vehicle of disease transmission).

	Increasing the concentration of pathogen from sources other than direct shedding by swimmers and surfers by an order of magnitude did not significantly increase the prevalence of disease in the population above background. The risk assessment modeling results therefore suggest that water quality management strategies designed to prevent additional pathogens from point and non-point sources from reaching the beach would not appear to affect the disease prevalences associated with the selected pathogens. This conclusion is based on available water quality pathogen monitoring and modeling results and the assumption that the uncertainty in the water quality modeling results used as input to the risk assessment model is less than approximately an order of magnitude.

	Based on a simple preliminary analysis for Giardia and Ala Moana beach, the rate of water flow out of the swimming/surfing area was not important in determining the prevalence of disease. Despite a decrease in pathogen concentration with increasing flow rate out, average daily prevalences did not vary and remained at or below the background prevalence level of approximately 10 per 100,000.

	Based on the results of the uncertainty analysis for Giardia and Ala Moana beach, the most important determinants of prevalence were the same for Scenario 1 (background - exposure to recreational water not the vehicle of disease transmission) and Scenario 4 (shedding of pathogen by swimmers/surfers, pathogen from non-shedding sources). These parameters were , the fraction of individuals in state D who move to state Z per day (based on the duration of disease symptoms) and 0, the background transmission rate. The fact that the uncertainty analysis produced the same results for Scenarios 1 and 4 is consistent with the fact that the mean, variance, minimum and maximum values for average daily prevalence modeled for Scenario 4 did not vary significantly from Scenario 1, the modeled background prevalence.