EFFECTS OF EFFLUENT FROM THE BARBERS POINT AND SAND ISLAND OUTFALLS ON THE MAMALA BAY ECOSYSTEM

William J. Kimmerer

Romberg Tiburon Center, San Francisco State University

Joseph M. O'Connor

Kinnetic Laboratories Inc.

1.0 Introduction

The purpose of this report is to present results of analyses of existing data on chemical and biological conditions in Mamala Bay. The objective of these analyses was to determine whether ecological effects of sewage discharge from the Sand Island and Barbers Point outfalls could be discerned in the data, and the severity of any effects.

1.1  Conceptual Model

The influence of sewage discharge on coastal ecosystems is well known. Sewage affects coastal ecosystems through several pathways: nutrient and organic enrichment, sediment loading, and toxicant input. The first two are discussed in this report (Figure 1)

In most cases the effect of sewage discharge on the water column is a stimulation of primary and secondary production and an enhancement of biomass at most or all trophic levels (e.g. Smith et al. 1981; Figure 1). At high levels this stimulation can result in the dominance by a few opportunistic pelagic species. At still higher levels, elevated oxygen consumption by heterotrophs and by autotrophs near the bottom or at night can reduce or eliminate dissolved oxygen from the water column. This makes both the water column and the benthos unsuitable as habitat for most organisms.

The best-described example of the enrichment of a coastal ecosystem by sewage is Kaneohe Bay, Oahu (Smith et al. 1981, Maragos 1985, Taguchi and Laws 1987). In the study by Smith et al. (1981) the discharge of sewage was terminated, providing an opportunity to examine its effects. Among the effects were elevations in all of the pools and fluxes of organic matter and nutrient measured, including dissolved and particulate organic and inorganic N and P, organic carbon, phytoplankton, zooplankton, primary production, and nutrient fluxes out of the sediments. The studies of Kaneohe Bay did not examine effects on benthic communities except for the metabolic activities reflected in the nutrient fluxes.

Coral reefs are susceptible to sewage pollution in several ways; Some pathways of impact are illustrated in Figure. Some coral species are sensitive to sediment loading and are excluded from areas of high sediment input from sewage (Pastorok and Bilyard 1985). High levels of organic matter and nutrients can stimulate algae that overgrow corals, or phytoplankton that shade them (Maragos 1985). In addition, elevated levels of phosphate can reduce calcification rates of corals (Kinsey and Davies 1976), which could ultimately result in erosion and degradation of the reef.

Sediment loading, from sewage or from particulate matter produced on nutrients in sewage, can have an influence on soft-bottom benthic communities (Figure 1). Sediments in the vicinity of an outfall can become enriched with organic matter including petroleum products and toxicants (Swartz et al. 1986. The organic matter can stimulate the growth of benthic populations, particularly of certain opportunistic species (Pearson and Rosenberg 1978). Species sensitive to toxins or to high organic loading can be suppressed. At high loading rates, deposition of organic-rich sediments can cause eutrophication, anoxia, and suppression of species near the outfall (Pearson and Rosenberg 1978, Swartz et al. 1986).

No evidence has come to light that eutrophication has occurred in Mamala Bay. The reports of several years of benthic monitoring and sediment analysis have concluded that no effect of sediment enrichment has been observed in the sediments or in the benthic communities (Nelson et al. 1993, 1994). However, these results have not yet been compared across years to determine whether some effects might be evident in the combined data.

This report examines the effects of nutrient and organic loading from the sewage outfalls on Mamala Bay. Data used include benthic monitoring and sediment data (Nelson et al., ), water quality data from the City and County of Honolulu, and data on effluent characteristics. This report analyzes the effects of sewage discharge in the following ways:

1.	Examine data on concentrations of nutrients, chlorophyll, and other water quality measurements for evidence of a potential effect on pelagic ecosystems or reefs.

2.	Examine data on sediment composition near the outfalls for evidence of a sewage "signal" in several commonly-measured indicators, including percent organic carbon in sediments, oil and grease, and oxidation-reduction potential

3.	Calculate expected sediment loading rate to the bottom based on sewage input, assumed settling rate, and a simple analytical model

4.	Examine benthic species composition and abundance of selected species for evidence of spatial relationships that might be attributed to sewage effects

All the data used herein were taken with the sewage outfalls in operation. There are no data from before outfall construction. This is unfortunate because spatial patterns in the data cannot be unequivocally attributed to effects of sewage. Control sites were chosen far enough from the outfalls to be unaffected by the discharges, but this also meant that they may have been in different environments with respect to depth, current patterns, and proximity to reefs and major nonpoint sources of materials. In other words, without pre-discharge data there is no way to evaluate how representative the control stations are.

1.3   Data Sources

Details of the data used and analyses performed are given in individual sections below.

Annual benthic monitoring near each of the two outfalls provided most of the data analyzed here (Nelson et al. 1993, 1994, and references). Samples at Barbers Point were taken in the zone of initial dilution (ZED), at four locations along the (presumed) ZED boundary, and 3600 m east and 3700 m west of the ZED station (Figure 2a). At Sand Island samples were taken in the ZID, at three ZID-boundary stations, and approximately 1600 m east and 2000 and 2700 m west of the ZID station (Figure 2b). At both locations the stations roughly followed the depth contours at which the diffuser is located, so they nominally occur at about the same depth. However, even within stations, depth variation has been large because of navigational uncertainties (Tetra Tech 1987).

Replicate samples were taken at each station using a Van Veen grab sampler. Subsamples were taken for analysis of grain size distribution and several chemical components of the surficial (top 2 cm) sediments, and for several categories of benthic fauna in 5 cm deep subcores. Replicate data were retained for the analysis of chemical variables, but the mean of the replicates was used for most of the analyses of benthic fauna in Section 5.

Tetra Tech (1987) evaluated the benthic monitoring program conducted at Barbers Point in 1986 and made many comments, some of which led to improvements in the program.

This report examines three aspects of the series of reports produced since then: sediment composition, benthic species composition, and abundance of selected benthic taxa. We assume that methods of sample collection and analysis were applied correctly, consistently, and without bias, so that differences among stations can be taken as real differences rather than methodological artifacts.

We also analyzed the water quality data from the City and County of Honolulu's monitoring program. This data set includes several years of data, and includes the benthic sampling stations and additional stations close to and far from the outfalls. The data appear to be of high quality; values from control stations far from the outfalls have similar values to those previously reported for open-ocean waters near Oahu (see below). However, different stations were visited each year, so the analysis could not be based on a factorial model as in the sediment data (Section 2). Only data available in electronic format were analyzed.

This report focusses on differences among stations attributable to the influence of sewage. However, we also compared results between sites (Barbers Point and Sand Island) and years to attempt to explain the patterns observed within each site. We also looked for relationships with grain size of sediments, which is often related to chemical content.

2.0 Water Quality

The ocean waters surrounding Oahu are highly oligotrophic, meaning that nutrient concentrations in these waters are very low and often undetectable by the best routine analytical techniques available. One consequence of this is that the endemic phytoplankton community may be extremely nutrient-limited but still capable of rapid growth (Laws et al. 1984) Adding nutrients to this water may result in rapid uptake by the phytoplankton, so that the nutrients are stripped from the water fairly rapidly. Therefore, nutrient addition to these waters may manifest itself as increased chlorophyll concentration rather than increased nutrient concentration per se, except in the immediate vicinity of the outfall.

2.1 Approach

We focused on ammonium as a tracer of inorganic nutrients in the sewage plume. Ammonium is typically high in primary or secondary sewage and low in oligotrophic waters; nitrate is more characteristic of runoff and upwelling sources. Phosphate can be more useful than nitrogen species for tracing inputs, but it was measured only at Barbers Point and not as often as ammonium. Therefore we estimated phosphate concentrations from ammonium based on their ratio in samples with elevated ammonium near the outfall (see below).

Data analysis included graphical and analytical techniques. The goal of this analysis was to determine whether stations close to the diffuser had higher nutrient concentrations than stations far from the diffuser. First we plotted the data vs. distance from the diffuser using box plots to examine the apparent trend in median and spread with distance.

The second approach was designed to detect extreme values. The upper quartiles (75th percentiles) of all the data for each year were determined. The frequency at which each station contributed values exceeding the upper quartiles for all stations was examined for differences among stations. For example, if the ZID station had 2 samples in the upper quartile in 1990, 1 in 1991, and none in 1992-93, then it contributed 3 samples to the upper quartile. The expected value for each station is 25% of the total values from that station.

The proportion of samples in the upper quartiles of all stations was plotted against distance from the outfalls to test for increase in water quality parameters by sewage discharge.

We also examined measures of particulate matter including total organic N and P, extinction coefficient, and chlorophyll. Most of the measures of particulate matter were rather invariant with distance from the outfalls.

Chlorophyll concentrations were examined using various graphical and regression techniques. In addition, the expected concentration of chlorophyll as a function of dilution was calculated from the nutrient input of the sewage outfalls, assuming that all inorganic N was converted to phytoplankton with a Redfield C:N molar ratio of 106:16 and a carbon:chlorophyll ratio of 50.

2.2 Results

2.2.1 Nutrient concentrations

Ammonium concentrations varied in time, distance from the outfalls, and sample depth. The spatial trend at each of the sites is illustrated by Figure 3. The box plots show that stations close to the outfalls had more frequent high values of ammonium than stations further than 1000 meters away, particularly for the deep samples. However, the median values do not look particularly different. In fact, if the values over 1 g-at/l are removed, the median values from the stations near the outfall were lower than those from the distant stations.

Because the difference in ammonium among distance categories was apparently confined to the extreme values, and the data are obviously heteroscedastic and asymmetrically distributed, parametric tests of differences among means were neither warranted nor suitable. Instead, we focused on the extreme values as represented by the upper quartiles for each year.

The proportion of samples in the upper quartiles decreased with distance from the diffusers (Figure 4). This relationship was apparently unrelated to direction from the outfall; slopes are not significantly different for eastern and western stations (analysis of covariance, p>0.1).

Ammonium values greater than 0.6 g-at/l were rare in the stations more than1000m from the outfalls. Any measured values greater that were taken as plume values.The proportion of values from near-ZID stations 30 meters or deeper that were over 0.6 g-at/l was 26% for Sand Island and 14% for Barbers Point. That is, the plume can be detected at about those frequencies at the stations near the outfall. The median elevation of ammonium in the plume where it occurred was 1.1 g-at/l at Sand Island and 0.8 g-at/l at Barbers Point.

The mean concentration of ammonium in the two sewage discharges was estimated from data in Stevenson et al. (1995), which gives flow and loading from each plant. These concentrations are 1000 g-at/l NH4 at Sand Island and 1300 g-at/l NH4 at Barbers Point. This translates to dilutions of about 900 at Sand Island and 1800 at Barbers Point.

Ammonium concentrations calculated for the effluent were 6000-7000 times higher than background (the median concentration at the control stations). A similar calculation for other nutrients for which data are available gives ratios of 60-80 for nitrate plus nitrite, 210-290 for total nitrogen, and 2100-2900 for total phosphorus. Thus, nutrient elevation due to sewage should be most readily detected in ammonium (even accounting for differences in detection limits among these nutrients).

At the dilutions necessary to eliminate the excess ammonium signal, ca 2000, the salinity signal should be undetectable (at ca. 0.015 psu) in field data gathered over a period of time. Indeed, there was no relationship between distance from the outfall and salinity. A slight relationship between ammonium and salinity was statistically significant but had no explanatory power (p<0.0001,r2=0.03, linear regression), and inspection of graphs did not suggest a sewage origin of this relationship. Initial dilutions calculated by the near-field modeling had a median value of 550 (P. Roberts, Georgia Tech, personal communication)

Phosphate data from Barbers Point also showed some elevated values (Figure 5), but the influence of the sewage discharge was not as clear. The median molar ratio of ammonia-N to phosphate-P was 2 in all samples, and 4 in samples with elevated ammonium and phosphate (i.e. over 0.6 g-at/l and 0.2 g-at/l). Total N and P (most of which is dissolved) show similar patterns at both outfalls (Figure 6), with somewhat more high values close to the outfall. Applying the upper-quartile method to TN and TP, however, revealed a significant (negative) slope only for TP at Sand Island (linear regression, p<0.05).

Based on the nutrient results, the sewage plume could bring excess concentrations of ammonium (>1 g-al/l) and phosphate (0.25 g-at/l) to the bottom within 500 meters of the outfalls about 25% of the time. These results can be used to determine whether a hazard exists to coral reefs in the vicinity due to elevated nutrient concentrations. The control sampling stations were along the same depth contours as the outfalls, and the plume probably does not extend across the contours to a great an extent. Nevertheless, if we assume that the sewage plume visits stations within 500 m in any direction at about 1 time in 4, we can estimate the range of impacts conservatively. A distance of 500m from either outfall is at about the 20 meter depth contour. This is deeper than most corals in the Hawaiian Islands, mainly because of poor light penetration to this depth. In addition, Mamala Bay is not known for extensive coral reefs, mainly because of strong currents and wave action.

There is not much information on the effect of elevated nutrient concentrations on coral reefs. Smith et al. (1981) presented an extensive data set on nutrient concentrations in Kaneohe Bay, Hawaii, along with some data on reef responses to the cessation of sewage input to the bay. In the northern and central reaches of the bay, where coral growth on some patch reefs was luxuriant, ammonium and phosphate concentrations averaged 0.6 and 0.2 g-at/l respectively before sewage discharge was terminated, and 0.4 and 0.1 g-at/l after, with frequent excursions to twice the mean values (Smith et al. 1981 Figures 20-21). This does not rule out an influence of nutrients on the reefs, but if it existed it must have been subtle. Maragos (1985) has described the extensive recovery in the south bay several years after sewage discharge was terminated; in 1985-86, ammonium concentrations averaged around 0.5 g-at/l and phosphate 0.2 g-at/l (Taguchi and Laws 1987).

Kinsey and Davies (1979) reported calcification of corals was reduced about 50% by 3-hour periods at a phosphate concentration of 2 g-at/l. This represents a 10-fold increase over ambient phosphate, which in turn is about equal to the upper quartile of phosphate values at stations near the outfall in Mamala Bay. Again, this does not rule out effects on calcification rates, but given the relatively low levels of phosphate within 500 m of the outfall, and the occurrence of corals mostly in shallower waters than this, significant effects of elevated phosphate on Mamala Bay reefs appear to be extremely unlikely.

2.2.2 Particulate matter

Particulate matter was represented by total suspended solids (SS), turbidity, chlorophyll, and at Sand Island, extinction coefficient. Turbidity showed no pattern with distance from the outfall, but was generally higher inshore (Figure 7). SS was nearly invariant and low (Figure 7). Extinction coefficient was generally low with high values only at inshore stations.

No trend in chlorophyll with distance from the outfalls was detected by either graphical or regression techniques (Figure 8). At Sand Island, but not at Barbers Point, stations at distances of 1000-3000 meters had a somewhat greater proportion of high values of chlorophyll than distant or near-ZID stations. If chlorophyll growth were stimulated by the nutrient input from the outfall, the time scale for development of a bloom should be on the order of several days. That is much larger than the time scale to mix over distances less than 1000 meters, so if chlorophyll were elevated at a distance from the outfall equal to a few days' travel time, it would also be elevated at the outfall through mixing. Therefore, the elevation in chlorophyll must be due to other causes.

Inspection of time trends of chlorophyll indicates substantial interannual variability not associated with sewage flow, the physical environment, or any obvious meteorological events. Values were higher in 1990-1992 than in earlier years. In addition, different groups of stations were sampled in each year. The interannual variation, together with shifts in the stations visited in each year, accounts for most of the variation in chlorophyll with station.

The "excess" ammonium concentrations referred to above, about 1 g-at/l, translate to a chlorophyll concentration of about 0.1 g/l, similar to the values observed. Thus, after initial dilution there is no opportunity for blooms of phytoplankton to develop from the nutrients supplied by the outfall.

3.0 Sediment Chemical Composition

Check timing and years of sampling Samples were taken at each benthic station at Sand Island during August 1987 and 1990-1993 for oxidation-reduction potential (ORP, mv, also taken in 1987), total oil and grease (OG, mg/kg), weight percent total organic carbon (TOC, 1991-93 only), and total Kjeldahl nitrogen (TKN, mg/kg). At Barbers Point samples were taken in January of 1990-1994 for ORP, OG, and TOC. Grain size distribution was determined at each station.

Analysis of the data was exploratory, using graphical representations of the data to reveal patterns, then applying suitable statistical tests to the data where appropriate. Parametric statistical tests were unsuitable for the analysis because:

•	Variances were heterogeneous and distributions sometimes skewed

•	For all of the variables, significant interactions existed between station and year; thus, significance tests for differences between station means across all years would be difficult to interpret using an analysis of variance model

•	Generally the extreme values were of as much interest as the means, because the sewage effect could be episodic and patchy. Therefore special statistical methods were required.

The goal of this analysis was to determine whether stations close to the ZID had higher levels of the various measurements (lower for ORP) than stations far from the ZID and, if possible, to determine possible causes of any differences observed. Two general approaches were used to detect differences. These approaches were designed to minimize assumptions, and to minimize effects of the sometimes large interannual variability in the data. The first approach was designed to examine differences among median values. For each variable, medians were calculated from each year's data set. These were ranked, then the ranks were summed across all years. Sums of ranks were then examined for differences among stations that might result from sewage input. In addition, Friedman's test was applied to the medians to test for differences in rank among stations.

The second approach was to compare extreme values among stations. For each variable, the upper quartiles (lower for ORP; see below) of all the data for each year were determined. These were tested for rank-order using Friedman's test. In addition, the frequency at which each station contributed to the upper quartiles of all years was used to examine for differences among stations. For example, if the ZID station had 2 samples in the upper quartile in 1990, 1 in 1991, and none in 1992-93, then it contributed 3 samples to the upper quartile. The expected value for each station is 25% of the total values from that station.

3.1 Results

Initial examination using correlation coefficients and graphical analysis showed no apparent relationships among the chemical variables used in the analysis. Therefore each was treated as a separate entity for examining the possible effect of the outfalls. Table 1 gives the median values and the number of samples reported for each station in each year.

3.1.1 Oxidation-Reduction Potential

If sewage-derived particulate organic matter is deposited in substantial concentrations, oxidation can reduce oxygen supply or cause anoxia in subsurface sediments near the outfall. If so, ORP would be lower at sites nearest the outfall. Therefore this analysis examined the data to detect low rather than high values from stations near the outfall.

Figure 9 shows box plots of ORP for each of the stations within each of the two areas, arranged by distance from the ZID station from west to east. Box plots are useful for examining the central tendency of the data as well as the extreme values. These results can be examined in two stages: first by comparing the ZID station with the 3 or 4 ZID-boundary stations, and second by comparing the stations in or near the ZID with the distant stations.

The only apparent pattern in the medians in Figure 9 is that the far-field stations (HB1 and HB7) at Barbers Point appeared to have generally higher medians, indicating more oxidized sediments, than the stations in and near the ZID. Conversely, the only station at Sand Island with a substantially different median is Station B4, where the median is elevated. However, Friedman's test did not indicate a significant difference among medians (p>0.1).

The values of the lower quartiles likewise did not differ among stations (p>0.07). However, the frequency of values in the lower quartiles for each year is a more powerful statistic. At Barbers Point, the distant stations had significantly fewer values in the lower quartile than stations closer to the outfall (p<0.01, chi-square test), but near the ZID no pattern was apparent. By contrast, the distant stations at Sand Island had significantly more of the lower-quartile values (p<0.01), while two stations close to the ZID (B4 and B5) had no values in the lower quartiles for any year.

Comparison of the results for the two areas revealed slightly lower medians and a wider spread, with a few negative values, at Sand Island (Figures 9, 10). Negative values occurred only in 3 samples in 1987, and were clearly different from other replicates from the same stations. These measurements were probably erroneous, but the Sand Island stations still had more low values than the Barbers Point stations (Figure 10). Possible explanations for the difference between areas are differences in temperature, sediment grain size, water motion, or terrigenous loading.

Seasonal differences could arise because samples were taken at Barbers Point in January-February, and at Sand Island in August. The temperatures during these periods were 24.5C and 26.5C (median of all data from each station from City and County of Honolulu samples; depth stratification in temperature was usually small). The difference of 2C does not seem sufficient to account for the difference in ORP at the two sites. Furthermore, since increasing temperature would increase rates of both oxidation and reduction reactions, it is not clear in which direction ORP would go as temperature increased. Check literature on this: must be something?? However, seasonal differences in runoff or other environmental variables may contribute to the difference.

ORP was not correlated with median grain size of sediments, either within each site or between sites (linear regression, p>0.1, and inspection of scatter plots). Median grain size did not differ among individual stations by more than 1 phi unit (factor of 2 in diameter), and there was no consistent difference in grain size between the two areas. Overall median grain size was 3.1, which is medium sand. The proportion of silt and clay in the samples had a median value of 4%, and only 3% of samples had a silt and clay proportion larger than 10%. Thus, the sediments can be characterized for all stations as medium sand with little fine sediment. Sediments of this size range are not normally associated with reducing conditions.

The Barbers Point outfall is on an area of open coast with the only major nearby source of terrigenous matter being Pearl Harbor. By comparison, the Sand Island outfall is close to several major streams with only limited areas for settling of sediments (e.g. Ala Wai Canal, Honolulu Harbor, Keehi Lagoon). Thus there could be a slightly greater terrigenous component of sediments at Sand Island that contribute to a somewhat more reducing sedimentary environment. However, this finding may be contradicted by the higher proportion of total organic carbon in the sediments at Barbers Point (see below).

3.1.2  Oil and Grease

Oil and grease measurements showed large differences between sites (Figure 11), but no apparent effect of the sewage outfalls (Tables 1-3; Friedmans test on medians and upper quartiles, p>0.1). Data from Barbers Point were fairly uniform, with all stations having medians ranking near the upper and lower ends of the range of medians in different years (Table 2), and an even frequency of values in the upper quartile (Table 3). Data from Sand Island were more variable in space and time: values from 1992 were much higher than any of the other values. Although stations differed in rank order of their median values (Table 2) and the frequency of values in the upper quartile (Table 3), these differences were not significant, nor did they suggest a sewage effect.

The reason for the high values from Sand Island in 1992 is unknown. Possible explanations of such a widespread elevation in oil and grease include an oil spill in the area, or a widespread bloom of phytoplankton that subsequently was deposited in the sediment. Data on chlorophyll in the vicinity of the Sand Island outfall (City and County of Honolulu monitoring data) do not show particularly high values during late 1991 or early 1992.

3.1.3 Total organic carbonAdd info re oil spill.  Check chlorophyll records from C&C

The percentage of total organic carbon (TOC) measured in the sediments was greater at Barbers Point than it was at Sand Island (Figure 12); medians overall were 0.26% at in samples from Barber’s Point and 0.12% for samples from Sand Island, respectively. This difference is opposite from those seen for ORP (Figure 10) and oil and grease (Figure 11). Differences among stations are variable (largely because of low sample size at Sand Island; Tables 1-3), but none was significant (Friedman's test, p>0.1). The medians and upper quartiles of the ZID and near-ZID stations do not suggest a sewage influence.

The discrepancy in results between TOC and ORP could be due to several factors, but without synoptic sampling at the two locations a seasonal or temperature effect cannot be ruled out. In either case, the values of TOC are very low, consistent with the location of these samples in an oligotrophic environment with little organic matter input to the sediments. There is no evidence in the chlorophyll data of significant blooms that could have affected sediment organic carbon or O&.

3.1.4  Total Kjeldahl Nitrogen

The concentration of total nitrogen (TKN) was measured only at Sand Island. Box plots (Figure 13) show a tendency for more extreme values in and near the ZID, as does the summary of upper quartile values (Table 3). This effect was confined to 1990. In other years there was no relationship between distance from the ZID and TKN, and the medians and upper quartiles did not differ among stations (p>0.1, Friedman's test).

During 1990, three of the highest four values were from station Z, and 8 of the highest 10 from the ZID and near-ZID stations. Median values were 251, 282, and 368 mg/kg at far-field, ZID boundary, and ZID stations respectively. The median value for the ZID station in 1990 differed from that of all other stations (p=0.014, Mann-Whitney U test), while that of the stations within 1000m of the ZID station differs from that of all other stations (p=0.001). Even in view of the multiple testing implicit in the exploratory data analysis conducted here, the latter value indicates a strong probability that the near-ZID stations were indeed higher in TKN than the far-field stations, possibly suggesting a sewage effect. However, this effect did not appear in the other measures of sediment chemistry.

3.2  Discussion

Sediment chemical composition in the two areas showed considerable spatial and interannual variability. However, almost none of that variability suggested sewage input. The single exception was for TKN in 1990, which was somewhat elevated at and near the ZID station. This general lack of clear gradients in sediment chemistry with distance from the outfalls could arise for several reasons

1.	Rates of input of sewage-derived material from the outfall are too low to produce a gradient in deposition rates with distance from the outfall

2.	A gradient in deposition rate of organic matter exists, but benthic oxidation of organic matter is so rapid that a gradient in organic matter concentration cannot be detected

3.	The spatial scale of sampling is too small: either the ZID station is outside the area of influence of the outfall, or the far-field stations are substantially affected by effluent

4.	Sampling or analytical techniques are not sensitive enough to detect a sewage effect.

These possibilities are discussed in the conclusions to Section 5. Briefly, some evidence shows an effect of sewage inputs on sediments near the outfall, but the flux of sewage-derived sediment to the bottom is small and the benthic oxidation of these sediments seems to be rapid.

High values of some of the variables occurred at all stations at a site in one year, but never in both areas in the same year. This suggests that much of the variability is either seasonal or irregular in time. Without further information on the spatial and intra-annual variability in these data, the cause of the variability cannot be determined.

4.0 Sediment Supply to the Benthos

The purpose of this section is to analyze the rate at which sediment is supplied to the benthos. In particular, we are concerned with the rate of deposition of organic matter and the effects that this material may have on the benthos.

4.1 The Model

We constructed a simple analytical model of particle flux to the bottom to examine the extent to which suspended sediments might affect the benthos in the vicinity of the Barber’s Point and Sand Island outfalls. The model was based on a single measurement of settling rate distributions made on effluent from Sand Island. The structure of the model is depicted in Figure 14.

Assumptions of the model are:

1.	The single experimental measurement is reasonably representative (see below for sensitivity analysis).

2.	Most of the particle settling rates are small, and the distribution of settling rates for the more rapidly settling particles decreases exponentially with increasing settling rates.

3.	The plume is subjected to a net (i.e. tidally-averaged) velocity U with direction that varies randomly on time scales of days.

4.	Sediments settle evenly over the area affected by the plume.

5.	Turbulence, stratification, flocculation, and shear effects on particle settling are ignored.

6.	Particles are not resuspended once they reach the bottom.

Assumption 2 was based on examination of the data, and results in a model that approximates the form of the data. Assumptions 3-5 represent a simplification of the model necessary to make it analytically tractable. Assumption 3 is equivalent to assuming that the plume is equally likely to flow in any direction, and the frequency of visitation of any point is averaged over time. This model gives a higher sediment flux at the edges of the plume than would be expected, and is therefore conservative. Assumption 6 is probably not correct (Zapka and Krock 1983) but is also conservative.

The data consist of a single set of 6 measurements of settling rate. Effluent was mixed with seawater (1 part in 40), then poured into cylinders 35 cm high and allowed to settle for various periods. The water was then siphoned off and the particulate matter was weighed. The red esults (Figure 15) show that after 4 hours only 17% of the weight of sediments had settled.

Under assumption 2 above, the settling rate can be modeled as a probability density:

where W is settling velocity, PdW is the proportion of particles with settling velocities between W and W + dW, and A and b are positive parameters.

(0)

We applied this model to the experimental data to obtain the equation:

where S is the proportion of mass that settled to the bottom of the cylinder by time t. This equation was solved for A and b using a nonlinear curve-fitting procedure to give values of A=17.5, b=106 (for velocities in cm/s). Equation 1 with these parameters gives a value of 6% of the particles settling faster than 0.01 cm/s, and 0.004 settling faster than 0.1 cm/s (Figure 15). Values of b of 89-154 produce curves that include all but 1 data point (Figure 15).

(0)

Under assumptions 3 and 4, the sediments either settle out within some distance from the outfall, or they remain suspended. The distance from the outfall is equal to the distance the mean flow takes the particles in the time it takes them to settle. In other words, particles must have a minimum settling velocity W0:

where H is the depth below the plume and r is the distance from the outfall. Under assumption 4 the sediment flux to the bottom within the distance r is given by:

(0)

Where  is the settling flux (g m-2), =LA/(b), and =bHU, where L is the mass loading rate (grams) from the outfall.

(0)

The settling flux  has a maximum value in the radius r that can be determined by differentiating equation 4 with respect to r and setting the result to 0. Maxima in  for various conditions are given in Figure 16, assuming a loading rate L of 1.6 106 grams/day of suspended sediments (the upper 95% confidence limit for monthly means at Sand Island). All values fall below a sediment flux of 10 g m-2, and most are less than 1% of that value.

4.2  Discussion - Sediment Supply to the Benthos

Sedimentation rates have been obtained using sediment traps in the vicinity of the outfalls on several occasions. M&E Pacific (1984) reported sedimentation rates of organic material of 2-3 g m-2 d-1 at sites within 25 m of the Sand Island outfall, compared with 1 g m-2 d-1 at a control site. However, the sediment traps were on the bottom and affected by sedimentation: carbonate material made up 94.3% of the trap material at the control site and 94.5% at the outfall sites (M&E Pacific 1984, Table IV-18). This material originates from resuspension or deposition of biogenic material from reefs and sand. The proportions of organic matter in the sediments collected at the control and outfall stations were very similar (2.8% at diffuser sites, 2.7% at the control site), suggesting that nearly all of the difference between control and diffuser sites was due to the deposition of biogenic material. Therefore most of the difference between the control and outfall sites was probably due to differences in the background sedimentation rather than the sewage discharge.

Smith and Dollar (1986) reported sedimentation rates of 25 and 12 mmole C m-2 d-1 near the Sand Island outfall, 17 and 16 at Barbers Point, and 12 and 8 at two control sites in 1984 and 1985, respectively. Particulate nitrogen sedimentation rates were 1.8 and 1.1 at Sand Island, 1.2 and 1.0 at Barbers Point, and 0.2 in both years at the control site. Sediment traps were placed 10, 70, and 140 m from the diffusers, and data were combined from the three distances. Differences between Sand Island values and controls for carbon, and between both sites and controls for nitrogen, were significant in each year (p<0.05, analysis of variance; Smith and Dollar 1986), comprising about 13 and 4 mmole C m-2 d-1 in each year. 	Smith and Dollar (1986) discussed the possibility of variation in background sedimentation rates between their sites and the controls. Based on the C:N ratios in the organic sediments they concluded that most of the excess organic sediment in the traps near the outfalls was of sewage origin. This contrasts with the conclusion drawn above on the basis of ratios of organic to inorganic matter.

A sedimentation rate of 10 mmole C m-2 d-1 due to sewage inputs is equivalent to about 0.3 g m-2 d-1 of organic sediment assuming that carbon comprises about 40% of the dry weight of the sediment. This falls within the envelope of values for sedimentation rate based on particle settling rates (Figure 16). However, the rate calculated from trap data is for stations very close to the outfalls, and sedimentation rate should decline with distance from the outfalls.

Organic sediment settling to the bottom will be resuspended, oxidized, or eventually buried. Resuspension is probably frequent given the strength of currents in the region (Zapka and Krock 1983), but is ignored here as intractable (which makes calculations conservative). The chemical composition and N isotope ratios of sediments in profiles through the upper 3-4 cm suggest that no component of the sediment is buried (Smith and Dollar 1986). Therefore, we assume that all of the sediment deposited in the vicinity of the outfalls is oxidized.

The portion of organic matter that is oxidized contributes to an upward flux of nutrients and dissolved inorganic carbon (DIC), and a downward flux of oxygen. Smith and Dollar(1986) determined the fluxes in nutrients, DIC, and oxygen into and out of the bottom near and far from the outfalls. Fluxes did not differ by direction or distance from the outfall within 140 meters at Sand Island, although there was an increasing trend to the northwest at Barbers Point. Significant differences were found in fluxes of all materials between controls and the Sand Island site in 1984, and in fluxes of some materials between controls and Sand Island in 1985 and Barbers Point in both years.

The calculated DIC fluxes near the outfalls exceeded the controls by 8-27 mmoles C m-2 d-1. This range of values is in rough agreement with the data from sediment traps, and also in accord with the sediment flux model presented above. In addition, comparisons of fluxes of different materials gave results consistent with a sewage source of the material near the outfalls, and predominantly aerobic metabolism of the sediments.

Analysis of pore-water concentrations of nutrients also suggested an aerobic environment and lack of long-term net burial of organic sediments (Smith and Dollar 1986). Furthermore, the sediments are poor in organic carbon (see Section 1), and there is no spatial trend in the percentage organic carbon in bulk sediments.

The model demonstrates the inverse relationship between the sediment flux and the distance over which that flux is felt. Because of the low settling velocities of most of the sediment particles, only a small percent of them actually reach the bottom. Smith and Dollar (1986) estimated that only about 1% of the organic material reaches the sediment near the outfalls, and that essentially all of that is remineralized in the top few cm of sediment. Based on the model calculations above, about 1% of the sediment reaches the bottom within about 400 meters, and 10% reach the bottom within about 2 km (except where sediment drifts into deeper water). Thus, the bulk of the sediment discharged by the sewage outfalls leaves Mamala Bay in suspended form.

It is arguable, though, to what extent the finer materials transported away from the outfalls ever reach the bottom in the form in which they are discharged. The settling rates of the more slowly settling particles are poorly known. However, the currents in Mamala Bay are sufficient to mobilize sand grains at times; there being apparently no location in Mamala Bay where fine sediments aggregate to produce well-packed sediments, it is reasonable to expect the finer sediments to be mobilized as well. Even measurements taken with sediment traps are misleading in this regard, since they include an unknown proportion of resuspended material. Thus, the estimates above for sediment impacts should be viewed as upper limits.

5.0 Faunal Abundance

Faunal abundance analyses were based on data from six years of sampling near each outfall. Data reported consist of abundance (number per sample) of a large variety of taxonomic groups, divided grossly into molluscs and other taxa. The latter consists of polychaete worms, crustaceans, and a variety of other groups. We used the data to test the following hypotheses:

1.	Total abundance of major categories of benthos (as a surrogate for biomass) differs among stations

1a	Total abundance is related to distance from the outfall

2.	Benthic diversity differs among stations

2a.	Diversity patterns are related to distance from the outfalls

3.	Abundance of common species differs among stations

3a.	Abundance of common species is related to distance from the outfall

In general, there were substantial differences among stations in abundance patterns of some taxa. Most of the patterns persisted in time, so years could be treated as replicates for analytical purposes. These analyses revealed a possible sewage effect for some attributes of some taxonomic groups.

Total abundance was examined for molluscs, polychaetes, oligochaetes, nematodes, total crustaceans, and copepods. Table 4 shows the dependence of each of these on variables such as depth, sediment grain size, distance along an east-west transect, and distance from the outfalls. Two of the groups differed significantly among stations at Barbers Point and four at Sand Island. Distance from the outfall was inversely related to mollusc abundance at both sites: molluscs were more abundant near the Sand Island outfall, and less abundant near the Barbers Point outfall. Both polychaetes and oligochaetes were more abundant near the Sand Island outfall than far from it, although polychaete abundance also depended on sediment grain size. Copepods (not shown) were vastly more abundant near the Sand Island outfall than far from it, but the same was not true at Barbers Point. Nematodes were more abundant at the stations at and west of the ZID station at Sand Island than at the two easternmost stations. Thus, the gross patterns of abundance of these groups show some response to distance from the outfall especially at Sand Island, but this response is ambiguous and confounded with differences in geographic position, depth, and sediment grain size.

We examined species diversity by using the number of species per replicate sample, and by comparing curves of relative abundance vs. rank order of abundance for the 20 most abundant species in each sample. These curves were compared by first subtracting the mean pattern, then using principal component analysis to characterize the remaining variability. In all cases the first principal component was related to dominance by the most abundant species, and the second to dominance of the second and third most abundant species or, conversely, higher relative abundance of the species ranking between 5 and 20.

There were significant spatial patterns in gross diversity levels, but these were not clearly related to distance from the outfalls. For example, among molluscs, Barbers Point station H1 (east control) was heavily dominated by the most abundant species, while the westernmost stations were dominated by the second most abundant species; stations closer to the outfall were somewhat more diverse. At Sand Island there was an east-west decrease in dominance of the most abundant species except that the easternmost station had the least dominance; there was also a break in the evenness of species between eastern and western stations with the ZID station having relatively high evenness. Total numbers of species differed among stations but were not related to distance from the outfalls.

Abundances of the 15 most common mollusc species at all stations within a site (Barbers Point or Sand Island) were compared using cluster analysis of the mean values for each station, and principal component analysis on the species abundances. Cluster analysis (Figure 2) shows that the two most remote stations at Barbers Point were similar to each other and dissimilar from the other stations. Principal component analysis yielded 4 components that explained 82% of the variance in the 15 species. The first of these was very abundant at the near-ZID stations and low at stations far from the outfall. This component was closely correlated with the abundance of Finella pupoides, which is considered an indicator of anaerobic conditions (Kay 1980). However, note that reducing conditions have not been found near the outfall (see above). The remaining components had significant spatial gradients but not related to the sewage outfall.

At Sand Island there were two groups of stations that did not cluster along geographic lines. The first four principal components explained 88% of the variance in species abundance. The first component declined from west to east and was significantly related to depth, grain size, and position along an east-west transect (p<0.001, r2=0.76, linear regression). This component also was related to Finella pupoides, which was very abundant only at station B1 to the west. This result appears to contradict that from Barbers Point. The second component was highest at station B2 and moderate to low elsewhere; these first two components are the key to the cluster analysis in Figure 2.

Species abundance patterns among the non-mollusc component also showed differences among stations, but no differences other than those mentioned above that could be attributed to sewage effects.

5.1 Conclusions Regarding Faunal Abundance

The apparent effects of sewage discharge on sediment and benthic fauna in the vicinity of the two outfalls are small and confined to a few taxonomic groups. There was no significant trend in sediments that could be explained as a sewage effect. In spite of the lack of sewage effect on sediments, there was an apparent effect on some of the taxonomic groups in the region. Molluscs, copepods, polychaetes, and oligochaetes were elevated near the Sand Island outfall, but molluscs were less abundant near the Barbers Point outfall than far from it. There were also several effects of depth, sediment grain size, and distance along an east-west transect that make it difficult to determine whether sewage was having an effect.

Swartz et al. (1986) documented major changes in benthic abundance and diversity near an outfall in southern California, accompanied by substantial changes in sediment chemistry. None of the trends they observed in sediment chemistry (in oxidation-reduction potential, oil and grease, or organic carbon) have been observed in Mamala Bay. Furthermore, the trends in benthic species abundance in Mamala Bay are far more subtle than those observed by Swartz et al. (1986).

The Mamala Bay outfalls apparently do not provide sufficient loading of organic matter to overwhelm the processes of dissipation and oxidation in Mamala Bay. The effects on benthic fauna are minor and localized, and probably confined to the soft-bottom communities within ca. 500 meters of the outfalls.

6.0 Conclusions

The potential impact of sediment loading on the benthos is a major concern for alteration of the function or structure of the benthic communities in the vicinity of the outfalls. The structure of these communities is discussed in the Section 5. As discussed above, the function of these communities has not been greatly altered. Although nutrient and oxygen fluxes differ between outfall and control stations, there is no evidence of buildup of organic sediments on the bottom, and no evidence that anaerobic metabolism in sediments is more common near than far from the outfalls. In other words, the soft-bottom benthic community appears able to absorb the excess organic matter without undergoing qualitative changes in metabolism, as are seen in eutrophication.

References

Joe O’ConnorJune 4, 19951:21:36 PMNeed these refs checked and the rest added

Bailey-Brock, J.H. and R. Fujioka. 1991.  Benthic faunal sampling adjacent to Sand Island ocean outfall, Oahu, Hawaii, August 1990. Water Resources Research Center, University of Hawaii.

Laws E. A., D. G. Redalje, L. W. Haas, P. K. Bienfang, R. W. Eppley, W. G. Harrison, D. M. Karl, and J. Marra. 1984. High phytoplankton growth and production rates in oligotrophic Hawaiian coastal waters. Limnol. Oceanogr. 29: 1161-1169.

Kay, E.A. 1980.  Micromollusks: techniques and patterns in benthic marine communities.  Pp. 93-112 in Environmental survey techniques for coastal water assessment, Conference proceedings. Water Resources Research Center, University of Hawaii at Manoa.

Maragos, J.E. C. Evans, and P. Holthus.  1985.  Reef corals in Kaneohe Bay six years before and after termination of sewage discharges.  Proceedings of the Fifth International Coral Reef Congress Volume 4 pages 189-194

M&E Pacific. Inc. 1983.  Sand Island outfall inaugural water quality monitoring report 1970 -1983. Report to Department of Public Works, City and County of Honolulu, Division of Wastewater Management.

Nelson, W.G. 1987.  Benthic faunal sampling adjacent to Barbers Point ocean outfall, Oahu,Hawaii. Water Resources Research Center, University of Hawaii

Nelson, W.G. 1991.  Benthic faunal sampling adjacent to Barbers Point ocean outfall, Oahu,Hawaii, February 1990. Water Resources Research Center, University of Hawaii

Nelson, W.G. J.H. Bailey-Brock, W.J. Cooke, E. A. Kay 1992.  Benthic faunal sampling adjacent to Barbers Point ocean outfall, Oahu,Hawaii, February 1992

Water Resources Research Center, University of Hawaii

Nelson, W.G. J.H. Bailey-Brock, W.J. Cooke, E. A. Kay 1992.  Benthic faunal sampling adjacent to Barbers Point ocean outfall, Oahu,Hawaii, July 1991.

Water Resources Research Center, University of Hawaii

Nelson, W.G. J.H. Bailey-Brock, W.J. Cooke, E. A. Kay 1992.  Benthic faunal sampling adjacent to Sand Island ocean outfall, Oahu,Hawaii, August 1991.

Water Resources Research Center, University of Hawaii.

Nelson, W.G. J.H. Bailey-Brock, W.J. Cooke, E. A. Kay 1992.  Benthic faunal sampling adjacent to Sand Island ocean outfall, Oahu,Hawaii, August 1992.

Water Resources Research Center, University of Hawaii.

Pastorok, R.A. and G.R. Bilyard.  Effects of sewage pollution on coral reef communities.  Marine Ecology Progress Series 21:175-189

Pearson, T.H. and R. Rosenberg.  1978.  Macrobenthic succession in relation to organic enrichment and pollution of the marine environment.  Oceanogr. Mar. Biol. Ann. Rev. 16:229-311

Smith, S.V. and S.J. Dollar. 1986.  Response of benthic ecosystems to deep ocean sewage outfalls in Hawaii.  A Nutrient Cycling Approach to Biological Impact Assessment and Monitoring.  Report to U.S. EPA, Newport, Oregon.  Grant # CR811180-01-1.

Smith S. V., W. J. Kimmerer, E. A. Laws, R. E. Brock, and T. W. Walsh. 1981. Kaneohe Bay sewage diversion experiment: perspectives on ecosystem responses to nutritional perturbation. Pac. Sci. 35: 279-396.

Stevenson, M., J. O’Connor, and J. Aldrich. Mamala Bay Study. MB-3. Pollutant Source Identification. This Volume.

Swartz R. C., F. A. Cole, D. W. Schults, and W. A. DeBen. 1986. Ecological changes in the Southern California Bight near a large sewage outfall: benthic conditions in 1980 and 1983. Mar. Ecol. Progr. Ser. 31: 1-13.

Taguchi S. and E. A. Laws. 1987. Patterns and causes of temporal variability in the physiological condition of the phytoplankton community in Kaneohe Bay, Hawaii. J. Plankt. Res. 9: 1143-1157.

Zapka, M.J. and H.-J. Krock.  1983.  Sand Island outfall diffuser sand effects study.  University of Hawaii J.K.K. Look Laboratory Technical Report No. 54.