2.0	Introduction

Basic to the Mamala Bay Program is an understanding of the transport and fate of material released from waste water diffusers in Mamala Bay.  The primary tool being used to help resolve the path of material from the diffusers (point sources) as well as material from non-point sources, such as Pearl Harbor and the Ala Wai Canal, is a hydrodynamically based water quality model.  To provide useful insights, such a model must resolve and duplicate the three-dimensional circulation patterns occurring in the study area.  To accomplish this, actual oceanographic variables at selected locations in Mamala Bay must be known and used by the regional model if it is to simulate circulation and transport accurately.  Additional, separate observations are used to verify that this model reproduces the observed patterns.

A numerical dispersion model is used to provide the regional model with an estimate of the concentration of discharged material at short distances (the near field) from the diffusers.  Detailed current and temperature profiles were made at the two diffusers in order to provide information needed to model accurately this initial mixing(See MB-4).  Providing key physical oceanographic variables and insights to support numerical models for near field dispersion and regional circulation  has been the primary thrust of MB-6.

2.1	Scope of Work

During MB-6: Ocean Currents Measurements, a comprehensive suite of oceanographic and meteorologic variables were measured concurrently and used to document changing circulation patterns, temperature and density fields, water level fluctuations and surface winds.  The types of observations and measurement sites were selected (1) to resolve oceanographic patterns expected to be active in the study area, and (2) to provide data that is tailored to the specific needs of the numerical modeling tasks; MB-4: Initial Dilution, and MB-5: Transport Fate and Effects.

Sites in the study area at which measurements of currents, temperature, and water level were made are shown in Figure 2.1.  These observations focused either on conditions at the two primary diffuser sites in Mamala Bay (Sand Island and Honouliuli), or conditions along the boundary of the general study area.  Time series of current and temperature profiles were made at most of these measurement sites, however, resolution in the vertical varied depending on the expected data use.  Additional water level measurements were made at or obtained from selected sites  around the island of Oahu.  A more detailed description of measurements is given in Section 3.

Figure 2.1	Map of the island of Oahu showing the general location of instruments/ moorings in Mamala Bay and around the island.

This experimental design is in keeping with the Task Objectives of providing a data base appropriate:

•	to describe and characterize forcing mechanisms and resulting circulation patterns in Mamala Bay which affect the dispersion and transport of both point and non-point sources of pollution, and

•	to support the concurrent numerical modeling activities, i.e. variables needed to initialize models as well as provide appropriate oceanographic boundary conditions.  This involves measuring selected variables at appropriate and compatible locations and sampling intervals.

2.2	Project Organization

Three key SAIC participants were Dr. Evans Waddell, Program Manager, Dr. Peter Hamiton - PI for Data Analsysis and Synthesis, and Mr. James Singer, PI for Field Operations.  Additional local field and operational support was provided by Edward K. Noda and Associates, a Honolulu-based coastal engineering and environmental consulting

firm.  Vessel support was obtained initially from American Workboats of Honolulu and later from the University of Hawaii.  Primary liaison between MB-6 and modeling tasks was done by Dr. Hamilton.