MAMALA BAY STUDY

POLLUTANT SOURCE IDENTIFICATION

PROJECT MB-3

Principal Investigators

Marty Stevenson and Joseph O'Connor

Kinnetic Laboratories, Inc.

55-1 Puapake Place

Lahaina, Hawaii  96761

John Aldrich

Camp Dresser & McKee

100 Pringle Ave., Suite 300

Walnut Creek, CA  94596

JULY 1995

# 1  EXECUTIVE SUMMARY

Pollutant loads to Mamala Bay from both point and non-point sources were assessed based upon existing data.  Pollutant loads from point source discharges were based primarily on data submitted to the Hawaii Department of Health by dischargers as part of federally mandated NPDES discharge reports.  Pollutant loads estimates for the two major point source dischargers, the Sand Island and Honouliuli Wastewater Treatment Plants (WWTPs), were augmented by measurements of nutrients and microbiological constituents conducted by other elements of the Mamala Bay study team.  Non-point source pollutant load estimates were developed using two models, CDM’s Watershed Management Model (WMM) and the HEC-STORM model.  The WMM was used to develop annual and seasonal estimates of pollutant loads while the HEC-STORM model was used to develop an hourly time series estimate of pollutant loads at seven locations over the time period of January 1994 through April 1995.  In both cases, load estimates for each constituent were developed based upon the best estimates of appropriate Event Mean Concentrations (EMCs) obtained from local and national data sets.  Upper and lower bounds of estimated pollutant loads were assessed based upon available information regarding the  variability of EMC data for each pollutant.  Estimates were developed for the Ala Wai Canal, Kewalo Basin, Keehi Lagoon/Honolulu Harbor, each major Loch in Pearl Harbor, and the Ewa Plain at the western boundary of the study area.

Average annual pollutant loads to Mamala Bay from point and non-point sources were compared to assess relative importance of each source.  On an annual basis, approximately 361,000 acre-feet of water are discharged to Mamala Bay.  Approximately one-third of this volume is attributable to point source dischargers.  Non-point discharges are the dominant sources of sediment, copper and zinc.  Point source discharges are the most significant sources of BOD, COD, indicator bacteria, nutrients, cadmium and lead.

Although the average annual load of indicator bacteria in non-point source discharges is small in comparison to point sources, non-point source discharges can be significant sources during more intense storm events.  On an annual basis, non-point sources contribute 0.1 to 5% of the total load of indicator bacteria to Mamala Bay.  Results of the HEC-STORM model indicated that short-term loading rates of indicator bacteria during larger storm events can be highly significant.  Discharge loading rates of enterococci from the Ala Wai Canal were estimated to be equivalent to discharges from all point sources during an 8-hour event in late February 1995.  Discharges of fecal coliform were estimated to be 6 percent of discharge rates from the Sand Island WWTP during the same event. This occurrence is particularly significant with respect to the fact that the loading points for non-point source discharges are typical within semi-enclosed water bodies that have been identified as Water-Quality Limited Segments (WQLSs) by the Department of Health.  These water bodies, in turn, discharge to the shallow, nearshore waters of Mamala Bay.

A total of 29 NPDES permits is currently in effect for industrial, agricultural or thermal discharges to Mamala Bay.  Twenty-five of the permits allow for discharges directly or indirectly to WQLSs.  The remaining four permits are for discharges that enter directly into nearshore waters.  In comparison to non-point source pollutant loads, these sources are considered to be only minor contributors to total pollutant loads.

The two major point source dischargers, Sand Island and Honouliuli WWTP, discharge through offshore outfalls.  Together, these two point source discharges clearly dominate all other point source dischargers to Mamala Bay in terms of flow, conventional pollutants, microbiological indicators, nutrients and metals.  Combined discharges from these two treatment plants account for 88% of the total flow and 96 to 100% of the sediment, BOD, COD, oil and grease, microbiological constituents, nitrogen and phosphorus discharged by all point source dischargers.  These two discharges account for 89 to 94% of the cadmium, copper, lead, and zinc entering Mamala Bay in point source discharges.  The Navy’s Ft. Kamehameha STP accounts for most of the remaining cadmium, copper, lead and zinc discharges from point sources.

# TABLE OF CONTENTS

1  EXECUTIVE SUMMARY

TABLE OF CONTENTS

LIST OF FIGURES

LIST OF TABLES

2  INTRODUCTION

2.1  Scope Of Work

2.2  Objectives

2.3  Project Organization

3  METHODS

3.1  Task Summary

3.2  Task Methodology

3.2.1  Mamala Bay Watersheds

3.2.2  Pollutants of Concern

3.2.3  Point Source Assessment

3.2.4  Non-Point Source Assessment

4  RESULTS AND DISCUSSION

4.1  Point Source Assessment

4.1.1  Identification of Point Source Discharges

4.1.2  Point Source Pollutant Loads

4.2  Nonpoint Source Assessment

4.2.1  Average Annual Pollutant Load Projections

4.2.2  Time Series Pollutant Load Estimates

4.2.3  Pollutant Delivery to Mamala Bay

5  CONCLUSIONS

6  RECOMMENDATIONS

7  REFERENCES

APPENDIX A

# LIST OF FIGURES

Figure 3.1  Mamala Bay Watersheds and Subbasins

Figure 3.2  Generalized Land Uses - Island of Oahu

Figure 3.3  Rainfall Isohyets (mm)- Island of Oahu

Figure 4.1  Regional Map of NPDES Discharges

Figure 4.2  Keehi Lagoon/Honolulu Harbor NPDES Discharges

Figure 4.3  Relative contribution of non-point source pollutants by watershed under current land use conditions

# LIST OF TABLES

Table 3.1  Land Use of Mamala Bay Watersheds

Table 3.2  Impervious Percentages by Land Use

Table 3.3  Average Annual Baseflows

Table 3.4  Mean Event Mean Concentrations for Mamala Bay Watersheds

Table 3.5  Base Flow Concentrations

Table 3.6  Concentrations of Bacterial Constituents

Table 4.1  Average annual load estimates for conventional and microbiological pollutants from point source discharges

Table 4.2  Average annual load estimates for nutrients and trace metals from point source discharges

Table 4.3  Annual Pollutant Loading for Mamala Bay Watersheds - Existing Development Conditions

Table 4.4  Average EMCs for Mamala Bay Watersheds Existing Land Use Conditions

Table 4.5  Mean Annual Bacteria Loads from Mamala Bay Watersheds

Table 4.6  Annual Pollutant Loading Rates for Mamala Bay Watersheds - Buildout Land Use Conditions

Table 4.7  Average EMCs for Mamala Bay Watersheds - Buildout Land Use Conditions

Table 4.8  Summary of storm events and HEC-STORM estimates of discharge in the Ala Wai watershed

Table 4.9  Pollutant Delivery Ratios into Mamala Bay

Table 5.1  Comparison of average annual pollutant loads from point and non-point sources

Table 5.2  Comparison of point and nonpoint source load estimates for conventional and microbiological pollutants

Table 5.3  Comparison of point and non-point source discharges load estimates for nutrient and trace metals

Table A.1  Location and nature of existing (as of 12/93) permitted industrial dischargers direct to Mamala Bay

Table A.2  Location and nature of existing (as of 2/13/93)  permitted industrial dischargers to Keehi Lagoon

Table A.3  Location and nature of existing (as of 2/13/93)  permitted industrial dischargers to Honolulu Harbor

Table A.4  Location and nature of existing (as of 2/13/93)  permitted industrial dischargers to Pearl Harbor

Table A.5  Location and nature of existing (as of 2/13/93)  permitted industrial dischargers to the Ala Wai Canal

# 2  INTRODUCTION

This report provides a summary and analysis of pollutant loads to Mamala Bay from both point and nonpoint sources.  Estimates of point source pollutant loading to Mamala Bay were based upon data that were available through the end of 1993 and limited to parameters measured by dischargers in compliance with requirements of NPDES self-monitoring requirements.  When available, NPDES data sets were augmented by special studies conducted to characterize specific constituents that were poorly characterized in the NPDES data sets or were not required to be monitored by the NPDES Permits.  Estimates of annual loads from nonpoint sources were developed based upon long-term average rainfall in the Mamala Bay watershed.  Hourly precipitation data were then used to develop estimates of nonpoint source pollutant loading over a one-year time period.  Data from this investigation provide spatial and temporal pollutant loading information necessary as input data for Project MB-4 Initial Dilution: Point Sources and Project MB-5 Modeling Transport and Fate of Pollutants.

## 2.1  Scope Of Work

The Scope of Work incorporated the following specific tasks in order to address the overall objective of the study:

•	Review existing data regarding point and nonpoint discharges to Mamala Bay.

•	Evaluate the reliability of existing data based upon quality control/quality assurance records.

•	Using available data, determine the location, quantity and quality of significant nonpoint source discharges to Mamala Bay.

•	Provide first order estimates of nonpoint source pollutant loading associated with stormwater runoff based upon land use information and water quality data from local studies, the National Urban Runoff Program (NURP), Soil Conservation Service and other relevant studies.

•	Compare total loading of toxic contaminants from point and nonpoint sources.

•	Determine the relative contributions of point and nonpoint source discharges as a function of seasonal changes in flow.

•	Develop time series on nonpoint flows and pollutant loads for a one year period.

•	Identify significant data gaps and recommend an approach for filling data gaps and refining loading estimates.

## 2.2  Objectives

The overall objective of the Pollutant Source Identification Program was to provide the best possible estimates of pollutant loading to Mamala Bay from all potential sources in order to provide input to modeling efforts and allow assessment of potential cumulative impacts.  All significant point and non-point sources of pollution were to be identified and, when possible, quantified.

## 2.3  Project Organization

Marty Stevenson (Kinnetic Laboratories, Inc. - KLI) served as Project Manager.  Joseph O'Connor (KLI) was the principal investigator for quantification of point source loads.  John Aldrich (Camp, Dresser & McKee - CDM) was the principal investigator for quantification of non-point source loads.  Christian Kocher and Peter Wilde (KLI) compiled point source load information and provided electronic files to the Mamala Bay Database.  Larry Roesner and Ken Payne (CDM) assisted in developing estimates of non-point source loads to Mamala Bay using WMM and HEC-STORM models.

# 3  METHODS

## 3.1  Task Summary

This assessment of sources of pollutants to Mamala Bay (Project MB-3) was divided into two separate tasks.  The first task addressed pollutant loads from point source dischargers.  These included loads from the three major municipal wastewater dischargers (Sand Island Wastewater Treatment Plant, Honouliuli  Wastewater Treatment Plant, and the Navy's Ft. Kamehameha Sewage Treatment Plant) as well as some 30 industrial/agricultural discharges permitted under the State-administered National Pollutant Discharge Elimination System (NPDES) program.  Point source loading information was obtained primarily from self-monitoring reports submitted to the Hawaii State Department of Health by the dischargers.  The second task addressed pollutant loads from non-point sources.  Non-point source loads were estimated using a simple, statistically based deterministic model (CDM's Watershed Management Model - WMM) to estimate annual pollutant loads.  A continuous hydrologic simulation model, HEC-STORM, was then used to develop time-series estimates of pollutant loading over a period of one year.  In both cases, estimates of load for each constituent were developed based upon the best estimates of appropriate Event Mean Concentrations (EMCs) obtained from regional or national data sets.

## 3.2  Task Methodology

### 3.2.1  Mamala Bay Watersheds

It was necessary to delineate watersheds and aggregation points for both the point and non-point assessments (Figure 3.1). The boundaries of the Mamala Bay watershed were defined by the Waianae Range,  Schofield Plateau and the Koolau Range. The western edge of the study area was defined by the ridge of the Waianae Range.  The divide of the Schofield Plateau marked the northwestern boundary of the watershed and the ridge of the Koolau Range marked the north-eastern boundary.  The western edge of the watershed was defined by Wa'ahia Ridge and Diamond Head.  All water from this region drains to Mamala Bay.

Figure 3.1  Mamala Bay Watersheds and Subbasins

Five major watersheds were defined based on topography and major outlet locations into Mamala Bay.  The largest of the five watersheds, Pearl Harbor, was further divided into six subbasins for the non-point source modeling effort (Figure 3.1).  This allowed non-point source pollutant loads to be aggregated within each of the three major Lochs that comprise the Pearl Harbor Estuary.  Of the five major watersheds, all but the Ewa Plain watershed drain into water quality limited segments (WQLS's) as defined in the Water Quality Management Plan for the City and County of Honolulu (Department of Public Works, City and County of Honolulu and the Hawaii State Department of Health 1990).  The WQLS's were identified by their consistent failure to meet state water quality standards established by Title 11, Chapter 54 of the Hawaii Administrative Rules.  Violations for total nitrogen, total phosphorous, turbidity, fecal coliform and other parameters are frequently reported in the WQLSs, with nutrients (nitrogen and phosphorous) the most widely reported parameter in violation.

### 3.2.2  Pollutants of Concern

Pollutants most commonly detected in point and non-point source discharges were the primary targets of the Mamala Bay Pollution Source Identification Project.  The following were considered to be the primary pollutants of concern.

Sediments:  Sediment discharges are the most common and visible non-point source pollutant in Hawaii and are often associated with other contaminants in point and non-point source discharges.  Sediment discharges are primarily associated with poor erosion and sediment controls along unstable stream banks, unstable slopes, and cleared land (e.g., agricultural cropland, urban construction sites).  Heavy metals and other toxicants frequently adsorb to sediment particles.  Sediments discharged into the Bay can destroy coral reef communities and can drastically alter aquatic habitat.

Nutrients:  Nutrients, such as phosphorous and nitrogen, are essential for plant growth.  High concentrations of nutrients, typically nitrogen in marine waters, can result in accelerated growth of algae and other aquatic vegetation.  Algal growth also interferes with coral reef habitat, and can also deplete oxygen when algae dieoff and decomposition occur.  Phosphorous and nitrogen compounds are typically high in effluent from wastewater treatment plants.  Nutrient loads in urban runoff can be as high as 10 to 20 times that of undeveloped areas, such as forested areas (Camp Dresser & McKee, 1994).  Agricultural runoff also typically contains significant nutrient loading from fertilizers.  This study projects nutrient loads for total phosphorus (TP), dissolved phosphorus (DP), total kjeldahl nitrogen (TKN), and nitrate + nitrite (NO3+NO2).

Oxygen Demand:  The decomposition of organic material depletes dissolved oxygen (DO) levels in slower moving receiving waters, such as the Ala Wai Canal, and may cause fish kills in the tributary streams.  Oxygen demand is often measured by the biochemical oxygen demand (BOD) and chemical oxygen demand (COD).  BOD expresses the amount of easily oxidized organic matter in water; whereas, COD measures all oxidizable matter in water.

Heavy Metals:  Heavy metals are common in urban runoff and  in sewage effluent.  High concentrations of dissolved heavy metals can be toxic to aquatic life.  For the Mamala Bay study we projected both point and non-point loading for lead (Pb), copper (Cu), zinc (Zn), and cadmium (Cd).

Pathogens.  Discharges from point sources (e.g. wastewater treatment plants) and non-point sources (e.g. septic tanks, sewer spills, animal feedlots, and urban runoff) can contain human pathogens that can result in health problems at beaches and in boating areas where body contact occurs.  Fecal coliform is considered to be an  indicator of pathogens.  However, recent monitoring data from streams in O'ahu (Fujioka, 1988) indicate that fecal coliform is not always a good indicator of human pathogens:

•	Fecal coliform bacteria indicate a potential human health risk only where treated or untreated sanitary sewerage discharges occur.

•	Fecal coliform bacteria are commonly found in natural water bodies, where they do not necessarily indicate a sewage source or a human health risk.

•	Although urban storm water discharges on O'ahu typically contain fecal coliform concentrations on the order of 104 CFU(colony forming units)/100 ml, other indicators of sewage are not present in urban runoff and thus it is unlikely that fecal coliform in runoff indicates human pathogen pollution.

•	Clostridium perfringens, a more reliable indicator of sewage, is not commonly measured in runoff monitoring studies and thus representative EMC values for this parameter are not available.

Thus, fecal coliform concentrations projected for non-point sources do not necessarily indicate the presence of human pathogens.

### 3.2.3  Point Source Assessment

Point source discharges are defined as discharges that "enter a body of water from a specific, identifiable point such as a pipe, ditch, tunnel, channel, or similar discrete conveyance" (DPW/DOH 1990).  By strict definition, stormwater discharges from urban environments are considered as point source discharges since they discharge through stormwater conveyances.  For purposes of this report, however, stormwater discharges from both urban and non-urban environments were treated as non-point source discharges.

The primary point source discharges to Mamala Bay consisted of 1) the three municipal sewage treatment plants  (Sand Island WWTP, Honouliuli WWTP, and the Navy's Ft. Kamehameha STP), 2) thermal discharges from steam electric generating plants, 3) thermal process water from sugar mill facilities.  Discharges from the three major sewage treatment plants are treated separately from the various industrial and agricultural discharges regulated as point source discharges under the Federally-mandated National Pollutant Discharge Elimination System (NPDES) program implemented under Hawaii's Department of Health, Clean Water Branch.  Data were obtained directly from monthly Discharge Monitoring Reports (DMRs) submitted by permitted discharger, electronic files created by DOH based upon the DMRs, and from annual summary reports compiled by individual dischargers.

The vast majority of data reported between 1989 and 1993 in NPDES self-monitoring reports consist of flow, total suspended solids (TSS), biochemical oxygen demand (BOD5) and oil and grease (O&G).  The three major sewage treatment facilities reported data on concentrations of bacteria in their effluent.  Monitoring requirements stated in NPDES permits for the most dischargers did not include nutrients or known toxic metals and organic compounds. In those instances where toxic metals and organic compounds were required by permits, detection limits for the required analyses were rather high, and the majority of data on toxic metals and organic compounds were below detection limits or below reporting limits.  In such cases, one-half of the detection limit or reporting limit was utilized for estimation of pollutant loads.

Data files assembled and submitted to the Mamala Bay database incorporated all available data for each individual discharger, however this report focuses on constituents of primary concern to the Mamala Bay study effort.  These constituents include indicator organisms (fecal coliform, enterococci, Clostridium perfringens), TSS and BOD5.  In addition, constituents typically considered to be important components in stormwater runoff were also quantified when data were available.  These included nutrients (NO2/NO3-N, TKN, Total N, and PO4-P), chemical oxygen demand (COD), copper, lead and nickel. Average loading data for some pollutants such as nutrients and COD could only be roughly approximated based on very few measurements and the likelihood that these pollutants were present in relatively constant concentrations in the wastewater streams.

The data used in establishing loading estimates for pollutants discharged to Mamala Bay were used essentially as they were reported in NPDES self-monitoring reports prepared by the dischargers and submitted to the regulatory authorities of the State of Hawaii.  Few-to-none of the data could be checked against quality assurance and quality control data (QA/QC).  While the full suite of data was available to us for use in this report, we were unable to obtain sufficient QA/QC data for validation or verification of the numbers that were contained in the reports.  For that reason the data reported herein must be assumed to represent estimates of pollutant loading only.

### 3.2.4  Non-Point Source Assessment

The non-point source assessment was performed using planning level models to evaluate the timing, location and magnitude of pollutant loading from non-point sources.   Two types of pollutant loads were defined:

•	Average annual loads were calculated using CDM's Watershed Management Model (WMM).  WMM uses local hydrologic data and pollutant loading factors representative of local land uses to compute pollutant loads.

•	Time series of loading from critical precipitation events were determined using the U.S. Army Hydrologic Engineering Center Storage Treatment Overflow Runoff Model (HEC-STORM).  HEC-STORM is a continuous hydrologic simulation model that calculates time series of storm flow and pollutant discharge events under alternative land use, storage, and treatment scenarios.

Both evaluations relied on existing land use information and local projections of buildout within the watershed.  Pollutant loading factors for this study are based on land use specific event mean concentrations (EMCs) generated from the National Urban Runoff Program (NURP, EPA 1983) and previous studies of storm water quality conducted within the Mamala Bay study area between 1972 to 1993.  Insufficient recent local storm water quality data are available to determine characteristic EMC's for urban land uses.  Available local data were compared to NURP EMC's and found to be comparable.  This study assumed that median NURP EMC values could be applied for the analysis.

The non-point source assessment involved a five step process.  First, the existing and projected land use of each major watershed was defined.  Then, hydrologic factors such as precipitation patterns, impervious surface area and base stream flow were evaluated.  Third, appropriate EMC data were selected for application to specific land use types based on both local and national data sets.  Next, annual pollutant loads were projected for each of the watersheds and subbasins using WMM.  Finally, a continuous one-year simulation of pollutant loading time histories was developed using the HEC-STORM model and EMC data.  The model was run for the period of January 1994 through July 1994 using actual rainfall records.  Rainfall records from August through December 1985 were used to complete the simulation since more recent data were not yet available and this time period was determined to be most representative of long-term average conditions during these months.  The following paragraphs briefly describe the methodology used in these five steps.

#### 3.2.4.1  Existing and Buildout Land Use

Several sources of land use data were used to define existing and buildout land use projections for the study area.  A generalized GIS land use map prepared by the Office of State Planning, State of Hawaii (Figure 3.2) for defining existing urban, agricultural, and forest conservation areas.  This map was last updated in March 1991.  The report Revised Total Maximum Load Estimates for Six Water Quality Limited Segments, Island of O'ahu, Hawaii (Freeman, 1993) was used to determine approximate urban land use breakdowns.  The following assumptions were made in order to project a potential buildout land use distribution within the study area.  First, existing agricultural and forest conservation lands are not expected to be developed anywhere within the study area.  This assumption is based upon continued availability of water imported from the windward side of the island and agricultural tax incentives.  Second, remaining undeveloped open land along the east side of the Bay were projected to develop according to the existing urban land use percentages.  Finally, the preliminary map/exhibit from the Kapolei Area Long Range Master Plan (Helber Hastert & Fee, 1993) was used to represent buildout conditions along the north and west sides of the Bay.  Table 3.1 summarizes existing and buildout land use of each major Mamala Bay watershed.

Figure 3.2  Generalized Land Uses - Island of Oahu

Table 3.1  Land Use of Mamala Bay Watersheds

#### 3.2.4.2  Hydrologic Factors

Rainfall/runoff from the Mamala Bay watersheds was estimated using precipitation data and a runoff coefficient based on soil characteristics and impervious surfaces.  Baseflow in each major watershed was based on USGS flow gage statistics for hydrologically similar areas.

Annual precipitation patterns on the island of O'ahu are directly related to the topography on both sides of Mamala Bay.  Along the east side of the Bay, significant median annual rainfall variations occur due to the orographic lifting of trade winds along the Koolau Range.  This results in an increase of rainfall that is directly proportional to the increasing topographic elevations of the Koolau's.   Along the west and north portions of the Mamala Bay watersheds, the Waianae Mountains cause similar precipitation patterns.  The median annual rainfall varies from over 100-inches near the Kolekole Pass to 30-inches in the Ewa Plain.  Figure 3.3 shows the median annual rainfall on the island of O'ahu.

Table 3.2  Impervious Percentages by Land Use

| Land Use | Percent Impervious |
| --- | --- |
| Residential • Low Density • Med. Density • High Density | 10 30 50 |
| Industrial • Light • Heavy | 70 80 |
| Commercial | 90 |
| Institutional | 30 |
| Highways/Roads | 90 |
| Agricultural | 1 |
| Forest Conservation | 1 |
| Water | 100 |
| Wetlands | 1 |

Figure 3.3  Rainfall Isohyets (mm)- Island of Oahu

Urban non-point pollution loadings tend to be governed by the amount of imperviousness associated with each land use category.  The impervious cover percentage for each land use was estimated from values accumulated from USEPA NPDES Storm Water permit programs conducted by Camp Dresser & McKee in over 30 areas of the United States. Table 3.2 shows the impervious cover percentage for each land use.  Annual baseflow (i.e. dry weather flow) was determined from the USGS Water Resources Data, Hawaii and other Pacific Areas, Volume 1 Hawaii for the water years 1986 through 1991.  Twenty-four USGS stream gages lie within the Mamala Bay Watershed.  Water quality monitoring is periodically conducted at eight of the gaging sites.  Available baseflow volumes were totaled in cfs per square mile and extrapolated to ungaged streams in watersheds with similar hydrologic characteristics.  Table 3.3 presents the average annual baseflow values defined for each Mamala Bay watershed in the manner described above.  In general, baseflow results from percolated runoff in forest conservation areas and from agricultural areas.

Table 3.3  Average Annual Baseflows

| Watershed | Average Baseflow |  |  |
| --- | --- | --- | --- |
|  | cfs/mi2 | in/yr |  |
| Ala Wai | 0.46 | 6.2 |  |
| Ke'ehi Lagoon/Honolulu Harbor | 0.29 | 4.0 |  |
| Kewalo Basin | 0.43 | 5.9 |  |
| Pearl Harbor: West Loch 1 | 0.43 | 5.9 |  |
| Pearl Harbor: West Loch 2 | 0.43 | 5.9 |  |
| Pearl Harbor: Middle Loch 1 | 0.13 | 1.8 |  |
| Pearl Harbor: Middle Loch 2 | 0.29 | 4.0 |  |
| Pearl Harbor: East Loch 1 | 0.34 | 4.6 |  |
| Pearl Harbor: East Loch 2 | 0.34 | 4.6 |  |
| Ewa Plain | 0.29 | 4.0 |  |

#### 3.2.4.3  Non-point Pollutant Loading Factors

This section describes the types of non-point source pollutants, their impact on Mamala Bay, and land use based pollutant concentrations representative of rainfall runoff and baseflow.  The representative pollutant concentrations are based on the recent data on discharge flows and runoff water quality data from various local studies, NURP data, Soils Conservation Service and other relevant studies.

The most widely accepted parameter for characterizing pollutant loading from rainfall runoff events is the event mean concentration (EMC), defined as the average of the pollutant mass loading during a storm event divided by the storm event's runoff volume.  To define EMC's representative of rainfall runoff from non-point sources in the Mamala Bay watersheds, over one hundred reports on water quality and water quality parameters for the Mamala Bay watersheds were reviewed for applicability to non-point source pollutant load estimates.  Only 28 of these reports contained data on non-point source discharges. Table 3.4 shows the EMC values characteristic of the predominant land uses in the Mamala Bay watershed.

#### 3.2.4.4 Urban Stormwater Runoff

Available local water quality data were compared with the National Urban Runoff Program (NURP) national median EMC's (EPA, 1983) to determine if NURP EMC's adequately characterized local urban runoff quality.  The following reports best characterize urban runoff conditions in the Mamala Bay Watersheds:

•	Quality of Storm-Water Runoff, Mililani, Oahu, Hawaii, 1980-84 (Yamane and Lum, 1985)

•	Storm Water Runoff and Urban Street Contamination Study:  Intensive Survey Report (DOH 1980)

•	Management Plan for Ala Wai Canal Watershed (Fox and Freeman, 1992)

These limited local urban runoff data generally lie within one standard deviation of median EMC's from the USEPA National Urban Runoff Program (EPA 1983).  Thus NURP EMC's are considered to be representative of urban non-point source pollutant discharges for purposes of determining the relative order of magnitude loading of pollutants to Mamala Bay.  NURP values are used for this study because the NURP database is large, thus its median EMCs and the coefficient of variation of the data are considered to be more representative than the limited local data.

Table 3.4  Mean Event Mean Concentrations for Mamala Bay Watersheds

|  | Oxygen Demand & |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sediment |  |  |  |  | Nutrients |  |  |  |  | Heavy |  |  |  | Metals |  |  |
| Land Use | BOD | COD | TSS | TDS | TP | DP | TKN | NO23 | Pb |  | Cu | Zn | Cd |  |
|  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L |  |
| Forest/Conservation | 8.0 | 51 | 650 | 94 | 0.23 | 0.06 | 1.36 | 0.73 | 0.00 |  | 0.01 | 0.02 | 0.00 |  |
| Open - Vacant | 8.0 | 51 | 325 | 100 | 0.23 | 0.06 | 1.36 | 0.73 | 0.00 |  | 0.00 | 0.00 | 0.00 |  |
| Cropland | 8.0 | 51 | 500 | 100 | 0.54 | 0.21 | 3.26 | 1.60 | 0.00 |  | 0.00 | 0.00 | 0.00 |  |
| Low Density Single Family | 10.8 | 83 | 250 | 100 | 0.47 | 0.16 | 2.35 | 0.96 | 0.02 |  | 0.05 | 0.18 | 0.002 |  |
| Med/High Density Single Family | 10.8 | 83 | 250 | 100 | 0.47 | 0.16 | 2.35 | 0.96 | 0.02 |  | 0.05 | 0.18 | 0.002 |  |
| Commercial | 10.8 | 83 | 150 | 100 | 0.47 | 0.16 | 2.35 | 0.96 | 0.02 |  | 0.05 | 0.18 | 0.002 |  |
| Industrial | 9.7 | 61 | 150 | 100 | 0.24 | 0.10 | 1.28 | 0.63 | 0.02 |  | 0.04 | 0.33 | 0.002 |  |
| Parks/Golf Courses | 8.0 | 51 | 330 | 100 | 0.43 | 0.10 | 1.36 | 0.68 | 0.00 |  | 0.00 | 0.00 | 0.00 |  |
|  |  |  | COEFFICIENT OF VARIATION |  |  |  |  |  |  |  |  |  |  |  |
|  | Oxygen Demand & |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sediment |  |  |  |  | Nutrients |  |  |  |  | Heavy |  |  |  | Metals |  |  |
| Land Use | BOD | COD | TSS | TDS | TP | SP | TKN | NO23 | Pb |  | Cu | Zn | Cd |  |
|  | CV | CV | CV | CV | CV | CV | CV | CV | CV |  | CV | CV | CV |  |
| Forest/Conservation | 0.5 | 0.5 | 0.5 | 0.5 | 0.7 | 0.7 | 0.5 | 0.5 | 0.0 |  | 0.0 | 0.0 | 0.0 |  |
| Open - Vacant | 0.5 | 0.5 | 0.5 | 0.5 | 0.7 | 0.7 | 0.5 | 0.5 | 0.0 |  | 0.0 | 0.0 | 0.0 |  |
| Cropland | 0.5 | 0.5 | 0.5 | 0.5 | 0.7 | 0.7 | 0.5 | 0.5 | 0.0 |  | 0.0 | 0.0 | 0.0 |  |
| Low Density SF (1-2 ac) | 0.4 | 0.6 | 1.0 | 1.0 | 0.7 | 0.5 | 0.7 | 0.8 | 0.8 |  | 1.0 | 0.8 | 0.8 |  |
| Med/High DSF (0.5-1 ac) | 0.4 | 0.6 | 1.0 | 1.0 | 0.7 | 0.5 | 0.7 | 0.8 | 0.8 |  | 1.0 | 0.8 | 0.8 |  |
| Commercial | 0.3 | 0.4 | 0.9 | 0.9 | 0.7 | 0.7 | 0.4 | 0.5 | 0.7 |  | 0.8 | 1.1 | 1.1 |  |
| Industrial | 0.3 | 0.4 | 0.9 | 0.9 | 0.7 | 0.7 | 0.4 | 0.5 | 0.7 |  | 0.8 | 1.1 | 1.1 |  |
| Parks/Golf Courses | 0.5 | 0.5 | 0.5 | 0.5 | 0.7 | 0.7 | 0.5 | 0.5 | 0.0 |  | 0.0 | 0.0 | 0.0 |  |
|  |  |  |  |  | HIGH EMCS |  |  | Pctle = | 90% |  |  |  |  |  |  |
|  |  |  |  |  |  |  | z = | 1.28 |  |  |  |  |  |  |
|  | Oxygen Demand & |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sediment |  |  |  |  | Nutrients |  |  |  |  | Heavy |  |  |  | Metals |  |  |
| Land Use | BOD | COD | TSS | TDS | TP | SP | TKN | NO23 | Pb |  | Cu | Zn | Cd |  |
|  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L |  |
| Forest/Conservation | 13.1 | 83 | 1065 | 154 | 0.43 | 0.11 | 2.24 | 1.20 | 0.00 |  | 0.01 | 0.02 | 0.000 |  |
| Open - Vacant | 13.1 | 83 | 533 | 164 | 0.43 | 0.11 | 2.24 | 1.20 | 0.00 |  | 0.00 | 0.00 | 0.000 |  |
| Cropland | 13.1 | 83 | 819 | 164 | 0.99 | 0.39 | 5.34 | 2.62 | 0.00 |  | 0.00 | 0.00 | 0.000 |  |
| Low Density Single Family | 16.6 | 141 | 508 | 203 | 0.85 | 0.25 | 4.39 | 1.86 | 0.04 |  | 0.10 | 0.34 | 0.004 |  |
| Med/High Density Single Family | 16.6 | 141 | 508 | 203 | 0.85 | 0.25 | 4.39 | 1.86 | 0.04 |  | 0.10 | 0.34 | 0.004 |  |
| Commercial | 15.2 | 126 | 294 | 196 | 0.84 | 0.29 | 3.66 | 1.55 | 0.04 |  | 0.09 | 0.37 | 0.004 |  |
| Industrial | 13.7 | 92 | 294 | 196 | 0.44 | 0.18 | 2.00 | 1.03 | 0.04 |  | 0.07 | 0.69 | 0.004 |  |
| Parks/Golf Courses | 13.1 | 84 | 541 | 164 | 0.79 | 0.18 | 2.23 | 1.11 | 0.00 |  | 0.00 | 0.00 | 0.000 |  |
|  |  |  |  |  | LOW EMCS |  |  | Pctle = | 10% |  |  |  |  |  |  |
|  |  |  |  |  |  |  | z = | -1.28 |  |  |  |  |  |  |
|  | Oxygen Demand & |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sediment |  |  |  |  | Nutrients |  |  |  |  | Heavy |  |  |  | Metals |  |  |
| Land Use | BOD | COD | TSS | TDS | TP | SP | TKN | NO23 | Pb |  | Cu | Zn | Cd |  |
|  | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |  | mg/L | mg/L | mg/L |  |
| Forest/Conservation | 3.9 | 25 | 317 | 46 | 0.09 | 0.02 | 0.67 | 0.36 | 0.00 |  | 0.01 | 0.02 | 0.0000 |  |
| Open - Vacant | 3.9 | 25 | 159 | 49 | 0.09 | 0.02 | 0.67 | 0.36 | 0.00 |  | 0.00 | 0.00 | 0.0000 |  |
| Cropland | 3.9 | 25 | 244 | 49 | 0.20 | 0.08 | 1.59 | 0.78 | 0.00 |  | 0.00 | 0.00 | 0.0000 |  |
| Low Density Single Family | 6.0 | 38 | 64 | 26 | 0.17 | 0.08 | 0.82 | 0.29 | 0.01 |  | 0.01 | 0.05 | 0.0006 |  |
| Med/High Density Single Family | 6.0 | 38 | 64 | 26 | 0.17 | 0.08 | 0.82 | 0.29 | 0.01 |  | 0.01 | 0.05 | 0.0006 |  |
| Commercial | 7.0 | 48 | 44 | 30 | 0.18 | 0.06 | 1.27 | 0.48 | 0.01 |  | 0.01 | 0.04 | 0.0004 |  |
| Industrial | 6.3 | 35 | 44 | 30 | 0.09 | 0.04 | 0.70 | 0.32 | 0.01 |  | 0.01 | 0.07 | 0.0004 |  |
| Parks/Golf Courses | 3.9 | 25 | 161 | 49 | 0.16 | 0.04 | 0.66 | 0.33 | 0.00 |  | 0.00 | 0.00 | 0.0000 |  |

3.2.4.5  Non-Urban Stormwater Runoff

NURP did not evaluate non-urban pollutant loading, thus EMC's for non-urban runoff are based on available local water quality studies:

•	Quality and Quantity of Non-point Pollution Sources in Rural Surface Water Runoff on O'ahu, Hawaii (Yim and Dugan 1975)

•	Revised Total Maximum Load Estimates for Six Water Quality Limited Segments -- Island of O'ahu, Hawaii (Freeman, 1993)

•	Preliminary Assessment of Possible Anthropogenic Nutrient Sources in the Lahaina District of Maui (Tetra Tech, 1993)

Tetra Tech (1993) was used to represent agricultural non-point source discharges of  nutrients.  Nutrient loads are in two components:  sediment-borne nutrients (2,000 mg-N/kg sediment and 1,000 mg-P/kg sediment) and dissolved nutrients (2.6 mg-N/L and 0.21 mg-P/L).  The resulting EMC values correlate well with the range of nutrient concentrations reported in the older Yim and Dugan (1975) study.  Sediment loading rates were based upon Yim and Dugan (1975) and Freeman (1993).  Maintenance dredging records from Pearl Harbor and the Ala Wai Canal were utilized to calibrate the loading rates. Metals are not commonly found in agricultural runoff, and therefore are assumed to be zero for this study.  Local values for BOD and COD from agricultural areas were also not found in the literature, thus these parameters were set at NURP values for open space land uses.  Pathogens are not commonly associated with agricultural crops.  Six years of water quality data (1986 through 1991) from a USGS gage on North Halawa Stream was used to set nutrients and metals EMCs for runoff from forest conservation areas.

3.2.4.6 Baseflow Concentrations

Ambient water quality monitoring data collected within the watershed were used to represent baseflow (dry weather) concentrations of nutrients and heavy metals.  The locations of the ambient water quality sampling stations within the watershed are limited.  Median baseflow concentrations were estimated using data from the eight USGS gage and monitoring stations between 1986 through 1991. By comparing the sampling dates to the daily baseflow volumes and eliminating the sampling data that are clearly part of rainfall events, this method provides a reasonable method to estimate baseflow concentrations.  As with the baseflow volumes, no monitoring stations were available in the Ewa Plain (EP) and West Loch (WL1 & WL2) areas.  The median values computed for the eastern and middle sub-basins were taken as regional values and applied throughout the study area.  Table 3.5 shows the baseflow concentrations applied in the model to estimate non-point pollutant loads.

Table 3.5  Base Flow Concentrations

|  | Annual Base Flow Concentrations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | General/Physical (mg/L) |  |  |  |  |  |  | Nutrients (mg/L) |  |  |  |  |  |  |  |  |  | Metals (ug/L) |  |  |  |  |  |  |  |  |  |  |
| Land Use | BOD | COD | TSS | TDS | TP | DP |  | TKN |  | NO2+NO3 |  | Pb |  | Cu |  | Zn |  | Cd |  |  |
| Urban | 5 | 0 | 0 | unk | 0.06 |  | 0.05 |  | 0.39 |  | 0.20 |  | 3 |  | 2 |  | 9 |  | 2 |  |
| Agriculture | 5 | 0 | 0 | unk | 0.19 |  | 0.17 |  | 0.89 |  | 1.41 |  | 3 |  | 1 |  | 5 |  | 1 |  |
| Forest | 5 | 0 | 0 | unk | 0.02 |  | 0.02 |  | 0.16 |  | 0.08 |  | 1 |  | 1 |  | 3 |  | 1 |  |

3.2.4.7  Bacterial Constituents

Three bacterial constituents were evaluated in this study:  fecal coliform, enterococci (conservatively based upon 100 percent of observed fecal streptococci concentrations) and Clostridium perfringens.  Two monitoring studies on O'ahu collected data on these constituents:  the Mililani Town study (Yamane and Lum 1985) and in-depth monitoring on South Ahuimanu stream and Waikele stream by Fujioka (1988).  In addition, bacterial data collected in the Ala Wai Canal was used to calibrate the non-point source pollutant loading models.  Table 3-6 lists the calibrated bacterial concentrations used in this study.  There is little evidence that significant amounts of sanitary sewage are discharged into the surface drainage system.  For instance, the East Mamala Bay Wastewater Facilities Plan Draft EIS reports that annual sewer spills and bypasses ranged between 0.7 and 7.4 acre-ft/year within an area roughly corresponding with the Ala Wai Canal, Kewalo Basin, and Keehi Lagoon/Honolulu Harbor watersheds.  Such volumes of sewer spills and bypasses are insignificant compared with the average annual runoff of nearly 80,000 acre-ft/yr from these watersheds.  The Draft EIS reported that about one-fourth of the urban areas of these three watersheds exhibited high wet weather infiltration/inflow, a condition that could result in unreported sewer bypasses during storm events.  No data exist to either confirm or deny if wet weather bypasses occur.  Finally, there are only 15 locations in these watersheds where septic tanks are used, an insufficient number to cause significant sanitary sewage discharges.  For these reasons, this study assumed that no additional sanitary sewage discharges entered non-point source runoff, and thus fecal coliform concentrations typical of observed non-point source runoff were used.

Table 3.6  Concentrations of Bacterial Constituents

|  | Baseflow Concentrations (CFU/100ml) |  |  |  |  | Runoff EMCs (CFU/100ml) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Land Use | Fecal Coliform | Enterococci | C. perfringens | Fecal Coliform | Enterococci | C. perfringens |
| Non-Urban | 1,000 | 100 | 0 | 1,000 | 100 | 0 |
| Urban | 1,000 | 100 | 0 | 26,000 | 33,000 | 450 |

3.2.4.8 Qualitative Information on Non-point Source Pollutant Sources

A wide array of largely unquantifiable occurrences of spills, leaks, and improper disposal can result in localized, short duration elevation of non-point source discharges.  While such discharges can occur anywhere, they are most likely to be discharged in larger quantities, with a higher risk of containing hazardous materials, in industrial and commercial areas.  The many harbors and shipyards along Mamala Bay are particularly susceptible to such spills.  For instance, the US Coast Guard has recorded 179 oil spills from 1991 through 1993 (average spill volume = 141 gallons).  Industrial activities (including shipyards, marinas, and military bases) are subject to stormwater NPDES permitting requirements.  These programs are currently focusing on "good housekeeping" practices to control incidental spills and leaks that may enter the drainage system or a receiving water.  As these programs collect monitoring data and prepare annual reports, additional data will be available identifying significant, localized sources of spills and leaks.

# 4  RESULTS AND DISCUSSION

## 4.1  Point Source Assessment

### 4.1.1  Identification of Point Source Discharges

#### 4.1.1.1  Municipal/Industrial Point Source Dischargers

Mamala Bay serves as the receiving water body for three separate municipal/industrial sewage treatment plants.  These wastewater treatment facilities include the Fort Kamehameha STP, the Sand Island  WWTP and the Honouliuli WWTP (Figures 4.1 and 4.2).

The Fort Kamehameha STP is operated by the U.S. Navy and discharges secondary effluent near the entrance to Pearl Harbor.  This plant receives industrial and domestic wastewater from the Pearl Harbor Naval Base Complex, Hickham Air Force Base, Hickham Village, Moanalua Terrace, Ford Island, Makalapa and Camp Smith.

The Sand Island WWTP discharges primary effluent to Mamala Bay through an outfall and diffuser system terminating at 21o17' 01"N latitude and 157o 54' 24" longitude and 70 meters depth.  This facility treats domestic and industrial wastewater from the City and County of Honolulu's metropolitan area.  The service area extends from Niu Valley-Paiko Peninsula on the east to the Moanalua-Aliamanu district on the west, and from the forest reserve boundary to the coastal waters.  The Sand Island WWTP has a design capacity of 82 MGD.

The Honouliuli WWTP provides primary treatment for domestic and industrial wastewater from the West Mamala Bay region.  The service area extends from Halawa on the east to the Waianae coastline on the west.  Effluent is discharged to Mamala Bay through the Barbers's Point Outfall located approximately 9000 feet offshore of Oneula Beach.  The terminus of the outfall is located at 21o 17' 00" N latitude and 158o 01' 50" W longitude and 60 meters depth.

Figure 4.1  Regional Map of NPDES Discharges

Figure 4.2  Keehi Lagoon/Honolulu Harbor NPDES Discharges

#### 4.1.1.2  Industrial/Agricultural Point Source Dischargers

As of December 1993, the Hawaii Department of Health reported a total of 30 industrial or agricultural point source discharges (Figures 4.1 and 4.2) permitted within the Mamala Bay watershed.  Of these 30 discharges, four discharge directly to the coastal waters of Mamala Bay.  The remaining 27 NPDES permits are for discharges that directly or indirectly enter water bodies that have been identified as Water Quality Limited Segments (WQLSs).  The location and waste stream characteristics of these discharges are summarized in Appendix A, Tables A.1 through A.5.

The majority of the industrial point source discharges are intermittent in nature.  These intermittent discharges are most often associated with storm events.  A number of the permitted discharges have never reported discharges.  Others have implemented Best Management Practices (BMPs) in recent years that have reduced or eliminated discharges.

Only six of these discharges are continuous flows.  The greatest volume of continuous discharges are thermal discharges from two Hawaiian Electric Company power plants (670 mgd) and thermal agricultural process water from the C & H Sugar Refinery (3 mgd).  The remaining continuous discharges consist of flows from exhibit tanks of the Waikiki Aquarium (0.7 mgd) and various thermal discharges from large air conditioning cooling systems.

Mamala Bay Coastal Waters.  The four permitted industrial/agricultural dischargers to coastal waters of Mamala Bay include AES Barbers Point Inc., Hawaiian Milling Corporation, DLNR's Demonstration Desalinization Plant, and the Waikiki Aquarium.   AES Barber's Point Inc. is a cogeneration facility that stores coal and limestone process materials on site.  Hawaiian Milling Corporation maintains feedlot operations at the Campbell Industrial Park.  Both AES Barber's Point Inc. and the Hawaiian Milling Corporation are permitted for stormwater discharges if rainfall in exceeds 8-inches in a 24-hour period.  With one possible exception, no discharges have been reported from these sites within the past three years.

The Demonstration Desalinization Plant started operation in October 1991.  The Plant pumps slightly brackish basal water from two wells and uses electrodialysis to remove salts.   Small quantities of brackish water (0.026 to 0.38 mgd) resulting from the process are discharged into a ditch that flows to Mamala Bay.  The discharge infiltrates through the porous limestone within a short distance from the discharge point.  No surface discharges have ever been documented as reaching Mamala Bay.

The Waikiki Aquarium continuously discharges overflow from aquatic animal exhibits.  The design capacity of the outfall is 0.73 mgd with current discharges.  The Aquarium currently (1993) reports continuous discharges of 0.45 to 0.56 mgd.

Keehi Lagoon/Honolulu Harbor.  Keehi Lagoon is the receiving water body for three permitted discharges.  These include Ameron HC&D, Hawaii Fueling Facilities, and Keehi Marine Inc.  All are minor or intermittent dischargers.

Ameron HC&D, Ltd. is authorized to discharge overflow from an enviromatic slurry system consisting of treated process wastewater, stormwater runoff and conveyer belt tunnel seepage.  Historically, discharges have averaged less than 0.003 mgd. No discharges have occurred since March 1991.

The Hawaii Fueling Facilities is permitted for stormwater runoff from an aviation turbine  fuel storage facility.  Discharges are typically estimated at less than 1 to 2  mgd for days when stormwater runoff is reported.

Keehi Marine Center is permitted to discharge wash water from a marine railway facility.  No discharges were reported from this site during the years 1990 through 1993.

Honolulu Harbor is the designated receiving water body for nine separate industrial discharges.  With the exception of Hawaiian Electric Company's Honolulu Generating station, all discharges are intermittent or the permitted activity has not reported any discharges.

Hawaiian Electric Company (HECO) is currently permitted to discharge a maximum of 157 mgd of cooling water, 0.06 mgd of low volume wastes and 0.065 mgd of metal cleaning wastes into Honolulu Harbor.  Monitoring requirements for HECO's thermal discharge are limited to flow and temperature, thus were not considered in developing flow and pollutant load estimates.

Six of the permitted discharges are related to petroleum product handling or storage facilities.  These include three Chevron USA Inc. facilities (Honolulu Terminal, Honolulu Distribution Center, and Kapalama Terminal), Pacific Resources Terminals, Shell Oil Company's Marketing Distribution Plant, and Mike's Vineyard Shell.

Discharges from most of the petroleum product handling or storage facilities have the been reduced or eliminated in recent years.  The permit for Mike's Vineyard Shell is for treated petroleum contaminated groundwater.  The treatment system was not installed as of December 1993, thus no discharges have been reported.  Discharges from the two Chevron facilities have been substantially reduced in recent years.  The Chevron USA Inc. Honolulu Distribution Center has reported only two discharges between 1991 and 1993.  The Chevron USA Inc. Kapalama Terminal has not discharged since February of 1992.   Similarly, the Shell Oil Company Marketing Distribution Plant did not discharge during the three-year period examined for this study.  The last reported discharge was in December 1990.

Hawaiian Cement is permitted for stormwater runoff from a cement receiving, storage and distribution facility.   No discharges have been reported since October 1991.

Honolulu Shipyard, Inc. discharges at three locations.  These include the AFDL-40 Floating Drydock, a marine railway system, and the Kekaulana Floating Drydock.  Pollutant loads cannot be calculated from NPDES monitoring data since measurements include ambient harbor water as well as potential contributions from the shipyard operation.  During the period of interest, no discharges occurred from the AFDL-40 Floating Drydock.

Pearl Harbor.  A total of 11 NPDES permits exist for industrial/agricultural discharges to Pearl Harbor.  Four of these dischargers (Hawaiian Cement Halawa Batch Plant, Oahu Sugar Company, Pearl Harbor Naval Shipyard Dockside Chlorinators, and Pearl Harbor Naval Submarine Base Dockside Chlorinators) reported no discharges from 1991 through 1993.  Three other dischargers (Oahu Sugar Company Field Facilities, Naval Public Works Center Building No. 826 and the Pearl City Fuel Annex) reported discharges in four or fewer months during the three year period from 1991 through 1993.

NPDES permits have been issued for two shipyard operations within Pearl Harbor.  These include four dry-docks at the Pearl Harbor Naval Shipyard and a floating dry-dock (AFDM-6) at the Naval Submarine Base.  Discharges from air compressors and emergency generators are included as part of the NPDES permit for the Submarine Base floating dry-dock.  Monitoring requirements for these two shipyard operations do not allow for calculation of pollutant loads since measured concentrations incorporate pollutants present in ambient waters of Pearl Harbor.  Permits for shipyard operations are principally based upon enforcement of good housekeeping practices in order to assure that discharges of pollutants are minimized.

California & Hawaiian Sugar operates a sugar refinery in Aiea that discharged thermal condenser cooling water and boiler blowdown through Aiea Stream into East Loch at an average rate of 2.95 mgd.  This discharge ceased in mid 1994 when the crystallization phase of the process was eliminated at the refinery.

Hawaiian Electric Company operates the Waiau Generating Station which discharges cooling water, low volume waste and metal cleaning waste to the East Loch of Pearl Harbor.  This power plant is permitted to discharge 512 mgd of cooling water, 0.70 mgd of low volume waste and 0.4 mgd of metal cleaning waste.  Monitoring of the thermal discharges is limited to flow, temperature and total residual oxidants.  Total flow and pollutant loading from the thermal discharge of HECO's Waiau Generating Station were therefore excluded from the point source assessment.

Ala Wai Canal.  The DOH has issued only two NPDES permits for industrial discharges to the Ala Wai Canal.  Super Hawaii Ala Wai Marine is permitted to discharge wash water from a marine railway facility.  No discharges have been reported under this permit.  A condominium unit, the Yacht Harbor Towers, is permitted to discharge air conditioning cooling water to the Canal.  Discharge from this site is reported to be 2.6 mgd.

### 4.1.2  Point Source Pollutant Loads

Point source discharges were evaluated in terms of discharge volume, TSS, BOD, COD, oil and grease, microbiological constituents (Clostridium perfringens, Escherichia coli, enterococci, and fecal coliform), nutrients (NH4-N, NO2/NO3-N, TKN, total nitrogen and total phosphorous), and trace metals (cadmium, copper, lead, and zinc).  Estimates of load were typically based upon data from 1991 through 1993.  In cases where data were not available for this time period, it was necessary to rely on limited measurements or data from studies conducted as early as 1988.

#### 4.1.2.1  Wastewater Flow

Point source discharges (exclusive of thermal discharges from the HECO generation stations in Pearl Harbor and Honolulu Harbor) average approximately 122,460 acre-feet per year (110 million gallons per day).  The vast majority of wastewater discharged into Mamala Bay from point source discharges comes from the three major sewage treatment plants: Sand Island, Honouliuli, and Fort Kamehameha.  Together these three facilities contribute about 115,300 acre-feet per year (103 million gallons per day) directly to Mamala Bay, or more than 99% of the total direct wastewater flow to Mamala Bay.  Only three other wastewater sources are permitted to discharge directly to Mamala Bay: the Waikiki Aquarium, the Hawaii Milling Corp. and AES Barber's Point (Figure 4.1 and 4.2). Of these only the Waikiki Aquarium reported discharges during the three year period utilized for this assessment. All other point-source wastewater discharges enter Mamala Bay indirectly being first discharged to the waters of the Ala Wai Canal, Keehi Lagoon, Honolulu Harbor or to the waters of Pearl Harbor.

#### 4.1.2.2  Sediment

An average of  8,360 tons of sediment per year was discharged from point source discharges between 1991 and 1993 (Table 4.1).  The three treatment plants accounted for 99.8 percent of this annual discharge.  The minor point source discharges accounted for an average of 17.1 tons per year.

#### 4.1.2.3  BOD, COD, and Oil and Grease

An average of 15,590 tons of BOD, 42,700 tons of COD and 4,127 tons of oil and grease are discharged each year.  In all cases, the three treatment plants comprise the major source of contaminants.  Estimates of COD loading from Sand Island and Honouliuli are based upon ratios of BOD:COD developed by the Mamala Bay Special Studies Task.  For both treatment plants, measurements were limited to a period of several days and are considered to be a rough approximation.  Estimates of COD loads are provided only as an indication of the general magnitude relative to non-point source loading estimates.  Comparisons of COD loads discharged by the two wastewater discharges would not be valid or appropriate (Table 4.1).

#### 4.1.2.4  Microbiological Pollutants

Microbiological pollutants were measured as part of either standard NPDES monitoring efforts or special studies (Fujioka, 1991).  A special study concentrations of Escherichia coli, enterococci and Clostridium perfringens in effluent was conducted by Fujioka at both the Sand Island WWTP and the Honouliuli WWTP between September 1988 and December 1990.  These were the only available measurements of both Escherichia coli and Clostridium perfringens in effluent.  Estimates of loading rates for enterococci were the geometric mean of 6 months of measurements starting in June of 1993 and data from earlier Fujioka study.  Fecal coliform loads were based exclusively on NPDES monitoring data from 1991 through 1993.  Load estimates are presented in units of 1015 CFU (or "organisms") per year (Table 4.1).

The Sand Island and Honouliuli WWTPs were the only significant point source dischargers of microbiological pollutants (Table 4.1).  Due to secondary treatment and disinfection, discharges of enterococci from the Fort Kamehameha STP were seven orders of magnitude less than reported at the other two treatment plants. Insignificant quantities of fecal coliform are discharged directly to the coastal waters of Mamala Bay in effluent from the Waikiki Aquarium.

#### 4.1.2.5  Nutrients

The Sand Island WWTP, Honouliuli WWTP and Fort Kamehameha STP are the major documented dischargers of nutrients to Mamala Bay (Table 4.2).  Nutrient loading estimates for the Honouliuli WWTP are based upon a very limited number of measurements reported by the University of Hawaii (Ed Laws, pers. comm.) and the Sand Island analytical laboratory in 1992.  More recent and complete data were available for effluent from the Sand Island WWTP.  Nutrients in Sand Island effluent were analyzed over a one-year period (December 1993 to December 1994) as part of the MB-7 investigations (Fujioka pers. comm.). Relatively low loads of nitrite/nitrate-nitrogen reported for Pearl Harbor were primarily attributable to the C&H sugar refinery which has since eliminated all discharges.

#### 4.1.2.6  Metals

The three treatment plants also discharge the largest quantities of cadmium, copper, lead and zinc to Mamala Bay (Table 4.2).  Small quantities of copper and lead are reported in association with minor discharges to Keehi Lagoon/Honolulu Harbor and Pearl Harbor.

## 4.2  Nonpoint Source Assessment

### 4.2.1  Average Annual Pollutant Load Projections

Average annual pollutant loads from the Mamala Bay watersheds were projected with the WMM model.  WMM generates reasonable order of magnitude estimates of pollutants conveyed by rainfall runoff and in baseflow using land use based parameters defined in Section 3.2.2.  Because all but the bacterial nonpoint pollution loading factors used in the Watershed Management Model were derived from national statistics, the model includes the capability to perform an uncertainty analysis with a range of literature values for each land use category.  The estimated mean EMC's listed in Table 3.4 are assumed to be representative of a "medium" or "most probable" estimate of the nonpoint pollution loading factor for each land use.  The purpose of the uncertainty analysis is to develop statistical estimates of the extremes, high and low values of pollutant loadings and to assess whether estimates from the extremes would result in different management decisions.

Based upon the NURP study results (USEPA, 1983), a coefficient of variation (CV) is assigned to each pollutant and to each land use category to provide an indication of the relative degree of uncertainty associated with the EMC estimates (Table 3.4).  "High" and "low" pollutant load estimates were computed for the mean EMC and CV for each pollutant and land use category based on a 90 percent probability of exceedance.  In other words, the high and low pollutant load estimates will be exceeded during only 10 percent of storm events.  CVs for bacterial constituents were not available, thus high and low estimates were set at +/- 90% of the mean EMC.

Table 4.1  Average annual load estimates for conventional and microbiological pollutants from point source discharges

|  |  | Conventional Pollutants |  |  |  |  |  |  |  | Microbiological Pollutants |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Flow | TSS | BOD | COD | O&G |  | Clostridium perfringens | E. coli | Enterococci | Fecal Coliform |
| Discharge Location | (ac-ft/yr) | (tons/year) |  |  |  |  |  |  |  | (10^15 CFU/year) |  |  |  |  |  |  |
| Ala Wai Canal | 2,912 | 11 |  |  | 10 |  |  |  |  |  |
| Keehi/Honolulu Harbor | 850 | 3.3 |  |  | 1.5 |  |  |  |  |  |
| Pearl Harbor | 3,360 | 1.2 | 19 |  | 0.049 |  |  |  |  |  |
| Coastal Mamala Bay | 72 | 1.6 | 7.6 |  |  |  |  |  |  | 0.00003 |
| Sand Island WWTP | 80,651 | 6,137 | 11,052 | 31,600 | 2,920 |  | 29 | 30,675 | 476 | 17,205 |
| Honouliuli WWTP | 26,884 | 1,997 | 4,425 | 11,100 | 1,157 |  | 13 | 8,533 | 131 | 5,074 |
| Ft. Kamehameha STP | 7,729 | 209 | 83 |  | 38 |  |  |  | 0.00037 |  |
| TOTAL | 122,460 | 8,360 | 15,590 | 42,700 | 4,127 |  | 42 | 39,208 | 607 | 22,280 |

Table 4.2  Average annual load estimates for nutrients and trace metals from point source discharges

|  |  | Nutrients |  |  |  |  |  |  |  |  |  | Metals |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Flow | NH4-N | NO2/ NO3-N | TKN | Total N | Total P |  | Cadmium | Copper | Lead | Zinc |
| Discharge Location | (ac-ft/yr) | (tons/year) |  |  |  |  |  |  |  |  |  | (tons/year) |  |  |  |  |  |  |
| Ala Wai Canal | 2,912 |  |  |  |  |  |  |  |  |  |  |
| Keehi/Honolulu Harbor | 850 |  |  |  |  |  |  |  | 0.00009 | 0.010 |  |
| Pearl Harbor | 3,360 | 0.26 | 3.7 |  |  | 0.28 |  |  | 0.0013 |  |  |
| Coastal Mamala Bay | 72 |  |  |  |  |  |  |  |  |  |  |
| Sand Island WWTP | 80,651 | 1,573 | 8.8 | 2,070 | 2,079 | 301 |  | 0.30 | 1.8 | 4.6 | 6.5 |
| Honouliuli WWTP | 26,884 | 676 | 2.0 | 879 | 881 | 135 |  | 0.093 | 1.1 | 6.8 | 2.8 |
| Ft. Kamehameha STP | 7,729 | 18 | 60 | 59 | 120 | 18 |  | 0.040 | 0.18 | 0.29 | 1.1 |
| TOTAL | 122,460 | 2,267 | 71 | 3,008 | 3,080 | 454 |  | 0.43 | 3.08 | 11.7 | 10.4 |

#### 4.2.1.1  Pollutant Loading for Existing Development Conditions

Table 4.3 lists the estimated annual runoff and the high, medium, and low estimates of annual pollutant loads from the Mamala Bay watersheds for existing development conditions, while Table 4.4 lists the range of average EMC's of discharges from Mamala Bay watersheds.  The relative contribution of key pollutants within each watershed is summarized in Figure 4.3. The following sections discuss these results in detail.

Runoff.  Each year, about 243,000 acre-ft of runoff enters Mamala Bay from its tributary watersheds.   Nearly 80 percent of this runoff is the direct result of rainfall events, with the rest consisting of baseflow.  About 25 percent of the rainfall over the Mamala Bay watersheds becomes rainfall-runoff.  On average, about 17 inches per year of rainfall runoff and 5 inches per year of baseflow are discharged from the Mamala Bay watersheds.

Total runoff ranges from about 30 inches per year in forest conservation areas to about 10 inches per year from the Ewa Plain.  Average annual runoff rates range from 5 cfs (2.3 cfs/mi2 ) from the Kewalo Basin to the 112 cfs (1.5 cfs/mi2 ) that enters the West Loch of Pearl Harbor.

Sediment.  Overall, between 50,000 and 185,000 tons of suspended sediment are washed off the Mamala Bay watersheds each year.  The largest contributor of sediment is natural erosion from the mountainous forest conservation areas of the watersheds.  Bedload sediments are not included in these estimates because most of this bedload is retained within the watershed (Freeman, 1993).  About 5 percent of the sediment load is from urban areas, where sediments are known to be associated with other pollutants, such as metals, nutrients, and hydrocarbon byproducts.  Total projected sediment loads correlate with measured sediment EMC's from small storms that contribute the most long-term pollutant loading.  The more critical urban sediment loadings are characteristic of national EMCs.  Sediment loading rates range between 39 mg/L (the heavily urbanized Kewalo Basin) to 453 mg/L (the largely forested and agricultural areas draining to the Middle Loch of Pearl Harbor).

Table 4.3  Annual Pollutant Loading for Mamala Bay Watersheds - Existing Development Conditions

Table 4.4  Average EMCs for Mamala Bay Watersheds Existing Land Use Conditions

Figure 4.3  Relative contribution of non-point source pollutants by watershed under current land use conditions

Nutrients.  Annual total phosphorus discharged from the Mamala Bay watersheds ranged between 79,000 and 321,000 lb-P/yr, while total Kjeldahl nitrogen loading ranged between 527,000 and 1,680,000 lb-N/yr (Table 4.3).  Urban runoff is the primary "per acre" source of nutrient loading from the Mamala Bay watersheds.  Table 4.4 shows the range of average total phosphorus EMC's characteristic of discharges from the Mamala Bay watersheds.  For this study, nutrient EMC's characteristic of urban runoff, derived from NURP, are substantially higher than locally reported nutrient concentrations.  These higher estimates are considered acceptable for this order of magnitude level estimate of pollutant loading because of the limited availability of local data and the lack of rationale provided in local studies to support lower nutrient loading for vegetated urban areas of O'ahu (e.g., fertilization and irrigation practices).

Metals.  Significant heavy metal loading is characteristic of urban runoff.  With the near universal use of unleaded gasoline, the most prevalent metals in urban runoff are now zinc and copper.  Zinc loading from the Mamala Bay watersheds (Table 4.3) range between 18,000 and 110,000 lb-Zn/yr, while copper loading ranges between 4,200 and 18,000 lb-Cu/yr.  Largest metals loads are discharged from the Keehi Lagoon/Honolulu Harbor and West Loch Pearl Harbor watersheds, as shown on Table 4.3.  The watersheds with the largest percentage of commercial and industrial land uses (Kewalo Basin and Keehi Lagoon/Honolulu Harbor) possess the highest metals loading rates (Table 4.3; Figure 4.3).

Bacterial Constituents.  Table 4.5 lists the mean annual loading and average concentration of fecal coliform, enterococci and Clostridium perfringens from each Mamala Bay watershed.  The largest bacterial loads are contributed by the Keehi Lagoon/Honolulu Harbor watersheds.  Bacterial concentrations match well with bacterial sampling conducted in the Ala Wai Canal for this project.

Table 4.5  Mean Annual Bacteria Loads from Mamala Bay Watersheds

Total Mean Annual Load (CFU/yr)

Fecal		Clostridium

Watershed			Coliform	Enterococci	perfringens

Ala Wai Canal			3.20E+15	3.86E+15	5.24E+13

Kewalo Basin			0.94E+15	1.18E+15	1.61E+13

Keehi Lagoon/Honolulu H.	7.86E+15	9.58E+15	13.00E+13

East Loch, Pearl Harbor		2.15E+15	2.28E+15	4.31E+13

Middle Loch, Pearl Harbor	2.15E+15	2.28E+15	3.05E+13

West Loch, Pearl Harbor		8.51E+15	9.97E+15	13.50E+13

Ewa Plain			0.66E+15	0.73E+15	0.98E+13

Average Bacterial Concentration (CFU/100ml)

Fecal		Clostridium

Watershed			Coliform	Enterococci	perfringens

Ala Wai Canal				11,119	13,417	182

Kewalo Basin				21,043	26,477	361

Keehi Lagoon/Honolulu H.		12,583	15,343	208

East Loch, Pearl Harbor			6,066	6,767	91

Middle Loch, Pearl Harbor		4,708	4,979	67

West Loch, Pearl Harbor			8,470	9,931	134

Ewa Plain				5,702	6,288	85

#### 4.2.1.2  Increased Pollutant Loads under Buildout Conditions

WMM was also used to project increases in runoff and annual pollutant loads under buildout conditions.  For this study, only about 5,400 acres of vacant land within the Mamala Bay watersheds (about 4 percent of the total area) was considered developable (see section 3.2.4.1). Table 4.6 lists the estimated annual runoff and the high, medium, and low estimates of annual pollutant loads from the Mamala Bay watersheds for buildout conditions, while Table 4.7 lists the range of average EMC's of discharges from Mamala Bay watersheds.  Annual runoff and pollutant loading increase slightly under buildout conditions. Runoff increases by about 4 percent in the Keehi Lagoon/Honolulu Harbor watershed, with lesser increases in runoff from the other watersheds.

### 4.2.2  Time Series Pollutant Load Estimates

A time series of hourly pollutant loads contributed by the Mamala Bay watersheds was prepared for the two periods:  August 1 through December 31, 1985, and January 1, 1994 through April 30, 1994.  Hourly pollutant loads were developed in the following manner:

•	Baseflows and baseflow concentrations are the same as those developed for the WMM.

•	The HEC-STORM model was used to predict hourly runoff volumes from each Mamala Bay watershed using hourly precipitation data from the Honolulu NWS gage multiplied by the ratio of average annual precipitation in each watershed and at the gage, and land use based runoff coefficients developed for the WMM.

•	Hourly loading for total suspended solids, biochemical oxygen demand, fecal coliform, enterococci, and Clostridium perfringens were determined by multiplying the hourly runoff flows by average EMCs determined by the WMM for each watershed.

The time series of hourly flows and pollutant loads for each loading point is available in the file nps.wk1 on the Mamala Bay database.  These data were provided to the MB-5 study team for use in evaluating the effects of nonpoint source loads on nearshore water quality.

Table 4.6  Annual Pollutant Loading Rates for Mamala Bay Watersheds - Buildout Land Use Conditions

|  | Ala Wai | Kewalo | Keehi Lagoon/ | Pearl Harbor |  |  |  |  | Ewa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Canal | Basin | Honolulu Harbor | East Loch | Middle Loch | West Loch | Plain |
| Constituent |  |  | Medium Annual Loading (lb/yr) |  |  |  |  |  |  |
| Runoff (ac-ft/yr) | 23,400 | 3,687 | 52,572 | 38,839 | 37,334 | 83,443 | 10,354 |
| BOD | 528,000 | 99,200 | 1,240,000 | 842,000 | 800,000 | 1,790,000 | 216,000 |
| COD | 3,186,637 | 511,704 | 7,471,648 | 4,974,852 | 4,726,742 | 9,525,458 | 1,096,027 |
| TSS | 20,700,000 | 1,350,000 | 44,100,000 | 45,500,000 | 45,600,000 | 61,100,000 | 6,110,000 |
| TDS | 4,724,037 | 818,060 | 11,634,294 | 8,254,037 | 8,230,065 | 16,296,703 | 1,791,684 |
| Total-P | 17,230 | 2,237 | 36,611 | 24,818 | 29,122 | 75,513 | 8,396 |
| Dissolved-P | 5,743 | 923 | 12,594 | 7,630 | 10,221 | 34,296 | 3,418 |
| TKN | 91,460 | 11,815 | 195,805 | 136,924 | 164,719 | 417,135 | 47,446 |
| NO2&NO3 | 42,350 | 5,750 | 93,435 | 68,232 | 88,883 | 264,105 | 22,661 |
| Lead | 488 | 159 | 1,409 | 451 | 312 | 1,444 | 177 |
| Copper | 1,316 | 301 | 3,362 | 1,600 | 1,187 | 3,071 | 366 |
| Zinc | 5,411 | 2,560 | 19,675 | 6,348 | 4,762 | 22,364 | 2,446 |
| Cadmium | 49 | 16 | 141 | 45 | 31 | 144 | 18 |
|  |  |  | High Annual Loading |  |  | (lb/yr) |  |  |
| BOD | 786,000 | 137,000 | 1,830,000 | 1,250,000 | 1,240,000 | 2,580,000 | 306,000 |
| COD | 5,275,322 | 781,278 | 12,049,348 | 8,156,502 | 7,734,165 | 15,208,804 | 1,766,957 |
| TSS | 36,100,000 | 2,630,000 | 77,200,000 | 76,400,000 | 75,900,000 | 104,000,000 | 10,600,000 |
| TDS | 8,675,226 | 1,600,058 | 21,554,266 | 14,366,210 | 14,059,914 | 29,155,099 | 3,248,741 |
| Total-P | 31,127 | 3,974 | 66,181 | 45,052 | 52,271 | 128,562 | 14,889 |
| Dissolved-P | 9,355 | 1,607 | 21,324 | 12,973 | 17,303 | 53,573 | 5,718 |
| TKN | 157,947 | 18,423 | 328,683 | 229,193 | 269,665 | 651,292 | 76,718 |
| NO2&NO3 | 74,094 | 9,241 | 159,282 | 114,869 | 141,842 | 380,585 | 36,874 |
| Lead | 914 | 290 | 2,605 | 841 | 580 | 2,652 | 327 |
| Copper | 2,451 | 584 | 6,235 | 2,602 | 1,865 | 5,816 | 726 |
| Zinc | 10,270 | 5,323 | 39,293 | 11,494 | 8,516 | 45,212 | 4,912 |
| Cadmium | 96 | 33 | 286 | 90 | 62 | 297 | 36 |
|  |  |  | Low Annual Loading |  |  | (lb/yr) |  |  |
| BOD | 318,000 | 66,700 | 755,000 | 482,000 | 448,000 | 1,130,000 | 143,000 |
| COD | 1,524,661 | 288,491 | 3,785,425 | 2,430,333 | 2,318,548 | 4,927,559 | 556,136 |
| TSS | 8,810,000 | 399,000 | 18,600,000 | 21,100,000 | 21,500,000 | 27,200,000 | 2,610,000 |
| TDS | 1,755,422 | 244,831 | 4,204,782 | 3,533,378 | 3,679,227 | 6,500,910 | 689,402 |
| Total-P | 6,708 | 916 | 14,196 | 9,507 | 11,615 | 35,346 | 3,480 |
| Dissolved-P | 2,905 | 405 | 5,843 | 3,512 | 4,819 | 19,647 | 1,661 |
| TKN | 40,254 | 6,469 | 91,891 | 64,509 | 81,559 | 230,467 | 24,314 |
| NO2&NO3 | 18,134 | 2,970 | 42,402 | 31,783 | 47,022 | 171,669 | 11,489 |
| Lead | 169 | 59 | 507 | 158 | 110 | 530 | 64 |
| Copper | 499 | 92 | 1,273 | 876 | 697 | 1,062 | 103 |
| Zinc | 1,877 | 592 | 5,604 | 2,629 | 2,055 | 6,046 | 677 |
| Cadmium | 14 | 4 | 36 | 12 | 8 | 35 | 5 |

Table 4.7  Average EMCs for Mamala Bay Watersheds - Buildout Land Use Conditions

|  | Ala Wai | Kewalo | Keehi Lagoon/ | Pearl Harbor |  |  |  |  | Ewa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Canal | Basin | Honolulu Harbor | East Loch | Middle Loch | West Loch | Plain |
| Constituent |  |  | Medium EMC's (mg/L) |  |  |  |  |  |
| Runoff (cfs) | 32 | 5 | 73 | 54 | 52 | 115 | 14 |
| BOD | 8.4 | 9.7 | 8.7 | 8.0 | 7.9 | 7.9 | 7.7 |
| COD | 50.2 | 51.1 | 52.4 | 47.2 | 46.7 | 42.1 | 39.0 |
| TSS | 327 | 132 | 310 | 432 | 450 | 270 | 218 |
| TDS | 74.4 | 81.8 | 81.6 | 78.3 | 81.2 | 72.0 | 63.8 |
| Total-P | 0.27 | 0.22 | 0.26 | 0.24 | 0.29 | 0.33 | 0.30 |
| Dissolved-P | 0.09 | 0.09 | 0.09 | 0.07 | 0.10 | 0.15 | 0.12 |
| TKN | 1.44 | 1.18 | 1.37 | 1.30 | 1.63 | 1.84 | 1.69 |
| NO2&NO3 | 0.67 | 0.57 | 0.65 | 0.65 | 0.88 | 1.17 | 0.81 |
| Lead | 0.01 | 0.02 | 0.01 | 0.00 | 0.00 | 0.01 | 0.01 |
| Copper | 0.02 | 0.03 | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 |
| Zinc | 0.09 | 0.26 | 0.14 | 0.06 | 0.05 | 0.10 | 0.09 |
| Cadmium | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  |  | High EMC's (mg/L) |  |  |  |  |  |
| BOD | 12.4 | 13.4 | 12.9 | 12.2 | 12.2 | 11.4 | 10.9 |
| COD | 83.1 | 78.1 | 84.5 | 77.4 | 76.3 | 67.2 | 62.9 |
| TSS | 571 | 258 | 542 | 726 | 750 | 461 | 379 |
| TDS | 137 | 160 | 151 | 136 | 139 | 129 | 116 |
| Total-P | 0.49 | 0.40 | 0.46 | 0.43 | 0.52 | 0.57 | 0.53 |
| Dissolved-P | 0.15 | 0.16 | 0.15 | 0.12 | 0.17 | 0.24 | 0.20 |
| TKN | 2.49 | 1.84 | 2.30 | 2.17 | 2.66 | 2.88 | 2.73 |
| NO2&NO3 | 1.17 | 0.92 | 1.12 | 1.09 | 1.40 | 1.68 | 1.31 |
| Lead | 0.01 | 0.03 | 0.02 | 0.01 | 0.01 | 0.01 | 0.01 |
| Copper | 0.04 | 0.06 | 0.04 | 0.02 | 0.02 | 0.03 | 0.03 |
| Zinc | 0.16 | 0.53 | 0.28 | 0.11 | 0.08 | 0.20 | 0.17 |
| Cadmium | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  |  |  | Low EMC's (mg/L) |  |  |  |  |  |
| BOD | 5.0 | 6.5 | 5.3 | 4.6 | 4.4 | 5.0 | 5.1 |
| COD | 24.0 | 28.8 | 26.5 | 23.1 | 22.9 | 21.8 | 19.8 |
| TSS | 139 | 39 | 131 | 200 | 212 | 120 | 93 |
| TDS | 27.6 | 24.5 | 29.5 | 33.5 | 36.3 | 28.7 | 24.5 |
| Total-P | 0.11 | 0.09 | 0.10 | 0.09 | 0.11 | 0.16 | 0.12 |
| Dissolved-P | 0.05 | 0.04 | 0.04 | 0.03 | 0.05 | 0.09 | 0.06 |
| TKN | 0.63 | 0.65 | 0.64 | 0.61 | 0.81 | 1.02 | 0.87 |
| NO2&NO3 | 0.29 | 0.30 | 0.30 | 0.30 | 0.46 | 0.76 | 0.41 |
| Lead | 0.00 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Copper | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.00 | 0.00 |
| Zinc | 0.03 | 0.06 | 0.04 | 0.02 | 0.02 | 0.03 | 0.02 |
| Cadmium | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

A summary of significant rainfall events and simulated discharge from the Ala Wai Canal is presented for the 16-month period of January 1994 to April 1995.  Significant events were defined as any event producing a minimum of 2,000 kilocubic feet of runoff from the watershed.  Significant runoff occurred on 53 different occasions during this time period  averaging approximately 3.3 events per month.  Events ranged in duration from 1 to 54 hours although 75% of the runoff events lasted less than 12 hours and resulted in less than 12,000 kilocubic feet of runoff.

On four occasions, estimated runoff exceeded 50,000 kilocubic feet.  Two of these events occurred in February 1994.  The Ala Wai Canal wet weather study used to calibrate runoff loading factors was conducted in association with the third event.  This event lasted 54 hours and was the among the largest runoff events (95,900 kilocubic feet) occurring during the Mamala Bay Study.  The event ended on the 25th of March.  The wet weather study captured runoff from the latter part of this event.  The fourth major event occurred on January 28, 1995.  This latter event was relatively brief and intense.  The storm lasted only eight hours and was estimated to produce 114,000 kilocubic feet of runoff.

A review of the modeled pollutant load estimates for the January 28, 1995 storm event indicates that bacterial loads can be extremely high during such intense events.  The loading rate of fecal coliform over the eight hour event was estimated to be 6 percent of the average loading rate of the Sand Island WWTP.  Over the same period, enterococci loading rates were estimated to be equivalent to combined discharge of Sand Island and Honouliuli WWTPs.

Table 4.8  Summary of storm events and HEC-STORM estimates of discharge in the Ala Wai watershed

| Date |  |  |  |  | Duration (hours) | Discharge Volume (kilocubic ft) |  | Date |  |  |  |  | Duration (hours) | Discharge Volume (kilocubic ft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 17 | 94 | 23 | 6,940 |  | 10 | 31 | 94 | 1 | 2,880 |
| 1 | 25 | 94 | 36 | 18,700 |  | 11 | 16 | 94 | 5 | 3,860 |
| 2 | 11 | 94 | 18 | 4,510 |  | 11 | 29 | 94 | 5 | 2,430 |
| 2 | 12 | 94 | 14 | 4,970 |  | 12 | 1 | 94 | 8 | 5,370 |
| 2 | 14 | 94 | 20 | 119,000 |  | 12 | 4 | 94 | 8 | 3,940 |
| 2 | 17 | 94 | 37 | 75,200 |  | 12 | 23 | 94 | 7 | 2,770 |
| 2 | 21 | 94 | 8 | 3,940 |  | 12 | 24 | 94 | 3 | 2,660 |
| 3 | 5 | 94 | 22 | 11,800 |  | 1 | 12 | 95 | 3 | 5,230 |
| 3 | 20 | 94 | 3 | 5,130 |  | 1 | 14 | 95 | 6 | 3,030 |
| 3 | 25 | 94 | 54 | 95,900 |  | 1 | 26 | 95 | 7 | 3,340 |
| 4 | 8 | 94 | 10 | 3,140 |  | 1 | 28 | 95 | 2 | 3,480 |
| 4 | 12 | 94 | 9 | 3,400 |  | 2 | 5 | 95 | 13 | 22,400 |
| 4 | 28 | 94 | 10 | 7,420 |  | 2 | 10 | 95 | 7 | 3,910 |
| 6 | 19 | 94 | 8 | 14,800 |  | 2 | 14 | 95 | 15 | 28,700 |
| 6 | 21 | 94 | 2 | 2,490 |  | 2 | 18 | 95 | 22 | 20,300 |
| 6 | 22 | 94 | 2 | 8,190 |  | 2 | 28 | 95 | 8 | 114,000 |
| 6 | 25 | 94 | 3 | 39,300 |  | 3 | 1 | 95 | 5 | 9,850 |
| 7 | 1 | 94 | 1 | 10,700 |  | 3 | 2 | 95 | 10 | 7,430 |
| 7 | 6 | 94 | 1 | 2,170 |  | 3 | 5 | 95 | 10 | 4,570 |
| 7 | 19 | 94 | 8 | 10,900 |  | 3 | 12 | 95 | 11 | 11,200 |
| 7 | 22 | 94 | 8 | 12,000 |  | 3 | 28 | 95 | 11 | 7,460 |
| 7 | 24 | 94 | 4 | 22,200 |  | 3 | 29 | 95 | 9 | 3,110 |
| 8 | 31 | 94 | 2 | 3,770 |  | 4 | 3 | 95 | 11 | 12,900 |
| 9 | 18 | 94 | 15 | 3,860 |  | 4 | 14 | 95 | 11 | 5,460 |
| 9 | 20 | 94 | 3 | 3,800 |  | 4 | 18 | 95 | 11 | 3,460 |
| 10 | 3 | 94 | 17 | 27,000 |  | 4 | 25 | 95 | 8 | 12,500 |
| 10 | 15 | 94 | 2 | 13,500 |  |  |  |  |  |  |

### 4.2.3  Pollutant Delivery to Mamala Bay

Virtually all runoff from the Mamala Bay watersheds pass through major embayments before reaching Mamala Bay.  Residence time in these embayments range from 2 to 3 days (Ala Wai Canal) to several months (Pearl Harbor).  Thus a portion of the annual load of settleable constituents is not discharged into Mamala Bay.  Furthermore, biological processes may consume dissolved nutrients, further reducing Mamala Bay loading.  Dieoff of bacteria is also likely within these embayments.

Estimating the pollutant delivery to Mamala Bay begins by determining the settleable fraction of each constituent.  The suspended fractions of the following pollutants are based on review of national and regional pollutant loading databases and CDM's national experience with nonpoint source pollution loading studies:

Pollutant     	Suspended Fraction

BOD	0%

COD	0%

TSS	100%

TDS	0%

TP	40%

DP	0%

TKN	20%

NO2&NO3	50%

Pb	90%

Cu	60%

Zn	40%

Cd	70%

Fine to very fine silt particles that make up the bulk of suspended sediments in runoff settle at a rate of about 0.1 ft/hr.  This settling rate, coupled with an estimate of the average residence time in each water body is used to determine the percent of the suspended fraction that settles during an average storm.  While actual residence times will vary storm to storm, the average residence time gives a good approximation of the average annual percent of the pollutant load trapped in the embayment, defined as follows:

Ala Wai Canal.  The recent management plan for Ala Wai Canal used dye tests to define an average residence time of 40 to 60 hours. With this residence time, suspended sediment will settle about 4 to 6 feet.  Since the average depth of Ala Wai Canal is about 8 to 10 feet, about 50 percent of the settleable solids should be removed.

Pearl Harbor.  Pearl Harbor, with a surface area of about 5,000 acres and an average inflow rate of about 200 cfs, has an average retention time of about 4 months.  Virtually all the suspended solids will be trapped by Pearl Harbor on an average annual basis.

Kewalo Basin.  Kewalo Basin has a surface area of about 23 acres, with an average inflow volume of 5 cfs.  Thus, Kewalo basin has a retention time of about 3 to 4 weeks, sufficient to remove virtually all suspended solids.

Honolulu Harbor/Keehi Lagoon.  With a total surface area of 1661 acres and an average inflow rate of about 70 cfs, Honolulu Harbor and Keehi Lagoon have a residence time of about 2 months, again sufficient to remove virtually all suspended solids.

The final parameter for determining pollutant delivery from the embayments to Mamala Bay is an estimate of pollutant uptake and bacteria dieoff within each water body.  Such phenomena are best estimated with a water quality model, a task beyond the scope of this order of magnitude pollutant loading estimate.  Therefore, this study assumes that all dissolved materials and bacterial constituents are delivered to Mamala Bay, a conservative yet reasonable approximation.  Table 4.9 lists the delivery ratio into Mamala Bay of pollutants discharged into Mamala Bay embayments.

Table 4.9  Pollutant Delivery Ratios into Mamala Bay

|  |  | Average Percent of Total Load Delivered to Mamala Bay |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pollutant | Suspended Fraction | Ala Wai Canal (1) | Pearl Harbor (2) | Kewalo Basin (2) | Keehi Lagoon/ Honolulu Harbor (2) |
| BOD | 0% | 100% | 100% | 100% | 100% |
| COD | 0% | 100% | 100% | 100% | 100% |
| TSS | 100% | 50% | 5% | 5% | 5% |
| TDS | 0% | 100% | 100% | 100% | 100% |
| TP | 40% | 80% | 62% | 62% | 62% |
| DP | 0% | 100% | 100% | 100% | 100% |
| TKN | 20% | 90% | 81% | 81% | 81% |
| NO2&NO3 | 50% | 75% | 52% | 52% | 52% |
| Pb | 90% | 15% | 10% | 10% | 10% |
| Cu | 60% | 70% | 43% | 43% | 43% |
| Zn | 40% | 80% | 62% | 62% | 62% |
| Cd | 70% | 65% | 34% | 34% | 34% |
| Fecal Coliform | 0% | 100% | 100% | 100% | 100% |
| Enterococci | 0% | 100% | 100% | 100% | 100% |
| C. perfringens | 0% | 100% | 100% | 100% | 100% |

1.   Pollutant delivery equals 50% of the suspended fraction and 100% of the dissolved fraction.

2.   Pollutant delivery equals 5% of the suspended fraction and 100% of the dissolved fraction.

# 5  CONCLUSIONS

On an annualized basis, non-point sources account for roughly twice the volume of point source discharges (Table 5.1).  The majority of non-point source discharges pass through Pearl Harbor, Keehi Lagoon/Honolulu Harbor, Kewalo Basin and the Ala Wai Canal.  The various pollutants associated with the non-point source discharges are known to impact water and sediment quality causing these water bodies to be classified as Water Quality Limited Segments (WQLSs).  Many of the pollutants entering the WQLSs are known to be trapped or transformed as the flows pass through the WQLSs.  The degree to which this occurs is primarily a function of the volume or flushing rate of the water body, the flow rates generated by non-point source discharges, and partitioning of the pollutant between particulate and dissolved forms.

The average annual contribution of pollutants from point and non-point source discharges (Table 5.1) differ substantially for the various classes of pollutants.  Non-point discharges are the primary source of sediment, copper and zinc.  Point source discharges are the dominant sources of BOD, COD, indicator bacteria, nutrients, cadmium and lead.

Although the annual load of indicator bacteria in non-point source discharges is 0.1 to 5% of the point source loads, the HEC-STORM model indicated that loading rates of indicator bacteria can be substantial during more intense storm events.  The model estimated loading rates for fecal coliform at 6 percent of long-term average loading rates for fecal coliform from the Sand Island WWTP.  Estimates of entercoccus loading rates for the same 8-hour event were equivalent to loading rates from all point source discharges combined.

In comparison to non-point source pollutant loads, permitted point source discharges contribute only minor quantities of pollutants to the Ala Wai Canal, Keehi Lagoon/Honolulu Harbor, Pearl Harbor and directly to coastal waters of Mamala Bay (Tables 5.2 and 5.3).  The two major point source dischargers, Sand Island WWTP and Honouliuli WWTP, discharge through offshore outfalls.  These two wastewater treatment plants are clearly the dominant point source dischargers to Mamala Bay in terms of flow, conventional pollutants, microbiological indicators, nutrients and metals.

Pearl Harbor, being the largest watershed, receives the largest non-point source flows and total pollutant loads.  However, pollutant loading rates from the smaller, more urbanized watersheds of the Ala Wai Canal, Kewalo Basin and Keehi Lagoon/Honolulu Harbor are relatively elevated due to both increased runoff associated with urbanization and contributions of urban-related pollutants such as copper and zinc.

Table 5.1  Comparison of average annual pollutant loads from point and non-point sources

|  | Total | Percentage |  |  |  |  | Total | Percentage |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Load | Point | Non-Point1 |  | Parameter | Load | Point | Non-Point |
| Flow | 361,205 ac-ft | 34 | 66 |  | NH4-N | 2,267 tons | 100 | - |
| TSS | 119,815 tons | 7 | 83 |  | NO2/NO3-N | 367 tons | 19 | 81 |
| BOD | 18,160 tons | 86 | 14 |  | TKN | 3,527 tons | 85 | 15 |
| COD | 57,795 tons | 74 | 26 |  | Total N | 3,895 tons | 79 | 21 |
| O&G | 4,127 tons | 100 | - |  | Total P | 547 tons | 83 | 17 |
| C. perfringens | 4.2E16 CFU | 99 | 1 |  | Cadmium | 0.63 tons | 68 | 32 |
| E. coli | 3.92E19 CFU | 100 | - |  | Copper | 8.17 tons | 38 | 62 |
| enterococci | 6.38E17 CFU | 95 | 5 |  | Lead | 13.7 tons | 85 | 15 |
| fecal coliform | 2.23E19 CFU | 99.9 | 0.1 |  | Zinc | 38.5 tons | 27 | 73 |

1 Only a fraction of the non-point source load reaches Mamala Bay.

Table 5.2  Comparison of point and nonpoint source load estimates for conventional and microbiological pollutants

|  |  | Conventional Pollutants |  |  |  |  |  |  |  | Microbiological Pollutants |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TSS | BOD | COD | O&G |  | Clostridium perfringens | E. coli | Enterococci | Fecal Coliform |
| Point Source Discharge Location | Flow (ac-ft/yr) | (tons/year) |  |  |  |  |  |  |  | (10^15 CFU/year) |  |  |  |  |  |  |
| Ala Wai Canal | 2,912 | 11 |  |  | 10 |  |  |  |  |  |
| Kewalo Basin | 0 |  |  |  |  |  |  |  |  |  |
| Keehi/Honolulu Harbor | 850 | 3.3 |  |  | 1.5 |  |  |  |  |  |
| Pearl Harbor | 3,360 | 1.2 | 19 |  | 0.049 |  |  |  |  |  |
| Coastal Mamala Bay | 72 | 1.6 | 7.6 |  |  |  |  |  |  | 0.00003 |
| Sand Island WWTP | 80,651 | 6,137 | 11,052 | 31,600 | 2,920 |  | 29 | 30,675 | 476 | 17,205 |
| Honouliuli WWTP | 26,884 | 1,997 | 4,425 | 11,100 | 1,157 |  | 13 | 8,533 | 131 | 5,074 |
| Ft. Kamehameha STP | 7,729 | 209 | 83 |  | 38 |  |  |  | 0.00037 |  |

|  |  | TSS | BOD | COD | O&G |  | Clostridium perfringens | E. coli | Enterococci | Fecal Coliform |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-point Source Discharge Location | Flow (ac-ft/yr) | (tons/year) |  |  |  |  |  |  |  | (10^15 CFU/year) |  |  |  |  |  |  |
| Ala Wai Canal | 23,350 | 10,350 | 256 | 1,585 |  |  | 0.052 |  | 3.9 | 3.2 |
| Kewalo Basin | 3,690 | 635 | 44 | 256 |  |  | 0.016 |  | 1.2 | 0.9 |
| Keehi/Honolulu Harbor | 50,590 | 21,800 | 571 | 3,522 |  |  | 0.13 |  | 9.6 | 7.9 |
| Pearl Harbor (E. Loch) | 38,320 | 22,700 | 403 | 2,417 |  |  | 0.043 |  | 3.2 | 2.9 |
| Pearl Harbor (M. Loch) | 37,013 | 22,750 | 387 | 2,320 |  |  | 0.031 |  | 2.3 | 2.2 |
| Pearl Harbor (W. Loch) | 81,336 | 30,250 | 822 | 4,547 |  |  | 0.13 |  | 10 | 8.5 |
| Ewa Plain | 4,446 | 2,970 | 87 | 448 |  |  | 0.0098 |  | 0.73 | 0.66 |

Table 5.3  Comparison of point and non-point source discharges load estimates for nutrient and trace metals

|  |  | Nutrients |  |  |  |  |  |  |  |  |  | Metals |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NH4-N | NO2/ NO3-N | TKN | Total N | Total P |  | Cadmium | Copper | Lead | Zinc |
| Point Source Discharge Location | Flow (ac-ft/yr) | (tons/year) |  |  |  |  |  |  |  |  |  | (tons/year) |  |  |  |  |  |  |
| Ala Wai Canal | 2,912 |  |  |  |  |  |  |  |  |  |  |
| Kewalo Basin | 0 |  |  |  |  |  |  |  |  |  |  |
| Keehi/Honolulu Harbor | 850 |  |  |  |  |  |  |  | 0.00009 | 0.010 |  |
| Pearl Harbor | 3,360 | 0.26 | 3.7 |  |  | 0.28 |  |  | 0.0013 |  |  |
| Coastal Mamala Bay | 72 |  |  |  |  |  |  |  |  |  |  |
| Sand Island WWTP | 80,651 | 1,573 | 8.8 | 2,070 | 2,079 | 301 |  | 0.30 | 1.8 | 4.6 | 6.5 |
| Honouliuli WWTP | 26,884 | 676 | 2.0 | 879 | 881 | 135 |  | 0.093 | 1.1 | 6.8 | 2.8 |
| Ft. Kamehameha STP | 7,729 | 18 | 60 | 59 | 120 | 18 |  | 0.040 | 0.18 | 0.29 | 1.1 |

|  |  | NH4-N | NO2/ NO3-N | TKN | Total N | Total P |  | Cadmium | Copper | Lead | Zinc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-point Source Discharge Location | Flow (ac-ft/yr) | (tons/year) |  |  |  |  |  |  |  |  |  | (tons/year) |  |  |  |  |  |  |
| Ala Wai Canal | 23,350 |  | 21 | 46 | 67 | 8.6 |  | 0.024 | 0.65 | 0.24 | 2.7 |
| Kewalo Basin | 3,690 |  | 2.9 | 5.9 | 8.8 | 1.1 |  | 0.008 | 0.15 | 0.079 | 1.3 |
| Keehi/Honolulu Harbor | 50,590 |  | 45 | 93 | 138 | 17 |  | 0.062 | 1.5 | 0.62 | 8.6 |
| Pearl Harbor (E. Loch) | 38,320 |  | 44 | 67 | 111 | 12 |  | 0.020 | 0.74 | 0.20 | 2.8 |
| Pearl Harbor (M. Loch) | 37,013 |  | 43 | 81 | 124 | 14 |  | 0.014 | 0.56 | 0.14 | 2.2 |
| Pearl Harbor (W. Loch) | 81,336 |  | 130 | 204 | 334 | 37 |  | 0.064 | 1.4 | 0.63 | 9.9 |
| Ewa Plain | 4,446 |  | 10 | 22 | 32 | 3.7 |  | 0.005 | 0.09 | 0.044 | 0.58 |

# 6  RECOMMENDATIONS

This study provided an initial evaluation of the primary sources of pollutants to Mamala Bay.  Data for the point source pollutant load evaluation were obtained for all dischargers with existing National Pollutant Discharge Elimination System (NPDES) discharge permits.  Point source pollutant loads were calculated using measurements of discharge volume and pollutant concentration reported in monthly discharge monitoring reports (DMRs) submitted to the  Hawaii Department of Health.  Standard EPA forms are used for submittal of discharge monitoring data.  QA/QC data are not routinely submitted with the NPDES reports.   Evaluation of sources of non-point source pollutants relied primarily on national data sets with use of local data whenever possible.  Few studies have been conducted in Hawaii using appropriate methods to establish statistically valid EMC data for local conditions.  Nevertheless, limited data from local studies were typically in the range of values reported from the National Urban Runoff Program.  Appropriate EMC data from non-urban areas were even more limited requiring that estimates be based upon data sets that used a wide variety of sampling strategies.

The following recommendations are provided for 1) improving the quality and usefulness of the point source database and 2) confirming appropriateness of model inputs for assessing non-point source loads.

A number of changes are recommended in order to improve the reliability and usefulness of NPDES self-monitoring data reports.  These actions would need to be implemented by the Hawaii Department of Health although assistance could be provided by the Mamala Bay Commission.

·	Develop standard electronic submittal formats for all NPDES dischargers that would include submittal of QA/QC data.

·	Classify dischargers into major categories and provide a consistent set of monitoring requirements for each group.

·	Specify target detection limits for each constituent in the permits

Non-point source EMC data that were not available during this study should be obtained and reviewed to further evaluate appropriateness of input parameters used in the loading model.  The only known set of data for which we did not have access is the City and County of Honolulu Stormwater Characterization Study.  These data are extremely important since they will represent a significant proportion of the local non-point source database.

The City and County of Honolulu stormwater discharge permit will require that monitoring be continued over the life of the permit.  It is recommended that this ongoing monitoring effort incorporate a microbiological component to assist in validation of EMC data and verification of loading estimates.  Further time-series data, together with flow measurements, from sites representative of typical land use are needed in order to           1) improve loading estimates and 2) assist in evaluation of improved source control measures.

# 7  REFERENCES

Chinn, S.S., G.A. Tateishi and J.J.S. Yee 1988.  Water Resources Data Hawaii and other Pacific Areas Water Year 1986, Volume 1. Hawaii.  U.S. Geological Survey Water- Data Report HI-87-1

Ching, R.K.Y. 1972.  Water quality effects of urban runoff.  Master's Thesis, University of Hawaii at Manoa

Christakos-Comack, E. and G.L. Dugan 1982.  Water Quality of Airport Storm Runoff.  Technical Progress Rpt., University of Hawaii at Manoa, Water Resources Research Center.

Department of Health 1980.  Storm Water Runoff and Urban Street Contaminants: Intensive Survey Report.  State of Hawaii, Department of Health, Pollution Investigation and Enforcement Branch, Environmental Protection and Health Services Division.  Honolulu, June.

Department of Public Works 1991.  Part 1- Application for National Pollutant Discharge Elimination System (NPDES) Permit for Stormwater Discharges into Waters of the United States from Municipal Separate Storm Sewer Systems.  October 17, 1991.  DPW, City and County of Honolulu, HI

Department of Public Works 1990.  Water Quality Management Plan for the City and County of Honolulu.  September, 1990.  City and County of Honolulu, Hawaii State Department of Health, HI.

Department of Wastewater Management 1993.  East Mamala Bay Wastewater Facilities Plan.  Environmental Impact Statement.  August 23, 1993.  Belt Collins Hawaii and City and County of Honolulu, HI.

Department of Land and Natural Resources and Division of Water and Land Development, State of Hawaii 1992.  Ala Wai Canal Improvement.  Feasibility Report R-89b.  October.

El-Swaify, Samir  A. and K. R. Cooley 1980.  Sediment Losses from Small Agricultural Watersheds in Hawaii (1972-77).  U. S. Department of Agriculture Science and Education Administration Agricultural Reviews and Manual.  Oakland:  Agricultural Research (Western Region).

Fox, J. and W. Freeman 1992.  Management Plan for the Ala Wai Canal Watershed.  Report prepared for Edward K. Noda and Associates, Inc. and DLNR

Freeman, W., 1993.  Revised Total Maximum Daily Load Estimates for Six Water Quality Limited Segments Island of O'ahu, Hawaii.  Report prepared for Environmental Planning Office Hawaii State Department of Health Honolulu, Hawaii.

Fujioka, R.S., S.K. Tenno, and S. Kansako 1988.  Naturally occurring Fecal Coliforms and Fecal Streptococci in Hawaii's Freshwater Streams.  Toxicity Assessment 3:613-30

Fujioka, R.S. 1983.  Stream Water Quality Assessment based on Fecal Coliform and Fecal Streptococcus Analysis.  Technical Memorandum No. 70, University of Hawaii at Manoa, Water Resources Research Center.

Fujiwara, T.O. 1973.  Characterization of urban stormwater discharges from separate storm sewers.  Master's Thesis, University of Hawaii at Manoa

Fukunaga & Associates, Inc. 1980.  Statewide Silt Basin Investigation.  Report # R66.  Report prepared for Division of Water & Land Development Department of Land & Natural Resources State of Hawaii.

Matsuoka, I., V.E. Kunishige and M.G. Lum 1992.  Water Resources Data Hawaii and other Pacific Areas Water Year 1991, Volume 1. Hawaii.  U.S. Geological Survey Water-Data Report HI-91-1

Matsuoka, I., M.G. Lum and V.E. Kunishige 1991.  Water Resources Data Hawaii and other Pacific Areas Water Year 1990,  Volume 1. Hawaii.  U.S. Geological Survey Water-Data Report HI-90-1

Nakahara, R., J.J.S. Yee, I. Yamashiro, G.A. Tateishi and J.A. Dommingo 1989.  Water Resources Data Hawaii and other Pacific Areas Water Year 1988, Volume 1. Hawaii.  U.S. Geological Survey Water-Data Report HI-89-1

Nakahara, R., M.G.H.S. Lum, I. Tamashiro, G.A. Tateishi and V.E. Kunishige 1990.  Water Resources Data Hawaii and other Pacific Areas Water Year 1989, Volume 1. Hawaii.  U.S. Geological Survey Water-Data Report HI-89-1

Nakahara, R., J.J.S. Yee and J.A. Domingo 1988.  Water Resources Data Hawaii and other Pacific Areas Water Year 1987, Volume 1. Hawaii.  U.S. Geological Survey Water-Data Report HI-87-1

Office of State Planning, Land Use Commission - Land Use Maps

Office of State Planning 1992.  State Land Use District Boundary Review - Oahu.  Office of the Governor, Office of State Planning

U. S. Department of Agriculture Soil Conservation Service 1981.  Sediment Losses from Small Agricultural Watersheds in Hawaii (1972-77).  (with 1993 errata to the Universal Soil Loss Equation K and T values).

U.S. Department of Agriculture Soil Conservation Service and University of Hawaii Agricultural Experiment Station 1972.  Soil  Survey of Island of Kauai, Oahu, Maui, Molokai, and Lanai, State of Hawaii.  Washington, D.C., UlSl Government Printing Office.

Wong, M.F., B.R. Hill 1992.  Statistical Summary of Hydrologic and Water-Quality Data From the North Halawa, Haiku, and Kamooalii Drainage Basins, Oahu, Hawaii.  Water-Resources Investigations Report 92-4049.  U.S. Geological Survey, Honolulu, Hawaii.

Yamane, C.M., and M.G. Lum 1985.  Quality of Storm-Water Runoff, Mililani, Oahu, Hawaii, 1980-1984.  Water Resource Investigation Report 85-4265.  UlSl Geological  Survey, Honolulu, December.

Yim, S.K., and G.L. Dugan 1975.  Quality and Quantity of Nonpoint Pollution Sources in Rural Surface Water Runoff on Oahu, Hawaii.  Technical Report No. 93.  University of Hawaii at Manoa, Water Resources Research Center.

# APPENDIX A

| Table A.1 Location and nature of existing (as of 12/93) permitted industrial dischargers direct to Mamala Bay |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| AES Barbers Point Inc. 91-086 Kaomi Loop Ewa Beach, HI 96707 (HI 0021130) | 001 21°18' 16" 002 21°17' 56" | 158°06'38" 158°06'22" | Overflow from settling basins Coal or limestone pile runoff | Pacific Ocean |
| DLNR Demonstration Desal Plant (HI 0021148) | 001 21°19' 45" | 158°05'30" | Brine from desalinization process | Unnamed ditch to Pacific Ocean/groundwater recharge |
| Hawaii Milling Corporation Hawaii Meat Company Feedlot Campbell Industrial park (HI 0020656) | 21°18 "30' | 158°06"00' | Stormwater runoff (greater than 8 inches in 24 hours) | Pacific Ocean |
| Waikiki Aquarium University of Hawaii (HI 0020630) | 21° 16" 17' | 156°49"31' | Overflow water from aquatic animal exhibits through an outfall | Pacific Ocean |

| Table A.2 Location and nature of existing (as of 2/13/93) permitted industrial dischargers to Keehi Lagoon |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| AMERON HC&D, Ltd. 811 Middle Street Honolulu, HI 96820 (HI 0021075) | 21°19'30" | 157°53'40" | Treated process water, storm water runoff, and conveyer belt tunnel seepage | Keehi Lagoon |
| Hawaii Fueling Facilities 3181 and 3201 Aolele Street Honolulu, HI (HI 0020354) | 21°20"08' | 157°55"00' | Stormwater runoff from an aviation turbine fuel storage facility | Keehi Lagoon |
| Keehi Marine Inc. 24 Sand Island Access Rd. Honolulu, HI (HI 0020664) | 21°19"08' | 157°53"50' | Wash water from a marine railway facility | Keehi Lagoon |

| Table A.3 Location and nature of existing (as of 2/13/93) permitted industrial dischargers to Honolulu Harbor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| Chevron USA Inc. Honolulu Distribution Center 933 North Nimitz Highway Honolulu, HI (HI 0020923) | 21°19'01" | 157°52'37" | Stormwater runoff from discharge recovery pad Loading rack washdown Fire hydrant test water | Honolulu Harbor |
| Chevron USA Inc. Honolulu Marine Terminal 777 North Nimitz Highway Honolulu, HI (HI 0020931) | 21°18'49" | 157°52'39" | Tank water draws, loading rack/pier wash down, fire hydrant test water and storm water runoff from fuel storage facility | Honolulu Harbor |
| Chevron USA Inc. Kapalama Terminal (HI 0020940) | 001 21°19'21" 002 21°19'21" | 157°52'43" 157°52'46" | Tank water draws, yard storm water runoff and fire hydrant testing waters | Honolulu Harbor |
| Hawaiian Electric Company Honolulu Generating Station (HI 0000027) | 21°18 "33' | 157°52"01' | Cooling water 70 mgd (002) Low volume waste 0.06 mgd (002/003) Metal Cleaning Wastes 0.065 mgd (002/003) | Honolulu Harbor |
| Hawaiian Cement Pier 34 Distribution Center (HI 0021083) | 21°19 "04' | 157°52"42' | Stormwater runoff from a cement receiving, storage and distribution facility (001 and 002) | Honolulu Harbor (via ditch) |
| Honolulu Shipyard Pier 41 (HI 0020753) | 001 21°19 "20' 002 21°19 "13' 003 21°19 "10' | 157°53"07' 157°53"09' 157°53"10' | Harbor water flowing off docks after lifting/lowering Marine Railway Kekaulana Floating Drydock | Honolulu Harbor |
| Pacific Resources Terminals 739 North Nimitz Highway Honolulu HI 96817 (HI 0000663) | 21° 18" 50' | 157° 52" 30' | Stormwater runoff from petroleum bulk station and terminal | Honolulu Harbor |
| Shell Oil Company - Marketing Dist. Plant 789 North Nimitz Highway Honolulu Plant (HI 0000582) | 21° 19" 00' | 157° 52" 30' | Stormwater runoff for plant | Honolulu Harbor |
| Shell Oil Company (Mike's Vineyard Shell) 225 South Vineyard Boulevard Nov 91 Permit (HI 0021211) | 21° 19" 06' | 157° 52" 20' | Treated petroleum-contaminated ground water | Nuuanu Stream |

| Table A.4 Location and nature of existing (as of 2/13/93) permitted industrial dischargers to Pearl Harbor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| California & Hawaiian Sugar Company Aiea Refinery Aiea, HI 96701 (HI 0000281) | (Position estimated) |  | Sugar refinery process water | Aiea Stream tributary to Pearl Harbor (East Loch) |
| Hawaiian Electric Company Waiau Generating Station 559 Kamehameha Highway Pearl City, HI (HI 0000604) | 21°23 "30' | 157°57"56' | Cooling water 133 mgd (002) 162 mgd (003) 217 mgd (004) Low volume waste 0.350 mgd (003) 0.350 mgd (004) Metal cleaning waste 0.200 mgd (003) 0.200 mgd (004) | Pearl Harbor (East Loch) |
| Hawaiian Cement Halawa Batch Plant (HI 0000558) | 21°22 "44' | 157°54"44' | Treated process water (001) | Halawa Stream |
| Oahu Sugar Company Limited Waipio Peninsula Field Facilities (HI 0020699) | 21° 21"55' | 157° 59 "30' | Excess irrigation water for the purpose of mosquito control | Pearl Harbor (West Loch) |
| Table A.4 Location and nature of existing (as of 2/13/93) permitted industrial dischargers to Pearl Harbor |  |  |  |  |  |  |  |  |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| Navy Public Works Center Building No. 826 Navy Public Works Center Pearl Harbor, HI (HI 1120907) | 21° 21" 12' | 157° 56" 47' | Cooling water from the air compressor facility | Pearl Harbor |
| Pearl City Fuel Annex Naval Supply Center (HI 0000647) | 001 21° 23" 16' 002 21° 23" 12' | 157° 58" 38' 157° 58" 37' | Stormwater runoff and tank draw down water | Kaiapo Canal to Pearl Harbor |
| Pearl Harbor Naval Shipyard U.S. Naval Base Pearl Harbor (HI 0110230) | 001 21° 21" 15' 002 21° 21" 00' 003 21° 20" 45' 004 21° 20" 45' | 157° 57" 30' 157° 57" 30' 157° 58" 00' 157° 58" 00' | Wet sandblasting water, hydroblasting water, cooling water, drydock seepage water and stormwater from Drydocks 1-4 | Pearl Harbor |
| U.S. Navy Submarine Base Floating Drydock AFDM-6 (HI 1121032) | 21° 21" 37' | 157° 56" 51' | Discharge from floating drydock outfall 001 and cooling water from air compressors (002 and 005) and emergency generators (007 and 008) | Pearl Harbor |
| Table A.4 Location and nature of existing (as of 2/13/93) permitted industrial dischargers to Pearl Harbor |  |  |  |  |  |  |  |  |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| Navy Public Works Center Pearl Harbor, Hawaii Building no. 177 (HI 1121105) | 21° 21" 05' | 157° 58" 11' | Discharge non-contact cooling water from the air compressor facility at Building No. 177 | Pearl Harbor |
| Pearl Harbor, Naval Shipyard Dockside Chlorinator Units (HI 1121172) | no precise location |  | Discharge chlorinated, once-through, non-contact, cooling water from its vessel berthed in Drydock Nos. 1,2,3&4 and at piers B1-B21 | Pearl Harbor |
| Pearl Harbor, Naval Submarine Base Dockside Chlorinators (HI 1121181) | no precise location |  | Discharge chlorinated, non-contact, cooling water from vessels berthed at the piers, from S1 to S21 | Pearl Harbor |

| Table A.5 Location and nature of existing (as of 2/13/93) permitted industrial dischargers to the Ala Wai Canal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Discharger (NPDES Permit Number) | Latitude | Longitude | Type of Discharge | Receiving Water |
| Super Hawaii Ala Wai Marine, Ltd. 1651 Ala Moana Blvd. Honolulu, HI 96815 (HI 0020711) | 21°17'26" | 157°50'37" | Wash water from marine railway facility | Ala Wai Canal |
| Yacht Harbor Towers 1600 Ala Moana Honolulu, HI 96815 (HI 0020346) | 21°17 "37' | 157°50"48' | Air conditioning cooling water | Ala Wai Canal |