Appendix A - Model Parameterization

This appendix describes the use of the 20 sampling parameters in the model.  The parameters are divided into three groups: 12 biological, 5 community and 3 water quality/flow parameters.  Most of the biological parameters are based on properties of the microorganism under study.  Community parameters are based on properties of the community and the exposure scenario under study. Water quality/flow parameters are derived from fate and transport modeling data from other Mamala Bay study groups.

Biological Parameters

1. and 2.	T and P

The dependent parameter , the fraction of individuals in state Y who move to state D per day, is a ratio of sampling parameters p, the fraction in state Y that move to state D and T, the incubation period:

 = p / T

3.		Rand

The dependent parameter , the fraction of individuals in state Y who move to state D per day, depends on the value of .  For each unit of time, a fraction of the population in state Y will move to state D, at the rate .  The remaining population in state Y either remains in state Y or moves to state Z.  The sampling parameter Rand was sampled from a uniform distribution of 0 to 1 and determined the fraction of the remaining population in state Y that moves to state Z. Therefore  was defined as follows:

 = Rand  (1 - )

4.		

The parameter  is the fraction of individuals in state Z who move to state X per day. This parameter was based on the reciprocal of the estimated time for an immune person to become susceptible.

5.		

The parameter  is the fraction of individuals in state D who move to state Z per day. This parameter was based on the reciprocal of the estimated duration of symptoms.

6.		

The parameter  is the fraction of individuals in state D who die due to modeled disease per day. It was based on case-fatality rate data for the disease.

7.		a

The parameter a, the number of new susceptible individuals who migrate into the population per day, was set equal to the birth rate of the community, which was assumed to be equal to the global birth rate provided by Raven and Johnson.1

8.		

The parameter , the fraction of individuals who die from natural causes per day, was set equal to the death rate of the community, which was assumed to be equal to the global death rate provided by Raven and Johnson.1

9.		F

The dependent parameter  is the number of pathogen shed per liter of water in the swimming/surfing area per day per infectious/asymptomatic individual. This parameter is a function of three community parameters, Ts, the number of hours spent swimming per day, BM, the fraction of the population that visits the beach each day during a given month of the year, and SM, the fraction of beachgoers that swim or surf each day during a given month of the year; one biological parameter, F, the number of pathogen shed per swimmer per hour; and one water quality/flow parameter, VD, the volume of the swimming/surfing area:

 = (F  BM  SM  Ts)/VD

The parameters BM, SM, Ts, and VD will be further discussed later in this appendix.

The parameter F, the number of pathogens shed per infected swimmer per hour, was defined using water quality data provided by Mamala Bay Study monitoring groups (see Appendix B). This data included pathogen monitoring data from Ala Moana and Waikiki beaches. For each of the selected pathogens, 1,000 Monte Carlo simulations were performed for Scenario 2 (shedding of pathogen by swimmers and surfers but no pathogen from non-shedding sources) at Ala Moana beach. The initial sampling range for F was set from 1 to 1011 pathogen/hour.  This very wide range was selected to span the range of uncertainty and variability associated with this parameter.  For each simulation an average daily pathogen concentration was calculated.

Using the pathogen monitoring data at the beaches, an upper bound concentration of 0.1 pathogen per liter in the swimming area due to shedding by swimmers and surfers was defined (it should be noted that it was assumed that surfers would shed pathogen at the same rate as swimmers). A lower bound of pathogen concentration three orders of magnitude lower than the 0.1 upper bound was selected. Thus a range of pathogen concentration in the swimming area of 0.0001 to 0.1 pathogen per liter due to shedding by swimmers and surfers was defined as being approximately consistent with the beach water quality monitoring data. The approximate range of F values which produced pathogen concentrations within this range were then determined.

Figures A.1 through A.4 are scatter plots of the above Monte Carlo simulation results showing the relationship between average daily pathogen concentration and F. This relationship is approximately linear.  From these figures it was determined that 0.0001 to 0.1 pathogen per liter corresponds to the following ranges for F (in pathogen/hour):

Giardia		105 - 108

Cryptosporidium	106 - 109

Salmonella		105 - 108

Enteroviruses		102 - 105

The above ranges for F were used for the risk modeling.

To further support the use of the above ranges, we estimated the fecal coliform shedding rate using data from a report prepared by the Mamala Bay Study group MB-7 that appears to show a relationship between the number of swimmers at Waikiki Beach and the concentration of fecal coliforms.2 A copy of a page from this report with figures comparing the change over time in the number of people and the concentration of fecal coliforms at a 35-meter length of Waikiki Beach is attached at the end of this appendix. To calculate the approximate fecal coliform shedding rate using this data, we made the assumption that for the time period from 0800 to 1600 all of the increase in fecal coliform concentration was due to the increase in people in the water. For this eight hour time period the approximate areas under the number of people curve and the fecal coliform concentration curve were determined to yield people-hours and coliform concentration-hours. Fecal coliform concentration-hours were then converted to total fecal coliform number-hours using an assumed swimming area volume of 1,400 cubic meters (35 meters length x 2 meters depth x 20 meters width). The ratio of fecal coliform number-hours to people-hours was then calculated and divided by an assumed swimming time of 2 hours per person to yield a shedding rate of approximately 2 X 107 fecal coliforms per person per hour. This rate is in the same order of magnitude as the mean rate of shedding of total coliform by swimmers of 2.3 X 108 indicators per hour swimming determined in a study by Hanes and Fossa.3

The rate of indicator shedding can be related to the rate of pathogen shedding by making the following assumption: the ratio of concentration of pathogen to concentration of indicator organisms in an infected person's feces is equal to the ratio of the rate of shedding of pathogen by an infected person during swimming or surfing to the rate of shedding of indicators during swimming. For total coliforms, the concentration of indicators in feces is approximately 107 to 109 per gram of feces4 (we assume that this range is independent of whether a person is infected or not). For Giardia, infected persons may shed 106 cysts per gram of feces.5 Thus it could be assumed that the shedding rate of Giardia by an infected person would be about 1 to 3 orders of magnitude lower than the shedding rate of indicators. This is generally consistent with the rates of shedding of indicators given above and the 105 to 108 pathogen per hour rate used for Giardia for the modeling.

While the above calculations are very approximate and based on numerous assumptions, they do appear to support the rate of shedding ranges used for Giardia in the risk modeling.

10.		0

The parameter 0, the background transmission rate, is based on the expected background incidence of disease in the community.  The sampled range of 0 is established by running a series of calibration simulations.  First, simulations of the model are run with the value of 0 sampled from an arbitrarily large range.  The incidence rates generated from these calibration simulations are compared with the expected background incidence rate range, and if within this range, are classified as passes.  The sampling range of 0 is adjusted to reflect the distribution of 0 for the passed simulations.  Hence, the sampling range of 0 is narrowed by these calibration simulations to provide a high likelihood of matching the expected background incidence of disease in the community.

11.		Exp

For Giardia, Cryptosporidium and Salmonella, the standard single-hit exponential model to describe the probability of infection when an individual is exposed to a certain dose of pathogen was used.  This model assumes that infection is a two-step process: 1) the host is exposed to a certain number of microorganisms, and 2) a fraction of the microorganisms ingested survive and cause infection.  From these assumptions the probability of an infection resulting from the ingestion of d organisms is:6

Pij = 1 - exp ( -Exp  d)

where d is the dose that an individual is exposed to and Exp is the fraction of ingested organisms that survive.

For enteroviruses, an alternative model called the beta poisson model was used. This model results in a more gradual response to increasing dose by describing Exp in the above dose-response model with a beta probability distribution:

where the parameters Bp and Bp characterize the dose-response curve and d is the dose to which an individual is exposed.

Both of the above functions have been used to calculate the risk of disease due to exposure to various waterborne pathogens, including viral diseases7 and Giardia lamblia.8 The range of the sampled parameters Exp, Bp and Bp were determined using a maximum likelihood estimator (MLE) approach, derived in a previous study on risk assessment of pathogens in drinking water.8 The likelihood equation for the both of the functions was maximized:

1

where pj is the number of infected at each dosage, nj is the number of non-infected at each dosage and Pij is the probability of infection, as described above. Tables A.1 through A.4 present the dose-response data used for each of the selected pathogens. It should be noted that rotavirus dose-response data was used to represent the dose-response relationship for enteroviruses.

For this study, the pathogen dose d was described by the following function:

d = W  I  Ts

where W is the output variable representing the concentration of pathogen in water in the swimming area, I is the volume of water ingested per hour swimming, and Ts represents the amount of time in hours spent swimming or surfing per day. The parameters I and Ts will be described later in the community parameters section.

12.		

The parameter  is the fraction of pathogen in the recreational use water that become non-viable per day. It was based on the reciprocal of the estimated survival time of pathogen in ocean water.

Community Parameters

1.		X0

X0 is both the initial susceptible population and the total population, since it is assumed that all members of the population are susceptible at the beginning of each simulation. A range for X0 was selected based on the 1992 de facto population of the City and County of Honolulu.9

2. and 3.	BM and SM

BM is the fraction of the population that visits the beach each day during a given month of the year. Monthly ranges for this time-varying parameter were estimated using daily attendance records in a lifeguard station log database provided to EOA by PRC. This database included daily attendance counts at lifeguard stations for three years: 1990 - 1992. The daily attendance at a given beach was determined by summing the daily attendance records from the lifeguard stations located at that beach (see Table A-5 and Figure 2.2) and averaging for each month of the year for each of the three years. For a given month of each of the three years, the lowest of the three monthly average attendance values was divided by the upper bound of the population range (932,000). This value was used as the lower bound of the sampled range of BM. The upper bound of BM was calculated by dividing the highest of the three monthly average attendance values by the lower bound of the population range (800,000). Figures A.5 and A.6 give the ranges that BM was sampled from for each month of the simulation for each of the two beaches included in the risk assessment. Appendix C contains this data in tabulated form.

The parameter SM is the fraction of beachgoers that swim or surf each day during a given month of the year. Monthly ranges for this time-varying parameter were estimated using daily records of the number of individuals who swim or surf and daily beach attendance records in the lifeguard database provided to EOA by PRC. The daily fraction of swimmers and surfers at a given beach was determined by summing the daily number of swimmers and surfers recorded at lifeguard stations at that beach (see Table A-5 and Figure 2.2) and dividing by the daily attendance at that beach.  The daily fractions were averaged and the standard deviation calculated for each month of the year for all three years. The sampling range for SM used in the model was determined for a given month of the year by taking the lowest value of the monthly average minus a standard deviation and the highest value of the monthly average plus a standard deviation for that month over the three years.

Figures A.7 and A.8 give the ranges that SM was sampled from for each month of the simulation for each of the two beaches included in the risk assessment. Appendix D contains this data in tabulated form.

The parameter , the infection rate due to ingestion of pathogen in the recreational use water, was calculated by multiplying the dose response equation describing the probability of a susceptible swimmer or surfer becoming infected by BM and SM:

 = (1-exp (-Exp  d)BMSM

to give the probability of a susceptible individual in the population becoming infected due to the ingestion of pathogen in the recreational water.  As described earlier, BM and SM are also used to calculate , reflecting that only the fraction of the population in state Y that swims or surfs sheds pathogen directly into the recreational water.

4.		I

The parameter I is the amount of water ingested per swimmer or surfer per hour.  As shown earlier, it was used in the calculation of the pathogen dose.

5.		Ts

The dependent parameter , the infection rate due to ingestion of pathogen in recreational water, is a function of the number of hours spent swimming or surfing per day, Ts.  As described earlier, these parameters are used in the  transmission function, where dose is dependent on the fraction of time spent swimming or surfing, and to calculate , reflecting that the infected population contributes pathogen directly to the swimming water only when swimming or surfing.

Water Quality/Flow Parameters

1.		WNS

The parameter WNS is the average daily concentration of in the water in the swimming/surfing area from sources other than shedding by swimmers and surfers. This time-varying parameter was set by averaging over each day of the simulation hourly pathogen concentrations modeled by HydroQual. Figures A.9 through A.16 give the values of WNS used for each day of the simulation for each of the two beaches and four microorganisms included in the risk assessment. Appendix E contains this data in tabulated form.

2.		FD

The parameter FD is the flow rate out of water from the swimming/surfing area. FD is used to calculate the number of pathogen leaving the swimming/surfing area due to water flow. This time-varying parameter was set by averaging over each day of the simulation the sum of the hourly flows and dispersions out of the beach segment modeled by HydroQual. Dispersion values from HydroQual were given in a volume per unit time basis and were treated as flows for our purposes. Figures A.17 and A.18 give the values of FD used for each day of the simulation for each of the two beaches included in the risk assessment. Appendix E contains this data in tabulated form.

3.		VD

The parameter VD is the volume of the swimming/surfing area. This time-varying parameter was set by averaging over each day of the simulation hourly beach segment volumes modeled by HydroQual. Figures A.19 and A.20 give the values of VD used for each day of the simulation for each of the two beaches included in the risk assessment. Appendix E contains this data in tabulated form.

VD is used to calculate the number of pathogen leaving the swimming/surfing area due to water flow and is used in the differential equation describing the change over time in the state variable WS, the concentration of pathogen in the swimming/surfing area due to shedding by swimmers and surfers:

dWS/dt = -(FD  WS)/VD + Y - WS

where (FD  WS)/VD is the rate that pathogen shed by swimmers and surfers leave the swimming/surfing area per liter of water in this area, Y is the rate of shedding by infected/asymptomatic individuals swimming or surfing per liter of water in the swimming/surfing area and WS is the rate of pathogen die-off per liter of water in the swimming/surfing area. The following two assumptions were made: 1) pathogen shed by swimmers and surfers completely mix in the swimming/surfing area and 2) pathogen shed by swimmers and surfers that leave the swimming/surfing area do not return to this area.

The state variable W, the concentration of pathogen in the swimming/surfing area water, is the sum of the pathogen concentration from non-shedding and shedding sources:

W = WNS + WS

Appendix A - References

1.	Raven, P.H., and G.B. Johnson, Biology (Times Mirror/Mosby College Publishing, Saint Louis, Missouri, 1989), 2nd ed.

2.	Dobbs, C.D., M.R. Landry and L. Campbell, "Microbial Aspects of Point- and Nonpoint Source Pollution in Mamala Bay: Bather and Light Effects on Sewage-Indicator Bacteria, Mamala Bay Study Project MB-7, (May 31, 1995).

3.	Hanes, N.B., and J. Fossa, "A Quantitative Analysis of the Effects of Bathers in Recreational Water Quality," Proceedings of the Fifth International Water Pollution Research Conference, HA-9/1-9 (1970).

4.	Sanitation and Disease: Health Aspects of Excreta and Wastewater Management, Feacham, R.G. et al. (John Wiley and Sons, New York, N.Y., 1983).

5.	Cooper, R.C., A.W. Olivieri, R.E. Danielson, P.G. Badger, R.C. Spear and S. Selvin, Evaluation of Mililtary Field-Water Quality, Volume 5: Infectious Organisms of Military Concern Associated with Consumption: Assessment of Health Risks and Recommendations for Establishing Related Standards (Lawrence Livermore National Laboratory, 1986).

6.	Haas, C.N., "Estimation of Risk Due to Low Doses of Microorganisms:  A Comparison of Alternative Methodologies," American Journal of Epidemiology, 55, 573-82 (1983).

7.	Haas, C.N., "Effect of Effluent Disinfection on Risks of Viral Disease Transmission Via Recreational Water Exposure," Journal of the Water Pollution Control Federation, 55, 1111-15 (1981).

8.	Regli, S., J.B. Rose, C.N. Haas, and C.P. Gerba, "Modeling the Risk from Giardia and Viruses in Drinking Water," Journal of the American Water Works Association, 83(11), 76-84 (1991).

9.	The State of Hawaii Data Book, 1992, A Statistical Abstract, Hawaii Department of Business, Economic Development and Tourism (Honolulu, Hawaii, 1993).

10.	Rendtorff, R.C., and C.J. Holt, "The Experimental Transmission of Human Intestinal Protozoan Parasites, IV. Attempts to Transmit Entamoeba Coli and Giardia lamblia Cysts by Water," American Journal of Hygiene, 60, 327-338 (1954).

11.	Rendtorff, R.C., "The Experimental Transmission of Human Intestinal Protozoan Parasites, II. Giardia lamblia Cysts Given in Capsules," American Journal of Hygiene, 59, 209-220 (1954).

12.	Dupont, Herbert L. et al, "The Infectivity of Cryptosporidium parvum in Healthy Volunteers," New England Journal of Medicine, 332(13), 855-9 (1995).

13.	Blaser, M.J., and L.S. Newman, "A Review of Human Salmonellosis: I. Infective Dose," Review of Infectious Disease, 4, 1096-1106 (1982).

14.	A Collaborative Report, "A Waterborne Epidemic of Salmonellosis in Riverside, California, 1965, Epidemiologic Aspects," American Journal of Epidemiology, 93, 33-48 (1971).

15.	Feldman, R.E., W.B. Baine, J.L. Nitzkin, M.S. Saslaw, and R.A. Pollard, "Epidemiology of Salmonella typhi Infection in a Migrant Labor Camp in Dade County, Florida," Journal of Infectious Disease, 130, 334-342 (1974).

16.	Naylor, G.R.E., "Incubation Period and Other Features of Foodborne and Waterborne Outbreaks of Typhoid Fever in Relation to Pathogenesis and Genetics of Resistance," Lancet, 1:8329, 864-866 (1983).

17.	Ward, R.L., D.I. Bernstein, E.C. Young, J.R. Sherwood, D.R. Knowlton and G.M. Schiff, "Human Rotavirus Studies in Volunteers: Determination of Infectious Dose and Serological Response to Infection," Journal of Infectious Disease, 154, 871-880 (1986).