Table 2. 	Percentage of various pollutants entering Mamala Bay from four retention basins based upon proportion of the pollutant in the suspended fraction and percent solids retained in each basin. From Stevenson et al., (1995)

| Pollutant | Percent Suspended | Ala Wai Canal (50% Retained) | Pearl Harbor (95% Retention) | Kewalo Basin (95% Retention) | Keehi Lagoon (95% Retention |
| --- | --- | --- | --- | --- | --- |
| BOD | 0 | 100 | 100 | 100 | 100 |
| COD | 0 | 100 | 100 | 100 | 100 |
| TSS | 100 | 50 | 5 | 5 | 5 |
| TDS | 0 | 100 | 100 | 100 | 100 |
| TP | 40 | 80 | 62 | 62 | 62 |
| DP | 0 | 100 | 100 | 100 | 100 |
| TKN | 20 | 90 | 81 | 81 | 81 |
| NO2/NO3 | 50 | 75 | 52 | 52 | 52 |
| Pb | 90 | 15 | 10 | 10 | 10 |
| Cu | 60 | 70 | 43 | 43 | 43 |
| Zn | 40 | 80 | 62 | 62 | 62 |
| Cd | 70 | 65 | 34 | 34 | 34 |

BOD = Biochemical Oxygen Demand; COD = Chemical Oxygen Demand; TSS = Total Suspended Solids; TDS = Total Dissolved Solids; TP = Total Phosphorus; DP = Dissolved Phosphorus; TKN = Total Kjeldahl Nitrogen.