IDENTIFICATION OF STRESSORS OF CONCERN

IN THE MAMALA BAY ECOSYSTEM

1.0  Introduction	Mamala Bay is a bight in the southern shoreline of the Island of Oahu delimited on the east by Diamond Head, and on the west by Barber’s Point (Figure 1). The shoreline of Mamala Bay is cut by three sub-embayments; Pearl Harbor, Kewalo Basin, and Keehi Lagoon/ Honolulu Harbor (Figure 1). Development along the shoreline of Mamala Bay includes metropolitan Honolulu (The City and County of Honolulu) with its attendant manufacturing and service industries, the Pearl Harbor Naval Base, Hickam Air Force Base, Honolulu Harbor, numerous parks and recreation facilities, and the tourist center of Waikiki.

Mamala Bay and its sub-embayments receive the treated wastes from a population of about one million people. The same waters receive the treated wastes generated by the industry and commerce in the City and County of Honolulu, as well as some eight-to-ten million tourists and visitors each year (Hawaii Visitors Bureau Reports, 1988 to 1992).

Mamala Bay and its sub-embayments also receive fresh water runoff and storm drain effluent from within the Mamala Bay watershed (Stevenson et al., 1995; O’Connor and Courtney, 1995), and support human activities known to distribute wastes in the marine environment (in-water and shoreline-based construction, dredging and dredged material disposal, naval operations, marine transportation, and marine recreation; OTA, 1987; White, 1984; El-Swaify, 1991; Freeman et al., 1994). A summary of known discharges to Mamala Bay is found in Stevenson et al. (1995; discharge locations given in Figures 1 and 2).

The combined effect of the human activities listed above may place a burden on the ability of the environment of Mamala Bay to maintain stable populations of marine biota. It is the purpose of this report to evaluate waste discharges in Mamala Bay and its major sub-embayments, and to define those components that may pose a threat to the ecology of the Mamala Bay ecosystem. The report is organized to provide first, an overview of the scope of work, and in subsequent sections, to evaluate the available data on various stressors in the Mamala Bay ecosystem as they relate to potential effects on the Mamala Bay ecosystem.

For the purposes of this report the terms “stressor” and “receptor” will be used to define waste components and environmental components of Mamala Bay, respectively. A “receptor” is a habitat, population, or individual within the environment that requires specific conditions to allow that population to be maintained within sustainable limits. An exception will be made to include individual members of rare and endangered species, since individual members of such populations (as well as their populations) are subject to full and complete protection under state and federal law.

A “stressor” is any material or condition which, when present in sufficient quantities, concentrations, or intensities, acts to impair the ability of a habitat to support populations of endemic species, or acts to impair the ability of an organism or a population of organisms to survive, grow, and reproduce successfully in a given habitat.

While numerous stressors and receptors might be identified in the Mamala Bay system, the discussion in this report is limited to specific categories of stressors and receptors. The categories of stressors discussed here will be physical, chemical and biological stressors associated with point source and nonpoint source waste discharge. The receptors will be the populations comprising reef flat and coral reef communities, and rare and endangered species. Table 1 provides a brief summary of stressors, their possible sources, and the general effects that stressors may have on some receptors in Mamala Bay.

2.0 Scope of Work

The Scope of Work for Project MB-10 was as follows:

1. 	To develop a conceptual model representative of the interactions among and between ecosystem compartments and discharged wastes;

2.	To identify physical, chemical and biological stressors of concern; and

3.	To evaluate stressors of concern for their potential to cause environmental harm in the Mamala Bay ecosystem.

A simple conceptual model of the Mamala Bay ecosystem was developed in order to identify the interrelationships between waste discharges, the pathways of stressor movement among compartments in the ecosystem, and the potential for stressors to accumulate in, and impose effects upon, the biota of Mamala Bay. The conceptual model for Mamala Bay is described and discussed in Section 3, below.

Potential stressors of concern included the pollutants and stressors discharged into Mamala Bay. Stressors discharged into Mamala Bay were identified from Project MB-3 (Pollutant Source Identification; Stevenson et al., 1995). Other stressors for consideration were identified from monitoring studies conducted in Mamala Bay. Yet other stressors not measured in dischargers or in other monitoring efforts were to be evaluated for their likelihood of occurrence in Mamala Bay based upon their common presence in discharges in other major urban/industrial areas.

Stressors in Mamala Bay were evaluated for their potential to cause environmental harm in a two-phase process. The first phase was to evaluate stressor concentrations at the margin of the zone of initial dilution (ZID) for the outfalls from the Sand Island and Honouliuli Waste Water Treatment Plants (WWTPs) and chemical stressors present in other discharges to the bay and its contiguous estuaries. The second Phase was to evaluate the distribution and concentrations of stressors after mixing in Mamala Bay. In Phase I evaluations, stressors were evaluated to determine if concentrations in the zone of initial dilution of the discharge were within two orders of magnitude (factor of 100) of an existing criterion for potential effect (e.g., a water quality standard, or a “Lowest Observed Effect Level” (LOEL) determinable from the literature. If concentrations were predicted to have the potential for an effect, or if there were special circumstances involved, such as discharge to or near bathing beaches or coral reefs, the stressor would be evaluated further in Phase II.

Evaluations in Phase II were carried out using the fate and transport model for Mamala Bay developed by Blumberg and Connolly (1995) as part of the Mamala Bay Study. The fate and transport model incorporates Mamala Bay oceanographic data (SAIC, 1995) and generates a suite of time-varying dilution isopleths for conservative and non-conservative substances discharged into in the Bay. The Phase II analysis considered the concentrations of each of the stressors predicted to occur in the Bay or in outflow from the sub-embayments, included in Phase II, and used toxicological exposure/response information available in the literature to determine the extent to which the stressor might pose a threat to the Mamala Bay ecosystem.

3.0 Candidate Stressors and Receptors of Concern in Mamala Bay

The direct charge for evaluating impacts on Mamala Bay was specific in identifying stressors for consideration as 1) chemicals discharged to the Bay, and 2) those physical and biological conditions that could be altered by waste discharges into Mamala Bay.

3.1 Candidate Stressors of Concern

Stressors included physical effects of discharges (freshwater discharge, suspended sediments, altered temperatures, altered current flows and solids wastes). Candidate chemical stressors included chemicals and chemically definable pollution parameters with the potential to have adverse impacts in Mamala Bay including nutrients, trace metal toxicants, and trace organic toxicants. Candidate biological stressors included bacteria, viruses and parasites in waste discharged to Mamala Bay, as well as some human activities such as the introduction of exotic species and over fishing of finfish and shellfish stocks.

3.2 Candidate Receptors of Concern

The receptors for evaluation in Mamala Bay were defined as coral reefs, coral reef communities, and populations of endangered species as defined by State and federal law. An additional receptor population for consideration was the human population deriving beneficial use from the waters of Mamala Bay. Receptor populations and communities are described in detail in Brock and Am (1995), Grigg (1995), and Laws and Ziemann (1995).

3.2.1 Reefs, Corals, and Coral Reef Communities

The Mamala Bay ecosystem contains numerous reefs and reef flats in the shoal waters immediately offshore from beaches, extending in depth to >20 meters (m). Brief descriptions of the reef communities are presented in Grigg (1995). The reef flats in Mamala Bay comprise coralline limestone and are populated with algae or corals. Pockets of sand and silt are interspersed throughout the reef and reef flat areas. The reefs contain sediment transported to the Bay from land and coralline sand from natural processes of erosion, weathering, and biological activity. Hard coral cover in Mamala Bay is quite restricted; in some areas living hard coral cover on reefs and reef flats is virtually nonexistent, or is restricted to deeper waters (>10m depth).

The fishes and invertebrates that comprise coral reef communities are dependent upon reef structure. In areas that have been strongly affected by shoreline-based and in-water construction, reefs and reef communities no longer exist. Historical activities such as dredging, island construction, filling wetlands, and channelization have done much to alter the habitat and eliminate some communities from some areas. The seaward slopes of existing reefs support reef-building corals and a diverse community of fishes and invertebrates.

3.2.2   Rare, Endangered, and Threatened Species

With the exception of whales that may migrate through Mamala Bay, and an occasional sighting of a Hawaiian monk seal, there is only one rare and endangered aquatic species known to occur in Mamala Bay; the Hawaiian green turtle, Chelonia mydas. The green turtle is endangered throughout its range, at least partly due to the disease green turtle fibropapillomatosis (GTFP). A viral agent is suspected to be the cause of GTFP. Aguirre et al. (1994) have been studying the relationship between chemical pollution and activation of latent viruses responsible for the disease. Other major threats to the green sea turtle are the destruction of suitable nesting habitats throughout its range, the ingestion of debris, as well as poaching of turtles for meat and shell products.

3.2.3   Human Populations Deriving Benefit From Mamala Bay

The human population is a receptor population for stressors in Mamala Bay for several reasons; 1) the aesthetic appeal of the beaches of Mamala Bay is a major attraction for Hawaiians as well as for millions of tourists, 2) the human population makes use of the waters of Mamala Bay for fishing, surfing, diving, and other recreational pursuits, and 3) the human population may be exposed to various pathogens in Mamala Bay waters (Fujioka and Loh, 1995; Hill et al., 1995; Landry et al., 1995). This report addresses humans as receptors only insofar as discharges to Mamala Bay affect aesthetic appeal and potential exposure to chemical toxicants accumulated in fish and shellfish resources. The human health risk associated with pathogens in Mamala Bay waters is addressed by Cooper and Olivieri (1995).

4.0 Ecosystem Assessment Conceptual Model

Wastewater discharges, storm drains, illicit discharges, sediment dredging and dumping, and many other activities are sources of stressors to Mamala Bay (Fujioka and Loh, 1995; Stevenson et al., 1995). The conceptual model for Mamala Bay provides an overview of discharges and potential transport pathways to receptors in the Mamala Bay ecosystem.

4.1 Conceptual Model - Sources of Pollutants to Mamala Bay and Sub-embayments

The generalized conceptual model for the Mamala Bay ecosystem shows three major categories of discharge contributing waste materials to either the open waters of Mamala Bay, or to its tributaries and sub-embayments (Figure 3). The major contributors of stressors to the open waters of Mamala Bay are three ocean outfalls from WWTPs. Wastewater discharges contribute only small loads to sub-embayments. Urban runoff, stormwater runoff, industrial discharges and flow from the drainage basin contribute most of their flows to sub-embayments (Figure 3; Table 2). Only a fraction of the pollutant load entering the sub-embayments actually reaches Mamala Bay (Stevenson et al., 1995; see Table 3). This is because the sub-embayments acts as settling basins for the suspended solids in urban runoff water and storm water, and many pollutants in urban runoff and storm water are either in particulate form, or adsorbed to particulate matter (Figure 4).

Only small quantities of stressors from urban runoff, drainage, industrial discharges and illicit discharges enter Mamala Bay directly (Stevenson et al., 1995). The full load of stressors from the three WWTPs enters directly into Mamala Bay, as well as the dissolved stressors from the sub-embayments and whatever fraction of the particle-associated stressors enters the Bay after particles have settled out in the sub-embayments (Figure 3; Table 3). It is important to note that the Sand Island and Honouliuli WWTP discharges to Mamala Bay are not located in direct proximity to the discharge points of the sub-embayments. The Fort Kamehameha WWTP discharge is located adjacent to the mouth of Pearl Harbor (Figure 1).

4.2  Conceptual Model - Ecosystem Stressor Distribution

Wastes entering Mamala Bay are either in particulate or dissolved form, and distribute exclusively to the water column (Figure 3). Chemical stressors in the dissolved, particulate and deposited phases can change association; dissolved stressors can become adsorbed to particulate matter; or stressors adsorbed to particulate matter in the water column can desorb to the dissolved state. Stressors associated with particulate matter can deposit to the sediments, and stressors in the sedimentary compartment can move to the water column either by desorption/dissolution, or by resuspension of deposited sediments (Figures 3 and 4).

Stressors in the dissolved or particulate phase in the water column can be assimilated by the biota either by direct accumulation from the water (bioconcentration), or by ingestion of stressor-contaminated particles. In either case, stressors that are not eliminated or metabolized by the biota are subject to transport in food webs (O’Connor and Pizza, 1987; Thomann et al., 1989). Stressor accumulation occurs in proportion to the ability of the organism to accumulate the stressor in question and the concentration of the stressor in the “exposure zone” of the organism, i.e., those surfaces of the receptor organism that are exposed to the stressor in a biologically active form. In general, such surfaces are vascularized epithelia at the gill surface and in the digestive tract.

Benthic invertebrates, for example, can assimilate stressors from the water by absorption across respiratory epithelia (gills and equivalent organs) as well as by ingestion of particulate matter with sorbed stressors or ingestion of contaminated phytoplankton and zooplankton (Figure 3). The nektonic community can assimilate stressors in the same way; stressors dissolved in the water column can enter the bodies of fishes directly across the gills, or can be assimilated into the body with stressor-contaminated food. Some stressors in deposited sediments can be assimilated by benthic-feeding fishes when contaminated sediments are eaten coincidentally with food organisms. Needless to say, edible fishes and edible invertebrates (crustaceans, molluscs) serve as the vector for transport of stressors from the marine food web into humans; humans complete the cycle, moving stressors back to the marine ecosystem by elimination of stressors into the wastewater stream (Figure 3).

4.3  Conceptual Model - Ecosystem Stressor Effects

With the exception of chemical stressors that occur in very high concentrations and may physically damage respiratory surfaces in marine biota, stressors impose their effects based upon dose-response relationships. That is, in order for a stressor to elicit an effect it must be present in target tissues long enough, and at concentrations high enough, to cause an impact. Higher concentrations of stressors in water or food will cause more rapid accumulation and will result in high concentrations in tissues in a short period of time, thereby eliciting a rapid, often lethal, (acutely toxic) effect. Lower concentrations of stressors in water or food will accumulate more slowly and may result in a lower stressor concentration in target tissues due to the kinetics of tissue accumulation and elimination. The lower stressor concentrations in tissues arising from slower accumulation may elicit less rapid (chronically toxic) effects. Acute toxicity and chronic toxicity caused by a single chemical stressor may be due to different modes of action in the organism that is affected; nonetheless, the outcome is the same in that the survival, growth, and reproduction of the population may be impaired.

The critical aspects of the flow of chemical stressors in the ecosystem (Figure 3) are that stressors can be accumulated from all compartments (from water, from particulates, and from deposited sediments), and that stressors can be transported throughout the food web, often in increasing concentrations (biomagnification).

The ecosystem conceptual model documents the complexity of the network of relationships in the Mamala Bay ecosystem, and it identifies the fact that once stressors enter the system, each compartment and each receptor, will, eventually, be exposed to that stressor. Further analyses of the dynamics of stressor transport in Mamala Bay are beyond the scope of this report; however, the quantitative distribution of chemical stressors in the ecosystem and rates of transport of stressors among and between compartments can be estimated using various models (Thomann, 1989; EPA, 1989; Blumberg and Connolly, 1995).

4.4  Conceptual Model - Human Population Interactions

The human population is listed as a receptor population for stressors in the Mamala Bay conceptual model for three reasons; 1) the aesthetic appeal of the beaches of Mamala Bay is a major attraction for Hawaiians as well as tourists, 2) the human population makes use of the waters of Mamala Bay for fishing, surfing, diving, and other recreational pursuits, and 3) the human population may be exposed to various pathogens in Mamala Bay waters. The conceptual model also shows that the human population is a contributor to sources of stressors as well (Figure 3; see State of Hawaii, 1990). The human health risk associated with pathogens in Mamala Bay waters is addressed in the water quality Management Plan for the City and County of Honolulu (State of Hawaii, 1990; see also Cooper and Olivieri, 1995; Fujioka and Loh, 1995; Hill et al., 1995; Landry et al., 1995)

5.0  Identification of Stressors Discharged to Mamala Bay

5.1  Physical Stressors

Physical stressors in the Mamala Bay ecosystem are those factors with the potential to physically change the ecosystem to a point at which the physical limits of indigenous organisms are exceeded. Potential physical stressors are all phenomena which, by virtue of human activity, might come to exceed the limits normally experienced by the Mamala Bay community; these physical stressors are:

	fresh water discharge;

	suspended and deposited sediment discharge;

	altered temperature regimes;

	altered current flows; and

	solid waste disposal (flotsam and jetsam).

Suspended solids present a special case in that they are, in one sense, a physical stressor. In another sense, suspended solids are a vector for chemical stressors because many potential toxicants adsorb to and desorb from suspended solids in aquatic environments. The complex pathways and interactions between suspended solids and receptors of concern via numerous pathways in the Mamala Bay ecosystem are depicted in Figure 3.

5.1.1	Freshwater Discharges Into Mamala Bay

Stevenson et al. (1995) estimated that about 443.9 million cubic meters (m3 x 10 6; 360,000 acre-feet) of freshwater enter Mamala Bay and its sub-embayments each year (Table 2). The majority of freshwater flow to Mamala Bay enters the estuaries first, where natural mixing processes take place (Blumberg and Connolly, 1995). A fraction of the total freshwater flow, (about 148 x 106 m3; 120,000 acre-feet) enters Mamala Bay directly, through the discharge pipes of the Honouliuli, Sand Island, and Fort Kamehameha WWTPs.

SAIC (1995) measured salinity and density of Mamala Bay waters. Detailed analyses of salinity and density effects due to freshwater discharge were provided by several projects in the Mamala Bay Study (SAIC, 1995; Roberts et al., 1995). Kimmerer and O’Connor (1995) evaluated historical data to evaluate impacts of WWTP effluent discharge as related to Mamala Bay as a habitat for tropical marine organisms with a low tolerance for exposure to decreased salinities. The effect of the freshwater discharges on salinity profiles in Mamala Bay was easily detected, but freshwater flow from the effluents reduced salinity by no more than a fraction of a part-per-thousand. Such small changes would not have any impact on the receptor organisms (see Grigg, 1995; Brock and Am, 1995).

5.1.2  	Deposited and Suspended Sediment

We were unable to locate data on sediment bed-load transport from major sub-embayments of Mamala Bay. The remainder of this discussion is limited to suspended solids.

Stevenson et al. (1995) estimated the annual load of suspended solids loads to Mamala Bay at about 108 x 106 kg (120,000 tons) per year. About 102 x 10 6 kg (111,000 tons) were discharged to the sub-embayments of Mamala Bay, while the remainder (7.6 x 106 kg; 8,400 tons) was discharged to Mamala Bay through WWTP effluents. Of the 102 million kilograms discharged to Mamala Bay’s sub-embayments, Stevenson et al. (1995) estimated that 95% of the particle load to Pearl Harbor, Keehi Lagoon and Kewalo Basin settled out; 50% of the suspended sediment load to the Ala Wai Canal settled (see Table 3).

Sediment discharge has always been a potential problem for Hawaii’s coastal waters (El Swaify, 1991). Suspended sediment acts as a vector for nutrients, many potentially toxic metals, and organic compounds (O’Connor et al., 1982; Stevenson et al., 1995). Recent studies have shown that suspended sediment in natural watersheds may also act as a vector for soil bacteria (Fujioka and Loh, 1995; Kinnetic Laboratories, 1994a). Suspended solids may pose a problem in tropical marine waters by increasing turbidity, depositing on and impacting reef-building corals (Table 4; see, e.g Pastorok and Bilyard, 1985), and altering habitats in areas of high sediment deposition (Figure 5).

Concentrations of suspended sediments in the main discharges to Mamala Bay (Sand Island and Honouliuli WWTP effluents) ranged between 20 and 75 mg/L (see below; Section 5.2.1). Assuming that initial dilution and mixing decrease the concentration of the suspended solids in the WWTP effluent by a factor of 400 (Roberts et al., 1995), suspended solids discharged to Mamala Bay with waste water effluents would decline rapidly to concentrations of from 0.05 to 0.19 mg/L. Potential increases in suspended solids concentrations from WWTP discharges should be no more than 0.2 mg/L. Such very low increases in suspended solids concentrations will have no impact on fishes and invertebrates in the study area (O’Connor, 1992), and will not impact light penetration in the Mamala Bay system to any measurable extent (Kimmerer and O’Connor, 1995).

The suspended sediments from the Sand Island and Honouliuli WWTP discharges could, however, be ingested by filter-feeding organisms and thereby serve as a vector for sediment-sorbed contaminants in the food chain (Figure 3). Kimmerer and O’Connor (1995) evaluated the extent to which suspended sediments from the Sand Island and Honouliuli WWTP discharges would impact benthic communities in Mamala Bay. They estimated deposition of carbon and sediments from the discharges to the floor of Mamala Bay as about 0.3 g m-2 d-1 (3 mg cm-2 d-1), a value estimated by Pastorok and Bilyard (1985) to pose no threat to coral reef communities (Table 4; Pastorok and Bilyard, 1985; Grigg, 1995).

5.1.3  Solid Waste Disposal In Mamala Bay

Solid wastes disposed in Mamala Bay include flotsam (materials floating in the water) and jetsam (materials discarded in water that may be floating or may settle to the bottom). Flotsam may include materials discarded into the water, materials present in waterways adjacent to the Bay that are carried into the Bay with currents, and materials from the beaches bordering the Bay that may be picked up by advancing tides and transported into the open Bay. Jetsam includes materials lost or thrown overboard from recreational and commercial vessels, materials discarded overboard in sub-embayments and transported on currents out into the Bay, and materials purposefully dumped into the Bay.

The primary problem with solid wastes in Mamala Bay is aesthetic; however, certain items of flotsam and jetsam may have impacts on the ecosystem. For example, plastic bags and containers lost in Mamala Bay may pose a threat to green sea turtles because green sea turtles can mistake billowing, semi-submerged plastic bags and other materials as prey items such as coelenterate medusae (jellyfish), ingest them, and die as a result of an intestinal blockage. In like fashion flashing, silvery pop-tops from cans, or bottle caps can be mistaken by turtles or fishes as potential prey items, ingested, and lead to digestive stress.

5.2  Chemical Stressors

Chemical stressors are those elements and chemical compounds discharged into Mamala Bay by point source and nonpoint source discharges. The State of Hawaii, Department of Health, includes 82 elements, compounds, and chemical classes in their list of chemicals that may pose a hazard to aquatic life and, therefore, are limited to certain concentrations by standards for water quality (Table 5). Other “chemicals” for which the State of Hawaii maintains standards are the more conventional water quality measurements of total suspended solids (TSS; see above, section 5.1.2) nutrients, biochemical oxygen demand (BOD), chemical oxygen demand (COD), and oil and grease (State of Hawaii, Department of Health, 1990).

Data on the water column concentrations of most chemical stressors in Mamala Bay are not available. The identity of potential stressors of concern came from four sources:

1. 	Discharge Data: Self-monitoring reports from WWTPs and other dischargers were examined to determine concentrations of chemical stressors in effluents for the five-year interval from 1989 through the end of 1993.

2.	Nonpoint Source Discharge Data: Estimated nonpoint source loads (from Stevenson et al., 1995) were used to predict concentrations of chemical stressors in sub-embayments, and to calculate stressor concentrations in outflows from sub-embayments into Mamala Bay.

3.	Environmental Monitoring Data: Environmental monitoring data were evaluated to determine the identity of chemical stressors known, or suspected to be present, in the waters of Mamala Bay. The primary monitoring data used in this evaluation were from the National Status and Trends Program, National Mussel Watch (NOAA, 1989; 1995a, 1995b), which maintained two stations in Mamala Bay (Barber’s Point and Honolulu Harbor).

4. 	Discharge Data from Other Sources: Historical discharge data from two other urban/industrial centers (New York and San Francisco) were examined to determine the roster of common chemical stressors detected in monitoring programs. The chemical lists were evaluated in order to determine whether contaminants common in other discharges might be present in discharges to Mamala Bay, and whether those chemical stressors could threaten the Mamala Bay ecosystem.

5.2.1  Mamala Bay Discharge Data

5.2.1.1 WWTP Discharges to the Open Waters of Mamala Bay

Chemical data from WWTP effluents are limited by the sensitivity of the analytical methods used to satisfy federal and state NPDES monitoring requirements. Concentration data from effluent waters are restricted to “conventional” pollutants (TSS, BOD, COD, Oil and Grease), and a small number of chemical stressors that are predominantly metals (Stevenson et al., 1995). Effluent data from WWTPs contain primarily “ND” (not detected) qualifiers for most measurements of metals and virtually all of the organic compounds. “ND” values are problematic for calculation of loading values and for making assessments of potential acute or chronic toxicity (see, e.g. D’Elia et al., 1989; Davis et al., 1992).

Stevenson et al. (1995) compensated for “ND” pollutant concentrations in NPDES self-monitoring reports by assigning a value equal to one-half the detection limit to ND values; all loading data published by Stevenson et al. (1995), and the concentration data herein were derived from databases that included such corrections. While such data manipulation may pose a problem as to the accuracy of the concentrations reported (our calculated concentrations are probably overestimated), it compensates well for the use of insensitive analytical methods, and assures that the concentration estimates reported are conservative.

Calculated mean pollutant concentrations in the effluents from the Sand Island, Honouliuli, and Fort Kamehameha WWTPs for the period 1989 through 1993 are presented in Table 6, along with existing Hawaii Water Quality Standard values for those pollutants having water quality standards. Effluent values in Table 6 are for all pollutants that had at least one measurement above the detection limit. With the exception of measurements for BOD, Oil and Grease, TSS, and free chloride ion (Fort Kamehameha only), there were nondetect values in each data set.

Stressors for Phase I evaluations were identified from Table 6 by comparing measured effluent concentrations with existing Hawaii saltwater standards. Ammonia, cadmium, chloride ion, chromium, copper, and zinc were identified as present in effluent water at concentrations that were within a factor of 10 of existing standards or, in the case of copper, were within a factor of 10 of effluent limitations assigned to copper at other WWTPs. These chemicals were assigned to Phase I evaluations as potential stressors of concern.

5.2.1.2  Discharge of Chemical Stressors to Mamala Bay Waters from Sub-Embayments

Evaluation of point source discharges of chemical stressors to the sub-embayments revealed that the vast majority of measurements were “nondetects.” Measured concentrations of chemical stressors in discharges to sub-embayments were primarily for Cu (Hawaiian Electric - Honolulu and Waiau Stations; Honolulu Shipyard) and Pb (Shell Oil Honolulu, Pacific Resources, Chevron USA). Water samples from the Pacific Resources effluent were also contained toluene and xylene at low concentrations. The discharge from the Aiaea Hawaiian Sugar Refinery contained ammonium. In all cases the measured concentrations of chemical stressors were low, and, with the exception of ammonium at Hawaiian Sugar, positive measurements were infrequent (Kinnetic Laboratories MB-3 Data Base, Kinnetic Laboratories, Santa Cruz, CA, 1995).

Nonpoint source discharges were shown by Stevenson et al. (1995) to contribute large loads of chemical stressors to the waters of the Ala Wai Canal, Keehi Lagoon, Kewalo Basin, and Pearl Harbor. We calculated the concentrations of stressors entering the sub-embayments of Mamala Bay with runoff water, as well as the annual average concentrations of chemical stressors in water flowing out from the sub-embayments to the near shore waters of Mamala Bay under runoff conditions (Table 7). These estimates are based upon numerous assumptions, and the potential margin of error was large; therefore, we cross-checked and validated the calculated metals concentrations (Table 7) against raw stormwater metals data and primary-treated (settling only) stormwater metals data available to us from studies carried out in other US Cities. The comparison (Table 8) shows that the metals concentrations estimated in sub-embayment outflows were well within the range of metals concentrations measured in raw runoff water in Sacramento, CA, but lower than metals concentrations in stormwater discharged from the North Point Treatment Plant in the City of San Francisco; the North Point Plant is a primary treatment plant that receives only stormwater runoff (City and County of San Francisco Planning Department, 1994). We concluded that the estimates of metals concentrations in the outflow from sub-embayments to Mamala Bay were a fair description, within the same order of magnitude of what the actual stormwater metals concentrations might be.

Metals concentrations in the outflow from sub-embayments that were in the same order of magnitude as existing Hawaii Water Quality Standards were identified as potential stressors of concern and included in further Phase I evaluations. As can be seen from the comparison of sub-embayment outflow concentrations with Hawaii water quality standards (Table 9), the calculated concentrations of copper in the sub-embayment outflow were in the same order of magnitude as copper standards for San Francisco Bay, and the concentration of zinc in the runoff outflow from Kewalo Basin exceeded the Hawaii acute standard. Zinc concentrations in runoff outflows from all four sub-embayments are within a factor of three of the Hawaii acute standard and the Hawaii chronic standard (Table 9). The results of these analyses do not justify the inclusion of lead in the list of chemical stressors of concern in Mamala Bay.

5.2.2  Environmental Monitoring Data

There is a dearth of environmental monitoring data for toxic and potentially toxic chemicals in the water column of the Mamala Bay ecosystem. Sediment monitoring programs have been directed primarily toward characterizing the concentrations of toxic and potentially toxic chemicals in the sub-embayments of Mamala Bay (Pearl Harbor, Keehi Lagoon, Kewalo Basin, and Ala Wai Canal). The NOAA National Status and Trends Program (NOAA, 1995b) has sediment chemistry data from two sites adjacent to Mamala Bay (the Barber’s Point Boat Basin and Keehi Lagoon/Honolulu Harbor).

Harding Lawson reported on a study of some chemical stressors (selenium, cyanide and PCBs) in marine biota sampled in Mamala Bay on the reef flat off Kakaako (immediately west of Kewalo Basin)(AECOS, 1990). Two samples of achilles tang (Acanthurus achilles) from Kakaako contained selenium (0.03 and 0.02 mg/kg from the east and middle portions of the reef flat, respectively). A single sample of aholehole (Kuhlia sandvicensis)contained PCBs (Aroclor 1260) at a concentration of 260 ng/g (wet weight).

Most of the monitoring data of use in an evaluation of chemical stressors in the Mamala Bay ecosystem come from the US Department of Commerce, National Oceanic and Atmospheric Administration National Status and Trends Program and National Mussel Watch Program (NOAA, 1989; 1995a). Important cognate data come from stormwater monitoring efforts and monitoring carried out in sub-embayments (State of Hawaii, Department of Health, 1978, 1980)

National Mussel Watch is based on the concept that bivalve molluscs integrate long-term exposures to bioavailable chemical stressors in their environment by accumulating residues of the stressor and storing it in tissue (Phillips, 1980; O’Connor, 1992; SFEI, 1995). Bivalves accumulate contaminants from both ingested food and from the water passing over the gills. Bivalve molluscs also have slow rates of turnover for chemical stressors; metals are distributed and removed from body compartments slowly, and the rate of metabolism of organic compounds, such as DDTs and PAHs, is slow, by comparison to other aquatic biota such as fishes (Phillips, 1980; SFEI, 1995). The use of bivalve molluscs as time-integrating biological sentinel organisms for pollution studies has been successfully applied in many environments, worldwide (Phillips, 1980; Claisse, 1989; SFEI, 1995). By sampling appropriate bivalve molluscs, NMW has built a consistent data base documenting trends in contamination (NOAA, 1989, 1995a; O’Connor, 1992; see also Claisse, 1989).

The National Mussel Watch Program maintained two stations in waters adjacent to Mamala Bay. One station was located in the boat harbor at Barber’s Point, at the western end of Mamala Bay. The other was located in Honolulu Harbor (Keehi Lagoon). Neither NMW station can be considered representative of the open waters of Mamala Bay; however, the sites represent fairly the water bodies that receive chemical stressor input from the drainage basin as well as from a variety of marine activities.

Native oysters (Ostrea sandvicensis) were sampled from these sites from 1986 through 1988, and from 1990 through 1992. Tissue samples from the oysters were analyzed for a suite of metals, pesticides, polychlorinated biphenyls (PCBs), polycyclic aromatic hydrocarbons (PAHs), and organotins. The analytes measured in 1986 and 1987 (Table 10) comprise the baseline data; analytes measured in succeeding years included additional stressors. Two “classes” of PAHs were represented in summary documents; low molecular weight PAHs (LMWPAH; all 2- and 3-ring PAHs) and high molecular weight PAHs (HMWPAH; all 4- and 5-ring PAHs). All data discussed in this report were prepared from the NOAA NMW database made available through Dr. Tom O’Connor of NOAA, ORCA.

Analyses of tissue composites from about 30 oysters sampled at each site in each year showed that they contained measurable concentrations of metals and toxic organic contaminants (Tables 11 and 12). Accumulation of silver, chromium, cadmium, selenium and elemental tin apparently declined in oysters from Honolulu Harbor/Keehi Lagoon between 1986 and 1992 (Figure 6). Trends in lead, nickel, copper and zinc apparently increased at the same time (Figure 6B and C). Similar trends were apparent in oyster tissue data from samples taken at Barber’s Point (Table 11; Figure 7). Organotins were measured only in 1990, 1991, and 1992. Monobutyltin apparently declined in concentration over that time period, whereas dibutyl and tributyl tins were accumulated at much higher concentrations in 1992 than in 1990 or 1991 (Figure 8). Organotin accumulation in oysters was due to residues from anti-fouling paints (Salazar, 1986; Henderson, 1986; Huggett et al., 1992).

The accumulation of pesticides, PCBs and PAHs in oyster samples from Barber’s Point and Honolulu Harbor (Table 12) was due to the transport of these compounds from agricultural, industrial, and urban sources, as well as direct input by commercial and recreational vessels to the basins where the samples were taken (especially for the PAHs). PCB accumulation in oyster tissues declined at both sites between 1986 and 1992 (Figure 9). The decline in PCB accumulation in oysters from Honolulu Harbor was especially dramatic, falling from almost 600 to about 80 ng/g (dry weight). Accumulation of HMWPAH and LMWPAH apparently declined in oysters from the Barber’s Point site (Table 12; Figure 10).  HMWPAH accumulation by oysters in Honolulu Harbor appeared to increase between 1987 and 1992, while LMWPAH remained about the same (Figure 10).

Pesticide accumulation in oysters from Barber’s Point declined (Table 12), and trends for chlordane accumulation reflected the banning of the use of chlordane (Figure 11). Trends in pesticide accumulation in oysters sampled from Honolulu Harbor were erratic (Table 12), showing apparent increases for total DDTs, chlordane, and dieldrin.

The Mussel Watch data (NOAA, 1995a) are corroborated by bioaccumulation measurements made during surveys carried out by the Hawaii Department of Health in the Ala Wai Canal and other water bodies adjacent to Mamala Bay (1978; and other years). In those surveys, samples of tissue from two crab species (the Hawaiian crab Thalamita crenata, and the blue claw crab, Podophthalmus vigil) contained substantial concentrations of copper, lead, nickel and zinc, as well as -chlordane and trans-nonachlor (a contaminant in, and a component of, chlordane). Interestingly, the Hawaii Department of Health (1978) measured sediments from the same stations in the Ala Wai Canal, and were able to measure not only -chlordane and trans-nonachlor, but also -chlordane, dieldrin, and PCBs. None of the latter three compounds were measured in Hawaiian crabs and blue claw crabs from the Ala Wai Canal samples.

Bioaccumulation of chemical stressors in the tissues of oysters in Mamala Bay is not evidence for an effect due to the chemicals in question. Rather, bioaccumulation demonstrates the availability to the biota of chemical stressors that are neither detected nor detectable in WWTP effluents or in the water column of Mamala Bay and its sub-embayments. What is more important, the bioaccumulation data show that the chemical stressors present in the waters adjacent to Mamala Bay are, in large part, decreasing. However, it must be pointed out that many chemical stressors are present at concentrations sufficient for residues to accumulate in oyster tissues. The substantial declines in organic chemical stressors in oysters at Barber’s Point (Table 12) suggest that nonpoint source control of land-based and water-based sources of agricultural and industrial chemicals is successful (O’Connor and Courtney, 1995).

The following chemical stressors from biomonitoring studies in Mamala Bay were added to the list of chemical stressors of concern that were judged to be worthy of further evaluation: arsenic, mercury, selenium, silver, PCBs, DDTs, Chlordane, dieldrin, LMWPAH and HMWPAH, and organotins.

5.2.3  Discharge Data From Other Sources

We examined two cases in which estuarine or coastal environments were studied in great detail as a means to determine the presence and concentration of chemical stressors deriving from the discharge of wastewater and storm runoff. These were 1) New York Harbor and the New York Bight (Squibb et al., 1990; EPA, 1990) and 2) the San Francisco Bay Estuary (Phillips, 1988; Gunther et al., 1988; Davis et al., 1991). Each of these water bodies receives wastewater and runoff from nearby urban-industrial areas as well as agricultural runoff from upstream sources. The chemical stressors for the New York Harbor/New York Bight are listed in Table 13.

Phillips (1988) generated a list of toxic substances (=chemical stressors) of concern in San Francisco Bay based upon the probable use of the chemical stressor in the watershed. That list has been incorporated into the framework of the San Francisco Estuary Project (Davis et al., 1991; Monroe and Kelly, 1992) as the list of contaminants of concern in San Francisco Bay (Table 14).

Both lists from the New York-New Jersey estuary and in San Francisco Bay identify several metals as stressors of concern that were either not detected in WWTP discharges to Mamala Bay, were present in very low concentrations in WWTP discharges and outflows from sub-embayments, or not often detected in oyster samples from Barber’s Point and Keehi Lagoon / Honolulu Harbor. These are lead, nickel, and silver. The fact that these stressors were measured in low concentrations (Pb), or were not identified in Mamala Bay analyses, leads us to the conclusion that they should not be included in further Phase I evaluations.

The lists of organic chemical stressors of concern in New York and in San Francisco include the organic chemical stressors identified in bioaccumulation studies in Mamala Bay (Tables 12 to 14), as well as several other organic chemicals known to have impacts on aquatic biota, malathion and parathion, the phthalate esters, and the dioxins (TCDDs).

Malathion (O-O-dimethyl dithiophosphate) and parathion (O-O-diethyl-O-p nitrophenyl thiophosphate) were listed by Phillips (1988; Davis et al., 1991) primarily because they are widely used for pest control in rice-growing areas in the Sacramento-San Joaquin drainage basin. We have no data on the volume of use of either pesticide in the Mamala Bay drainage basin; however, malathion has tolerances listed for pre-harvest pineapples and mangoes, (40 CFR 180.111) and parathion has tolerances listed for pineapples, mangoes, and sugarcane (40 CFR 180.121).

Malathion and Parathion

Both malathion and parathion are thiophosphate pesticides acting as cholinesterase inhibitors. They are moderately persistent, and are subject to breakdown in soil and water. Whatever residues occur in surface runoff and in sub-embayments would be likely to be metabolized by the high level of bacterial activity present in the system, resulting in stressor concentrations that pose no potential harm to marine biota. Malathion and parathion will not be included in any further Phase I evaluations.

Phthalate Esters

The phthalates are ubiquitous in the global environment, widely used in the plastics and synthetics industries, and enter the aquatic environment with industrial waste, WWTP discharge, and atmospheric deposition (Thuren, 1988). Phthalate esters are common components of urban runoff, and stormwater runoff from suburban and rural areas (Kinnetic Laboratories, 1994b). Their potential to cause chronic effects at relatively low concentration (below 500 g/L) is well known (e.g., Thuren, 1988). The phthalate esters were not detected in wastewater effluents in Mamala Bay because detection limits were too high. They were not detected in NMW bioaccumulation studies because they were not analyzed (NOAA, 1989, 1995a). Based upon the available data, we concluded that phthalate esters should not be considered as chemical stressors at this time; however, future monitoring should include efforts to measure phthalate esters in the Mamala Bay system so a complete evaluation can be made when an adequate data base is available.

Dioxins

The polychlorinated dibenzodioxins (PCDDs) are potential stressors of concern for Mamala Bay. The PCDDs comprise a group of congeners with varying chlorine substitutions at various positions on the dioxin molecule. The most hazardous are those containing four chlorine molecules: the tetrachlorodibenzo-p-dioxins (TCDDs). One TCDD (2,3,7,8-tetrachlorodibenzo-p-dioxin) has been identified as one of the most toxic organic chemicals ever produced and released by humans (Safe, 1982; EPA Science Advisory Board, 1994).

Dioxins have been listed as chemical stressors of concern for many water bodies. Dioxins were not analyzed for in the NOAA NMW program (1989, 1995a, 1995b), or in any monitoring program in the Hawaiian Islands. Dioxins typically have their origin as by-products and contaminants of chemical manufacturing processes, as contaminants in certain herbicides (e.g., 2,4,5-trichlorophenoxyacetic acid), and as pyrogenic by-products of incomplete incineration of wastes containing organic matter and chlorine, or other organochlorine compounds (EPA, 1987; Rappe et al., 1986). Stack gases of municipal incinerators and pulp mills may be assumed to contain some dioxins.

The environmental effects of dioxins are: bioaccumulation and transport to other portions of the food web (Rubinstein et al., 1989; Opperhuizen and Sijm, 1990); depression of the immune system; interference with reproduction; and action as a mimic of natural sex steroids (Lindstrom-Seppa and Oikari, 1989). Long-term impacts of dioxins in natural ecosystems range from elimination of certain fisheries products from consumption due to contamination to elimination of entire populations from water bodies due to dioxin-induced reproductive failure (EPA Science Advisory Board, 1994).

There is no evidence to suggest that dioxins are a stressor of concern in the Mamala Bay ecosystem. First, there is a minimum of chemical manufacturing in the City and County of Honolulu, and no plants in the drainage basin manufacture the herbicides likely to contain dioxins as a contaminant. Second, there are no paper mills operating in the Mamala Bay watershed, and paper mill stack gases have been identified as a significant source of dioxins (in stack gases) in previous studies. Third, municipal solid wastes are not incinerated in the Mamala Bay watershed, so there is a low probability that waste incineration would add dioxins to the air. However, these reasons, by themselves, do not eliminate consideration of dioxins as potential chemical stressors of concern in Mamala Bay. While it would be premature to include dioxins on the list of chemical stressors for Mamala Bay, it would be prudent to include the dioxins in future analyses of sediments and biota, especially in the contaminated waters of Pearl Harbor.

Diazinon

One common chemical stressor that has not, until recently, appeared on lists of potential hazards is the pesticide diazinon (O-O-diethyl O-[6-methyl-2-(1-methylethyl)-4-pyrimidinyl] phosphorothioate). Diazinon is listed with tolerances relative to use in culture of both pineapples and sugarcane (40 CFR 180.153). Diazinon is one of the most widely used pesticides on the market; it is sold commercially for the control of lawn and garden pests, and is marketed for domestic use. Toxicity Identification Evaluations (TIEs; EPA 1988a, 1988b) conducted on toxic storm water samples in California and in other locations across the US have identified diazinon as a primary toxicant of concern (Amato et al., 1992; V. Connor, personal communication).

Analysis for diazinon and the other thiophosphate pesticides has not been included in studies aimed at characterizing the chemical content of urban and suburban runoff water. Since diazinon may be used in pineapple and sugar cane culture, and since it is marketed in Hawaii for the control of lawn and garden pests, and since pest control is a matter of significance in tropical environments like Hawaii, it is likely that diazinon use in the Mamala Bay watershed is quite high. Based upon data available from studies of stormwater runoff in other urban/suburban centers we include diazinon in the list of chemical stressors to be evaluated in Phase I and we recommend that analysis for diazinon be included in any future monitoring programs undertaken.

5.3  Biological Stressors

Biological stressors in Mamala Bay may be divided into two areas for consideration; 1) bacterial viral, and protozoan pathogens in the Bay, and 2) the introduction of exotic species to the Mamala Bay ecosystem.

5.3.1 Pathogenic Bacteria, Viruses and Protozoans

Pathogens may enter the Bay via any of the following routes:

	discharge to the Bay from WWTP discharges;

	discharge to the Bay with storm water runoff;

	discharge to the Bay from illegal discharges from vessels; and

	discharge to the Bay by “shedding” from human populations in the water.

Pathogenic bacteria, viruses and protozoans pose a potential problem to biological resources in Mamala Bay only under circumstances where the pathogens are infectious to fishes, invertebrates, or plants. To the best of our knowledge there are no data showing that pathogens discharged to Mamala Bay infect fishes, invertebrates, or plants residing in the Bay. 	Some of the pathogens discharged to the Bay pose a threat to the human population. These considerations are not treated in this report; rather they are the topic of other reports from the Mamala Bay Project, some dealing with the identification of pathogens in Mamala Bay waters (Dobbs et al., 1995; Gerba et al., 1995; Fujioka and Loh, 1995; Paul and Rose, 1995), and others providing a human health risk assessment for Mamala Bay based upon mathematical models of disease transmission (Cooper and Olivieri, 1995).

5.3.2 Exotic Species

Numerous exotic species have undoubtedly been introduced to Mamala Bay by natural processes and by accident (see Carlton’s 1979 discussion of vectors for exotic species transport). The invasion of Hawaiian waters by exotic species that follow natural routes of dispersal is limited because of the extreme isolation of the Hawaiian Islands from any mainland, and the lack of “stepping-stone” land forms affording invading species suitable locations for dispersal to the Hawaiian Islands (Hobson, 1984). Relatively few accidental introductions have resulted in large populations of exotics, presumably due to the rather inhospitable environment afforded by the oligotrophic waters of the Hawaiian archipelago. Another factor may be that major shipping to the Hawaiian Islands originates in temperate estuarine climates, and marine and estuarine species invading Hawaii from temperate estuaries are ill-adapted to the tropical climate.

Several purposeful introductions of commercially important food fish to the Hawaiian Islands have taken place since the 1950s (Oda and Parrish, 1981) when 11 species of snappers and groupers were introduced. Only the blueline snapper (taape; Lutjanus kasmira) has become abundant, spreading to many of the Hawaiian Islands, including Oahu. Fishermen complain that blueline snapper are so abundant as to displace native fishes. Ecologists suggest that the reason that more blue snapper are caught is related to their more aggressive behavior and reaction to bait rather than their displacement of native species (Oda and Parrish, 1981). Studies investigating competition between the taape and native fishes show minimum overlap in diet (Oda and Parrish, 1981). It is probably true that blueline snapper have not caused any change in the abundance or distribution of native species.

6.0 Phase I Evaluation of Stressors

The following chemicals will be included in Phase I analysis; i.e., the concentrations of each of these chemicals should be evaluated to determine if they pose a potential threat to the ecological health of the Mamala Bay ecosystem after discharge and initial mixing:

	ammonia				dieldrin

	arsenic				mercury

	cadmium				PAHs (LMWPAHs and HMWPAHs)

	chloride ion				organotins

	chlordane				PCBs

	chromium				selenium

	copper					silver

	diazinon				zinc

	DDTs

6.1 Phase I Evaluation of Chemical Stressors in WWTP Effluents

Phase I evaluation of potential chemical stressors was carried out by comparing Hawaii water quality standard values with concentrations of pollutants in effluent water after initial dilution in Mamala Bay. A conservative initial dilution value of 400 was estimated by Roberts (1995), and was applied to the concentrations of chemical stressors in effluent water. If stressor concentrations were within two orders of magnitude of an applicable Hawaii Water quality standard after initial dilution, the stressors would be included in Phase II of the stressor evaluation. Although several chemical stressors measured in the WWTP effluents were at concentrations similar to Hawaii Water quality standards (Cd, Cr, Pb, Zn), stressor concentrations after initial dilution were all below the criterion (one one-hundredth of Hawaii water quality standards). The stressors that approached closest to Hawaii Water quality standards concentrations were Cr (0.1% of the water quality standards after dilution) and Zn (up to 0.3% of the Water quality standards after dilution)(Table 15).

6.2  Phase I Evaluation of Chemical Stressors in Outflows From Sub-embayments

A hydrodynamic model predictive of mixing processes at the point of outflow from sub-embayments was not available at the time of this writing. We have performed Phase I evaluations, therefore, using a factor of 50 as representative of dilution between sites of outflow and receptor populations (i.e., areas that may be routinely exposed to outflow waters).

Applying the criterion, we determined that concentrations of cadmium, copper, and zinc in outflow waters from the sub-embayments of Mamala Bay (Table 16) could pose a problem for receptor populations in the Bay, and should be moved forward for evaluation in Phase II.

6.3    Phase I Evaluation of Chemical Stressors in Biomonitoring Studies

Bioaccumulation of chemical stressors demonstrates sufficient concentrations of chemical stressors to result in increased body burdens. The metals in Table 11 that showed no apparent decreasing trends were moved forward to evaluation in Phase II (nickel, mercury, and tin). The organic chemical stressors chlordane, dieldrin, and HMWPAH were moved Phase II. The decision to move chlordane to Phase II was due to the lack of a downward trend in chlordane bioaccumulation in Honolulu Harbor (Figure 11). The decision to move dieldrin to Phase II was due to a possible increasing trend in accumulation in Honolulu Harbor. The decision to move HMWPAH to Phase II was due to high concentrations of HMWPAH accumulated in Honolulu Harbor (>5,000 ng/g; 1990, 1991, and 1992; Table 12).

6.4    Phase I Evaluation of Chemical Stressors Identified From Other Studies

The toxicological importance of diazinon in Mamala Bay has not been addressed in any studies in Mamala Bay. Because of the high probability of occurrence in the Mamala Bay watershed, diazinon will be included in Phase II evaluations.

7.0 Phase II Evaluations of Candidate Chemical Stressors

Phase II evaluation of chemical stressors was carried out by examining each stressor relative to its concentration in Mamala Bay waters after dilution, and the potential impact of the stressor using LOEL concentrations as determined from the available literature. Chemical stressors for evaluation in Phase II are those listed in Table 17.

7.1  Phase II - Metals Evaluations

Phase II evaluation for metals was limited to a description of the element and documented effects on aquatic biota at concentrations reflecting those calculated to occur in Mamala Bay waters. A description of the effects of chemical stressors on species of importance to the Mamala Bay ecosystem is available in O’Connor and Dorsey, 1995.

7.1.1 Cadmium

Cadmium can be acutely toxic to marine biota, but only at very high concentrations. Cadmium accumulated in tissues, especially in kidneys and hepatopancreas, can cause chronic effects. Cadmium concentrations in oysters from Barber’s Point and Honolulu harbor (Table 11) were measurable, but were in the range of concentrations reported from areas considered to be “uncontaminated” (Friberg et al., 1979). Cadmium bioaccumulation in saline marine waters is generally much lower than in estuarine waters (O’Connor et al., 1982). Bioaccumulation of cadmium is greatest in the hepatopancreas tissue of crustaceans and bivalves (Friberg et al., 1979). Although cadmium is not an essential element, it is regulated efficiently by most biota, usually by the same mechanisms involved in zinc regulation.

Ringwood (1993) demonstrated age-specific sensitivity of the Hawaiian bivalve Isognomon californicum exposed to cadmium. The 48-h Cd LC50s were as follows: 520 g/L for embryos, 530 g/L for 3-day veligers, 580 g/L for 10-day veligers, 3,800 g/L for 24-day veligers, 3,320 g/L for pediveligers, 16,520 g/L for spat, and 27,500 g/L for adults.  When the life stages were exposed to 2,000 g/L 115Cd for 24 hours, the relative rate of cadmium accumulation followed the order embryos> pediveligers>veligers>adults. The lowest effective concentrations of Cd that we found in the literature (LOEL; Table 18) showed that Cd concentrations as low as 0.1 g/L could have a negative impact on crab zoea development and survival. Cd concentrations of 0.5 g/L caused toxicity to purple sea urchin (Strongylocentrotus purpuratus) larvae. A depression of respiration was seen by Dawson et al. (1977 in EPA, 1994) wherein a 30-day exposure of striped bass (Morone saxatilis) to cadmium concentrations as low as 0.5 to 5.0 g/L reduced oxygen consumption.

The greatest calculated Cd concentrations in sub-embayment outflow water were 0.01 g/L. The very low cadmium concentrations in diluted WWTP effluent water and sub-embayment outflow water (from 0.003 to 0.01 g/L; Table 16) was judged not to pose a hazard to the biota of Mamala Bay; we conclude from these data that the cadmium bioaccumulation recorded by NOAA (1989, 1995a) in Mamala Bay are in no way suggestive of a cadmium contamination problem, and that cadmium should not be included among the chemical stressors of concern in Mamala Bay

7.1.2  Copper

The calculated concentrations of copper in outflows from sub-embayments to Mamala Bay were within two orders of magnitude of water quality objectives established for water bodies other than Mamala Bay (4.9 g/L; San Francisco Bay). Copper concentrations in sediments at Mussel Watch sites were between 25 and 36 g/g(dry weight) in 1986 and 1987 (NOAA 1995b).

Copper can have negative impacts on reef organisms at very low concentrations. Howard et al. (1986) investigated the effects of total copper, as Cu (II) on respiration, nutrient uptake and release, pigments, and zooxanthellae expulsion rate in coral branches of Montipora verrucosa from Kaneohe Bay. The 96-hour LC50 was determined to be 48 g/L Cu (II), well above the estimated concentrations in the open waters of Mamala Bay (0.045 to 0.082 g/L), and above the estimated concentrations of copper discharged with outflows from the sub-embayments of Mamala Bay prior to dilution (5.4 to 14.2 g/L; see Table 9).

Exposure of Montipora branch tips to concentrations of Cu (II) ranging from 10 - 50 g/L produced a visible gradation of bleaching and counted numbers of expelled zooxanthellae proportional to the concentration of copper. Such concentrations have been calculated to exist in the outflow waters of Mamala Bay’s sub-embayments (Table 9). A 96-hour bioassay (natural seawater, 27C) for copper was conducted on coral planula larvae (planulae) of the coral Pocillopora damicornis by Esquivel (1986). Probit analysis yielded a 24-hour LC50 of 114 g/L added Cu, a 48-hour LC50 of 87 g/L added Cu, and a 96-hour LC50 of 57 g/L added Cu.

No sublethal effects of 48 hours exposure to 10, 18, and 32 g/L added Cu were observed for the Pocillopora planulae, in contrast to the effects seen in adult Montipora after only one hour of exposure. It is generally suggested that the planula may be the more resistant life stage of the coral.

Heyward (1988) studied the inhibitory effects of copper and zinc sulphates on the fertilization success of two massive species of corals, Favites chinensis, and Platygyra ryukyuensis, from Okinawa. Copper totally inhibited fertilization in all species at concentrations equal to, or greater than, 500 g/L. A 50% reduction in fertilization was observed for F. chinensis between 10 and 100 g/L copper. This same concentration range reduced fertilization by 80-90% in P. ryukyuensis.

As little as 1.9 g/L copper caused sperm immobilization in red sea urchins (S. franciscanus) and 6.3 g/L copper disrupted embryo development in purple sea urchins (Strongylocentrotus purpuratus)(Table 18). Both LOEL concentrations are only slightly above the calculated concentrations of copper that might occur in diluted outflows from sub-embayments to Mamala Bay.

The LOEL value for copper in aquatic biota appears to be 1.4 g/L, a concentration that Moore and Stebbing (1976 in EPA 1994) determined inhibited enzyme activity in the hydroid, Campanularia. Erickson (1972 in EPA 1994) and Salfullah (1978 in EPA 1994) determined reduced growth in algae exposed to 5 g/L copper. Hollibaugh et al. (1980 in EPA 1994) concluded that natural phytoplankton populations exposed to as little as 6.4 g/L copper had a reduced growth rate. A similar value (5.8 g/L) was determined by Pesch et al. (1979 in EPA 1994) to cause reduced growth in bay scallops (Argopecten irradians).

The lowest LOEL determined for copper (1.4 g/L) is a factor of five greater than the greatest estimated copper concentration in diluted embayment discharge water (0.28 g/L). A factor of 5 is not considered a large margin of safety by environmental toxicologists; since these concentrations of copper might be expected to occur in nearshore waters of Mamala Bay under the influence of outflows from sub-embayments, it is conceivable that such effects could occur. It is our conclusion that copper should be listed as a chemical stressor of concern for the waters of Mamala Bay.

7.1.3  Lead

Lead in sea water is generally in the form of sparingly soluble lead carbonates; lead in aquatic systems tends to remain in the sediments. Bioaccumulation in filter feeders (such as oysters) and deposit feeders may occur due to suspension of sedimentary material containing lead. Bioconcentration factors for lead in aquatic biota range from 20 to more than 2000 (O’Connor and Kneip, 1986). Lead is not biomagnified in marine environments, and lead is well-regulated by shellfishes and finfishes (O’Connor and Rachlin, 1982).

Lead concentrations in WWTP discharges and outflow waters from sub-embayments to Mamala Bay were more than two orders of magnitude below Hawaii Water Quality Standards. Lead concentrations in sediments at Mussel Watch sites in Mamala Bay ranged from about 16 to 20 g/g (dry weight)(NOAA 1995b). The inclusion of lead in Phase II evaluations was due to accumulation in Ostrea sandvicensis (Table 11; NOAA, 1989, 1995a).

LOELs for lead (Table 18) were 600 g/L (larval development of Dungeness crab [Cancer magister]), and between 700 and 1,200 g/L (embryo development in the sand dollar [Dendraster excentricus]).

The low concentrations of lead in WWTP effluents and the low concentrations of lead calculated in outflows to Mamala Bay are far below the lowest concentrations of lead reported to have any impact on marine organisms. It is our conclusion that lead should not be included as a chemical stressor of concern in Mamala Bay.

7.1.4  Mercury

Mercury in aquatic systems is readily metabolized by bacteria to organomercurial compounds, primarily methyl mercury and dimethyl mercury. Organo-mercurials are far more toxic to aquatic biota than mercury in its elemental form. Mono- and dimethyl mercury are accumulated rapidly by aquatic biota. Bioconcentration factors of from 103 to 106 have been measured (Callahan et al., 1979; O’Connor and Kneip, 1986).

A lack of data prevented us from calculating expected concentrations of mercury in outflows from the sub-embayments of Mamala Bay. However, mercury concentrations in the sediments of the National Mussel Watch sites at Barber’s Point and Keehi Lagoon/Honolulu Harbor were very low, less than 0.2 g/g (dry weight)(NOAA, 1995b). We have no data on the form of the mercury in the sediment analyses.

LOEL values for elemental mercury in water may be lower than 1.0 g/L. The marine trochid snail Monodonta articulata reacted to 0.8 g/L and 1.0 g/L mercury by retracting into its shell (Saliba and Vella, 1977). Given the opportunity to crawl out of the water, M. articulata spent more time out of test tanks at concentrations of 0.5 to 1.0 g/L mercury. Thomas et al. (1977 in EPA 1994) showed that 1.0 g/L mercury inhibited growth of phytoplankton assemblages. DeCoursey and Vernberg (1972 in EPA 1994) determined that 1.8 g/L mercury caused an increase in the swimming behavior of fiddler crab larvae.

Methyl mercury LOEL values were as low as 0.4 g/L, a concentration shown by Harriss et al. (1970 in EPA 1994)to reduce photosynthesis in the diatom Nitschia. LOEL for methyl mercury effects on fish were as low as 10 g/L, a concentration that Weis and Weis (1978 in EPA 1994) showed inhibited fin regeneration in striped mullet (Mugil cephalus).

Although the data on mercury and organomercury concentrations in Mamala Bay are sparse, the very low concentration of mercury in Keehi Lagoon sediments (NOAA, 1995b) and the very low concentration accumulated in oyster tissues (Table 11; Figures 6A and 7A) argue against mercury as a chemical stressor of concern in the Mamala Bay ecosystem.

7.1.5  Nickel

Nickel concentration in seawater usually ranges from 0.1 to 0.5 g/L. Nickel concentration in the sediments from the Mussel Watch site at Barber’s Point was about 58 g/g (dry weight), while that in sediments from the Keehi Lagoon/Honolulu Harbor site was about 18 g/g (dry weight)(NOAA, 1995b). Insufficient data were available to calculate the concentration of nickel in outflows from sub-embayments into Mamala Bay. Nickel was included in Phase II analyses because accumulation in oysters from the vicinity of Mamala Bay showed an erratic and apparently increasing accumulation of nickel over time (Table 11; Figures 6B and 7B).

Even low concentrations of nickel can be toxic to coral reef receptor organisms. Goh (1991) reported that nickel concentration of 9 g/L caused 50% mortality in planula larvae of the coral Pocillopora damicornis from Kaneohe Bay. The same study showed impairment of settling success after exposure to only 1 g/L nickel. These latter data constitute what is the LOEL for nickel effects on aquatic biota. Zaroogian et al. (1982 in EPA 1984) showed that 10 g/L nickel caused a reduction of adenosine triphosphate levels in blue mussels (Mytilus edulis). Most LC50 concentrations for nickel effects on marine organisms were between 100 g/L and 2,000 g/L.

Despite bioaccumulation of nickel in oysters from Honolulu Harbor and Barber’s Point, it would appear that nickel is not a chemical stressor of concern to coral reef community receptor organisms in the Mamala Bay ecosystem.

7.1.6  Tin

The primary source of tin to Mamala Bay and its contiguous waters is the use of organotin-based anti-fouling paints. The use of tributyltin (TBT) anti-foulants on vessels under 25m in length was prohibited, and the maximum average daily release rate for TBT on larger vessels was set at 4 g/cm2 day by the 1988 Organotin Paint Control Act. The US EPA has concluded that very low concentrations (20 ng/L) of TBT in water could cause irreversible chronic effects to a broad spectrum of aquatic animals. A full review of organotin effects on aquatic biota is included in O’Connor and Dorsey (1995).

The available data are insufficient to allow us to calculate organotin concentrations in the waters of Mamala Bay and its sub-embayments; however, organotin accumulation in oysters from the two mussel watch sites in Honolulu Harbor and Barber’s Point was quite high (Figure 8).

Based upon the review of Widdows and Page (1993), the apparent LOEL for tin in water is 4 ng/L (0.004 g/L), a concentration that was shown by Gibbs et al. (1987) to cause imposex in the mollusc Nucella lapillis. The summary of Widdows and Page (1993) showed that molluscs with tissue concentrations as low as 0.52 g/g (520 ng/g) showed chronic effects due to TBT. Interestingly, tissues of oysters sampled from Barber’s Point and Honolulu Harbor had TBT concentrations above 500 ng/g in 1990 and in 1992 (Figure 8). Tissues of oysters sampled from Barber’s Point in 1992 had TBT concentrations in excess of 1,000 ng/g (Figure 8).

The very high concentration of organotins in oyster tissues from the Mussel Watch sites in Mamala Bay, and the likelihood that organotin concentrations in the sub-embayments of Mamala Bay are also high suggests to us that organotins are a chemical stressor of concern in at least the sub-embayments of Mamala Bay. We include organotins in the list of chemical stressors of concern primarily because there are so few data available, and because the organotins have the potential to have a devastating effect on the reef flat and reef communities of Mamala Bay.

7.1.7  Zinc

Zinc concentrations in seawater are about 10 g/L (Friberg et al., 1979). Zinc is an essential element acting as a coenzyme in physiological processes common to most eukaryotic species; not unexpectedly, zinc is well regulated by most aquatic biota (O’Connor and Rachlin, 1982).

Stevenson et al. (1995) determined zinc to be a major contaminant in both WWTP effluents and in nonpoint source runoff to Mamala Bay (Tables 6 and 8). We calculated that the zinc concentration in outflow from sub-embayments to Mamala Bay after mixing (Table 16) ranged from 0.77 g/L to as high as 3.22 g/L. The Hawaii water quality standards for zinc in seawater are 95 g/L (acute) and 86 g/L (chronic).

The apparent LOEL for zinc is 20 g/L, a concentration known to cause abnormal development of purple sea urchin embryos (Table 19; see Dinnel et al., 1989). Most other LC50 and EC50 data for zinc show sublethal effects to be caused by concentrations above 200 g/L (NRC, 1979; Rand and Petrocelli, 1985; Dinnel et al., 1989). A concentration of 500 g/L reduced fertilization in the coral Platygyra ryukyuensis by 45% (Heyward, 1988).

Since zinc LOEL concentrations (about 20 g/L) were measured for echinoderms and corals (Table 19), and since zinc concentrations in the outflow from sub-embayments could be as high as 3.2 g/L, we conclude that zinc is a chemical stressor of concern for the Mamala Bay ecosystem.

7.2  Phase II Organics Evaluations

Phase II evaluations of organic chemical stressors in Mamala Bay are not straightforward; there exist no water column dissolved or particle-associated concentrations in either Mamala Bay or its sub-embayments, and the presence of diazinon has not been documented for Mamala Bay, its sub-embayments, or the watershed. Diazinon is listed as having use in pineapple and sugar cane culture and, due to its widespread use, would be expected to be present in stormwater runoff from agricultural, urban, suburban, and industrial sources.

Phase II evaluations of organic chemical stressors will be based upon use in the Mamala Bay watershed, the potential for continued discharge of the compounds with wastes, known concentrations of the compounds in the sediments of sub-embayments of Mamala Bay, and the potential for the observed concentrations to cause effects in aquatic species.

7.2.1 Chlordane

Chlordane is presently banned from open use as a pesticide. Historical use of chlordane in the Mamala Bay watershed was for pest control on crops and fields, and as a household pesticide often used for termite control. Despite the fact that chlordane was accumulated by bivalves at Mamala Bay sites (combined totals of a-chlordane, trans-nonachlor, heptachlor and heptachlor epoxide; Figure 11), analysis for chlordane in the sediments of the boat harbor at Barber’s Point and Honolulu Harbor showed only minor concentrations less than (0.5 ng/g; NOAA, 1995b).

The potential for such low concentrations of chlordane to cause effects in Mamala Bay is very low. Long and Morgan (1990) accumulated information on sediment stressor concentrations as related to effects, and reported that a concentration of 17.4 ng/g was the lethal threshold for chlordane in sediments, and that sediment concentrations of 0.6 and 0.3 ng/g chlordane represented the 95% and 99% chronic permissible levels, respectively, based upon the equilibrium partitioning approach to establishing sediment quality criteria (EPA, 1992). The Apparent Effects Threshold concentration for chlordane in sediments of San Francisco Bay was 2.0 ng/g (Long and Morgan, 1990). Long and Morgan (1990) determined that the “Effects Response - Lower” (ER-L) for chlordane in sediments should be set at 0.5 ng/g.

Pavlou and Weston (1983) calculated the chronic threshold tissue concentration for chlordane at 799 ng/g, a concentration far greater than the concentrations of chlordane measured in oysters from either Barber’s Point of Honolulu Harbor (Table 12).

It is possible that chlordane discharge to the sub-embayments of Mamala Bay may continue, as residues in soils gradually move from land into surface waters; however, the ban on use of chlordane suggests a high probability that chlordane concentrations in surface sediments of sub-embayments of Mamala Bay will decline over time. It is our conclusion, based upon the available data, that chlordane should not be considered a chemical stressor of concern in Mamala Bay. However, since chlordane is readily accumulated by shellfish (Figure 11), chlordane concentrations should be monitored in the future as a means of confirming the fact that concentrations are gradually declining.

7.2.2 Diazinon

We have no information on the distribution of the pesticide diazinon in the Mamala Bay watershed, the biota, or the sediments. Due to the apparent toxicity of diazinon to freshwater planktonic invertebrates (Ceriodaphnia dubia) and ichthyoplankton (striped bass larvae) reported in stormwater in California (Kinnetic Laboratories, 1994b; Foe, personal communication; Connor, personal communication), we conclude that diazinon is likely to occur in Mamala Bay waters, and, therefore, should be considered a chemical stressor of concern.

7.2.3 Dieldrin

Data from NOAA (1995b) on dieldrin concentrations in the sub-embayments of Mamala Bay are sparse. Measurable concentrations of dieldrin occurred in only one replicate from Honolulu Harbor in 1986 (0.7 ng/g). No dieldrin was measured in sediment samples from Barber’s Point Dieldrin accumulation in oyster tissues was erratic (Table 12).

Dieldrin can be highly toxic, but toxicity varies widely with the sediment in which the dieldrin is sequestered. Assays with sediment from San Francisco Bay showed that dieldrin at 10.3 ng/g was highly toxic to the amphipod Rhepoxinius abronius (Long and Morgan, 1990); however, in bioassays with the polychaete worm Nereis virens, Van Dolah et al. (1984) determined an LC50 of 13,000 ng/g for dieldrin.

Reflecting the very high apparent toxicity of dieldrin, ER-L concentrations were set at 0.02 ng/g by Long and Morgan (1990). The 99 percentile chronic marine value (by equilibrium partitioning; EPA, 1992) was set at 0.01 ng/g. The EPA interim marine sediment quality criterion for dieldrin has been established at 57.7 ng/g dry weight (EPA, 1988c).

Pavlou and Weston (1983) calculated a chronic dieldrin threshold concentration in tissues at 451 g/g, a factor of 50 greater than the concentrations of dieldrin measured in oyster tissues from the sub-embayments of Mamala Bay.

Lacking information on dieldrin concentrations in Mamala Bay, and the low probability that dieldrin would be mobilized from sediments to the water column in the sub-embayments, we consider the potential for dieldrin effects in the Mamala Bay watershed to be very low. We conclude that dieldrin should be not considered a chemical stressor of concern in Mamala Bay.

7.2.4 HMWPAH

High molecular weight polycyclic aromatic hydrocarbons are abundant in stormwater runoff from urban locations, including the City and County of Honolulu. Their abundance in the waters of the sub-embayments of Mamala Bay is shown by the high accumulation of HMWPAH in oyster tissues (Table 12; Figure 10), and their abundance in sediments from Mussel Watch sampling sites at Barber’s Point and Honolulu Harbor in 1986 and 1987 (Figure 12).

The polycyclic aromatic hydrocarbons have numerous effects in the environment, including chronic and acute toxicity, induction of various enzyme systems in fishes and invertebrates, and the potential to act as carcinogenic compounds (Mix, 1985). Long et al. (1994) established an ER-L for HMWPAH of 1,700 ng/g in sediments. As shown in Figure 12, ER-L concentrations for HMWPAH were exceeded at both the Barber’s Point and the Honolulu Harbor Mussel Watch stations in 1986 or 1987. We assume that the very high accumulation of HMWPAH among oysters sampled from Honolulu Harbor in 1986 through 1992 to be attributable to high concentrations of HMWPAH in the water column and the sediments.

Although efforts are underway in the state of Hawaii to control organic chemical spills and organic chemical discharge in stormwater runoff, it is likely that HMWPAH compounds will continue to be discharged to surface waters of the Mamala Bay watershed for a long time to come. We conclude from this that HMWPAH compounds should be considered a chemical stressor of concern for Mamala Bay.

8.0  Conclusions and Recommendations

8.1 Conclusions

The Mamala Bay ecosystem receives large loads of chemical stressors from WWTP effluents, runoff from the watershed, and a variety of industrial, agricultural, and other sources. While the pollutant loads are large, the concentrations of pollutants in Mamala Bay are small, in all cases below Hawaii Water Quality Standards and the lowest concentrations of chemical stressors likely to cause acute or chronic toxicity (LOELs).

We evaluated a series of known and likely chemical stressors to determine which stressors, if any, should be considered to be of concern to the Mamala Bay ecosystem. These were the metals cadmium, copper, lead, mercury, nickel, tin, zinc; and the organic compounds chlordane, diazinon, dieldrin, and high molecular weight polycyclic aromatic hydrocarbons (HMWPAHs). Careful comparison of the likely concentration of each of these chemical stressors in Mamala Bay with water quality standards and LOEL data showed that copper, tin (especially organotins), zinc, diazinon, and HMWPAHs should be considered as chemical stressors of concern in the Mamala Bay ecosystem. The reason that these stressors were designated as “of concern” had to do, in all cases, with their apparent high concentrations in the sub-embayments that discharge to Mamala Bay.

Zinc and copper are considered to be stressors of concern primarily because the concentrations of these two metals in water flowing out from sub-embayments to the shoreline areas of Mamala Bay are such that sensitive life stages of corals could be affected (rates of fertilization, larval development). The potential for either of these metals to affect the open waters of Mamala Bay, away from the shore zone, is very small.

Organotins (mono-, di- and tributyl tin) were accumulated to very high concentrations by oysters in sub-embayments of Mamala Bay at Barber’s Point and Honolulu Harbor. Their presence is due to the use of alkyl tins in the formulation of anti-fouling paint formerly used on all vessels, but currently restricted to vessels over 25 m in length. Organotin accumulation by oysters in the sub-embayments was well in excess of the threshold for induction of chronic effects. While organotin impacts are usually limited to anchorage areas, the possibility exists for organotins to be mobilized and discharged into the shore zone adjacent to outflows from sub-embayments. The highly toxic nature of the organotins and their apparent abundance in the Mamala Bay system requires that they be listed as a chemical stressor of concern.

Diazinon is an unknown quantity in the Mamala Bay system. We included diazinon as a chemical stressor of concern because diazinon is a widely used pesticide, it is known to cause toxicity among planktonic invertebrates and fish larvae, and it is coming to be known, nationwide, as a common, toxic component of stormwater runoff.

HMWPAHs are a common problem in ports and harbors around the country. In most instances HMWPAHs remain sequestered in port sediments. Their origin in and around port facilities is from petroleum fuels and lubricants, and from stormwater runoff draining to semi-enclosed water bodies. HMWPAHs were accumulated by oysters in sub-embayments of Mamala Bay to concentrations well in excess of sediment toxicity limits. The potential for mobilization of HMWPAHs out of the sub-embayments into the shore zone adjacent to outflow areas is high.

8.2  Recommendations

We recommend that stormwater monitoring for chemical stressors be expanded to include diazinon (and other organic chemical stressors) so that an objective assessment of diazinon (and other organic chemical stressors) in stormwater runoff can be made.

We recommend that stormwater management measures and management practices be implemented in order to minimize the loads of chemical stressors entering the sub-embayments that flow into Mamala Bay.

We recommend that construction and dredging activities in the sub-embayments that flow into Mamala Bay be managed carefully to minimize the loss of fine material to suspension and subsequent transport out into Mamala Bay. Such transport would, in all likelihood, result in the mobilization of potentially harmful concentrations of copper, organotins, zinc, and HMWPAHs into Mamala Bay.