MAMALA BAY STUDY

MODELING TRANSPORT AND FATE OF

PATHOGENIC ORGANISMS IN MAMALA BAY

PROJECT MB-5

Principal Investigator:

Alan F. Blumberg

HydroQual, Inc.

1 Lethbridge Plaza

Mahwah, New Jersey  07430

Co-Principal Investigator:

John P. Connolly

HydroQual, Inc.

1 Lethbridge Plaza

Mahwah, New Jersey  07430

September 15, 1995

1  EXECUTIVE SUMMARY

The research effort described herein addresses the fate of pathogenic organisms

released from outfall and shoreline sources into Mamala Bay.  In particular, this effort is

directed towards Project MB-5 of the Mamala Bay Study Commission and presents a

modeling program that was designed to define the temporal and spatial distributions of

the pathogens released from outfall and shoreline sources.  The results of this study

include a synthesis of the data and modeling results designed to provide an understanding

of Mamala Bay that is essential for assessing public health and environmental effects of

pollution.

The modeling program was divided into two parts: a hydrodynamic and a

pathogen fate model.  The hydrodynamic model provides quantitative information on the

movements of water that are used by the fate model to predict the distributions of

pathogens in the bay.  A near field initial dilution model (Project MB-4) is coupled to the

hydrodynamic model so that outfall loadings can be input properly.  The fate model

includes physical, chemical and biological processes and is designed to address the

differences between different types of pathogens.  The models were used to simulate a

complete yearlong period, from April 15, 1994 through April 15, 1995.

A whole island hydrodynamic model has been constructed with increased grid

refinement in Mamala Bay.  The grid cells in the vicinity of the Sand Island outfall are of

the order of 380 m on a side while the region around the Honouliuli outfall has cells of about 700 m on a side.  Along the oceanic boundaries of the domain, the resolution

increases into the range of 2 km to 4 km.  The grid, it should be noted, includes Pearl

Harbor, Keehi Lagoon/Honolulu Harbor and the Ala Wai Canal, allowing better

representation of water quality loadings from these areas.  The offshore extent of the

modeled domain extends out to roughly the 500 m isobath.  The vertical resolution is

accomplished by use of 10 equally-spaced levels so that important physical processes

operating in the vertical plane are resolved.

The hydrodynamic model predicts advective and dispersive processes in Mamala

Bay with sufficient accuracy that model results can be confidently used for pathogen fate

simulations.  This conclusion is based on favorable comparisons between data and model

predictions for water elevations, currents, temperature, salinity and a dye release.

Hydrodynamic model simulations reveal a circulation pattern in Mamala Bay that is

complicated both in space and time.  The mean flow is typically westward at all depths.

However, a permanent recirculation zone is found in the nearshore region off Waikiki

Beach.  During Kona wind conditions, the model shows significant departures from the

currents calculated under trade wind conditions in nearshore areas.  The general

circulation patterns, properties of the outfall plumes and characteristics of the

recirculation zone change markedly for these different wind conditions.  However, in

water depths deeper than 70 m, the circulation patterns exhibit little change.

The hydrodynamic model was also used to investigate possible physical

oceanographic mechanisms that could transport fecal contamination from the outfalls to

the eastern beaches.  The analyses suggested that during stratified water column and

Trade Wind conditions there were plausible physical oceanographic pathways for the

transport of fecal contamination from the outfall to the eastern beaches.  Moreover, it was

shown that an outfall plume initially trapped below the surface could still eventually

move into the nearshore waters of Mamala Bay.  This possibility has previously been

assumed to be highly unlikely.  It was also found that the most favorable conditions for

pathogen transport from the outfall to the beaches was during winter months when Kona

winds blow and the water column is weakly stratified allowing the outfall plume to

surface.

Pathogen fate was modeled using the Mamala Bay portion of the hydrodynamic

grid.  Three indicator organisms and four pathogens associated with fecal contamination

were modeled.  Analysis of model results for the four pathogens was limited to the

eastern portion of the bay where data permitted characterization of pathogen sources.

The organisms modeled included fecal coliform, enterococci, Clostridium, Salmonella,

enterovirus, Giardia, and Cryptosporidium.  Conclusions drawn from the pathogen fate

model are supported by a favorable comparison of predicted and observed distributions of

fecal coliform, Clostridium enterovirus and Cryptosporidium.  The model does

underpredict enterococci by a factor of two to three and overpredict Giardia and

Salmonella by similar factors. Model results aid in estimating the contributions of the

various sources to fecal contamination within the bay and at the beaches.

The plume from the Honouliuli outfall is generally carried alongshore in a

westward direction, occasionally reaching the island at the westernmost edge of Mamala

Bay (Oneula Beach).  It has no impact along most of the shoreline of the Bay.

The Sand Island outfall plume is transported throughout the bay.  It̓s contribution

to indicator organism counts at specific locations is temporally variable as a result of

variations in tidal and low frequency currents.  It is the dominant source of fecal coliforms at all of the beaches, and of enterococci and Clostridium at the beaches west of

Honolulu Harbor.  The greater impact to the west results from the predominantly

westerly, low frequency currents.  The analysis of specific pathogens in the eastern bay

indicates that the Sand Island outfall is the dominant source of Giardia and enterovirus at

Ala Moana, Waikiki and Queens Surf Beaches.

The Ala Wai Canal is also an important contributor of fecal contamination within

the bay.   The canal̓s significance is implied by a relationship between measured

indicator organism counts at the eastern beaches and rainfall.  It was confirmed by the

model which indicated that the canal is the major source of enterococci and Clostridium

at the eastern beaches.  In addition, waters from the Ala Wai Canal contribute

significantly to contamination in waters offshore of these beaches, particularly off of

Waikiki.  The Ala Wai Canal is also an important contributor to Salmonella and

Cryptosporidium counts at the eastern beaches.  Because the study period had less

frequent high rainfall events than the historical record, the contributions from the canal

may occasionally be larger than estimated in this study.  The other shoreline sources may

have local effects, but they do not contribute significantly to fecal contamination in the

bay.

In summary, the model indicates that the Sand Island Outfall and the Ala Wai

Canal are responsible for most of the fecal contamination observed in Mamala Bay

during the study period.

Analysis of the efficacy of source control indicates that upgrade of the Sand

Island Treatment Plant to chemically-enhanced primary treatment (CEPT) reduces fecal

coliform counts throughout most of the bay by about 50%.  It has a smaller effect at the

western edge of the bay (Oneula Beach) because of contributions from the Honuliuli

Plant.  CEPT has less of an effect on enterococci counts because of the impacts of

shoreline sources.  Its most substantive effects on enterococci occur in the deeper

offshore waters and at the western beaches.  CEPT does not reduce Clostridium counts.

Additional reductions are achieved by adding disinfection to CEPT or upgrading the plant

to secondary treatment.  Of theses options, disinfection has the greater effect.  As with CEPT, additional treatment is most effective at reducing fecal coliform levels.  Average

levels drop by about an additional 20%.  The fecal coliform reductions are noteworthy in

that they substantially reduce the frequency at which threshold values are exceeded.

Treatment beyond CEPT has little impact on enterococci counts at the beaches.

Disinfection reduces Clostridium counts by about 80% at Oneula and Ewa beaches and

between 30% and 50% at the eastern beaches.

Upgrade of the Honouliuli Treatment Plant has little benefit at the shoreline,

except at Oneula Beach.  At this location fecal coliform levels are reduced by about 50%

with CEPT and about 80% with either disinfection or secondary treatment.  Enterococci

and Clostridium counts are marginally reduced because of contributions from the Sand

Island outfall and shoreline sources.  Disinfection at Sand Island is more effective at

reducing these indicators than is disinfection at Honouliuli.  Treatment upgrades at Honouliuli do have effects in the offshore waters of the western bay.  For example,

disinfection reduces indicator organism counts at station HU2 (inshore of the diffuser at

the 10 m isobath) by about 85% for fecal coliform, 40% for enterococci and 30% for

Clostridium.

Specific control options for the shoreline sources were not investigated.  To assess

potential benefits of controlling these sources, the impacts of source elimination were

analyzed.  Three sources were considered: Ala Wai Canal, Pearl Harbor and Keehi

Lagoon.  Elimination of the loads to Pearl Harbor and Keehi Lagoon had little impact

within the bay.  In contrast, elimination of the loads from Ala Wai Canal had substantive

impacts in the eastern bay.  Fecal coliforms were reduced by about 30% at the eastern

beaches and by about 50% at station W2 (offshore of Waikiki Beach at the 10 m isobath).

Enterococci and Clostridium counts were reduced by about 50-60% at the eastern

beaches and by about 70-80% at station W2.

TABLE OF CONTENTS

1.	EXECUTIVE SUMMARY	  1

2.	INTRODUCTION  6

2.1	Scope of Work  6

2.2	Objectives  8

2.3	Project Organization  8

3.	METHODS 12

3.1	Task Summary 12

3.2	Task Methodology 12

3.2.1	Selection of Study Area 12

3.2.2	Hydrodynamic Model 15

3.2.2.1	Model Development 15

3.2.2.2	Forcing Functions 17

3.2.2.3	Skill Assessment 21

3.2.3	Pathogen Fate Model 21

3.2.3.1	Analysis of Data 21

3.2.3.2	Model Development 22

3.2.3.3	Skill Assessment 30

3.2.3.4	Analysis of Source Contributions to the Beaches 33

4.	RESULTS 34

4.1	Hydrodynamic Circulation 34

4.1.1	Comparison of Model and Data 34

4.1.2	Description of Currents 55

4.1.3	Sensitivity to Wind Data 57

4.1.4	Kona Circulation 57

4.1.5	Outfall to Beach Transport Mechanisms 58

4.2	Pathogens 66

4.2.1	Analysis of Data 66

4.2.1.1	The Distribution of Bacterial Indicator Counts 66

4.2.1.2	Annual and Seasonal Patterns 68

4.2.1.3	Potential Sources of Contamination 69

4.2.1.4	Conclusions 73

4.2.2	Zone of Influence of the Outfalls 75

4.2.2.1	Sand Island Treatment Plant Zone 75

4.2.2.2	Honouliuli Treatment Plant Zone 77

4.2.3	Comparison of Model and Data 77

4.2.4	Sensitivity Analysis 85

4.2.5	Analysis of Source Contributions	 85

4.2.6	Source Contributions for Specific Pathogens 95

4.2.7	Analysis of Loading Reduction Evaluations	101

5.	CONCLUSIONS112

6.	RECOMMENDATIONS115

LIST OF FIGURES

2-1.	The study area considered in Project MB-5 and the location of the various

field measurements stations used in the analysis.  Hydrographic data

station are marked in the top panel and water quality data stations in the

lower panel.  7

3-1.	Numerical grids for hydrodynamic model (top panel) and pathogen fate

and transport model (bottom panel, shaded area). 14

3-2.	Wind data collected at Honolulu Airport during the calibration period,

April 15, 1994 through April 15, 1995. 20

3-3.	Conceptual model of pathogen fate 23

3-4.	Cumulative probability distributions of fecal coliform, enterococci and

Clostridium counts measured in Ala Wai Canal at the Ala Moana Bridge

for 72-hour cumulative rainfall at Honolulu Airport £0.03 inches. 27

3-5.	Cumulative probability distributions of fecal coliform, enterococci and

Clostridium counts measured in Ala Wai Canal at the Ala Moana Bridge

for 72-hour cumulative rainfall at Honolulu Airport > 0.03 and £ 0.30

inches. 28

3-6.	Cumulative probability distributions of fecal coliform, enterococci and

Clostridium counts measured in Ala Wai Canal at the Ala Moana Bridge

for 72-hour cumulative rainfall at Honolulu Airport > 0.30 inches. 29

3-7.	Comparison of the cumulative probability distributions of indicator

organism counts measured in Ala Wai Canal at the Ala Moana Bridge and

estimated using the Monte Carlo procedure. 31

3-8.	Relationship between indicator counts measured in the surface (0-4 ft) and

bottom (>4 ft) waters of the Ala Wai Canal at Ala Moana Bridge.  The

regression lines shown in the plots were used to determine bottom water

counts from surface water counts. 32

4-1.	Comparison between predicted and measured water surface elevations at

five locations around Oahu during August 1994. 36

4-2.	Comparison Between Predicted and Observed Low Frequency Water

Surface Elevations for the Calibration Period, April 15, 1994 through

April 15, 1995. 37

4-3.	Comparison Between Predicted and Measured Currents at Station C0

during October-December, 1994. 38

4-4.	Comparison Between Predicted and Measured Alongshore Surface

Currents During August 1994. 39

4-5.	Comparison Between Predicted and Measured Alongshore Mid-Depth

Currents During August 1994. 40

4-6.	Comparison Between Predicted and Measured Alongshore Bottom

Currents During August 1994. 41

4-7.	Tidal Current Ellipses for Measured (top) and Predicted (bottom) Surface

Currents for the Period April 1 to December 26, 1994. 43

4-8.	Tidal Current Ellipses for Measured (top) and Predicted (bottom) 70 m

Currents for the Period April 1 to December 26, 1994. 44

4-9.	Forty-hour, Low-pass Variance Ellipses for Measured (top) and Predicted

(bottom) Surface Currents for the Period April 1 to December 26, 1994. 46

4-10.	Forty-hour, Low-pass Variance Ellipses for Measured (top) and Predicted

(bottom) 70 m Currents for the Period April 1 to December 26, 1994. 47

4-11.	Comparison Between Measured and Model Input Temperatures at Stations

B2 and D2 During August 1994. 48

4-12.	Dye Injection Points in the Ala Wai Canal and Grid Cell Reference

Locations for Model/Data Comparison. 49

4-13.	Comparison Between Measured and Predicted Dye Concentrations at Six

Grid Cell Near the Ala Wai Canal. 51

4-14.	Comparison Between Measured and Predicted Flow Rates at the Ala

Moana Bridge on July 6-7, 1994.  Note that +/- Denotes out of/into Ala

Wai Canal, respectively. 52

4-15.	Comparison Between Measured and Predicted Salt Flux at the Ala Moana

Bridge on July 6-7, 1994.  Note that +/- Denotes out of/into Ala Wai

Canal, respectively. 54

4-16.	Predicted Average Current Distributions in Mamala Bay at Depths of 0

and 70 m for April 1994 through March 1995. 56

4-17.	Monthly Average Currents at 0 m Depth During Trade (top) and Kona

(bottom) Wind Conditions. 59

4-18.	Monthly Average Currents at 70 m Depth During Trade (top) and Kona

(bottom) Wind Conditions. 60

4-19.	Predicted Locations of Particles in the Upper 5 m of the Water Column at

Noon on May 22. 64

4-20.	Predicted Locations of Particles in the Upper 5 m of the Water Column at

Noon on May 22.  Particle Tracking was Simulated with Zero Diffusion. 65

4-21.	Probability plots of fecal coliform concentrations at four beaches.  All

years combined.  Open symbols: STORET historical data; filled symbols:

MBSC project data. 67

4-22.	Temporal plots of the frequency of fecal coliform concentrations greater

than 10 cfu/100ml, calculated annually.  Beach stations. Symbols as in

Figure 4-21. 70

4-23.	Seasonal plots of the frequency of fecal coliform concentrations greater

than 10 cfu/100ml, calculated monthly. All data collected since 1988

combined.  Beach stations. Symbols as in Figure 4-21. 71

4-24.	Seasonal plots of parameters related to potential sources of contamination,

calculated monthly.  Symbols as in Figure 4-21. 72

4-25.	Relationships between concentrations at two beaches and parameters

related to potential sources of bacteria.  Symbols as in Figure 4-21. 74

4-26.	Contours of the average concentrations (mg/l) in logarithmic units

resulting from a unit loading (1 kg/d) of a conservative substance at the

Sand Island outfall. 76

4-27.	Contours of the average concentrations (mg/l) in logarithmic units

resulting from a unit loading (1 kg/d) of a conservative substance at the

Honouliuli outfall. 78

4-28.	Cumulative probability distributions of fecal coliform data and model

results at various locations. 80

4-29.	Cumulative probability distributions of enterococci data and model results

at various locations. 83

4-30.	Cumulative probability distributions of Clostridium data and model results

at various locations. 84

4-31.	Average concentrations of fecal coliforms and enterococci at five beach

stations calculated by the model under three conditions: only Sand Island

treatment plant, only Honouliuli treatment plant, and only shoreline

sources discharging 87

4-32.	Time series of computed daytime fecal coliform counts at Ala Moana

Beach with loadings to Mamala Bay from all sources, outfalls only, or

shoreline sources only. 88

4-33.	Time series of computed daytime fecal coliform counts at Ewa Beach with

loadings to Mamala Bay from all sources, outfalls only, or shoreline

sources only. 89

4-34.	Time series of computed daytime enterococci counts at Ala Moana Beach

with loadings to Mamala Bay from all sources, outfalls only, or shoreline

sources only. 90

4-35.	Time series of computed daytime enterococci counts at Ewa Beach with

loadings to Mamala Bay from all sources, outfalls only, or shoreline

sources only. 91

4-36.	Time series of computed daytime Clostridium counts at Ala Moana Beach

with loadings to Mamala Bay from all sources, outfalls only, or shoreline

sources only. 92

4-37.	Time series of computed daytime Clostridium counts at Ewa Beach with

loadings to Mamala Bay from all sources, outfalls only, or shoreline

sources only. 93

4-38.	Temporal profiles of observed (symbols) and predicted (lines) daytime

pathogen abundances at Ala Moana Beach. 97

4-39.	Temporal profiles of observed (symbols) and predicted (lines) daytime

pathogen abundances at Waikiki Beach. 98

4-40.	Temporal profiles of observed (symbols) and predicted (lines) daytime

pathogen abundances at Queens Surf Beach. 99

4-41.	Average fecal coliform concentrations at six stations calculated by the

model for four treatment levels at Sand Island treatment plant, current,

chemically enhanced primary treatment (CEPT), secondary treatment, and

disinfection102

4-42.	Average enterococci concentrations at six stations calculated by the model

for four treatment levels at the Sand Island treatment plant: current,

chemically enhanced primary treatment (CEPT), secondary treatment, and

disinfection103

4-43.	Average Clostridium concentrations at six stations calculated by the model

for four treatment levels at the Sand Island treatment plant: current

chemically enhanced primary treatment (CEPT), secondary treatment, and

disinfection104

4-44.	Average predicted fecal coliform concentrations at ten stations for six

scenarios: current conditions, disinfection at Sand Island and Honouliuli

treatment plants, and elimination of Ala Wai Canal, Pearl Harbor and

Keehi Lagoon shoreline sources106

4-45.	The number of days on which predicted fecal coliform concentrations

exceed 50 cfu/100 ml at ten stations for six scenarios: current conditions,

disinfection at the Sand Island and Honouliuli treatment plants, and

elimination of Ala Wai Canal, Pearl Harbor and Keehi Lagoon shoreline

sources107

4-46.	Average predicted enterococci concentrations at ten stations for six

scenarios: current conditions, disinfection at the Sand Island and

Honouliuli treatment plants, and elimination of Ala Wai Canal, Pearl

Harbor and Keehi Lagoon shoreline sources108

4-47.	Average predicted Clostridium concentrations at ten stations for six

scenarios: current conditions, disinfection at the Sand Island and

Honouliuli treatment plants, and elimination of Ala Wai Canal, Pearl

Harbor and Keehi Lagoon shoreline sources109

4-48.	The number of days on which predicted enterococci concentrations exceed

7 cfu/100 ml at ten stations for six scenarios: current conditions,

disinfection at the Sand Island and Honouliuli treatment plants, and

elimination of Ala Wai Canal, Pearl Harbor and Keehi Lagoon shoreline

sources111

LIST OF TABLES

2-1.	Water Quality Sampling Stations  9

4-1.	Descriptive Statistical Comparison of Model and Data for Indicator

Organisms 81

4-2.	Abundances of Pathogenic Organisms in Sand Island Effluent and in the

Ala Wai Ccanal at Ala Moana Bridge 96

2  INTRODUCTION

2.1  Scope of Work

This project focused on the hydrodynamic circulation within Mamala Bay and the

fate and transport of pollutants entering Mamala Bay from the Sand Island and

Honouliuli wastewater treatment plant outfalls and various shoreline source discharges

from the island itself.  The region under investigation is shown in Figure 2-1.  It entailed

the development of a mathematical representation of the bay and the constituents within it

(a model) for the purposes of providing a tool for predicting the fate and transport of

pollutants entering the bay.  The model describes the movement of water within the bay

and between the bay and the ocean, the transport of enteric bacteria by that movement

and the death of the bacteria due to phototoxicity and predation by larger organisms.

The model was developed by applying the principles of conservation of mass and

momentum and empirical descriptions of phototoxicity and predation to the physical

setting of Mamala Bay.  This involved the analysis of measurements in Mamala Bay of:

water depths (bathymetry)

water surface elevations

water currents

temperature and salinity

wind velocity

solar radiation

decay of light with depth of water

precipitation and runoff to the bay

rates at which enteric bacteria are discharged to the bay

concentrations of enteric bacteria within the bay.

Some of these measurements were used to construct the model.  Others were used

to determine the accuracy of the model's predictions of circulation and concentrations of

Figure 2-1. The study area considered in Project MB-5 and the location of the various

field measurements stations used in the analysis.  Hydrographic data stations are

marked in the top panel and water quality data stations in the lower panel.

enteric bacteria.  The locations of the various field measurement stations that are used in

this study are shown in Figure 2-1; Table 2-1 relates the beach names to their respective

station codes.

Model development also involved analysis of published information on the

mechanisms of inactivation and dieoff in enteric bacteria, including quantification of

predation rates and phototoxicity loss rates.

The model was used to predict the changes in the concentration of enteric bacteria

that occur as a result of meteorological variability (for example, wind speed and direction

or rain storms) or changes in the discharge of enteric bacteria to the bay (for example,

through additional treatment of the wastewater or treatment of stormwater inputs).

2.2  Objectives

The objectives of this project were to provide quantitative answers to the

following questions:

●	What are the sources of fecal contamination measured at the beaches?

●	What meteorological and hydrodynamic conditions cause the sources to be

transported to the beaches?

●	How effective are selected source control efforts in reducing or

eliminating the incidence of standards violation?

2.3  Project Organization

An organizational structure and set of systematic procedures was established to

insure that the approach was based on firm scientific principles and that the results have

scientific merit.  All the tasks of this project were the responsibility of the principal

investigators, Dr. Alan F. Blumberg and Dr. John P. Connolly, both of HydroQual.

Table 2-1.	Water Quality Sampling Stations

The program manager was Dr. Blumberg.  His responsibilities included:

oversight of scientific progress, coordination of tasks, identification of problem areas, and

communication with the representatives of the Mamala Bay Study Commission.  In his

role as principal investigator for transport modeling, Dr. Blumberg was in constant

contact with the entire modeling effort.  He also acted as a liaison between the modeling

and observational groups to coordinate their efforts.

As principal investigator for the pathogen fate modeling, Dr. Connolly was

responsible for developing the appropriate strategy for modeling the fate of Mamala Bay

pathogenic organisms and interfacing with the scientific community.  Dr. Connolly led

the task to understand the relationship between pollutant discharges to the bay and water

quality within the bay.

An experienced team of assistants was assembled to perform all the tasks required

by the study.  Dr. C. Kirk Ziegler of HydroQual and Dr. E. Eric Adams of the

Massachusetts Institute of Technology (M.I.T.) contributed significantly to the transport

modeling and coupling of the far field and near field hydrodynamic components.  The

pathogen data analysis was directed by Dr. David Glaser of HydroQual who has

examined the historical data bases, placed the more recent data in that context, and

assisted with the interpretation of the fate model results.  The success of the study was

also due to the efforts of HydroQual engineers, Mr. Nicholas Kim, Mr. James Quadrini

and Mr. Harry Zahakos who rearranged the often extensive data sets for use with the

models and conducted the actual simulations.  Mr. Xue Yong Zhang of M.I.T. developed

the particle tracking model used in the analysis presented in this report.

The ultimate success of this study has been dependent on interaction with all the

other projects included in the Mamala Bay Study.  The development and calibration of

the models required a considerable quantity of data.  Therefore, it has been important that

the investigators work closely with the groups conducting Projects MB-2, MB-3, MB-4

and MB-6, so that the information they compile would be consistent with the

requirements of the models and would be available as it was needed.  Special thanks to

Dr. Peter Hamilton of SAIC for his help with the use of the bottom pressure

measurements.

3  METHODS

3.1  Task Summary

This project involves four major tasks associated with the development and

application of mathematical models.  The first two tasks consist of the development of

models of water circulation and pathogen fate in Mamala Bay.  Within these tasks are

numerous sub-tasks directed to the compilation and analysis of data from which

conceptual models are developed,  detailed descriptions of the relevant physical, chemical

and biological processes are constructed and bases for assessment of model performance

are established.

The third major task is use of the models to simulate circulation and pathogen fate

in Mamala Bay for a twelve-month period beginning in April 1994 and ending in April

1995.  This task includes a detailed assessment of the capabilities of the models to

reproduce observed conditions in the bay.

The final task is the use of the models as diagnostic tools to define the relative

contributions of each of the quantified pathogen sources to organism levels at locations of

human contact (the beaches).

3.2  Task Methodology

3.2.1  Selection of Study Area

Mamala Bay is a challenging coastal region under the joint influence of density

stratification, variable depth, earth's rotation and bottom friction.  The local winds

(including the diurnal breezes and large-scale wind patterns), coastline shape, non-local

forcing and tides all contribute to the circulation.  For example, motions generated by

synoptic-scale atmospheric disturbances often dominate the circulation.  Particular care is

required when defining the numerical grid in the proximity of Barbers Point because it

has been shown observationally that fronts and eddies can be formed in association with

flow around the Point.  The presence of an oceanic current offshore presents additional

complexity because of its meanders and attendant mesoscale eddy activity which can

impinge on the Mamala Bay region.

The fundamental length scale which characterizes most of the above-mentioned

physical processes is the baroclinic radius of deformation.  Both upwelling and coastal-

trapped waves are constrained to exist within a few radii of the coast and cross-front

length scales are also typified by a few radii.  The baroclinic radius in the Mamala Bay

region is of the order 20 km.  It is therefore desirable that the numerical model grid

spacing be less than about 1 km, and that the total horizontal domain exceed 20 km.

Because of these issues, it is apparent that a model developed only for Mamala

Bay may be too geographically limited to permit a realistic simulation of the observed

circulation patterns.  Therefore, a whole island hydrodynamic model has been constructed

with increased grid refinement in Mamala Bay as shown on Figure 3-1.  The grid cells in

the vicinity of the Sand Island outfall are of the order of 380 m on a side while the region

around the Honouliuli outfall has cells of about 700 m on a side.  Along the oceanic

boundaries of the domain, the resolution increases into the range of 2 km to 4 km.  The

grid, it should be noted, includes Pearl Harbor, Keehi Lagoon/ Honolulu Harbor and the

Ala Wai Canal, allowing better representation of water quality loadings from these areas.

The offshore extent of the modeled domain extends out to roughly the 500 m isobath.

This depth corresponds to the shelf break for the Oahu island topography and is a good

location for a modeling open boundary.  The shelf break is where the low-frequency,

open-ocean forcing is decoupled from on-shelf dynamics.  The distance from the outfall

to the 500 m isobath is sufficient to incorporate the entire effluent plume within the study

area.  Overall, the study area spans a distance 10-15 km offshore.

The vertical resolution is accomplished by use of ten equally-spaced levels so that

important physical processes operating in the vertical plane are resolved.  These levels

provide a reasonable resolution of the temperature structure, and the important surface

Ekman and bottom frictional mixed layers.

Figure 3-1.	Numerical grids for hydrodynamic model (top panel) and pathogen fate

and transport model (bottom panel, shaded area).

3.2.2  Hydrodynamic Model

3.2.2.1  Model Development

For this study, the three-dimensional, time-dependent coastal ocean circulation

model developed by Blumberg and Mellor (1980) is used for computing the transport in

Mamala Bay.  This model is being used by over 80 research groups around the world for

simulations of various hydrodynamic systems.  The history of  the model begins with the

development of analytical turbulence closure models of small-scale turbulence at

Princeton University (Mellor, 1973; Mellor and Yamada, 1974 and 1982) such that

vertical mixing or the inhibition of vertical mixing of momentum, temperature and

salinity (or any other ocean property) can be predicted with considerable confidence.

Building upon the turbulence model, the first version of the circulation model, now called

ECOM, was created by Blumberg and Mellor (1980) based upon the formulations

previously developed for vertically (Blumberg, l977a) and laterally (Blumberg 1977b)

integrated models of estuarine dynamics.

Over the past 15 years, further development has proceeded to expand model

capabilities, improve skill and reduce costs.  Radiation boundary conditions have been

developed to improve treatment of open boundaries (Blumberg and Kantha, 1985).  The

introduction of a heat flux coefficient in the surface boundary conditions has resulted in

more realistic thermal layer behavior (Blumberg and Mellor, 1985).  Refinements in the

formulation of the diffusivity terms have increased accuracy in regions of rapidly

changing topography (Mellor and Blumberg, 1985).  In addition, the model has been

completely recast in orthogonal curvilinear coordinates (Blumberg and Herring, 1987),

greatly increasing model efficiency in treating irregularly shaped boundaries and

requirements for high resolution in specific local regions.  Finally, the model was

modified such that the barotropic pressure gradient in the momentum equations and the

horizontal velocity divergence in the continuity equation are treated implicitly (Casulli,

1990).  This improvement provides a significant increase in computational efficiency

because the semi-implicit algorithm allows time steps that greatly exceed the limit of the

CFL condition.

The model is three-dimensional, incorporating a turbulence closure model to

provide a realistic parameterization of the vertical mixing processes.  The prognostic

variables are the three components of velocity, temperature, salinity, turbulence kinetic

energy and turbulence macroscale.  The momentum equations are non-linear and

incorporate a variable Coriolis parameter.  Free surface elevation is also calculated

prognostically, with only some sacrifice in computational time so that tides and storm

surge events can also be simulated.  Other computed variables include the density,

vertical eddy viscosity and vertical eddy diffusivity.

The skill of ECOM has been extensively assessed in over 30 studies of estuarine

and coastal ocean regions.  Previous recent applications include Chesapeake Bay

(Blumberg and Goodrich, 1990), Massachusetts Bay (Blumberg, Signell and Jenter,

1993), Georges Bank (Chen and Beardsley 1995), the Oregon Continental Shelf (Allen,

Newberger and Federiuk, 1995) and the Gulf Stream Region (Ezer and Mellor, 1992).  In

a recently completed study (Blumberg and Pritchard, 1995), the model was verified using

long term deployments of acoustic doppler current profilers (ADCP) at two locations in

the East River, New York for tidal through seasonal time scales.  The model is also

actively being applied in an operational forecasting mode for the East Coast of the United

States, the Great Lakes and in Norwegian coastal waters.

Three modifications to the basic ECOM structure were made for this application

to Oahu/Mamala Bay.  The first of these involved changing the code so that cyclic lateral

boundary conditions could be employed and whole-island physics could be properly

addressed.  A hierarchy of tests were conducted on an idealized circular island, which

approximates the geometry and latitude of Oahu, to insure that both transient and steady-

state current patterns are simulated correctly by the model.

Another modification was required because of the role that internal waves play in

Mamala Bay.  The temperature records are extremely variable and complex because of

westward propagating events.  Given the limited offshore temperature data, there was

little hope for reproducing these complex events with the model.  Instead, the model code

was changed so that data were used to specify temperature in a diagnostic fashion;

temperature was not calculated directly.

The last change to the model was to incorporate a capability for simulating near

field plume dynamics.  The near field plume model, developed in Project MB-4, used

oceanographic data collected at the two outfall locations and outfall configuration data to predict the time-variable structure of the effluent plume in the water column.  Information

generated by the near field model (plume trap height and thickness) was then used by the

far field model to determine the time-variable vertical distribution of the outfall plume at

each of the diffusers in Mamala Bay.  This method produced the most realistic and

accurate simulation of the near field plumes in the far field fate and transport model that

is presently possible.

3.2.2.2  Forcing Functions

Hydrodynamic simulations of Mamala Bay require boundary condition data for

input to the model.  These data, called forcing functions, are needed to drive the

circulation for the whole-island model: water surface elevation; water temperature and

salinity; wind speed and direction; atmospheric pressure; and freshwater inflows from

land and outfall sources.  The forcing functions can be temporally and spatially variable.

Proper specification of the forcing functions is important because these data are the

primary factors controlling the predictive capabilities of the model.

The water surface elevation must be specified along the entire outer, or open,

boundary of the numerical grid that encircles Oahu (Figure 3-1).  The field program

(Project MB-6) deployed bottom mounted pressure gauges at the A2, B2, HI and HA

sites in Mamala Bay and around the island (see Figure 2-1 and Table 2-1 for station

locations).  There were two additional tide gauges, at Honolulu Harbor (HON) and in

Kaneohe Bay (MOK) maintained by NOS/NOAA.  To provide a consistent basis for use

in the hydrodynamic model, these two different types of measurements of the same

quantity, water surface elevation, have been converted to subsurface pressure (SSP).

Estimates of SSP from tide gauges and from bottom pressure sensors agree very well in

frequency bands corresponding to periods from tidal through 30 days (Harms and

Winant, 1994).  Periods beyond 30 days exhibited little energy in Mamala Bay (see

Project MB-6 Report) so that the combined set of SSP observations could be used with

confidence.

The water level forcing function, which is temporally and spatially variable, can

be decomposed into tidal and low frequency bands.  The tidal band was isolated from the

time series through the application of a 40-hour high pass filter.  The assumption was

made that the tidal band could be translated along a transect normal to the shoreline to a

corresponding location on the open boundary.  The low frequency band was translated

out in a similar manner except that only a portion of the amplitude was taken.  This band

is dominated by coastally trapped waves generated by atmospheric and offshore oceanic

events with characteristic amplitudes which increase as the shoreline is approached

(Brink, 1991).  To avoid false amplification of the shoreline measurements, half of the

measured low frequency signal was used offshore for the trade wind dominate conditions

(April - December 1994).  During strong Kona winds, a factor of 0.3 was used (January -

March, 1995).  The total water level signal was reconstructed by combining the results of

translated tidal and low frequency bands.  Observations from five of the six sites were

used in this manner; observations from the Honolulu Harbor (HON) site were saved for

use with the validation of the model results.  Finally, water levels at each grid cell along

the open boundary were determined by linearly interpolating between the five open

boundary cells that had water level information.

Temperature and salinity variations affect the currents in Mamala Bay and the

near field behavior of the outfall plumes.  The temporal variability of temperature in the

bay is very dynamic due to large internal waves; conditions at the outfalls can range from

vertically homogeneous to strongly stratified during a tidal cycle.  Temporal variations of

the vertical distribution of temperature in the upper 70 meters of Mamala Bay, which

affects the transport of outfall effluent, were specified in the hydrodynamic model using

thermistor data collected at the Sand Island and Honouliuli outfalls.  The model did not

directly calculate variations in temperature but used data to specify these distributions

instead.  This method ensured that the effects of thermal stratification on circulation in

the upper layers of the water column are simulated realistically by the hydrodynamic

model.

Thermistor strings at stations B2 and D2 measured the vertical distribution of

temperature every 30 or 60 minutes during the model calibration period.  These data were

prepared for use in the model by first passing the raw data through a three-hour low pass

filter.  Next, it was assumed that the instantaneous temperature in Mamala Bay is

horizontally constant at any particular depth, which is a good assumption given that the

coherency squared between stations B2 and D2 is about 0.6.  The vertical distribution of

temperature in the upper 70 meters was then specified by averaging instantaneous

temperature measurements at common depths at stations B2 and D2.  The common

depths used for the spatial averaging process were: 15, 25, 35, 45, 55 and 65 m.  The

hourly variation of temperature at these six depths was used to specify the vertical

temperature distribution for the whole island model.  The temperature at depths greater

than 65 m was set equal to the 65-meter temperature.

Salinity in Mamala Bay is approximately constant both spatially and temporally;

salinity gradients in the ocean waters around Oahu minimally affect the circulation

patterns.  Thus, the salinity value at the open boundary was held constant at 35 psu.

Freshwater inflows from the outfalls and Oahu streams can have significant impact on the

circulation near these sources (near the Ala Wai Canal). The transport and fate of

pathogens associated with these freshwater sources could also be strongly affected by

density driven currents.  Therefore, the transport of salinity was calculated so that these

effects can be realistically simulated.

The wind exerts a stress on the sea surface that generates currents; wind-generated

currents can be important in the upper layer of Mamala Bay.  Wind speed and direction,

and atmospheric pressure data measured every hour at Honolulu Airport were used as

model inputs.  Wind data collected at the airport during the calibration period are

presented on Figure 3-2 after being 40-hour low-pass filtered. Trade wind conditions

occur most of the time with an occasional shift to Kona conditions.  For input to the

hydrodynamic model, the wind field around Oahu was assumed to be spatially constant at

any particular time.  The high coherencies in the wind data collected at Barbers Point,

Kaneohe Bay and Honolulu (see Project MB-6 Report) indicated that this is a valid

approximation.  Wind stress is calculated in the hydrodynamic model from the wind

speed using the method developed by Large and Pond (1981).

As mentioned previously, freshwater enters the Mamala Bay system from the two

outfalls and from streams on Oahu.  Effluent flow rates are specified in the model using

data from the Sand Island and Honouliuli Wastewater Treatment Plants; the freshwater

flow from the plants varies hourly.  Freshwater from six streams that are tributaries to

Mamala Bay are included in the simulation.   Three streams flow into Pearl Harbor; two

streams enter the Honolulu Harbor/Keehi Lagoon; and one creek flows into the Ala Wai

Canal.  The time-variable flow rates in these tributaries were determined by Project MB-3

investigators using a watershed runoff model.

3.2.2.3  Skill Assessment

The capability of the hydrodynamic model to predict water surface elevations,

currents and advective/dispersive transport processes in Mamala Bay was determined by

comparing model results to data.  Temporal and spatial profiles were used to make these

comparisons.  Data collected by Project MB-6 and during a special dye release study

were used to evaluate model performance.

3.2.3  Pathogen Fate Mode

3.2.3.1  Analysis of Data

The goals of the data analysis were:

(1)	to provide a picture of spatial and temporal patterns of indicator organism

counts at beaches in Mamala Bay;

Figure 3-2.	Wind data collected at Honolulu Airport during the calibration period,

April 15, 1994 through April 15, 1995.

(2)	to relate counts to processes that potentially can affect bacterial levels, in

particular, rainfall and wind direction;

(3)	to relate counts at target sites (beaches) to counts at potential sources

(waste water treatment plants and shoreline sources); and

(4)	to compare counts observed during the period of model simulation with

patterns observed historically.

Two primary sources of data were used:  data retrieved from the STORET

database, covering the years 1973-1993 (hereafter referred to as STORET data) and data

collected during 1993-1995 as part of this project (hereafter referred to as MBSC data).

Data from all stations in Mamala Bay were analyzed.  Data from four beach stations, Ewa

Beach, Sand Island Beach Point #1 (the eastern end of Sand Island), Ala Moana Beach,

and Tavern Beach, are considered here because they represent several key regions of

Mamala Bay and these stations are used in model/data comparisons (see Figure 2-1 and

Table 2-1 for station locations).  Important differences in bacterial levels between these

stations and other locations are noted below.

Data analysis focused on the major microbial groups that have been used as

indicators of fecal pollution: fecal coliforms and enterococci.  Patterns in measurements

of both of these have been analyzed; data for fecal coliforms are discussed below.

Patterns in enterococci levels are qualitatively similar to those of fecal coliforms,

although the degree of variation in the data is greater.

Historically, two methods of analysis of fecal coliforms have been used:

membrane filter (MF) and most probable number measurements using EC medium (EC) .

In the MBSC project, the MF technique was used.  On average, EC and MF

concentrations are equal, based on historical samples in which both techniques were used.

Therefore, in the analysis discussed below, membrane filter values were combined with

EC medium values.

Figure 3-3.	Conceptual model of pathogen fate

3.2.3.2  Model Development

The conceptual model of enteric organism fate in Mamala Bay is illustrated on

Figure 3-3.  Organism abundances are determined by rates of input (loads) to the bay

from outfall and shoreline sources, transport due to currents and dispersion and loss

processes that include phototoxicity, predation, starvation and settling.  A mathematical

model was developed from this conceptual model by application of the principal of

conservation of mass and empirical mathematical expressions describing each of the loss

processes.  The model was applied to three indicator organisms: fecal coliforms,

enterococci and Clostridium perfingens.  In addition, it was applied to particulate matter

(total suspended solids, TSS) and biochemical oxygen demand (BOD).  TSS was

modeled because of its presumed affect on light attenuation in nearshore areas during

storms.  The bay was modeled using the portion of the hydrodynamic grid illustrated by

the shaded region of Figure 3-1 with the transport (currents and dispersion coefficients)

determined by the hydrodynamic model.

The phototoxicity component of the loss rate was modeled as a second-order

kinetic process dependent on irradiance and organism abundance.  Project MB-7 studies

of sunlight inactivation of E. coli, enterococci and Clostridium in Mamala Bay water

were the basis for the rate constants of 0.80 m2/MJ for fecal coliforms, 0.45 m2/MJ for

enterococci and 0 for Clostridium that were used in the model (see Appendix E).  These

values are similar to values  reported for other marine waters (Davies-Colley et al., 1994;

Gameson and Gould, 1985).  Hourly measurements of surface irradiance at Kuria were

used with the rate constants to compute surface phototoxicity rates.

The decline of phototoxicity with depth in the water column was considered to be

proportional to the decline of photosynthetically active radiation (PAR).  Diffuse

attenuation coefficients for PAR (Kpar) were measured in Mamala Bay as part of a study to

evaluate water quality impacts of treatment plant discharges to the bay (M&E Pacific,

Inc., 1989).  Measurements were taken several times per year from October 1982 through

February 1989 at 20 stations in the bay.  Mean Kpar values are similar at all the deep water

stations (i.e., those in the vicinity of the outfalls and away from the outfalls along the

outfall isobaths).  They range from 0.034/m to 0.076/m with an overall mean of 0.054/m.

In contrast, the shallow water stations (i.e., 5 stations along the 10 meter isobath) have

much greater attenuation, with station means ranging from 0.137/m to 0.289/m and an

overall mean of 0.212/m.

The Kpar data were incorporated in the model in the following manner.  The

offshore average was assigned to the portion of the bay with depths greater then 10 m.

Near shore grid elements with water column depths of about 10 m were assigned the near

shore average.  This average was not extrapolated to shore because of the presumed

greater attenuation in bay grid elements adjacent to the shoreline and elements in Keehi

Lagoon, Honolulu Harbor and Pearl Harbor.  Attenuation coefficients in these elements

were estimated from a spectral attenuation model considering attenuation due to clear

water, dissolved organic matter (DOM), chlorophyll pigments and non-chlorophyll

particulates (Appendix E).

The loss rate of indicator organisms due to predation was calculated from

measured abundances and estimated clearance rates of heterotrophic flagellates.  Marra et

al. (1988) measured flagellate abundances in Mamala Bay of about 103/ml, consisting of

chrysophytes 4-6 mm in diameter.  This abundance is equivalent to a biomass of about

0.01 mg C/l.  The clearance rate of the flagellates was estimated to be about 25 l/mg

flagellate C-d.  This value was calculated from a representative physiological maximum

volumetric clearance rate of 105 cell volumes per hour (Fenchel, 1987), an assumed cell

volume of 100 mm3 and an assumed carbon content of 10-10 mg C/mm3.  A maximum rate

was presumed because of the low prey carbon concentrations [picophotoplankton of

about 0.0005 mg C/l (Laws et al., 1984) and bacteria of about 0.01 mg C/l (based on

abundance of 106/ml reported by Marra et al., 1988)].  The first order loss rate due to

predation is the product of the biomass and clearance rate (0.25/d).

Inactivation of organisms due to starvation was not considered because of

uncertainties in defining substrate concentrations and substrate uptake rates.  The

persistence of culturable cells in dark incubations indicates that this loss mechanism

contributes little to inactivation rates (Davies-Colley et al., 1994).

Particulate matter entering the bay from the Sand Island and Honouliuli outfalls

and the shoreline sources was assumed to be comprised of flocculent, fine-grained

particles.  A settling speed of 0.008 cm/s was assigned.  This value is the approximate

average of speeds measured in salt water over a wide range of floc diameters (Burban et

al., 1990).  Particle-associated indicator organisms were not considered under the

assumption that they would be deposited in the proximity of the sources and not represent a significant component of counts at the beaches.  Therefore, settling losses were applied

only to TSS.

BOD oxidation was assigned a single rate constant under the assumption that the

reactivity of the organic matter entering from all sources was similar.  A value of 0.2/d

was used as characteristic of organics in primary and secondary effluent (Thomann and Mueller, 1987).

Daily discharges of the organisms (cfu/d), TSS and BOD (kg/d) to Mamala Bay

from each of the contributing watersheds were estimated in Project MB-3.  These

loadings were incorporated in the model with one exception.  The indicator organism

loadings to Ala Wai Canal were insufficient to generate indicator organism counts

comparable to those observed in Ala Wai Canal at Ala Moana Bridge.  A fairly

substantial database exists at this station as a result monthly and quarterly sampling and

three intensive studies.  As an alternative to the loadings estimated in Project MB-3, this

database was used in conjunction with the water flow into and out of the canal calculated

by the hydrodynamic model (see Section 4.1.1) to determine indicator organism

discharges from the canal.

To establish daily indicator organism concentrations in the canal, the surface (0-2

ft) measurements of indicator organism counts were grouped according to the previous 72

hour cumulative rainfall measured at Honolulu Airport.  Three groupings were used: less

than 0.03 inches; 0.03 to 0.3 inches; and greater than 0.3 inches.  The frequency

distributions of the grouped data (Figures 3-4 to 3-6) were sampled using a Monte Carlo

procedure to determine a continuous record of concentrations at the Ala Moana Bridge

from April 15, 1994 to April 15, 1995.  Each day of the record a value was chosen from

the distribution applicable to the 72 hour cumulative rainfall for that day.   The resulting

record of daily concentrations was assigned to the surface layer at the bridge.

Comparisons between the distributions of concentrations chosen in this manner and those

observed at the bridge are shown in Figure 3-7.  In addition to reproducing the statistical

distribution of concentrations, the computed values maintain the relationship to rainfall

evident in the data.

The surface layer concentrations were assumed to extend from the surface to a

depth of 4 feet.  This depth was chosen based on vertical profiles of indicators and

salinity obtained in intratidal studies conducted in March and July, 1994.  From 4 feet to

the canal bottom the concentrations was assumed to be uniform at a concentration

proportional to the surface layer concentration.  Again, the vertical profiles were the basis

for assigning a uniform concentration over this depth.  The proportionality between

surface and bottom layers was determined by regression of paired data (Figure 3-8).

The discharges from the Sand Island and Honouliuli treatment plants were

obtained from Project MB-11.  The distribution of these discharges within the water

column were determined from near-field models of the outfall plumes (Project MB-4).

Figure 3-4.  Cumulative probability distributions of fecal coliform, enterococci and

Clostridium counts measured in Ala Wai Canal at the Ala Moana Bridge for 72-

hour cumulative rainfall at Honolulu Airport < 0.03 inches.

Figure 3-5.  Cumulative probability distributions of fecal coliform, enterococci and

Clostridium counts measured in Ala Wai Canal at the Ala Moana Bridge for 72-

hour cumulative rainfall at Honolulu Airport > 0.03 and < 0.30 inches.

Figure 3-6.  Cumulative probability distributions of fecal coliform, enterococci and

Clostridium counts measured in Ala Wai Canal at the Ala Moana Bridge for 72-

hour cumulative rainfall at Honolulu Airport > 0.30 inches.

The discharge estimates do not differentiate between free-living and particle-

associated organisms.  In the absence of data, the conservative assumption that all

organisms are free-living was invoked.  Thus, the model estimate of the impact of the

sources on the beaches may contain an upward bias.  This bias is counter-balanced by

entrainment of settled organisms into the water column, a process also ignored because of

data limitations.

Discharge of organisms by bathers was not considered.  A series of diel studies,

conducted as part of Project MB-7, found no evidence of a bather source in six

experiments (3 beaches at 2 times) and an apparent relationship in one experiment.

3.2.3.3  Skill Assessment

The capability of the model to predict the abundance of fecal coliform,

enterococci and Clostridium at the beaches was assessed by comparisons to data.

Probability distributions and time series of abundance were used as the bases of

comparison.  These comparisons were conducted using data collected by Project MB-7

and historical data collected by the Hawaii Department of Health.

3.2.3.4  Analysis of Source Contributions to the Beaches

The model was used as a diagnostic tool to examine the relationship between

organism abundance at the beaches and organism discharges from each of the sources.

The abundances calculated at the beaches were partitioned into the contributions from

each of the sources.  These contributions were contrasted by presenting the fractional

contribution of each source to the level computed at the beach.

Figure 3-7.  Comparison of the cumulative probability distributions of indicator organism

counts measured in Ala Wai Canal at the Ala Moana Bridge and estimated using

the Monte Carlo procedure.

Figure 3-8.  Relationship between indicator counts measured in the surface (0-4 ft) and

bottom (>4 ft) waters of the Ala Wai Canal at Ala Moana Bridge.  The regression

lines shown in the plots were used to determine bottom water counts from surface

water counts.

4  RESULTS

4.1  Hydrodynamic Circulation

4.1.1  Comparison of Model and Data

The horizontal eddy viscosity was the model parameter adjusted during the

calibration of the model.  It was determined during the calibration process that the

horizontal eddy viscosity had to be spatially variable to achieve physically consistent

results.  A low viscosity value of 0.5 m2/s was used in Mamala Bay and a higher value, 5

m2/s, was applied outside of the bay and also along the open boundary.  The eddy

viscosity values are primarily determined by grid spacing because viscosity represents

those physical processes occurring at scales smaller than the grid can resolve.  Nearshore,

the grid cell spacing in the model is about 0.5 km and, using the rule of thumb that 6-8

grid cells are required to resolve realistic processes, the model should resolve mixing

processes occurring at scales of 3-4 km.  Offshore, where the grid spacing is of order 2-4

km, processes can be resolved at scales of 20 km.  From Okuko (1971), horizontal mixing

at 3-4 km scales should be less than about 1 m2/s and at 20 km scales it should be about

10 m2/s, values not unlike those used here.  Because the pathogen fate model uses the

same grid as the hydrodynamic model and is driven by transports which resolve the

proper temporal scales of motion (hourly basis), the horizontal eddy diffusivity was

assumed to be equal to the eddy viscosity.

The calibration period extended over a yearlong period, from April 15, 1994

through April 15, 1995.  A fifteen day spinup prior to April 15, 1994 was used to remove

initial transients due to model startup.  Comparisons between model predictions and data

have been made to assess the ability of the hydrodynamic model to simulate advective

and dispersive processes in Mamala Bay.  Model evaluation has been conducted using the

following parameters: water surface elevation, current velocities, temperature, salinity

and conservative tracer (dye release).  Representative results are discussed in this section,

all of the model/data comparisons are included in Appendix A.

Predicted water surface elevations at five locations around Oahu are compared to

measured values for a thirty-day period in August 1994 on Figure 4-1.  Very good

agreement between predicted and observed values was achieved at all five locations.

Relative errors are less than 3%.  Particular attention should be paid to the Honolulu

station on Figure 4-1 because it was not used in deriving the offshore water elevation

forcing functions.  Comparisons at the other stations are useful, in any case, as they

demonstrate the overall consistency of the modeling framework.  The predicted low

frequency (40-hour lowpass filter) elevations for the entire calibration period are shown

on Figure 4-2.  In general, the agreement is quite good although there are some instances

where the model underestimates the low frequency fluctuations by 4 to 5 cm.  These

results indicate that: (1) translation of measured water surface elevations from the near

shore gaging stations to the open boundary is a valid approximation and (2) bathymetry

and bottom friction parameters have been specified accurately in the model.

The next model parameter to be evaluated is the currents.  In the ocean, currents

tend to flow along the bathymetric contours, so it becomes useful to rotate both the

current observations and predictions into alongshore (along the bathymetry) and cross

shore (normal to the bathymetry) components.  The alongshore current is typically the

much larger of the two.  Figure 4-3 contains  a comparison over a 90-day period,

October-December, 1994, at Station CO between the observed and predicted alongshore

and cross-shore currents.  This station is located near the mouth of the Ala Wai Canal and

is the closest station to the beaches.  The model appears to reproduce the data quite well

both in terms of current magnitude and tidal phase.

Moving out into Mamala Bay, comparisons along transects B and D are provided

next.  The alongshore current component at Stations B1, B2, B3, D1, D2 and D3 is shown

for surface (Figure 4-4), mid-depth (Figure 4-5) and near bottom (Figure 4-6) waters.

The time period corresponds to that of the water elevation comparisons, August 1994,

shown on Figure 4-1.  The agreement between model and data is quite reasonable given

our level of understanding of the circulation physics.

Figure 4-1.  Comparison Between Predicted and Measured Water Surface Elevations at

Five Locations Around Oahu During August 1994.

Figure 4-2.  Comparison Between Predicted and Observed Low Frequency Water Surface

Elevations for the Calibration Period, April 15, 1994 through April 15, 1995.

Figure 4-3.  Comparison Between Predicted and Measured Currents at Station C0 during

October-December, 1994.

Figure 4-4.  Comparison Between Predicted and Measured Alongshore Surface Currents

During August 1994.

Figure 4-5.  Comparison Between Predicted and Measured Alongshore Mid-Depth

Currents During August 1994.

Figure 4-6.  Comparison Between Predicted and Measured Alongshore Bottom Currents

During August 1994.

The dynamics of the Mamala Bay system (Task MB-6 Report) suggest that for

pathogen transport considerations the most relevant parameter that the hydrodynamic

circulation must reproduce is the observed variability in the currents.  This is because the

magnitude of the current fluctuations exceeds the mean current at every station where

measurements were taken (see Figure 2-1 and Table 2-1 for station locations).  The

variance (the measure of variability used here) of the circulation is produced by tides

propagating around Oahu in response to astronomical gravitational forces of the moon

and sun and by longer period (greater than 40 hr) forcings, such as the wind, various

types of large scale waves and the influences of the currents in the surrounding ocean.

The model's ability to correctly simulate the tidal dynamics of Mamala Bay is

illustrated by tidal current ellipses for near surface (Figure 4-7) and 70 m depth (Figure 4-

8) velocities.  The use of an arrowhead on the ellipses is to denote the rotational sense of

the tidal wave propagation.  Tidal flows, in both the model predictions and observations,

dominate the currents at Barbers Point and Diamond Head and diminish to very small

speeds in the vicinity of transect B.  The strength of the headland currents is only partially

reproduced by the model, however.  During flood and ebb tides, the model captures the

currents at the headlands flowing in opposite directions very well.  The spatial structure

and rotational sense of the ellipses appear quite similar in the model and data, indicating

that the tidal wave propagational characteristics are well simulated.

In general, within Mamala Bay (stations B1, B2, C0, D1, D2) the relative average

error between the predicted and observed alongshore tidal variance is about 5%.  The

cross shore variance is predicted with less than 7% relative average error.  The error tends

to increase with depth and movement offshore into deep water, where the errors are 32%

for the alongshore variance and about 55% for the cross shore variance.  The greatest

error is in the deep waters offshore of Diamond Head.  The character of the error is that

the model predictions almost always underestimate the observed tidal currents.  Given

this good level of calibration, the model physics will incorporate the large changes in

stratification occurring on tidal cycle time scales (Project MB-6 Report) at both outfall

sites in a realistic fashion.

The model is also able to reproduce the variance of the longer period motions.

The 40-hour low pass variance ellipses of the alongshore and cross shore velocities for

both the model results and observations are shown on Figures 4-9 and 4-10.  Near surface

ellipses are shown first, followed by those at 70 m depth.  As observed, the major

principal axes of the modeled currents align with the local topography.  An exception is at

station B3, where the observed axes rotate counterclockwise with depth while the

modeled axes do not.  There is a tendency in both the model results and observations for

the variance to increase in magnitude in the offshore direction and to decay with depth.

Within Mamala Bay, the relative average error for the alongshore variance is about 10%

while the cross shore variance error is less than 1%.  The error does increase offshore but

to a much less extent than for the tidal band.  There is no bias to the error in the 40-hour

low pass band, sometimes the model overestimates the observations and sometimes it

underestimates the observations.

As discussed in Section 3.2.2.2, the time-dependent temperature distribution was

specified in the model using average values of data obtained at stations B2 and D2.  The

validity of this averaging process was examined by comparing temperatures used in the model with measured values at B2 and D2.  As shown on Figure 4-11, for the August

1994 period, there is excellent agreement between surface temperatures at both locations.

This result is not surprising because surface temperatures exhibit little variability during

a tidal cycle.  The averaging process tends to reduce some of the variability in the bottom

temperatures, which are at depths of 65 to 70 m.  The amplitude of the temperature

oscillation in the model is smaller than that indicated by the data during some tidal

cycles.  However, the model generally reproduces the time-variable vertical temperature

stratification in an adequate manner.

Figure 4-7.  Tidal Current Ellipses for Measured (top) and Predicted (bottom) Surface

Currents for the Period April 1 to December 26, 1994.

Figure 4-8.  Tidal Current Ellipses for Measured (top) and Predicted (bottom) 70 m

Currents for the Period April 1 to December 26, 1994.

Figure 4-9.  Forty-hour, Low-pass Variance Ellipses for Measured (top) and Predicted

(bottom) Surface Currents for the Period April 1 to December 26, 1994.

Figure 4-10.  Forty-hour, Low-pass Variance Ellipses for Measured (top) and Predicted

(bottom) 70 m Currents for the Period April 1 to December 26, 1994.

Figure 4-11.  Comparison Between Measured and Model Input Temperatures at Stations

B2 and D2 During August 1994.

Figure 4-12.  Dye Injection Points in the Ala Wai Canal and Grid Cell Reference

Locations for Model/Data Comparison.

Additional validation of the ability of the hydrodynamic model to simulate the

advective and dispersive processes in the near shore region of the eastern portion of

Mamala Bay was accomplished using data obtained from a dye tracer study.  Rhodamine

dye was injected into the surface layer of the Ala Wai Canal during the evenings of April

2 and 4, 1995, see Figure 4-12 for dye injection locations.  After each injection, the ebb

tide transported the dye out of the canal and into Mamala Bay.  The dye concentration

was measured at various locations in Mamala Bay and at four stations on Ala Moana and

Waikiki Beaches for about twelve hours after injection.  The dye study is discussed in

more detail in Appendix B.

The dye concentration data resulting from this field study were then used to

validate the hydrodynamic model.  This task was accomplished by predicting the

transport of dye, which is a conservative tracer, in Mamala Bay during the four-day

period of interest, from April 2 through April 5.  Dye data were used to specify the time-

variable tracer concentration (dye loading) in the surface layer of the Ala Wai Canal

during the simulation.

A comparison between predicted and observed dye concentrations in Mamala Bay

is presented on Figure 4-13.  The six panels on this figure represent the time history of

dye concentration in different grid cells of the model.  Locations of the six cells examined

here, Cells A to F, are shown on Figure 4-12; these cells are immediately adjacent to the

shore and Cell C is at the mouth of the canal.  Agreement between model and data is very

good; the model accurately reproduces the spatial and temporal trends of the dye plume in

these near shore cells.  Similar success was obtained at other locations in Mamala Bay,

see Appendix B for additional comparisons.  These results lend strong support to the

capability of the model to transport pathogens in the near shore regions of Mamala Bay

accurately.

The last aspect of model validation to be discussed here concerns the circulation

and mixing near the mouth of the Ala Wai Canal.  Data from an intensively sampled 24-

hour period on July 6-7, 1994 was provided by Project MB-3.  These data consisted of

hourly measurements of currents and various water quality parameters collected

throughout the water column, at the Ala Moana Bridge and were used to help estimate

pathogen loadings to the bay.  Figure 4-14 compares the quantity of water flowing under

the Ala Moana Bridge (station AW1 on Figure 2-1) for the 24-hour period as computed

by the model to what was measured.  The volume of water and its temporal variability

over two tidal cycles is very well modeled.  The amount of salt being transported with the

currents was also estimated for the same period.  Comparisons of the predicted and

measured salt flux under the bridge are provided on Figure 4-15.  The model is able to

correctly simulate the movement of salt in the canal.  From these two comparisons, one

can conclude with considerable confidence that the exchange of water and materials

between the canal and the coastal waters is well represented by the model dynamics.  In

other words, pollutant loadings in the canal will be delivered to the nearshore waters of

the bay correctly.

Before leaving this section, some comments on the overall calibration are in order.

The uncertainties within the model result from an inadequate understanding of the

physical oceanography, trade-offs required to balance grid resolution and size of the

region under consideration, and knowledge of the boundary conditions.  In the sections to

follow, each of these uncertaintinties will be discussed.

Circulation patterns in Mamala Bay are complex with substantial changes

occurring over short distances and times.  These patterns are quite different than might be

expected from other coastal regions.  Without a more thorough understanding of the

processes governing the circulation, it is difficult to improve the calibration of the model.

For example, the waters of the bay have little horizontal temperature variability but show

quite strong temporally-variable vertical gradients below 50 m.  The presence of this

temperature structure and strong tides leads to internal waves and topographically trapped

wave modes.  These processes are known to lead to vigorous mixing along the sloping

bathymetry; they have been largely ignored in this modeling analysis.  The only way to

include these baroclinic effects in the model would be to obtain high quality temperature

measurements on an hourly basis in the offshore waters.

Figure 4-13.  Comparison Between Measured and Predicted Dye Concentrations at Six

Grid Cell Near the Ala Wai Canal.

Figure 4-14.  Comparison Between Measured and Predicted Flow Rates at the Ala Moana

Bridge on July 6-7, 1994.  Note that +/- Denotes out of/into Ala Wai Canal,

respectively.

Figure 4-15.  Comparison Between Measured and Predicted Salt Flux at the Ala Moana

Bridge on July 6-7, 1994.  Note that +/- Denotes out of/into Ala Wai Canal,

respectively.

The grid dimensions are typically chosen to balance overall model resolution with

computational cost; a greater number of grid boxes improves spatial coverage but

increases the computational burden.  The present grid structure of Mamala Bay is

adequate but probably does not cover enough of the offshore waters.  By expanding

oceanward, the physics of internal tidal waves generated by the interactions between the

barotropic, deep ocean tide and the Hawaiian Ridge may be incorporated into the model.

Boundary conditions must be provided to the model.  The limitations in our

understanding of the boundaries are reflected in the model results.  The key forcing

mechanism in the model is the water surface elevation.  Here, it is deduced from

measurements of bottom pressure and used dynamically in terms of subsurface pressure

(SSP).  An examination of bottom pressure measurements suggests that considerable

error may be present in the data since seemingly large gradients of SSP (water level

gradient equalivants of 10 cm or so) exist for periods up to five days.  Better

measurements would go far to improving the model results.

Given the favorable comparisons between the modeled and observed water levels,

currents, dye concentration temperatures and salinities, the hydrodynamic model

calibration appears to be quite satisfactory.  As it is true for all mathematical simulations

of nature, there are uncertainties and considerations to be kept in mind when interpreting

the results of the simulations.  However, none of these limitations preclude the confident

use of the hydrodynamic model results in pathogen transport and fate model

computations.

4.1.2  Description of Currents

Results of the model calibration show that the circulation pattern in Mamala Bay

can be complicated due to the temporal and spatial variability of the currents.  Examining

current velocities, either measured or predicted, at a few distinct locations can be valuable

but using this information to develop an understanding of the overall circulation patterns

in the bay is difficult.  However, the calibrated hydrodynamic model can be used to

investigate the circulation in Mamala Bay over a wide range of temporal and spatial

scales.  This application of the hydrodynamic model illustrates the usefulness of a model,

which has been properly calibrated and validated, to study the characteristics of a

complex coastal region.

Predictions of the circulation pattern in Mamala Bay, averaged over the yearlong

calibration period, are shown on Figure 4-16.  This long-term average is chosen because

it reveals some interesting aspects about the general transport pattern in the bay;

examination of current velocities on a tidal time-scale can obscure features of the net

circulation.  Surface currents in Mamala Bay generally flow westward with a

recirculation zone, or eddy, existing offshore of Waikiki Beach (Figure 4-16, top panel).

Maximum velocities, which occur in the shallow near shore areas, are approximately 15

cm/s.  Current velocities decrease with increasing depth, as is illustrated by the

circulation pattern at 70 m (Figure 4-16, bottom panel).  Similar to the surface circulation,

currents at this depth are primarily directed to the west.  There is a significant volume of

water flowing into Mamala Bay from the deep, offshore waters.  This flow joins the

westward currents on the shelf suggesting that the flux of water past a transect normal to

the coast at Barbers Point is greater than that past a similar transect at Diamond Head.

4.1.3  Sensitivity to Wind Data

Wind is one of the main forcing functions driving the currents in Mamala Bay.

Using wind data collected at Honolulu Airport as model input is a valid approximation, as

discussed in Section 3.2.2.2, but some uncertainty exists about the sensitivity of model

predictions to wind stress.  This uncertainty was addressed by repeating the calibration

simulation but with the wind velocity decreased by a factor of two.  Results of this

simulation showed that the surface currents decreased in magnitude, as expected, compared to the full wind case.  However, the circulation pattern in the bay was very

similar to the flow pattern during the original calibration simulation.  The currents at 70

m were nearly identical to those predicted during the calibration simulation.  These

Figure 4-16.  Predicted Average Current Distributions in Mamala Bay at Depths of 0 and

70 m for April 1994 through March 1995.

results show that only the surface layer, which is typically 20 to 30 m thick, is affected by

the wind.

4.1.4  Kona Circulation

Kona wind conditions occurred infrequently during the first nine months of the

calibration period but were prevalent during January, February and March of 1995.

These wind conditions, which are typified by variable winds that generally blow from a

southerly direction, have been thought to play a significant role in the surface circulation

pattern of Mamala Bay.  Hydrodynamic model results were examined to determine the

influence of Kona conditions on currents in the bay.

Short-term Kona periods (one to about five days in length) occurred between

April and December of 1994.  The impacts of these Kona conditions on surface currents

in Mamala Bay were noticeable but minor.  Surface currents still tended to flow eastward,

as during trade wind conditions, but nearshore currents, in water depths less than 50 m,

had a larger onshore component during Kona conditions than during trade wind

conditions.  Currents at depths greater than 50 m are minimally affected by winds.

Surface currents in the bay during January to March 1995 were significantly

affected by the Kona winds that occurred during that period.  Monthly average surface

currents in Mamala Bay during primarily trade wind (August 1994) and Kona wind

(February 1995) conditions are illustrated on Figure 4-17.  Westward flow dominates the

surface currents throughout the bay under trade wind conditions.  However, easterly

surface currents occurred in the shallow, nearshore regions, less than about 70 m depth,

during the strong Kona winds in February.  A comparison of predicted currents at 70 m

depth during August 1994 and February 1995 (Figure 4-18) shows that the general

pattern of westerly flow, at this depth is nearly unaffected by wind direction.

Figure 4-17.  Monthly Average Currents at 0 m Depth During Trade (top) and Kona

(bottom) Wind Conditions.

Figure 4-18.  Monthly Average Currents at 70 m Depth During Trade (top) and Kona

(bottom) Wind Conditions.

4.1.5  Outfall to Beach Transport Mechanisms

High fecal coliform events, when concentrations exceed background values, were

predicted by the pathogen fate model to occur on the beaches several times during the

model calibration period, see Section 4.2.4.  The model indicated that the Sand Island

outfall was the source of the high fecal coliform concentrations, which did not exceed the

beach standard of 200 CFU/100 ml, during some of these events. The hydrodynamic

model has been used to investigate the causes of the predicted high fecal coliform

concentrations during one of the events caused by the Sand Island outfall.

The event to be analyzed here, which occurred on May 22, 1994, is of special

interest because it happened under hydrodynamic conditions that have previously been

considered highly unfavorable for the transport of outfall effluent to the beaches.  Trade

wind conditions prevailed for at least one week before the May 22 event.  In addition, the

water column in Mamala Bay was stratified during this period.  The near field plume

model (Project MB-4) predicted that the Sand Island outfall had not surfaced during the

week prior to the event.  Near field model predictions showed that the average rise height

of the plume was 24 m, the maximum was 61 m and the one-percent occurrence of the

rise height was 43 m for this period.  Note that the water depth at the outfall is 70 m.

Thus, the Sand Island outfall plume was trapped in the lower half of the water column for

at least one week before the May 22 event.

Examination of the predicted currents in Mamala Bay during May 19 to 22

suggests the cause of the May 22 event.  On May 19, mean currents in eastern Mamala

Bay were directed westward, which is a typical condition.  On May 20, approximately 60

hours before the maximum fecal concentrations on the beaches were predicted to occur,

the currents in eastern Mamala Bay shifted toward the east.  Surface currents, which are

affected by the local wind,  in the shallow, nearshore region were still flowing westward

at times during this 60-hour period.   However, below the wind-driven surface layer, the

upper 20 m of the water column, eastward flowing currents were predicted throughout the

eastern portion of Mamala Bay which carry pathogens from the outfall towards the

beaches.

The cause of this change in mean current direction was a change in the low

frequency part of the water surface elevation forcing.  Prior to midnight on May 19, 1995,

low frequency (40-hour low-pass) water levels near Kaneohe Bay (Station MOK) were

higher than those in Mamala Bay (Stations A2 and E2).  When MOK levels are higher

than those in Mamala Bay, cross-shore water elevation gradients become established such

that in Mamala Bay the offshore water levels are lower than those of the nearshore

waters.  Scaling arguments for the low frequency coastal    flow fields imply that the

alongshore velocity is in geostrophic balance (Allen, 1980) with the elevation gradient

which, in this case, produces currents which flow towards the west.  After midnight, and

for the next three days, the low frequency water levels changed to a state where MOK

levels were lower than those in the bay, essentially reversing the previously existing

cross-shore elevation gradients.  Through geostrophic balance, strong eastward flowing currents in the eastern portion of Mamala Bay were generated.

It should be remarked that many such periods exist in the data when the water

levels at Station E2 are higher than at MOK.  In fact, it appears that they produce

eastward flowing currents in Mamala Bay for 1-2 day periods every 4 to 5 days.  These

events are unrelated to the local wind and are perhaps the result of coastal trapped waves

or edge waves that may be propagating along the Hawaiian ridge (see Project MB-6

Report).

Two different methods were used to analyze the mechanisms that transported

effluent from the Sand Island outfall to the beaches during this three-day period of

eastward currents.  The first procedure involved simulating the transport of a conservative

tracer released at the outfall.  The tracer was input continuously into the bottom grid

element at the outfall for five days, from May 19 through 23.  Results of this simulation

showed that the effluent plume began to be transported eastward along the 70 m isobath

on May 20.  Most of the tracer was trapped in the lower 35 m of the water column on this

day.  Stronger eastward currents on May 21 transported a large mass of tracer from the

outfall to a location offshore of Waikiki Beach.  The highest tracer concentrations were

still in the lower 35 m but some tracer was evident in the surface waters offshore of

Waikiki; tracer in the deeper water was being transported vertically to the surface waters.

During the next 12 hours, from midnight on May 21 to noon on May 22, surface currents

transported larger quantities of tracer to nearshore regions from Ala Moana to Waikiki

beaches.

The second method used to evaluate the transport of effluent to the beaches was

through the development of a particle tracking model for use in Mamala Bay. This model

can simulate the transport of discrete particles, due to advective and diffusive processes,

released from multiple sources into Mamala Bay.  The particle tracking model is an

extremely useful tool for analyzing transport processes in the bay and provides unique

capabilities for problem diagnosis that are lacking in conventional techniques, such as the

use of a conservative tracer.  A detailed description of the particle tracking model is

presented in Appendix G.

To corroborate the findings of the conservative tracer simulation, a continuous

release of particles was modeled during the same five-day period.  However, particles

were released at the Sand Island outfall at three locations in the water column: surface, mid-depth and bottom.  The locations of the particles in the upper 5 m of the water

column at noon on May 22 are presented on Figure 4-19.  Similar to the conservative

tracer results, most of the particles close to the beaches were released at mid-depth or

bottom.

The effect of turbulent diffusion on transport in Mamala Bay was also

investigated with the particle tracking model.  The five-day particle tracking simulation

was repeated but with horizontal and vertical diffusion set to zero.  This type of

simulation, i.e., no diffusion, can be difficult to do using a conservative tracer because of

numerical stability problems.  However, the particle tracking model can perform a zero

diffusion simulation with no difficulty.  Two important points can be determined from

Figure 4-20, which shows the positions of particles, transported only by advection, in the

upper 5 m at noon on May 22.  First, the impact of turbulent diffusion, both horizontally

and vertically, on transport processes in Mamala Bay is evident (compare Figures 4-19

and 4-20).  Second, particles released at the bottom are transported by advection to the

beaches whereas surface-released particles are not.

The results of these analyses suggest the following process for transporting Sand

Island outfall effluent to the beaches during stratified water column and trade wind

conditions.  First, strong easterly currents transport effluent along the 70 m isobath

toward Diamond Head.  Turbulent diffusion and advection transport the effluent, now

located offshore of Ala Moana and Waikiki Beaches, vertically from the lower half of the

water column up into the surface layer.  Advective and diffusive processes in the surface

layer then transport the effluent into the shallow, nearshore areas adjacent to the beaches.

The above simulation dealt with transport during stratified water column and trade

wind conditions, a situation that has previously been assumed to be highly unfavorable

for the transport of pathogens from Sand Island outfall to the local beaches.  Conditions

that are more favorable for pathogen transport from the outfall to the beaches occur

during winter months when the water column is weakly stratified and Kona winds blow.

The effects of Kona wind conditions on the circulation in Mamala Bay were discussed in

Section 4.1.4.  It was shown that the mean surface currents in eastern Mamala Bay are

directed from the Sand Island outfall toward Diamond Head during sustained Kona winds

(see Figure 4-17).  Stratification of the water column in Mamala Bay varied from weakly

stratified to unstratified during the months of January, February and March in 1995,

which allows the outfall plume to rise much higher in the water column than during

stratified conditions.  Thus, it should be expected that the impact of outfall effluent on the

beaches will increase during the winter when the water column is generally well mixed

and sustained Kona wind conditions occur.

Figure 4-19.  Predicted Locations of Particles in the Upper 5 m of the Water Column at

Noon on May 22.

Figure 4-20.  Predicted Locations of Particles in the Upper 5 m of the Water Column at

Noon on May 22.  Particle Tracking was Simulated with Zero Diffusion.

4.2  Pathogens

4.2.1  Analysis of Data

4.2.1.1  The Distribution of Bacterial Indicator Counts

Counts of fecal coliforms measured at the beaches of Mamala Bay range over

more than five orders of magnitude.  Sample distributions are presented on Figure 4-21.

These plots include data from STORET (open symbols) and MBSC (filled symbols).  The

horizontal dotted line represents the water quality criteria for fecal coliforms, 200 colony

forming units per 100 milliliters (200 CFU/100ml).  Data are roughly log normally

distributed (when the log of the concentration is plotted on a probability scale) the

concentration data from samples in which bacteria were detected are roughly linear.  The

causes of the great variation may include mixing processes within the bay, temporal

variation in source loadings, temporal and spatial variation in mortality and inactivation

rates (phototoxicity and predation), and measurement error.

The fecal coliform counts in the MBSC dataset were somewhat lower on average

than those of the STORET data.  The difference was within a factor of two to four at the

beach stations for which comparisons were performed (Figure 4-21), except for the Sand

Island stations.  It should be noted that the STORET station from Sand Island Point #2 is

presented on Figure 4-21;  values measured at Sand Island Point #1, on the eastern tip,

were lower than those at Point #2, within a factor of two of the MBSC data.

The frequency of violation of fecal coliform standards was calculated using the

historical STORET data.  At 13 of 20 beach stations in Mamala Bay, violations were

measured less than 5 percent of the time, and at no station were violations were measured

more than 20 percent of the time.  In contrast, violations were measured up to 98 percent

of the time at some nonbeach stations in Mamala Bay (data not shown); for example, at

the station located at the Ala Moana Bridge, violations were measured 65 percent of the

time.  The pattern of violation frequency of enterococci standards (33 CFU/100 ml) was

similar to that of fecal coliforms.

Figure 4-21.  Probability plots of fecal coliform concentrations at four beaches.  All years

combined.  Open symbols: STORET historical data; filled symbols: MBSC project

data.

4.2.1.2  Annual and Seasonal Patterns

Have bacterial indicator concentrations at the Mamala Bay beaches changed over

time?  To explore this question, a measure of bacterial concentration is needed.  From a

regulatory point of view, the frequency of standards violations is a key parameter.  At most beaches in Mamala Bay, standards have been violated in less than 5 percent of the

measurements.  This frequency is too low to allow the effective study of temporal

patterns.  Therefore, the frequency of fecal coliform counts greater that 10 CFU/100 ml

was used as a measure of "high" bacterial levels at the beaches. Hereafter, this measure of

concentration is referred to as the bacterial "level".

Annual average fecal coliform levels exhibit substantial variability.  In most cases

no trend is evident and the variability probably reflects the statistical uncertainty of

limited sampling of an event driven system.  The area of Keehi Lagoon is the one notable

exception.  Levels declined between 1973 and 1980 at stations SIP2, SIP3, SIP4 (SIP2

shown in Figure 4-22; other data shown in Appendix D).

The seasonal pattern of variation in bacterial levels may give some indication of

causation.  In general, bacterial levels appear to be lower during the summer months

(Figure 4-23), although not every station shows this pattern (Sand Island, Figure 4-23;

Appendix D).  The MBSC data are qualitatively consistent with the STORET data at Ala

Moana Beach and Tavern Beach, while no seasonal pattern is observed at Ewa and Sand

Island Beaches.  This lack of a pattern may be caused by small sample sizes.

Concentrations of contaminants at the Ala Moana Bridge show slightly lower

levels during summer months, in parallel with the beach stations, both in the STORET

and MBSC data sets (Figure 4-24, top left panel).  Total precipitation, which is correlated

with flow from shoreline sources, is markedly lower during the summer months, when

levels at the beaches are lower (Figure 4-24, top right panel).  In contrast, the bacterial

loads from the Sand Island and Honouliuli Treatment Plants are highest in the summer (Figure 4-24, bottom panels); thus, higher loads coincide with lower bacterial levels at the

Figure 4-22.  Temporal plots of the frequency of fecal coliform concentrations greater

than 10 cfu/100ml, calculated annually.  Beach stations.

Symbols as in Figure 4-21.

Figure 4-23.  Seasonal plots of the frequency of fecal coliform concentrations greater than

10 cfu/100ml, calculated monthly. All data collected since 1988 combined.  Beach

stations.

Symbols as in Figure 4-21.

Figure 4-24.  Seasonal plots of parameters related to potential sources of contamination,

calculated monthly.

Symbols as in Figure 4-21.beaches.

beaches.  The general seasonal pattern of bacterial levels at the beaches is consistent with

the pattern at Ala Moana Bridge and with total precipitation, and is opposite the pattern

of treatment plant loads.

4.2.1.3  Correlations Between Beaches and Parameters Related to Potential Sources

of Contamination

To explore the relationships between beach bacterial levels and potential sources

of contamination, the contaminant levels at two beaches on the eastern side of Mamala

Bay were plotted against rainfall, wind direction, concentration at a shoreline source (the

Ala Moana Bridge), and load from a outfall source (the Sand Island treatment plant).

Relationships for other beaches are presented in Appendix D.

The bacterial levels at the beaches are likely to represent contamination from

sources integrated over some period of time.  For this analysis, the values for rain and

wind were averaged over a three-day period leading up to the day of the bacterial

measurement, and values for shoreline and outfall sources were averaged over a five-day

period.

The bacterial levels at Ala Moana and Tavern Beaches exhibit positive

relationships to rainfall at high rainfall levels (Figure 4-25, leftmost panels).  This

suggests that more than one process may be controlling bacterial levels at the beaches:  a

low, background bacterial level that is controlled by outfall and/or shoreline sources, and

high levels related to rainfall, which is probably correlated with shoreline sources.  The

bacterial level at Sand Island Point #2 exhibits a weaker relationship with rainfall

(Appendix D).

Bacterial levels at Ala Moana and Tavern beaches exhibit a slight positive

correlation with levels at the Ala Moana Bridge, although the correlation coefficients are

not statistically significant (Figure 4-25, middle).   The correlations between levels at these beaches and loads from the Sand Island Treatment Plant are not consistently

positive or negative (Figure 4-25, rightmost panels); again, none are significant.

Figure 4-25.  Relationships between concentrations at two beaches and parameters related

to potential sources of bacteria.  Symbols as in Figure 4-21.

4.2.1.4  Conclusions

There is a great deal of variation in bacterial indicator measurements.  There does

not appear to have been an important change in the frequency of high bacterial counts

(that is, greater than 10 CFU/100ml) over the past several years.

Bacterial concentrations, as measured using the frequency of counts greater than

10 CFU/100ml, exhibit a greater correlation with factors associated with shoreline

sources than with outfall sources.  This statement is based on the following observations:

(1) bacterial levels at the beaches, bacterial levels at the Ala Moana Bridge, and

precipitation are all lower during the summer, when outfall source loads are greater; (2)

bacterial levels at the beaches are positively correlated with time-averaged precipitation;

and (3) correlations between levels at the beaches and levels at the Ala Moana Bridge are

uniformly positive, although they are statistically insignificant.

The MBSC data are qualitatively similar to the STORET historical data, based on

temporal patterns, seasonal patterns, and relationships to rainfall, where sufficient data

are available to observe patterns.  The MBSC data tend to be a factor of two to four lower

than the STORET historical data.

4.2.2  Zone of Influence of the Outfalls

The dynamic current patterns in Mamala Bay result in a temporally variable

dilution pattern of pollutants discharged from the treatment plant outfalls.  Over long time

scales the pollutants are distributed over a wide area.  One way to examine the

distribution of pollutants is to consider long-term average concentrations throughout the

bay.  To generate a generic distribution pattern, the model was used to compute the long-

term average (April 15 - December 30, 1994) concentrations resulting from the discharge

of a unit loading (1 kg/d) of a conservative substance at each of the treatment plant

outfalls.

4.2.2.1  Sand Island Treatment Plant Zone

A unit loading from the Sand Island treatment plant outfall is equivalent to an

average discharge concentration of about 0.0035 mg/l.  Contours of average

concentrations (mg/l) in the surface and bottom waters of the bay are shown in Figure 4-

26.  The contours are in logarithmic units with an interval of 0.5.  The highest contour of

-5.5 represents a dilution of about 1,000.  The lowest contour of -7.5 represents a dilution

of about 100,000.  The dilutions should not be interpreted as actual dilutions of the

effluent plume at any particular time.  Rather, they represent a combination of the plume

dilution and the visitation frequency of the plume at each location.

The contours indicate that the plume “visits” most locations within the bay.  The

orientation of the contours reflects the dominance of along shore currents.  They illustrate

the general western drift within the bay, but also that current reversals can take the plume

to the east.  Vertical gradients are evident particularly in the vicinity of the outfall where

bottom concentrations are about 3 to 5 times higher than surface concentrations.

4.2.2.2  Honouliuli Treatment Plant Zone

A unit loading from the Honouliuli treatment plant outfall is equivalent to an

average discharge concentration of about 0.01 mg/l.  Contours similar to those presented

for a unit loading from Sand Island are shown in Figure 4-27.  The highest contour of -5.5

represents a dilution of about 3,000 and the lowest contour of -7.5 represents a dilution of

about 300,000.

In contrast to Sand Island, the Honouliuli plume is never transported far to the

east.  The contours indicate dominant along shore transport to the west.  The contour

intervals are more compressed than for Sand Island, reflecting the more temporally

consistent currents in this part of the bay.  Shoreline concentrations exceed 10-7 mg/l only

at the western edge of the bay. In contrast, the unit load at Sand Island results in

concentrations exceeding 10-6.5 mg/l from Hickam Harbor to Barbers Point and exceeding

10-7 mg/l from Honolulu Harbor to Barbers Point. The vertical gradients are similar to

those observed from Sand Island.

4.2.3  Comparison of Model and Data

A one year simulation (April 15, 1994 to April 15, 1995), incorporating all major

outfall and shoreline sources, was performed for the following variables: biochemical

oxygen demand (BOD), total suspended solids (TSS), and culturable counts of fecal

coliforms, enterococci and Clostridium perfringens.  While the model does not directly

address viable-but-not-culturable bacteria, estimates were calculated by setting the

phototoxicity rate constant to zero (see Section 3.2.3).  To assess the capability of the model to predict BOD, TSS and bacterial concentrations at the beaches, model results and

data collected by Project MB-7 were compared.

BOD concentrations at the beaches were relatively constant with average values

of about 0.40 mg/L.  TSS concentrations were more variable than BOD levels;

concentrations ranged from about 0.03 - 0.10 mg/L, except during major storm events

where TSS levels were calculated as high as 25 mg/L.  Due to the lack of BOD and TSS

data, validation of these model results was not possible.

Observed and computed indicator organism counts were compared in several

ways.  Cumulative probability distributions and time series of data collected at the beach

and bay stations were visually compared to the model results at the same locations.  The

probability distributions are presented here.  The time series are presented in Section

4.2.5 (Figures 4-32 through 4-37) and Appendix F.  In addition, differences between

model and data at these stations were quantified as mean error and root mean square

error.  The mean error is the average difference in counts.  A negative value indicates that

the model under predicts the data, whereas a positive value indicates an over prediction.

The extent of bias is indicated by the magnitude of the error.  The root mean square

(RMS) error gives an estimate of the actual magnitude of the model error.  All

comparisons were between the daytime average computed concentration on the sampling

date and the average of observed values.

Figure 4-26.  Contours of the average concentrations (mg/l) in logarithmic units resulting

from a unit loading (1 kg/d) of a conservative substance at the Sand Island outfall.

Figure 4-27.  Contours of the average concentrations (mg/l) in logarithmic units resulting

from a unit loading (1 kg/d) of a conservative substance at the Honouliuli outfall.

Comparisons of the cumulative probability distributions of fecal coliforms at 6

beaches and 3 offshore stations are shown in Figure 4-28.  The model results are shown

by the open symbols and the data by the filled symbols.  At points in the distribution

where only a filled symbol is plotted, the model and data are equal.  In general, the slopes

and median values of the model and data are similar, indicating that the model accurately

reproduces both the variability and the mean concentration at each station.  Exceptions

include Iroquois and Sand Island beaches where the model tends to exceed the data and

Waikiki Beach where the model tends to underpredict the data.  However, even at these

beaches the slopes are similar.  The visual comparisons are supported by the mean error

(Table 4-1).  This statistic is small (less than 2 cfu/100 ml) at all the beaches except

Iroquois, Sand Island and Waikiki.  The RMS errors (Table 4-1) indicate that model is

more inaccurate in reproducing point values then it is in reproducing the distribution of

counts at a location.  The errors are typically 5 to 10 CFU/100ml.  Two notable

exceptions are Sand Island Beach and Queens Surf Beach, where the model and data

differ by about 26 and 23 CFU/100ml, respectively.  The difference in the capability of

the model to reproduce the probability distribution of counts and point values probably

results from the limited resolution of the model.  Diel studies at the beaches conducted in

Project MB-7 indicated that counts change on the time scale of hours.  The capability of

the model to capture variation on this time scale is limited by the spatial resolution of the

model and the daily resolution of loadings.  In addition, the loading from Ala Wai Canal

is determined by Monte Carlo procedures and captures the distributional characteristics of

this load but not its time series.

The cumulative probability distributions of enterococci are shown in Figure 4-29.

In contrast to the accurate reproduction of fecal coliform, enterococci counts tend to be

under predicted, particularly at the eastern beaches.  The differences are greatest at

Waikiki and Queens Surf beach, where the mean error is about -4.7 cfu/100 ml (Table 4-

1).  Despite this bias, the RMS errors are similar to those of fecal coliform (Table 4-1).

As with fecal coliforms, the differences between model and data are greatest at Sand

Island Beach.  Overall, the model under predicts enterococci at this station.  However,

Table 4-1.  Descriptive Statistical Comparison of Model and Data for Indicator

Organisms

Figure 4-28.  Cumulative probability distributions of fecal coliform data and model

results at various locations.

Figure 4-29.  Cumulative probability distributions of enterococci data and model results

at various locations.

Figure 4-30.  Cumulative probability distributions of Clostridium data and model results

at various locations.

this results from a single comparison in which a high observed value is matched with a

low computed value.  At all other times the model predicts counts that exceed the

measured value.

The cumulative distributions of computed Clostridium counts are generally

similar to the distributions of observed counts (Figure 4-30) with little bias (Table 4-1).

The reproduction is best in the eastern portion of the bay; both at the beaches and the

offshore stations.  At Sand Island Beach and the western beaches Iroquois and Ewa, the

model over predicts the median concentrations by about a factor of 2.  However, even at

these stations the variability is well described by the model.

In summary, the fates of the indicator organisms in Mamala Bay are well

described by the model as indicated by comparison of frequency distributions.  The

model is biased low for enterococci, suggesting that the loss of organisms may be over

estimated.  The model over predicts counts of all three indicators at Sand Island Beach.

This error could be caused by over estimation of the load to the bay from the Keehi

Lagoon/Honolulu Harbor watershed or of the transport of organisms from the Sand Island

outfall.  Comparison of the model and data within Keehi Lagoon (Station D1; results not

shown) indicates that the model does not over predict counts in this area.  Thus, it is

probable that the onshore transport from the outfall is not well predicted in this area.

4.2.4  Sensitivity Analysis

In an attempt to assess the sensitivity of the model to the process descriptions and

meteorological conditions, the following simulations were performed:

1.	No bacterial loss due to phototoxicity

2.	Bacterial loss due to phototoxicity increased by a factor of 5

3.	Light attenuation in the water column halved

4.	Light attenuation in the water column doubled

5.	Light attenuation in the water column excluding effect of solids

6.	No bacterial loss due to predation

7.	Bacterial loss due to predation increased by a factor of 4

8.	Wind speed halved

Bacterial abundances are most strongly influenced by the rate of inactivation due to

phototoxicity.  Daytime mean fecal coliform levels at Ala Moana Beach are 50 times

higher if the sunlight inactivation rate constant is zero rather than 0.32m2/MJ.  Thus, the

selection of an appropriate rate constant is vital to accurately estimating bacterial levels at

the beaches.

Loss due to predation has a minor effect on bacterial levels and changes in light

attenuation and wind speed result in minimal differences.

4.2.5  Analysis of Source Contributions

The main goal of the modeling effort was to provide a means of estimating the

contribution of the various sources to bacterial counts at the recreational beaches.  This

objective was accomplished by simulating the April 15, 1994 - April 15, 1995 period in iterative fashion with only the source of interest discharging to the bay.  In particular,

shoreline sources (Ala Wai Canal, Pearl Harbor and Keehi Lagoon), the Sand Island and

Honouliuli wastewater treatment plants together and each plant in isolation were

investigated.

Source contributions were explored by comparing mean counts of indicators at

five beaches in three scenarios: discharge only from the Sand Island treatment plant, only

from the Honouliuli treatment plant, and only from shoreline sources.  For each beach,

the set of three mean indicator concentrations are presented as a pie diagram, in which the

relative size of each slice represent the relative magnitudes of each mean (Figure 4-31).

The Honouliuli treatment plant is the dominant source of fecal coliforms at Oneula

Beach, and the Sand Island treatment plant is the dominant source for Ewa, Ala Moana,

Waikiki and Queens Surf Beaches (Figure 4-31, top row).  The greatest beach impact of

shoreline fecal coliform sources is at Waikiki Beach where their contribution is slightly

lower than that of the Sand Island treatment plant.  In addition, shoreline and outfall

sources contribute equally at the offshore Waikiki station.

In contrast, the Sand Island treatment plant is the dominant source of enterococci

at Oneula and Ewa Beaches.  Both shoreline sources and the Sand Island treatment plant

contribute to enterococci counts at the eastern beaches, with shoreline sources being somewhat greater (Figure 4-31, middle row).  The greatest impact of shoreline sources is

at Ala Moana and Waikiki Beaches, where their contribution is almost three times that of

the Sand Island treatment plant.  Shoreline sources contribute approximately 80 percent

of the counts at the offshore Waikiki station.  Clostridium follows a pattern similar to that

of enterococci (Figure 4-31, bottom row).

Source contributions were also explored by comparing time series of counts at the

beaches due to all sources, outfalls only and shoreline sources only.  Ala Moana and Ewa

beaches are presented as examples (Figures 4-32 to 4-37).  Other stations are shown in

Appendix F.  The plots show daytime values only to be consistent with the times of

sampling.  Counts at both beaches are highly variable in response to sporadic events

related to precipitation and to variation in currents within the bay.  Occurances of fecal

coliform are most often due to the outfalls and they account for the highest concentrations

(Figures 4-32 and 4-33) of the three indicator organisms.  In contrast, enterococci counts

at Ala Moana Beach are dominated by the shoreline sources (Figure 4-34).  Shoreline

sources are dominant through most of the year at Ewa Beach, although the outfall source

contribution is important in November and December (Figure 4-35).  The consistently

high enterococci levels at Ewa Beach in November-December cause the mean annual

contribution of the outfalls to be greater than that of the shoreline sources (Figure 4-31).

Shoreline sources account for the high Clostridium counts at Ala Moana Beach (Figure 4-

36), while the outfalls are the dominant source of Clostridium at Ewa beach (Figure 4-

37).

Figure 4-31.  Average concentrations of fecal coliforms and enterococci at five beach

stations calculated by the model under three conditions: only Sand Island treatment

plant, only Honouliuli treatment plant, and only shoreline sources discharging

Figure 4-32.  Time series of computed daytime fecal coliform counts at Ala Moana Beach

with loadings to Mamala Bay from all sources, outfalls only, or shoreline sources

only.

Figure 4-33.  Time series of computed daytime fecal coliform counts at Ewa Beach with

loadings to Mamala Bay from all sources, outfalls only, or shoreline sources only.

Figure 4-34.  Time series of computed daytime enterococci counts at Ala Moana Beach

with loadings to Mamala Bay from all sources, outfalls only, or shoreline sources

only.

Figure 4-35.  Time series of computed daytime enterococci counts at Ewa Beach with

loadings to Mamala Bay from all sources, outfalls only, or shoreline sources only.

Figure 4-36.  Time series of computed daytime Clostridium counts at Ala Moana Beach

with loadings to Mamala Bay from all sources, outfalls only, or shoreline sources

only.

Figure 4-37.  Time series of computed daytime Clostridium counts at Ewa Beach with

loadings to Mamala Bay from all sources, outfalls only, or shoreline sources only.

Finally, the number of days on which standards were exceeded were calculated for

beach stations.  Model results indicate that during the one-year simulation, there were

only 17 days on which fecal coliform counts exceeded the 200 cfu/100 ml standard at any

of the following beaches: Diamond Head, Queens Surf, Waikiki, Ala Moana, Sand

Island, Ewa and Oneula.  Sixteen exceedances were due solely to the outfall sources.

There were 402 days on which the enterococci standard of 7 cfu/100 ml were exceeded at

any of the above mentioned beaches; approximately 50 percent of these were due to

outfall sources and 50 percent to shoreline sources.  These results are consistent with the

results of the analysis of average levels.

Thus, the model indicates that both the treatment plants and the shoreline sources

can contribute measurable indicator organism counts at the beaches.  The relative

contributions of these sources differs among the indicators and among locations.  In

general, the outfalls are the greatest contributors to fecal coliforms at the beaches; the

shoreline sources are of relatively greater importance to enterococci and Clostridium.

This difference is due to the relative magnitudes of the indicators in the sources: the

average fecal coliform counts in the Sand Island and Honouliuli effluents are about 30

times greater than the enterococci counts, while the fecal coliform counts at the Ala

Moana Bridge (Station AW1) are only about 2 times greater than the enterococci counts.

Outfall source impacts are almost entirely due to the Sand Island plant, except at

Oneula Beach (Figure 4-31).  The effluent of the Honouliuli treatment plant has little

effect because persistent, westward flowing nearshore currents move the effluent toward

and beyond Barbers Point.  The impact of the Sand Island treatment plant is greatest at

Sand Island Beach.  Its impact at this beach appears to be related to the initial dilution of

the effluent plume and currents within the bay and not due to maximums in loading or

minimums in solar radiation.

4.2.6  Source Contributions for Specific Pathogens

As part of Project MB-7, abundances of the pathogens Salmonella, enterovirus,

Cryptosporidium and Giardia were measured monthly at several locations, including the

Sand Island Treatment Plant effluent, the downstream end of Ala Wai Canal (Ala Moana

Bridge) and the beaches.  These data were used to extend the fate model to pathogenic

organisms.  Means of the abundances in the effluent and the canal water were used as the

basis for defining pathogen loadings to the bay.  The analysis reported here was restricted

to the eastern beaches because other locations may be affected by sources not quantified

by the existing data.  The indicator organism results indicate that these other sources do

not significantly contribute to fecal contamination at the eastern beaches (see Section

4.2.5).

A summary of the measurements in samples of plant effluent and canal water is

presented in Table 4-2.  These data indicate quantitative and qualitative differences

between the Plant effluent and the canal.  The effluent is characterized by similar levels

of Salmonella, enterovirus and Giardia that exceed Cryptosporidium by about an order of

magnitude.  In contrast, Salmonella levels in the canal exceed the levels of the other

pathogens by a factor of 5 to 10 and Cryptosporidium and Giardia have similar

abundances. The pathogen levels in the Plant effluent are 5,000 (Salmonella) to 500,000

(enterovirus) times greater than in the canal.  Because of the lower levels, the mean

abundances in the canal have greater uncertainty, particularly for Salmonella and

enterovirus.  These pathogens were not detected in many of the canal samples (50% for

Salmonella and 17% for enterovirus).

The fate processes considered for the pathogens included transport and predation

at the same rate as the indicator organisms.  These organisms were assumed to be

resistant to phototoxicity.

| TABLE 4-2. ABUNDANCES OF PATHOGENIC ORGANISMS IN SAND ISLAND EFFLUENT AND IN THE ALA WAI CANAL AT ALA MOANA BRIDGE |  |  |  |
| --- | --- | --- | --- |
| Pathogen | Statistic | Sand Island Effluent | Ala Wai Canal at Ala Moana Bridge |
| Salmonella | number of samples | 10 | 12 |
|  | frequency of detection (%) | 100 | 50 |
|  | mean abundance (MPN/L) | 3800 | 0.69 (0.84)1 |
|  | coefficient of variation | 1.7 | 1.8 (1.6)1 |
| enterovirus | number of samples | 12 | 12 |
|  | frequency of detection (%) | 100 | 17 |
|  | mean abundance (MPN/L) | 3300 | 0.0058 (0.017)1 |
|  | coefficient of variation | 1.2 | 2.4 (0.9)1 |
| Cryptosporidium | number of samples | 13 | 13 |
|  | frequency of detection (%) | 100 | 85 |
|  | mean abundance (cysts/L) | 220 | 0.01 (0.011)1 |
|  | coefficient of variation | 1.1 | 0.7 (0.6)1 |
| Giardia | number of samples | 13 | 13 |
|  | frequency of detection (%) | 100 | 77 |
|  | mean abundance (cysts/L) | 2300 | 0.016 (0.017)1 |
|  | coefficient of variation | 1.0 | 1.2 (1.1)1 |
| 1first value includes non-detected values at zero; value in parentheses includes non-detected values at the limit of detection. |  |  |  |

Temporal comparisons of the model and data are presented for Ala Moana Beach

(Figure 4-38), Waikiki Beach (Figure 4-39) and Queens Surf Beach (Figure 4-40).  The

model results are qualitatively similar at all of the beaches.  Temporal variability

consistently increases from Salmonella to Cryptosporidium to Giardia to enterovirus.

These differences in temporal variability are the result of differences in the relative

contributions of the two sources.  Applications of the model with the sources separated

(not shown) indicate that temporal variability reflects contributions from the Sand Island

outfall.  The Ala Wai Canal contribution of all of the pathogens is relatively constant

because the limited data set forces the use of constant concentrations in the canal and

because of the short distance between the canal to the beaches.  In contrast, the transport

of constituents from the Sand Island outfall varies greatly in response to variation of tidal

and low frequency currents.  Thus, the relatively constant levels of Salmonella indicate

that the Ala Wai Canal is the dominant source.  The increasing variability across the other

pathogens indicates increasing importance of the Plant outfall.

Figure 4-38.  Temporal profiles of observed (symbols) and predicted (lines) daytime

pathogen abundances at Ala Moana Beach.

Figure 4-39.  Temporal profiles of observed (symbols) and predicted (lines) daytime

pathogen abundances at Waikiki Beach.

Figure 4-40.  Temporal profiles of observed (symbols) and predicted (lines) daytime

pathogen abundances at Queens Surf Beach.

Contamination events attributable to the outfall cause Cryptosporidium levels to

increase from about the 0.005 oocysts/L attributable to Ala Wai Canal to about 0.02 to

0.04 oocysts/L.  These events have greater effects on Giardia and enterovirus

contamination because of the order of magnitude higher Plant effluent levels in relation to

Canal levels (Table 4-2).  Giardia levels increase from the range of 0.005 to 0.01 cysts/L

to the range of 0.2 to 0.3 cysts/L.  Enterovirus levels increase from the range of 0.002 to

0.005 MPN/L to the range of  0.3 to 0.5 MPN/L.

Comparisons between observed and predicted levels at the beaches indicate that

the model results are of the same order as the data.  Some consistent biases are evident.

Predicted Salmonella values range from about 0.4 to 1 MPN/L, whereas, the measured

values are all below the detection limit of 0.3 MPN/L.  This may reflect the inaccuracy of

the Ala Wai Canal source, as discussed above.  The maximum detected level of Giardia

at the Waikiki and Queens Surf beaches was 0.005 cysts/L.  The model predicts values at

least a factor of two higher.  Either loads have been over estimated or loss mechanisms

have been under estimated.  Given the consistent detection of these organisms in the

sources, under estimation of the loss mechanisms may be more likely.

The indication from the model that both the Sand Island effluent and the Ala Wai

Canal contribute fecal contamination to the beaches is consistent with the results of the

indicator organism modeling.  Again, the importance of the sources is dependent on the

organism being considered.  The outfall tends to dominate for Giardia and enterovirus, as

it did for fecal coliforms.  The shore-based source tends to dominate for Salmonella

(subject to the loading uncertainty), and to a lesser extent, for Cryptosporidium, as it did

for enterococci and Clostridium.

4.2.7  Analysis of Loading Reduction Evaluations

The model was also used to estimate the reduction in indicator counts at various

locations in Mamala Bay that will result from chemically enhanced primary treatment

(CEPT), secondary treatment and disinfection (either with CEPT or secondary treatment)

at both the Sand Island and Honouliuli treatment plants.

At all of the beaches except Oneula, CEPT at the Sand Island treatment plant

reduces the mean fecal coliform count by about a factor of two (Figure 4-41).  Both

secondary treatment and disinfection (presumably with CEPT) reduce the mean fecal

coliform counts by about a factor of three.  At Oneula Beach, none of the treatment

options reduce counts more than 20 percent because of the dominance of the Honouliuli

outfall.  CEPT at Honouliuli treatment plant reduces the average fecal coliform count at

Oneula Beach by approximately a factor of two, and secondary treatment and disinfection

reduce the average by a factor of five (results not shown).  All treatments at Honouliuli

treatment plant have almost no effect at any other beach.

CEPT at the Sand Island plant reduces mean enterococci counts by approximately

65% at Ewa Beach and 40% at Oneula Beach (Figure 4-42).  It has a minor impact at

most of the other beaches.  CEPT at the Honouliuli treatment plant reduce counts at

Oneula Beach by 15% (results not shown).  These results are consistent with the

previously mentioned dominance of the Sand Island treatment plant at the western

beaches and the dominance of shoreline sources at the eastern beaches (Figure 4-31).

Secondary treatment and disinfection at Sand Island have only minor impacts on

enterococci beyond that achieved by CEPT.  All treatments at Honouliuli treatment plant

have almost no effect at any beach other than Oneula.

Clostridium is reduced only by disinfection (Figure 4-43).  The reductions are

dramatic at Oneula and Ewa Beaches, where levels are reduced by about 80%.  At the

other beaches the reductions range from about 30% at Ala Moana to about 50% at

Waikiki.

Figure 4-41.  Average fecal coliform concentrations at six stations calculated by the

model for four treatment levels at Sand Island treatment plant, current, chemically

enhanced primary treatment (CEPT), secondary treatment, and disinfection

Figure 4-42.  Average enterococci concentrations at six stations calculated by the model

for four treatment levels at the Sand Island treatment plant: current, chemically

enhanced primary treatment (CEPT),Figure 4-43.  Average Clostridium concentrations at six stations calculated by the

model for four treatment levels at the Sand Island treatment plant: current

chemically enhanced primary treatment (CEPT), secondary treatment, and

disinfection.

The Waikiki offshore station was the only offshore station to be considered here.

There is minimal impact to all three indicators from the various Honouliulu plant

treatment scenarios.  CEPT at the Sand Island treatment plant reduces fecal coliform

levels by about 30%, whereas secondary treatment and disinfection both archieve about

45% reductions (Figure 4-41).  Enterococci concentrations at this offshore station are

reduced by about 20% for Sand Island plant treatment scenarios.  Clostridium

concentrations only responded to total Sand Island treatment plant disinfection with a

20% reduction.

The relative impacts of disinfection of treatment plant effluent and of elimination

of shoreline sources vary with location and among indicators.  Disinfection has a greater

impact than shoreline source elimination in reducing both the average fecal coliform

counts (Figure 4-44) and the number of days with counts exceeding 50 cfu/100 ml

(Figure 4-45) at all the beach stations. A value of 50 was used to examine exceedances

because few of the stations exceed the standard of 200 cfu/100 ml.  This is consistent

with the dominance of the Honouliuli treatment plant at Oneula Beach and of the Sand

Island treatment plant at all other beaches.  In contrast to the eastern beach stations,

elimination of the Ala Wai Canal loading and disinfection have approximately equal

impact at the Waikiki offshore station (Figures 4-44 and 4-45) because the modeled

plume from the Ala Wai Canal tends to move more offshore.

Partial confirmation that the core of the Ala Wai Canal plume, which contains the

highest pathogen concentrations, is typically found about 1 km offshore of Ala Moana

and Waikiki Beaches was achieved using the results of a dye study (see Appendix B).  As

the surface plume debouches from the mouth of the canal, the plume generally tends to be

transported 0.5 to 1 km offshore before currents transport it either eastward or westward

along the shore.  Subsequent lateral advection and turbulent diffusion can then transport

pathogens in the plume to the beaches.

For enterococci and Clostridium,disinfection of the Sand Island effluent has a

greater impact than shoreline source elimination in reducing counts at the western

beaches of Oneula and Ewa (average values in Figures 4-46 and 4-47).  Similarly,

disinfection of both treatment plant effluents results in greater reduction of exceedences

in these beaches (number of days with enterococcus counts exceeding the marine

recreational standard of 7 cfu/100 ml in Figure 4-48).  However, the impact of the Ala

Wai Canal source elimination is greater at the eastern beaches and Waikiki offshore.

Elimination of the loadings to Pearl Harbor or Keehi Lagoon causes only minor

reductions in contamination.  These results are consistent with the previously mentioned

relationships between sources (Figure 4-31).

Figure 4-44.  Average predicted fecal coliform concentrations at ten stations for six

scenarios: current conditions, disinfection at Sand Island and Honouliuli treatment

plants, and elimination of Ala Wai Canal, Pearl

Figure 4-45.  The number of days on which predicted fecal coliform concentrations

exceed 50 cfu/100 ml at ten stations for six scenarios: current conditions,

disinfection at the Sand Island and Honouliuli treatment plants,

Figure 4-46.  Average predicted enterococci concentrations at ten stations for six

scenarios: current conditions, disinfection at the Sand Island and Honouliuli

treatment plants, and elimination of Ala Wai Canal, Pearl Harbor

Figure 4-47.  Average predicted Clostridium concentrations at ten stations for six

scenarios: current conditions, disinfection at the Sand Island and Honouliuli

treatment plants, and elimination of Ala Wai Canal, Pearl Harbor

Figure 4-48.  The number of days on which predicted enterococci concentrations exceed

7 cfu/100 ml at ten stations for six scenarios: current conditions, disinfection at the

Sand Island and Honouliuli treatment plants, and

5.  CONCLUSIONS

A coupled three-dimensional hydrodynamic circulation and pathogen fate model

has been developed for Mamala Bay.  It has been calibrated using observations from

April 1994 through April 1995, a complete yearlong period, and some ancillary

information extracted from the historical archive.  The conclusions drawn from the model

are supported by a favorable model skill assessment that indicates the relevant quantities

of interest can be predicted with confidence.

The predictions of the bay's advection and dispersive properties by the circulation

model appear reasonable enough to be useful for pathogen fate simulations.  This

conclusion is based on comparisons between the observations and model results for water

elevations, currents, temperatures and a dye release.  Trade wind and Kona wind

conditions were considered in the skill assessment.  Much of the discrepancy between the

observations and the model predictions is due to our limited understanding of the physical

oceanography and to a lack of precision in observations of the offshore water elevation

forcing functions.

The hydrodynamic model reveals a circulation pattern in Mamala Bay that is

complicated both in space and time.  The mean flow is typically westward at all depths.

However, a permanent recirculation zone is found in the nearshore region off Waikiki

Beach.  During Kona wind conditions, the model shows significant departures from the

currents calculated under trade wind conditions in nearshore areas.  However, at water

depths deeper than 70 m, the general circulation patterns do not change markedly for

these different wind conditions.

The causes of the predicted high fecal coliform concentrations at the eastern

beaches that are apparently caused by the Sand Island outfall have been investigated

through the use of the hydrodynamic model.  The results of the analyses suggested that

during stratified water column and Trade Wind conditions there were plausible physical

oceanographic pathways for the transport of fecal contamination from the outfall to the

eastern beaches.  This possibility has previously been assumed to be highly unlikely.  It

was also found that the most favorable conditions for pathogen transport from the outfall

to the beaches was during winter months when Kona winds blow and the water column is

weakly stratified allowing the outfall plume to surface.

A careful skill assessment was also conducted for the pathogen fate model.  The

assessment shows a favorable comparison of predicted and observed distributions of fecal

coliform, Clostridium perfringens, enterovirus and Cryptosporidium counts at most of the

beaches.  Of note is that the model does tend to underpredict the distributions of

enterococci and overpredicts the distributions of Giardia and Salmonella.  The model is

most sensitive to the rate of phototoxicity and overestimation of this loss mechanism may

be responsible for the underprediction of enterococci counts.

Model results aid in distinguishing outfall from shoreline impacts on average

bacterial levels.  In general, outfall sources dominate average fecal coliform levels at all

beaches.  Outfall sources dominate average enterococci and Clostridium levels at western

beaches; shoreline sources dominate at the eastern beaches.  The Sand Island treatment

plant is the dominate outfall source for bacterial contaminants.  Depending on the extent

of stratification at the outfall and currents within the bay, this contribution typically

ranges between non-detect (<1 cfu/100 ml) and about 50 fecal coliform cfu/100 ml and

20 enterococci cfu/100 ml.  The Honouliuli treatment plant outfall contributes

measurably to indicator organisms at only Oneula Beach.  Its effluent is consistently

transported alongshore to the west.

Analysis of the indicator organisms data suggest that more than one process may

be controlling bacterial levels at the beaches: a background level that is controlled by

outfall and/or shoreline sources, and high levels related to high rainfall events or inshore

transport of the outfall plume.  In the simulation, 16 of 17 fecal coliform violations and

50 percent of the 402 enterococci violations at seven beaches were due to outfall sources.

The outfall sources dominated the fecal coliform violations, at least in part because of the

inherent variability in the dynamics of bacterial contamination in Mamala Bay and the

relatively few periods with high rainfall during the year of simulation.

The impacts of additional treatments to the Sand Island effluent on levels at the

beaches parallel the results discussed above, being greater at beaches where outfall

sources are the dominant contributors to bacterial levels.  In general, CEPT with

disinfection or secondary treatment reduce fecal coliform levels at beaches more than

CEPT alone (approximately three-fold decreases vs. a two-fold decrease).  All three

treatments have similar impacts on average enterococci levels, ranging between 20 and 65

percent among beaches.

Finally, the relative impacts of disinfection and of elimination of shoreline

sources parallel the results described above; disinfection reduced average levels more

than shoreline source elimination where outfall sources dominate bacterial levels.

6  RECOMMENDATIONS

A number of recommendations can be suggested for strengthening the conclusions

reached in this study.  They are:

1.	Verification of the Waikiki Beach recirculation zone and Ala Wai Canal plume

dynamics through detailed observations and hydrodynamic modeling.

2.	Investigation of nearshore transport processes through high spatial resolution

modeling.

3.	Aquistion of detailed temperature and salinity observations in the offshore regions

to include them as predicted parameters within the model framework.

4.	Validation of the rates of phototoxicity used in the model by comparison to the

results of dieoff studies conducted in Mamala Bay itself.

5.	Determination of the relationship between culturable counts and viable organisms

so that the model may be used to examine infectivity potential.

6.	Quantification of the relationship between indicator organisms and specific

pathogens so that the model may be better extrapolated to disease causing

organisms.

7  REFERENCES

Allen, J.S., 1980.  Models of Wind-Driven Currents on the Continental Shelf, Ann. Rev.

Fluid Mech., 12, 389-433.

Allen, J.S., P.A. Newberger and J. Federiuk, 1995. Upwelling Circulation on the Oregon

Continental Shelf, J. Phys. Oceanogr., 35, 1843-1889.

Blumberg, A.F., 1977a. Numerical Tidal Model of Chesapeake Bay, J. of Hyd. Div.

ASCE, 103, 1-10.

Blumberg, A.F., 1977b. Numerical Model of Estuarine Circulation, J. of Hyd. Div.

ASCE, 103, 295-310.

Blumberg, A.F. and D.M. Goodrich, 1990. Modeling of Wind-Induced Destratification in

Chesapeake Bay, Estuaries, 13, 1236-1249.

Blumberg, A.F. and D.W. Pritchard, 1995. Rigorous Validation of Three Dimensional

Hydrodynamic Models.  In preparation.

Blumberg, A.F. and H.J. Herring, 1987. Circulation Modeling Using Orthogonal

Curvilinear Coordinates, in: Three-Dimensional Models of Marine and Estuarine

Dynamics, J.C.J. Nihoul and B.M. Jamart, eds., Elsevier Pub. Co., 55-58.

Blumberg, A.F. and L.H. Kantha, 1985.  Open Boundary Conditions for Circulation

Model, J. Hydraulic Engineering, 111, 237-255.

Blumberg, A.F. and G.L. Mellor, 1980.  A Coastal Ocean Numerical Model, In:

Mathematical Modeling of Estuarine Physics, Proceedings of an International

Symposium, Hamburg, August 24-26, 1978. J. Sundermann and K.P. Holz, Eds.,

Springer-Verlag, Berlin.

Blumberg, A.F. and G.L. Mellor, 1985.  A Simulation of the Circulation in the Gulf of

Mexico, Israel J. of Earth Sciences, 34, 122-144.

Blumberg, A.F. and R.P. Signell and H.L. Jenter, 1993.  Modeling Transport Processes in

the Coastal Ocean, J. Marine Env. Engr., 1, 3-52.

Brink, K.H., 1991.  Coastal-Trapped Waves and Wind-Driven Currents Over the

Continental Shelf. Annual Rev. Fluid Mech., 23, 389-412.

Burban, P-Y., Y-J. Xu, J. McNeil and W. Lick, 1990.  Settling Speeds of Flocs in Fresh

Water and Seawater., J. Geophys. Res., 95, 18,213-18,220

Casulli, V., 1990.  Semi-Implicit Finite Difference Methods for the Two-Dimensional

Shallow Water Equations, J. Comput. Phys., 86, 56-74.

Chen, C. and R.C. Beardsley, 1995. A Numerical Study of Stratified Tidal Rectification

over Finite-Amplitude Banks, J. Phys. Oceanogr., 25, 2090-2128.

Davies-Colley, R.J., R.G. Bell and A.M. Donnison, 1993.  Depth-Dependence of

Enterococcus Inactivation by Solar UV in Seawater, p. 535-541.  In L.C. Wrobel

and C.A. Brebbia (ed.), Water Pollution. II, Modeling, measuring and prediction,

Computational Mechanics Publications, Southampton, United Kingdom.

Davies-Colley, R.J., R.G. Bell and A.M. Donnison, 1994. Sunlight Inactivation of

Enterococci and Fecal Coliforms in Sewage Effluent Diluted in Seawater.  Appl.

Environ. Micorbiol. 60, 2049-2058.

Egbert, G.D., A.F. Bennett, and M.G.G. Foreman, 1994.  TOPEX/POSEIDON Tides

Estimated Using a Global Inverse Model, J. Geophys. Res., 99, 24,821-24,852.

Ezer, T. and G.L. Mellor, 1992.  A Numerical Study of the Variability and the Separation

of the Gulf Stream, Induced by Surface Atmosphere Forcing and Lateral

Boundary Flows, J. Phys. Oceanogr., 22, 660-682.

Fenchel, T., 1987.  Ecology of Protozoa: The biology of free-living phagotrophic protists.

Science Tech, Inc., Madison, WI, 197 p.

Gameson, A.L.H. and D.J. Gould, 1985. Investigations of Sewage Discharges to Some

British Coastal Waters.,  Chapter 8.,  Bacterial Mortality, Part 2.,  Technical

Report TR 222, Water Research Centre, Marlow, England, 72 p.

Harms, S. and C. D. Winant, 1994.  Synthetic Sutosurface Pressure Derived from Bottom

Pressure and Tide Gauge Observations, J. Atmos and Oceanic Tech., 11, 1625-

1637.

Large, W.G. and S. Pond, 1981.  Open Ocean Momentum Flux Measurements in

Moderate to Strong Winds, J. Phys. Oceanogr.,11, 324-336.

Laws, E.A., D.G. Redajie, L.W. Haas, P.K. Bienfang, R.W. Eppley, W.G. Harrison, D.M.

Karl and J. Marra, 1984. High Phytoplankton Growth and Production Rates in

Oligotrophic Hawaiian Coastal Waters,  Limnol. Oceanogr., 29, 1161-1169.

Marra, J., L.W. Haas and K.R. Heinemann, 1988.  Time Course of C Assimilation and

Microbial Food Web, J. Exp. Mar. Biol. Ecol., 115, 263-280.

Mellor, G.L. and A.F. Blumberg, 1985. Modeling Vertical and Horizontal Viscosity and

the Sigma Coordinate System, Mon. Wea. Rev., 113, 1379-1383.

Mellor, G.L. and T. Yamada, 1974.  A Hierarchy of Turbulence Closure Models for

Planetary Boundary Layers J. Atmos. Sci., 31, 1791-1806.

Mellor G.L. and T. Yamada, 1982.  Development of a Turbulence Closure Model for

Geophysical Fluid Problems, Rev. Geophys. Space Phys., 20, 851-875.

Mellor G.L. and T. Yamada, 1982. Development of a Turbulence Closure Model for

Geophysical Fluid Problems, Rev. Geophys. Space Phys., 20, 851-875.

Okuko, A., 1971. Oceanic Diffusion Diagrams. Deep-Sea Res., 18, 789-802.

Schwiderski, E.W., 1980.  Ocean Tides, Mar. Geol., 3, 161-255.

Thomann, R.V. and J.A. Mueller, 1987. Principles of Surface Water Quality Modeling

and Control, Harper & Row, Publishers, New York, 644 p.

Σ

1 Data obtained from Hawaiian Sugar Planter's Association, P.O. Box 1057, Aiea, Hawaii

96701-1057