| Station | Position |  | Approx. Water | Instrument | Variables |  |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Latitude (N) | Longitude (W) | Depth (m) | Types & (#) | Measured | Measurement Depths//Comments |
| A2* | 21°16.835' | 158°06.885' | 67 | BBADCP(1); W/T(1) | V,T,WL,W | Velocity Profile, Temp at bottom// |
| A4 | 21°14.799' | 158°06.373' | 505 | GO/Mk2(4) | V,T,P(top) | 75m, 125m, 240m 490m// |
| B1 | 21°17.462' | 158°01.455' | 17 | S4 (1) | V,T,C | 8m// |
| B2* | 21°17.017' | 158°01.142' | 78 | BBACDP(1) | V,T | Velocity Profile, Temp at bottom// |
| B2(T) | 21°16.994' | 158°01.248' | 74 | Thermistors(11) | T | Temperature Profile// |
| B3 | 21°16.150' | 158°01.035' | 250 | GO/Mk2(4) | V,T,P(top) | 30m,75m, 125, 240m// |
| D1 | 21°17.420' | 157°53.601' | 15 | S4(1) | V,T,C |  |
| D2* | 21°16.801' | 157°53.595' | 77 | BBADCP(1) | V,T | Velocity Profile, Temp at bottom. |
| D2(T) | 21°16.795' | 157°53.835' | 75 | Thermistors(11) | T | Temperature Profile |
| D3 | 21°16.343' | 157°54.050' | 249 | NBADCP(1) | V,T, | Vel. Profile//4/95-7/95; 2xGOs |
| E2* | 21°14.446' | 157°48.485' | 82 | BBADCP(1);W/T(1) | V,T,WL,W | Velocity Profile, Temp at bottom. |
| E4 | 21°13.242' | 157°48.771' | 496 | GO/Mk2(4) | V,T,P(top) | 75m, 125m, 240m 490m// |
| C0 | 21°16.800' | 157°50.767' | 6.4 | BBADCP(1) | V,T | Velocity Profile, Temp at bottom// |
| Hickum | 21°18.217' | 157°57.183' | 5.2 | W/T(1) | WL,W,T |  |
| Haleiwa | 21°35.580' | 158°06.370' | 3.0 | W/T(1) | WL,W,T |  |
| Waianae |  |  |  |  |  | //3/95-7/95 |
| F3 | 21°14.539' | 158°03.316' | 320 | NBADCP(1) | V,T | Vel. Pro.., Bottom Temp // Moved from D3, 4/95 |
| C1 | 21°16.448' | 157°50.829' | 19 | S4 (1) | V,T,C | //Moved from B1 4/95 |

–  Instruments:  BBADCP=RDI Broadband Acoustic Doppler Current Profiler; W/T=SeaBird Wave Tide Gauge; GO/Mk2=General Oceanics Mk2 Vector Averaging Current Meter; S4=InterOcean S4 Vector Measuring Current Meter; Thermistor=Aanderaa Thermistor String at 11 levels; NBADCP=RDI Narrow Band Acoustic Doppler Current Profiler.

–  Variables:  V=velocity vector (u,v); T=temperature; WL=water level; W=waves;C=conductivity; P=pressure.

Table 3.1  Information concerning the location, instrument type and variables measured at the various sites.