Table 16.  	Comparison of calculated chemical stressor concentration in outflows from sub-embayments after initial dilution (facor of 50) with Hawaii water quality standards.. Values in bold italics are within a factor of 100 of Hawaii water quality standards. All values are given as g/L. Based on data provided in Stevenson et al. (1995).

|  | Cadmium | Copper | Lead | Zinc |
| --- | --- | --- | --- | --- |
| Hawaii Saltwater Acute Standard | 43 | ns1 | 140 | 95 |
| Hawaii Saltwater Chronic Standard | 9.3 | ns | ns | 86 |
| Ala Wai Canal Outflow | 0.010 | 0.28 2 | 0.022 | 1.35 |
| Kewalo Basin Outflow | 0.011 | 0.26 2 | 0.032 | 3.22 |
| Keehi Lagoon Outflow | 0.006 | 0.19 2 | 0.018 | 1.55 |
| Pearl Harbor Outflow | 0.003 | 0.11 2 | 0.042 | 0.77 |

1. 	“ns” indicates no standard.

2.  	Concentration deemed high in relaton to effluent limits for WWTPs in CA.