MAMALA BAY STUDY

SUPPLEMENTAL STAPHYLOCOCCUS STUDY

PRINCIPAL INVESTIGATORS

Russell T. Hill

Center of Marine Biotechnology

Columbus Center, Suite 236

701 East Pratt Street

University of Maryland, Baltimore, Maryland 21202

Roger S. Fujioka

Water Resources Research Center

2540 Dole Street, Holmes Hall 283

University of Hawaii, Honolulu Hawaii 96822

November 30, 1995

PHASE I REPORT

TITLE:	 Assessing the Sources, Concentrations and Fate of Staphylococcus aureus

in  Mamala Bay

PRINCIPAL INVESTIGATOR:   Roger S. Fujioka

RESEARCH STAFF: 	Tuamasaga Unutoa, Bunnie Yoneyama,

Audrey Asahina, Fred Bonilla

ADDRESS:	Water Resources Research Center

2540 Dole street, Holmes Hall 283

University of Hawaii, Honolulu, Hawaii 96822

PROJECT PERIOD:	August 1, 1995 to December 31, 1995

I. INTRODUCTION AND DESCRIPTION OF STAPHYLOCOCCUS STUDY

A. Water-Contact Versus Water-Borne Diseases Associated with Swimming. Skin infections by staphylococcus bacteria following recreational use of water is classified as a water-contact disease defined as disease or microbial infection of eyes, ears, nose, throat, or skin of people obtained as a result of contact with water. The mode of transmission is contact with pathogenic microorganisms in water and the subsequent infection of some external part of man by one of more microorganisms. The normal habitats of microorganisms which cause water contact diseases are the water and/or  some external sources on the human body such as eyes, ears, nose, throat, and skin. Since many of these microorganisms are found on the skin or mucus membranes of humans, they are readily washed off into the water environment when someone swims. Water-contact diseases are frequently reported following swimming. One explanation is that the water environment is not a normal environment for humans and swimming allows water and microorganisms to gain entrance to many parts of the body to establish conditions for microbial infections. Moreover, swimming often results in cuts and abrasions to swimmers which provide opportunities for microbial infections.

In contrast to water-contact diseases, water-borne diseases are caused by different groups of microorganisms and are transmitted by ingestion of the causative microorganism. The normal habitats of the microorganisms responsible for water-borne diseases are the intestinal tracts of man and animal. As a result the primary environmental source of these microorganisms is feces or sewage.

B. Staphylococcus Skin Infections. Staphylococcus is a genus of bacteria comprised of over 32 species which are characterized as gram-positive cocci which occurs in pairs, short chains or grape-like clusters (Kloos and Bannerman, 1995). The normal habitats of staphylococcus bacteria are the skin and mucous membranes of mammals and birds. Most of the staphylococcus bacteria are not pathogenic to man and can be found in man’s environment, even in dust and soil. Staphylococcus aureus is the one species known to cause skin infections but even this pathogen is commonly found in the nose and skin of normal people who are not suffering from any skin infections. S. aureus is characterized by being coagulase positive or having enzymes capable of causing blood plasma to clot. Transmission of S. aureus skin infection is by contact but  a healthy intact skin is generally resistant to even high concentrations of this bacteria (Marples and Kligman, 1985).  However, when the skin is abraided, cut, and other defense mechanisms of man are lowered, much lower concentrations of S. aureus are required to establish a skin infection.

C. Staphylococcus Skin Infection Following Recreational Use of Water. Skin infection with  S. aureus is commonly observed following recreational use of waters. Pien et al ( 1983) reported that activities related to recreational use of waters create conditions (cuts, skin abrasions, softening of skin, penetration of water) which are conducive to skin infections. There are two mechanisms by which one contracts staphylococcus skin infection resulting from recreational use of water. In the first mechanism, S. aureus is carried on the external skin of an individual and swimming provides the opportunity for this staphylococcus bacteria to gain entrance under the skin of that individual to cause a skin infection. This mechanism results in self-infection of an individual and therefore does not constitute a public health problem. Many physicians believe in this mechanism because repeated staphylococcus skin infections often occurs on the same individual. In the second mechanism, S. aureus is washed off the bodies of one swimmer into the water and water serves as the vehicle for the transmission of skin infection to another swimmer. This mechanism results in the transmission of disease from one individual to another and therefore creates a public health problem. Epidemiologist support this second theory and believe that some people are more susceptible to staphylococcus skin infections than others.

Evans (1977)  was one of the first to evaluate the possibility that S. aureus could be transmitted by recreational use of water. He concluded that swimmers can be expected to become infected with S. aureus following recreational use of waters for two reasons. First, since the source of this bacteria is the skin and mucus membranes of people, S. aureus will be washed off the bodies of swimmers into the bathing waters. Second, since S. aureus is known to be much more resistant to environmental conditions such as drying, high salt concentrations and even disinfection than fecal coliform bacteria, this bacteria can be expected to persist in the water and to increase the potential time it can be infectious in the water environment. Evans (1977) also pointed out that S. aureus has very complex nutritional requirements and is therefore unlikely to multiply in most waters because the considerable array of organic nutrients it requires are not likely to be found in waters suitable for recreational use by humans.

To this day, a reliable method to selectively enumerate staphylococcus bacteria and especially S. aureus from all types of water has not been developed. It has been this lack of methodology that has prevented definitive studies on how S. aureus is being transmitted via recreational use of waters. Some studies monitor water for all species of staphylococcus (total staphylococcus), while others try to additionally determine the concentrations of the pathogenic S. aureus in the water sample. A review of the literature shows that studies conducted in Israel (Yoshpe-Purer and Golderman, 1987), in Spain (Alonso et al. 1989) in Canada (Seyfried et al. 1985)  and Hawaii (Charoenca and Fujioka, 1993)  have concluded that concentrations of total staphylococci bacteria or S. aureus in water can be correlated with swimmer density and can be used as an overall index of water quality. It should be noted that two major studies (Fujioka and Charoenca, 1991; Fujioka and Pennington, 1993) were completed in Hawaii which addressed the problem of staphylococcus bacteria in recreational waters. Some of the findings in these two studies are as follows: (1) A reliable but tedious and time consuming method was developed to enumerate the concentrations of  total staphylococcus and S. aureus from marine waters in Hawaii. (2) Beaches in Hawaii could be categorized as high or low concentrations of total staphylococcus bacteria and this classification correlated with the number of people using the beach and the flushing capacity of the water at the beach sites. (3) Based  on a 24 hour sampling study, the concentrations of staphylococcus in the water correlated with the numbers of swimmers during the day and persisted in the water after the swimmers left the water. (4) S. aureus recovered from south beaches (Waikiki, Ala Moana, Hanauma) of Oahu were similar in biotype (antibiotic sensitivity and phage typing) to S. aureus  recovered from skin lesions of patients from Dr. Jeremy Lam’s office in Honolulu. Dr. Lam had previously correlated staphylococcus skin infections of children and their recent experience in swimming in south beaches of Oahu. (5) Concentrations of S. aureus  diluted in waters from the Ala Wai Canal and from Hanauma Bay remained stable and did not multiply in the absence of sunlight but was inactivated in the presence of sunlight.

In a recent study, Ahtiainen et al (1991) reported median concentration of staphylococcus of 1-2 CFU/100 ml in pristine water, 60 to 110 CFU/100 ml in waters impacted by agricultural run-off and 1000 to 1600 CFU/100 ml in treated wastewater. When just treated wastewater was assayed, they reported a median concentration of 500 CFU/100 ml for staphylococcus, 16,900 CFU/100 ml for fecal streptococci and 17,800 CFU/100 ml for fecal coliform. Thus, fecal indicator bacteria are present in much higher concentrations in sewage than staphylococcus bacteria. In this same report, the authors reported that only 2% of the staphylococcus in sewage was identified as S. aureus whereas 18% of the staphylococcus recovered from waters in uninhabited areas was identified as S. aureus. In environmental waters, total staphylococcus can always be expected to be present in higher concentrations than S. aureus. Moreover, the concentrations of  S. aureus can be expected to vary considerably. For this reason, a reasonable limit for total staphylococcus for recreational water is not to exceed a geometric mean of 100/100 ml (Charoenca and Fujioka, 1993).

D. Decisions and Final Agreement on Staphylococcus Study. The MB-7 (Microbiological Assessment) Team of the Mamala Bay Study was aware of previous studies which reported  higher incidences of water-contact diseases (infections of eye, ear, nose, throat, and skin infections) of people following recreational use of waters than water-borne diseases (diarrheal diseases). Moreover, that in Hawaii, staphylococcus skin infections following recreational use (swimming, surfing, paddling) in the waters of Mamala Bay have been reported by a significant number of people.  However, the MB-7 Team made a deliberate decision against the inclusion of staphylococcus study because the prevailing scientific evidence indicated that the source of staphylococcus in recreational waters is swimmers and the major goal of the Mamala Bay Study was to determine the impact of discharges of ocean sewage outfalls versus discharges of non-point sources (streams, storm drains, canals, harbors) on the quality of waters in Mamala Bay. In keeping with the primary goal of the Mamala Bay Study, the MB-7 Team used its resources to monitor waters in Mamala Bay for the presence of as many microbial indicators and pathogens known to be present in sewage and which represent water-borne diseases.

In June of 1995, the Mamala Bay Commissioners held a meeting in San Francisco  to receive the preliminary final reports from all of the principal investigators. During this meeting, the Mamala Bay Commissioners reported that some citizens in Honolulu believed that the source of staphylococcus bacteria in Mamala Bay is sewage and requested the inclusion of a staphylococcus study as part of the Mamala Bay Study. As a result, the Commissioners requested that Russell Hill coordinate with Roger Fujioka to conduct a short, supplemental study to determine whether the concentrations of staphylococcus bacteria in Mamala Bay pose a threat to swimmers and to address the concerns raised by Honolulu citizens that sewage contamination of Mamala Bay is the cause of the increased level of staphylococcus skin infections among people using the waters in Mamala Bay. In response to the Commissioners request, Russell Hill and Roger Fujioka planned and implemented the completion of  a short term supplemental staphylococcus study in  two phases. Phase I Study was conducted by Roger Fujioka to assess the sources, the concentrations and fate of staphylococcus bacteria using cultural methods. Phase II Study was conducted by Russell Hill to assess the concentrations, the virulence and the fate of staphylococcus bacteria using molecular gene probe methods.

II. OBJECTIVES OF PHASE I STAPHYLOCOCCUS STUDY

This supplemental staphylococcus study was initiated after the completion of all the planned projects of the Mamala Bay Study because of continuing concerns expressed by citizens of Hawaii that recreational users (swimmers, surfers, paddlers, boaters) of Mamala Bay have had recurring staphylococcus skin infections. A spokesman for the citizens of Honolulu on the problem of staphylococcus skin infections is Mr. Robert Rodman, Vice President of the Waikiki Residential Association. Mr. Rodman scheduled a meeting with Mr. Felix Limtiaco, Director of  the Department of Wastewater Management during the last week of May 1995 to explain his concerns that sewage being discharged into Mamala Bay was responsible for causing staphylococcus skin infections in people using the waters in Mamala Bay. Roger Fujioka was asked to attend this meeting because of his experience in monitoring waters in Hawaii for staphylococcus bacteria. Having heard of Mr. Rodman’s concerns, Roger Fujioka invited Mr. Rodman to the Water Resources Research Center, University of Hawaii  on September 29, 1995 to meet with members of the Mamala Bay Team, to answer questions that Mr. Rodman had about staphylococcus, to clearly understand  his concerns, and to explain how experiments could be designed to address his specific claims that sewage was entering Mamala Bay (Waikiki Beach) from underground discharges either from hotel basements or from broken sewer lines. Mr. Rodman stated his belief that staphylococcus bacteria was entering Mamala Bay directly from this source of sewage or the nutrients provided by these sewage discharges allowed staphylococcus bacteria to multiply in the waters of Mamala Bay.

The objectives of Phase I as outlined below were determined based on the general Mamala Bay Study approach, discussions with Russell Hill and the concerns of Mr. Rodman:

1. To determine the concentrations of total staphylococcus bacteria and the pathogenic species of staphylococcus (Staphylococcus aureus) at beach sites (Ala Moana, Waikiki, Queen’s Surf, Sand Island park, Hanauma Bay, Ewa Beach) established earlier by the Mamala Bay Study on a weekly basis for two reasons. First to determine the concentrations of staphylococcus at these beaches. Second, to provide Dr. Hill with isolates of staphylococcus bacteria, especially  S. aureus from these sites for his molecular probe tests.

2. To coordinate and to plan for a field experimental study with Dr. Hill to sample all sites routinely monitored during the quarterly sampling by the MB-7 Team for total staphylococcus and S. aureus. By monitoring the concentrations of staphylococcus bacteria at these sites, a conclusion can be made whether the ocean sewage outfall is a major source of staphylococcus bacteria which is then being transported to the shoreline waters of Mamala Bay.

3. To determine the stability of S. aureus in the marine waters of Mamala Bay and to determine the effect of nutrients (peptone and 50% sewage) on the ability of this bacteria to grow. This objective was included specifically to test Mr. Rodman’s theory that S. aureus was multiplying in the waters of Mamala Bay.

4. To determine the stability of E. coli and S. faecalis in the marine waters of Mamala Bay and to determine the effect of nutrients (peptone and 50% sewage) on the ability of this bacteria to grow. This objective was included as a means to compare the response of S. aureus  with that of E. coli and S. faecalis under the same set of conditions   since most of the available monitoring data includes concentrations of these two fecal indicators.

5. To determine whether nutrients such as 50% sewage and peptone will have a stabililizing effect on the inactivation of S. aureus by sunlight. We previously determined that sunlight will inactivate S. aureus suspended in clean water. This objective was included to determine the effects of nutrients (consequence of pollution) on the inactivating effect of sunlight for S. aureus.

III. RESULTS: Sources and Concentrations of  Staphylococcus Bacteria.

A. Objective and Experimental Design. To address objectives 1 and 2 of this study,

grab samples of water were collected from popular recreational beach sites, offshore and deep ocean sites and analyzed for total staphylococcus bacteria and S. aureus using the method as previously described by Charoenca and Fujioka (1993). Beach water samples were collected once a week over a 4-week period in September and October. During the second week of October, all of the sites established for quarterly sampling by the MB-7 team were sampled for staphylococcus bacteria over a three day period using a boat to collect the offshore sites. During this field sampling study, Dr. Russell Hill from the University of Maryland and his assistant, Victoria Boccuzi, collected their own samples while personnel from the Hawaii team collected water samples from each of the MB-7 sites. The Hawaii Team also provided the University of Maryland Team with laboratory facilities to process their samples and also collected, clarified effluent from Sand Island Wastewater Treatment Plant (SIWWTP).

Samples were assayed on VJ agar supplemented with 0.005% sodium azide using the membrane filtration technique and incubated at 35°C for 44 ± 4 hours using the method developed in our laboratory (Fujioka and Charoenca, 1991, Charoenca and Fujioka, 1993).  It should be noted that this medium was optimized to selectively recover staphylococci bacteria from marine water samples as it inhibits the growth of marine bacteria. Black, shiny colonies were determined to be presumptive staphylococcus bacteria.  A representative number of presumptive staphylococcus colonies were streaked on Tyrpticase soy agar and incubated at 35°C for 22 ± 2 hours. The presumptive isolates were then gram stained and tested for catalase.  Gram positive cocci colonies which were positive on the catalase test were confirmed as total staphylococcus bacteria. The number of gram positive cocci that were also catalase positive divided by the number of isolates from each site was considered the percentage of total staphylococci from that site.  This number was then multiplied by the presumptive staphylococci count (all black, shiny colonies on VJ agar) to obtain the number of total staphylococci. These staphylococcus isolates were further tested for characteristics of S. aureus based on the latex agglutination and coagulase tube tests.  Positive results on both tests confirmed that the isolate was S. aureus. All positive isolates on the latex agglutination and coagulase tube tests were confirmed by a gene test for S. aureus using the Gen Probe Kit which detects specific ribosomal RNA sequences unique to S. aureus. Approximately 50 isolates, a representative number from each beach site, were screened using the Gen Probe. The results correlated well with the coagulase test results.

B. Discussion of Total Staphylococcus Counts. Results of total staphylococcus bacteria counts from the beach sites are summarized in Table 1. Waikiki, Queens Surf, Hanauma Bay, and Iroquois beaches had geometric mean concentrations of total staphylococcus bacteria which were above 100 CFU/100 ml, a level which was previously used (Charoenca and Fujioka, 1993)  to categorize beaches with high concentrations of staphylococcus. Waikiki beach had consistently high counts above this level for all four samples, with a range of 264 to 1,264 CFU/100 ml. It was also the beach with the highest numbers of swimmers. These results support previous observations that high concentrations of staphylococcus bacteria occur at beaches with high numbers of swimmers.

Only one sample of water was obtained from Hanauma Bay and from Black Point as these were our control sites. Hanauma Bay beach is a beach with high numbers of swimmers and poor flushing capacity. The concentration of total staphylococcus bacteria in water at this beach (148 CFU/100 ml) exceeded the 100 CFU/100 ml guideline while the concentration of total staphylococcus (88 CFU/100 ml) in water from Black Point beach characterized by fewer swimmers and better flushing capacity was below the guideline number. An attempt was made to determine the concentrations of total staphylococcus bacteria in the Sand Island sewage using the modified VJ medium, although this medium was known to be  unsuitable for  stream water and sewage samples.  When this medium was used to analyze the sewage sample, a presumptive staphylococcus count of 8.2 x 105 CFU/100 ml was obtained. However, only 1 out of 8 presumptive isolates was a gram positive coccus and 6 of these isolates were catalase positive. None of these isolates were both gram positive or catalase positive Therefore, none of these isolates were confirmed as staphylococcus bacteria. These results confirm that non staphylococcus bacteria in sewage are present in much higher concentrations than staphylococcus and the VJ medium is unsuitable to analyze sewage water. Based on the report by Ahtiainen et al (1991), one can expect a total staphylococcus count of 1,000 to 1600 CFU/100 ml with very low percentage of S. aureus in sewage. Dr. Hill’s gene probe method may be able to provide some information on the concentrations of total staphylococcus  and S. aureus bacteria in the Sand Island sewage sample.

Total staphylococci counts at the offshore Waikiki (W2) and offshore Ala Wai (AW2) sites were very low (<1 to 4 CFU/100 ml) compared to the near-shore sites (Table 2).  Similarly, at the deep ocean sites, including the Sand Island ocean outfall (D2), total staphylococci counts were very low. It should be noted that to increase the sensitivity of the method, a total of  1000 ml or ten times the volume of water sampled at beach sites were sampled at each of the deep ocean sites to recover staphylococcus bacteria. The geometric mean concentrations of total staphylococci bacteria at all of the offshore sites, including Sand Island ocean outfall site (D2), were less than 10 CFU/100 ml.The only slightly elevated site was at the mouth of Pearl Harbor where the total staphylococcus count from the surface sample was 39.4 CFU/100 ml and most probably reflect the contribution of discharges from the Fort Kamehameha WWTP and other land-based discharges in that area.

C. Results of S. aureus Counts. After the total staphylococcus counts are determined, the percent of the total staphylococcus bacteria recovered which are S. aureus must be determined based on further testing of the total staphylococcus isolates using the latex agglutination and coagulase tests. The isolates of S. aureus  were then confirmed using the Gen Probe assay, a specific molecular detection test for S. aureus. When these tests were applied, the geometric mean concentrations of   S. aureus was detected only at 3 CFU/100 ml at Waikiki Beach, and only 1 CFU/100 ml at  Ala Moana and Ewa beaches (Table 3). The low geometric mean concentrations of S. aureus reflect the fact that S. aureus was detected in only one of four samples from these three sites.  None of the total staphylococcus isolates collected at the offshore and deep ocean sites, including the Sand Island ocean outfall could be confirmed as S. aureus, using the latex agglutination and coagulase tests. It should be noted that these results reflect only 1 to 4 samples per site taken over a 1-2 month period. The brief period of this study is a limitation in the interpretation of data.

D.  Conclusions.  The data collected show that concentrations of staphylococcus bacteria were highest at shoreline beach sites and lowest from ocean sites located between the ocean outfall and the shoreline beach sites. The actual concentrations of total staphylococcus bacteria in sewage being discharged into Mamala Bay could not be determined because the culture method used was not suitable for sewage sample. However, based on the results of sampling the ocean water sites near and away from the sewage ocean outfall site, the concentrations of total staphylococcus at these sites were very low, even when ten times the volume of water sample were analyzed. These results provide evidence that sewage being discharged into the ocean via the Sand Island Outfall is not the source of staphylococcus bacteria being recovered from the waters from shoreline beach sites. The  best explanation for these results is that sewage discharged via the Sand Island Ocean Outfall is effectively diluted with sea water and ocean currents primarily transport the sewage away the beaches. Moreover, high salt concentrations, sunlight, and predation by other organisms are factors known to inactivate most of the sewage-borne microorganisms.

The concentrations of staphylococcus bacteria were consistently higher at beach sites and were higher at beach sites (Waikiki, Hanauma Bay) with greater numbers of swimmers. Since the normal habitat of staphylococcus is skin and mucus membranes of people, the best explanation is that swimmers are the major source of staphylococcus bacteria recovered from beach water samples. Moreover, based on the results of this study, only a small percentage of the total staphylococcus bacteria recovered  from beach water could actually be confirmed as being the pathogenic S. aureus. Clinical evidence show that staphylococcus skin infections is primarily caused by S. aureus and not by other species of the genus Staphylococcus which are measured as total staphylococcus. Thus, the data collected during this short-term study, indicate that the risk of skin infections by acquiring S. aureus from the waters in Mamala Bay was very low.

IV. RESULTS: Fate (stability, growth, death) of S. aureus in Environmental Waters

A. Objectives and Experimental Design.  Objective 3 of this study was included to address the specific concerns of citizens of Honolulu that sewage is entering the beaches near Waikiki and is the source of staphylococcus bacteria in these waters or the sewage is providing the nutrients to enable S. aureus to multiply in the marine water and thereby increase the risk of staphylococcus infections to people who use the waters for recreational uses. Low fecal indicator bacterial counts at Waikiki Beaches indicate that measurable amounts of sewage are not directly being discharged into these beaches. However, groundwater sources  with or without sewage  are probably entering the beaches at Waikiki and these may be sources of nutrients being discharged into Mamala Bay.  To determine whether sewage could serve as a nutrient source for the multiplication of S. aureus, an experiment was conducted to determine whether S. aureus would multiply in ocean water containing 50% sewage.

For this experiment, water samples from Black Point beach, Manoa Stream and primary sewage effluent from the Sand Island Wastewater Treatment Plant (SIWWTP) were collected and filter-sterilized through a 0.22 mm filter into a sterile flask. Samples of sewage and stream waters were pre-filtered in order to remove suspended sediments and large particles.  An estuary-type water was also prepared by preparing an equal mixture (50:50) of filter-sterilized ocean and stream water. The growth of S. aureus was then determined in these three types of waters (ocean, stream, stream/ocean mixture) in the absence of added nutrients and in the presence of nutrients in the form of 50% sewage and 0.05 to 0.5% peptone, a  nutrient used to prepare commercial bacteriological medium to culture S. aureus. The filter-sterilized samples were held in sterile glass beakers lined with foil and kept in the lab at room temperature (24 +/- 2°C). These samples were then seeded with approximately 104 cells/ml of an 18-24 hr overnight culture of S. aureus (ATCC: 25912). Growth was monitored over 6 days and samples were taken at 0 and 6 hours each day. Samples were assayed on VJ agar supplemented with 0.005% sodium azide and incubated at 35°C for 44 ± 4 hours.

B. Results and Discussions. The results of this experiment are tabulated in Table 4 and the results for each of the water samples graphed. The results graphed in Figure 1 show that in filter-sterilized stream water, the concentrations of S. aureus slowly declined when the stream water was not supplemented with any nutrients or when supplemented with 50% sewage. However, when the stream water was supplemented with 0.05% peptone, substantial growth (5 logs) of  S. aureus was observed. Similar response was observed when the suspending medium was filter sterilized marine water (Figure 2) and filter sterilized estaurine water (Figure 3). These results indicate that stream water, estaurine waters and marine waters do not have enough nutrients to support the growth of S. aureus. However, S. aureus can readily persist in these waters for at least 24 hours. The addition of 50% sewage to these waters did not result in sufficient nutrients to allow S. aureus to multiply but the presence of sewage allowed S. aureus to persist longer in these waters indicating that the bacteria was utilizing some nutrients from the sewage. The addition of 0.05% and 0.5% peptone to these three types of waters provided sufficient  nutrients required for S. aureus to multiply. It should be noted that peptone is the source of nutrients used to prepare culture medium for the growth of a wide variety of bacteria.

C. Conclusions.  The design of this experiment was to create conditions to specifically determine whether the nutrients available will enable S. aureus to multiply in environmental waters. This was done by filter sterilizing  the three types of environmental waters (stream, estuary, marine) to remove indigenous microorganisms but not the nutrients in these waters. Under natural conditions high concentrations of indigenous microorganisms will be present in the test water samples and can be expected to out-compete S. aureus in the uptake of nutrients and may even inhibit the growth of S. aureus using a number of known mechanisms. Moreover, the inactivating effect of sunlight was also excluded in these experiments. Since natural environmental conditions differ from the conditions of this experiment, caution must be applied in the interpretation of the results of this experiment.

Under the conditions of this experiment, the results showed that there is insufficient nutrients in natural marine, estuary or stream waters for S. aureus to multiply. However, S. aureus is able to persist for at least 24 hours indicating the resistance of this bacteria to high salt concentrations found in marine waters. In the absence of nutrients, the concentrations of S. aureus in these natural waters are reduced after 24 hours. A similar reaction was observed when 50% sewage was added to the three types of environmental waters. Thus, even 50% sewage does not contain all the essential nutrients required for S. aureus to multiply. The presence of sewage and nutrients in sewage did enable S. aureus  to persist longer in environmental waters. However, when 0.05% and 0.5%  peptone were added to the same three types of environmental waters, S. aureus was able to multiply and to persist even longer. These results support data in the literature that S. aureus is a hardy bacteria which can survive or persist under environmental conditions. However, due to its complex growth requirements (organic nutrients), growth of this bacteria under recreational water conditions is not likely, even when recreational waters is polluted with up to 50% with sewage.

V. RESULTS: Comparative Fate of E. coli and S. faecalis in Environmental Waters

A. Objectives and Experimental Design. The presence of sewage in environmental waters has traditionally been measured based on the concentrations of fecal bacteria such as E. coli and S. faecalis. Considerable more data is available on the concentrations of fecal bacteria in environmental waters than other kinds of bacteria such as S. aureus. Objective 4 of this study was to determine the fate of E. coli and S. faecalis, in the same three types of water under the same set of conditions as S. aureus so water quality data obtained based on concentrations of E. coli and S. faecalis can be applied to S. aureus.

To address objective 4 of this study, the same methodology and experimental design used for monitoring the growth of S. aureus was used. Filter-sterilized stream and ocean water samples were seeded with E. coli and S. faecalis (105 cells/ml for each bacterium) and monitored for growth over a period of 4 days. A 50:50 stream-ocean mixture was also prepared and seeded with the same bacteria to simulate an estuary-type water. The seeded samples were kept in the laboratory at room temperature (25°C) during the study period. The membrane filter technique was used to assay the samples. E. coli was recovered on mFC and incubated at 35°C for 24 ± 4 hours.  This was done in order to recover some of the stressed bacteria which would not have survived the standard temperature of 44.5°C. S. faecalis was recovered on mE and incubated at 41°C ± 4 hours 44 ± 4 hours.

B. Results and Discussions.  The results of this experiment are tabulated in Table 5 and the results for the individual water samples graphed in Figures 4-9. The results for E. coli (Figure 4) and  S. faecalis (Figure 7) in ocean waters show that in unsupplemented ocean water both  E. coli and S. faecalis did not grow but remained stable over the 4 day period. These results show that ocean water does not contain sufficient nutrients to support the multiplication of E. coli and S. faecalis and the high salt concentrations of ocean water did not inactivate these two fecal indicator bacteria. However, when this ocean water was supplemented with 50% sewage or with 0.05% and 0.5% peptone, multiplication of both E. coli and S. faecalis was evident. These results indicate that 50% sewage and as well as 0.05% peptone provided enough nutrients for the growth of both E. coli and S. faecalis in ocean water.

The same experiment was conducted in stream water and a 50:50 mixture of stream water and ocean water. Stream water is considered a better environment than ocean water because its low salt concentrations will provide less stress to the bacteria and because stream water can be expected to contain more nutrients than ocean water. In unsupplemented stream water, some multiplication of  E. coli (Figure 5) was observed while no multiplication of S. faecalis (Figure 8) was observed under these same conditions. These results indicate that stream water contains sufficient nutrients to allow E. coli but not S. faecalis to multiply in the absence of indigenous microorganisms. These results support previous reports that E. coli has simple nutritional requirements for growth  while S. faecalis has more complex nutritional requirements for growth. When stream water was supplemented with 50% sewage or peptone, multiplication of both  E. coli and S. faecalis was evident showing that nutrients in the form of 50% sewage and peptone will enhance the multiplication of both of these fecal indicators in stream water. Similar results were obtained when the water used was a 50:50 mixture of stream water and ocean water (Figures 6 and 9).

C. Conclusions. The conditions of this experiment were similar to the conditions of the previous S. aureus experiment and therefore the same precautions established for interpreting the results of the previous S. aureus experiment apply to this set of experiment. Although the results of this test do not reflect conditions under natural environment, the results measures the ability of E. coli and S. faecalis to utilize nutrients and multiply in environmental waters. Moreover, the results of this experiment should be comparable to the previous S. aureus experiment.

The following conclusions can be drawn from the results. Ocean water did not contain enough nutrients to allow E. coli and S. faecalis to grow whereas stream water contained enough nutrients to allow E. coli but not S. faecalis to grow. The addition of 50% sewage or 0.05% peptone to ocean water or stream water resulted in sufficient nutrients to allow both of the fecal indicator bacteria to grow. Since S. aureus did not grow in these same waters even when 50% sewage was added, it can be concluded that the nutritional requirement for growth of E. coli is the simplest, followed by S. faecalis and that S. aureus has the most complex growth requirements. These results are consistent with previous reports which have documented the simple growth requirements of E. coli, the more complex growth requirement of S. faecalis and the very complex growth requirements of S. aureus. Based only on these data, one can conclude that the impact of nutrients in environmental waters will allow E. coli to multiply first, followed by S. faecalis and lastly S. aureus. However in natural waters, indigenous microorganisms will outcompete S. aureus and fecal bacteria for available nutrients. The natural stability of bacteria to environmental factors also determines the final concentrations of bacteria in water. In this regard, it has been reported that E. coli is highly susceptible to inactivation by environmental factors (high salt, sunlight, predation, disinfection) whereas S. faecalis and S. aureus are known to be more resistant to these environmental factors. The principle of microbial ecology states that microorganisms whose natural habitat is not environmental waters will generally not be able to multiply in environmental waters unless they can adapt to environmental conditions.

VI.  RESULTS: Effects of Nutrients on Sunlight Inactivation of S. aureus.

A. Objective and Experimental Design. We previously determined that sunlight is a natural environmental agent which inactivated many bacteria, including S. aureus. However, in past experiments, the inactivating effect of sunlight was determined on S. aureus suspended in clean water samples. The results of this study showed that the presence of nutrients in environmental waters enabled S. aureus to persist longer (50% sewage) and to even multiply (0.05% peptone). The objective of  this experiment was to determine whether the presence of these kinds of nutrients, which generally follow a pollution event,  will prevent or retard the inactivating effect of sunlight.

In this experiment, ocean water from Black Point Beach was filter-sterilized and used to prepare the following three types of samples: a) unsupplemented ocean water, b) ocean water supplemented with 50% sewage, and c) ocean water supplemented with 0.05% peptone. All three water samples were clear to the naked eye. S. aureus was added to all three types of water. One set of these l liter samples were kept under laboratory conditions in the absence of sunlight and the concentrations of S. aureus determined at time 0 and after six hours in the absence of sunlight. The second set of these samples were placed into beakers with a magnetic stirrer and the samples continuously exposed to sunlight on the roof of Holmes Hall. As in previous sunlight experiments, the beakers were placed in a water bath to keep the water sample cool. Samples were obtained after the first 30 minutes exposure to sunlight and hourly over a four-hour period. All samples were measured for concentrations of S. aureus using the membrane filtration method and the modified VJ medium as previously described.

B. Results and Discussions. The results of this experiments are summarized in Table 6 and the data graphed in Figure 10. The results show that the concentrations of  S. aureus suspended in all three types of waters remained unchanged when held in the absence of sunlight indicating that ocean water itself did not inactivate this bacteria even after 6 hours. In contrast, the population of S. aureus, suspended in all three types of water was effectively inactivated  after 1-2 hour exposure to sunlight. The inactivation of  S. aureus  was most rapid in clean ocean water indicating that nutrients such as 50% sewage and 0.05 peptone slightly reduces the rate of sunlight inactivation. However, since these nutrients only slightly  reduced the rate of sunlight inactivation, it can be concluded that nutrients in the form of contamination are insufficient to protect against the inactivating effect of sunlight.

C. Conclusions.  The results of this experiment confirm our previous findings that S. aureus is susceptible to sunlight inactivation and demonstrated that sunlight inactivation even occurred when the ocean water was supplemented with enough nutrients to maintain cell structure (50% sewage) and to allow for the multiplication of S. aureus (0.05% peptone). These results provide evidence that natural sunlight is an effective mechanism for the inactivation of many types of bacteria suspended in marine waters.

VII. FINAL ASSESSMENT AND RECOMMENDATIONS

In Phase I of this study, the data and conclusions were based on using standard cultural techniques to enumerate bacteria which are viable and able to be cultured on bacteriological medium. These are the kinds of methods used in clinical laboratory  and  in water monitoring programs to enumerate and identify bacteria such as S. aureus. These methods are used to determine whether water quality standards are met and used to assess the risks to human infection. However, it is well known that cultural techniques recover the more healthy bacteria and bacteria which are injured, starved or have converted to some survival form may not or will not be detected by cultural methods. The presence and significance of bacterial forms which are non-culturable will be addressed in Phase II of this study.

Recognizing that people do acquire staphylococcus skin infections and that this study did not determine how people are becoming infected, the following recommendations are made:

1. The development of isolation and enumeration methods for staphylococcus bacteria and for S. aureus which is reliable, feasible and which can be applied to all types of samples (sewage, feces,  stream water, storm drain water, estuary water, ocean water).

2. The fate of total staphylococcus and S. aureus in sewage, stream, storm drain before and after it enters the natural ocean environment should be determined.

3. The concentrations of  total and S. aureus in sand above the water line and under the water in swimming area should be determined as these sites provide opportunities for human infection.

4. The contribution of total staphylococcus and S. aureus by feces and bodies of pigeons who  frequent the beach should be determined.

5. The contribution of total staphylococcus and S. aureus being washed off the bodies of  humans entering the coastal water should be determined.

6. A study should focus on people who chronically obtain staphylococcus skin infections following recreational use of water for the purpose of determining the mechanisms by which these people acquire staphylococcus skin infections. Only after the mechanism(s) of acquiring S. aureus skin infection are known can effective preventive management decisions be made. At the present time, the best approach may be to provide showers so swimmers will shower before they enter coastal waters. Showering will wash off most of the bacteria on the bodies of people. This guideline is already instituted for swimming pools.

7. Characterization of the biotype as well as the virulence factors of S. aureus recovered from people’s normal skin, from infected skin and from environmental sources (water, sand) should be determined and compared. Simpler methods should be developed for this characterization.

VIII. REFERENCES CITED

1. Ahtiainen, M. Niemi and H. Jousimies-Somer. 1991. Staphylococci in polluted waters and in waters of uninhabited areas. Wat. Sci. Tech. 24:103-108.

2.  Alonso, J.L., I. Amoros and E. Hernandez. 1989. Recovery of staphylococcus species from marine recreational waters of Puebla De Farnals (Valencia, Spain). Wat. Sci. and Tech. 21:239-241.

3. Charoenca, N. and R. Fujioka. 1993. Assessment of Staphylococcus bacteria in Hawaii’s marine recreational waters. Wat. Sci. Tech. 27:283-289.

4. Charoenca, N. and R. Fujioka. 1995. Association of staphylococcal skin infections and

swimming. Wat Sci and Tech. 31: 11-17.

5. Evans, J.B. 1977. Coagulase positive staphylococci as indicators of potential health hazards from water.  Pages 126-130. In, “Bacterial Indicators/Health Hazards Associated with Water”. ASTM STP 635. Editors: A.W. Hoadley and B.J. Dutka.

6. Fujioka, R. and N. Charoenca. 1991. Establishing the concentrations of staphylococcus, vibrio, pseudomonas bacteria and phosphate in Hawaii’s recreational waters. WRRC Project Completion Report 5:31:91.

7. Fujioka, R. and K. Pennington. 1993. Assessing the effects of natural events and management practices on the microbial quality of recreational waters. WRRC Project Completion Report 04:08:93.

8. Kloos, W.E. and T.L. Bannerman. 1995. Staphylococcus and Micrococcus. In, “Manual of Clinical Microbiology”, 6th Edition. pg. 282-298. Editors: P. Murray, E. Baron, M. Pfaller, F. Tenover, R. Yolken.

9. Marples, R.R. and A.M. Kligman. 1978. Experimental staphylococcal infections of the skin in man. In, “Staphylococci and Staphylococcal Diseases”, pg. 755-760. Editor: J. Jeljaszewicz.

10. Pien, F.D., K.S. Ang, N.T. Nakashima, D.G. Evans, J.A. Grote, M.L. Hefley and E.A. Kubota. 1983. Bacterial flora of marine penetrating injuries. Diag. Microbiol. Inf. Dis. 1:229-232.

11. Seyfried, P.L., R.S. Tobin, N.E. Brown and P.F. Ness. 1985. A prospective study of swimming-related illness. II. Morbidity and the microbiological quality of water. Am. J. Pub Health 75:1071-1075.

12. Yoshpe-Purer, Y. and S. Golderman. 1987. Occurrence of Staphylococcus aureus and Pseudomonas aeruginosa in Israeli coastal water. Appl. Environ. Microbiol. 53:1138-1141.