TOGA SEA LEVEL CENTER/JOINT ARCHIVE FOR SEA LEVEL

'ARCHIVING' FORMAT

FOR HOURLY SEA LEVEL DATA

FILE NAME CONVENTION:

INHOUSE METHOD:

Each file is given a name Hsssvyy.DAT where sss denotes the

the TSLC/JASL inhouse station number, v denotes the version or

independent series for a given station, and yy denotes the year.

For example, H003B87.DAT is the 1987 observed hourly data file in

the permanent archive for Baltra-B.  Baltra is a station with

more than one independent series due to a station replacement

that is not linked to the bench marks of the original station.

DESCRIPTIVE METHOD:

Each file is given a name nnnnnnyy.hsv where 'nnnnnn' is the first

six characters of the station name and 'yy' is the year.  The extension

character 'h' denotes hourly data; 's' denotes series version for cases

when the station has several time segments that are not linked to the

same tide staff zero; and 'v' denotes the version of the file, which is

for the convenience of the user to partition the data into separate files

if desired.  Note if the station name has less than 6 characters, then

the file name will also.  For example, yap86.ha1, is the hourly data file

for 1986 at Yap.

FORMAT DESCRIPTION:

A file exists for each year with data and every hourly value

in the year contains either data or a missing data flag (9999).

Only one header per file is given followed by two records per

day for the complete year.  All records are fixed-length, 80-byte.

The unique header for each file is coded as:

field          bytes   length             comment

-----------------  -------  ------  ------------------------------------

station number        1-3     3     exactly 3 digits

station version         4     1     letter from A to Z

station name         6-23    18     Abbreviated if necessary

region/country      25-43    19     Abbreviated if necessary

year                45-48     4

latitude            50-55     6     degrees, minutes, tenths

(implied decimal), and hemisphere

longitude           57-63     7     degrees, minutes, tenths

(implied decimal), and hemisphere

GMT offset          65-68     4     time data are related to in terms

of difference from GMT in hours

and tenths (implied decimal) with

East latitudes positive*

decimation method      70     1     Coded as 1 : filtered

2 : simple average of all

samples

3 : spot readings

4 : other

reference offset    72-76     5     constant offset to be added to each

each data value for data to be

relative to tide staff zero or primary

datum in same units as data

reference code         77     1     R = data referenced to datum

X = data not referenced to datum

units               79-80     2     always millimeters, MM

* Data always are in GMT (offset=0000) unless data are relative to a local

time zone that is not an increment of one hour from GMT.  For example,

Colombo, Sri Lanka has a GMT offset = 0055 which is 5.5 hours ahead

of GMT.

The data records are coded as:

field          bytes   length             comment

-----------------  -------  ------  ------------------------------------

station number        1-3     3     exactly 3 digits

station version         4     1     letter from A to Z

station name          6-9     4     Abbreviated if necessary

year                12-15     4

month               16-17     2     numerical value

day                 18-19     2

day record count       20     1     either 1 or 2

sea level data      21-80    60     12 sea level values

For example:

029A Kapingamarangi     Fd St Micronesia    1987 01059N 154466E 0000 1 00000R MM

029A Kapi  1987 1 11 1768 1661 1476 1267 1078  956  914  975 1067 1189 1280 1311

029A Kapi  1987 1 12 1250 1097  975  792  671  610  732  884 1006 1341 1585 1737

029A Kapi  1987 1 21 1829 1768 1692 1494 1219 1036  945  914 1006 1097 1158 1280

029A Kapi  1987 1 22 1341 1219 1097  945  792  701  671  762  884 1067 1341 1554

029A Kapi  1987 1 31 1737 1829 1768 1615 1433 1219 1036  945  914 1006 1036 1189

029A Kapi  1987 1 32 1280 1311 1219 1067 1006  823  732  701  792  884 1097 1372