Reading MEDS Format

There are two programs found here that show how to read MEDS ASCII format,

the format of the data on this CD. The first, called MEDS_ASCII_LIST, can

be used to produce a listing to a printer of the contents of a file. The

second , called OCPROC_TO_TABLES, reads a binary form of the format, and

outputs the contents of the FORTRAN structures to separate files in a comma

separated value format. This latter form can be used to put data data into

any application software such as spreadsheet programs or some relational

databases. You can use this second program in combination with the first to

go from the format of data on this CD to tables.

Here are a few examples of how to use the contents of the format to filter

out records of interest. The software changes shown in these examples fit

into appropriate places in the first program.

Example 1: Selecting only those stations that have a quality flag of 'good'

for the position.

The quality flag for position is in the FXD structure and is called Q_POS.

It is a single character field and good positions are flagged with the

character '1'. At the point marked C*** Example 1 in MEDS_ASCII_LIST simply

insert

IF (STAT.FXD.Q_POS.NE.'1') ILIKE = 0

The variable ILIKE is used as a switch to control whether or not the data

from the station being read are printed or not. If ILIKE is anything other

than 1, it is not printed. Instead of printing the station, you could write

the information out to a file.

Example 2: Selecting only stations with temperature and salinity profiles.

This process requires looking at the contents of the PROF structure of the

station record. You need to add some code at the point marked C*** Example

2 in MEDS_ASCII_LIST. First if there is only one type of profile, then you

can go on to the next station. So just before

DO I=1,STAT.FXD.NO_PROF

put in the following check

IF (STAT.FXD.NO_PROF.LT.2) ILIKE = 0

Again we have used variable ILIKE to turn off writing the station to

output.

But this is only part of the job. Now within the DO loop, noted above, you

must check if both temperature and salinity profiles are present. Variable

STAT.PROF(I).PROF_TYPE is a four character variable that describes what is

the profile. You must check for 'TEMP' for temperature and any of 'PSAL',

'SSAL' or 'USAL' for salinity. If both TEMP and one of the others are

present, set ILIKE to 1.

Example 3: Select only those data that have passed through scientific

quality control.

To find data that have under gone scientific quality control, you have a

couple of places in the format where you may look. One place is at the

point marked C*** Example 3-1. Each scientific centre records which of its

tests the data have undergone and which tests were failed. Each of these at

each science centre is identified by a different value of

STAT.SURF_CODES(I).PCODE. PCODE is a four character variable. Look for

PCODE to be 'QAO$' or 'QAP$' to identify tests performed at AOML. For those

performed at Scripps, look for 'QSP$' or 'QSF$'. For those performed at

CSIRO, look for 'QRF$' or 'QRP$'. If you find PCODE is any one of these,

the data have passed through scientific quality control at the indicated

centre.

An alternate way to find out where the data have been is to look in the

HISTORY structure. This is marked by C*** Example 3-2. This part of the

format is used to track where the data have been. As each agency handling

the data process them, they write at least one record in this history

structure. The variable STAT.HISTORY(I).IDENT_CODE is a two character

variable. The science centres write a record using their two character

identifiers. For AOML this is 'AO', for Scripps this is 'SI' and for CSIRO

this is 'CS'. So, you need only look for an IDENT_CODE matching one of

these to know that the data have passed through science centre processing.

Simply use

IF STAT.HISTORY(I).IDENT_CODE.EQ.'AO') ILIKE = 1

to select only those records that have been through processing at AOML.

Example 4: Select temperatures between 100 and 400 m depth only.

To find only those temperatures that lie between 100 and 400 m you must

look into the profile records. At the point marked C*** Example 4.1 check

that PRF.FXD.PROF_TYPE is 'TEMP' indicating a temperature profile. If so,

then in the listing at C*** Example 4.2 you need to put in something like

the following.

IF (PRF.PROF(I).DEPTH_PRESS.GE.100. AND.

PRF.PROF(I).DEPTH_PRESS.LE.400.) ILIKE = 1

Again ILIKE is used to indicate whether or not to select the results for

output. Of course, you could do other things at this point to only write

the temperatures at those depths.

You can also use PRF.PROF(I).Q_PARM to select only those values that have a

particular data quality flag (since this flag is stored in Q_PARM). The

software needed is similar to what was shown above in example 1.

You should be aware that the format contains at most 1500 depth-value pairs

in a single physical record. If a profile has more than 1500 depth-value

pairs they are divided into segments. The segment number is given by

PRF.FXD.PROFILE_SEG and will be a character string of '01', '02', etc. You

must take care to read all of the segments of a profile. That is what the

software is doing at the point in the program following the comment "Count

the number of profile segments to read".

Example 5: Find XBTs that have used the new fall rate equations.

Unless information is specifically present, you should assume the old fall

rate equations have been used. XBTs that have used the new fall rate

equations always have information about the probe, recorder and the

equations. This information is stored in the SURF_CODES structure. You need

to look for a PCODE set to 'PFR$'. The values stored in CPARM are the

values for the WMO code tables 1770 and 4770 in that order (see document on

WMO codes). You look for these codes in the same way as illustrated in

example 3-1 above. If the code is present, look at the first 3 characters

of the value in CPARM as these encode the probe type and the fall rate

equation used. Compare these to WMO code table 1770 to determine which

equation was used to calculate depth.

Example 6: Find high density XBT data.

To find these data you will need to make use of the ocean area, year and

ship identifier. The ocean area given in the list of high density lines

will tell you which ocean area files to search in. The year will narrow

down the search to 4 files (one for each quarter). Then you use the ship

identifier to select only those stations from each file. This is done by

inserting code at the position marked Example 6. Insert

IF (STAT.FXD.CR_NUMBER.EQ.'SHIP    95') ILIKE = 1

where, again, setting the variable ILIKE to 1 means you wish to select this

station. Note that you should insert the correct ship identifier and the

last two digits of the year in question. The example treats data from a

ship with identifier SHIP collected in year 1995.

Reading MEDS ASCII Format

PROGRAM MEDS_ASCII_LIST

C       Reads and lists the MEDS ASCII format written by NODC

CHARACTER*25568 INSTR

C       -------------------------------------------------------------

C...    STATION STRUCTURE

C       -------------------------------------------------------------

STRUCTURE /PR_STN/

STRUCTURE FXD

CHARACTER*8     MKEY

INTEGER*4       ONE_DEG_SQ

CHARACTER*10    CR_NUMBER

CHARACTER*4     OBS_YEAR

CHARACTER*2     OBS_MONTH

CHARACTER*2     OBS_DAY

CHARACTER*4     OBS_TIME

CHARACTER*2     DATA_TYPE

INTEGER*4       IUMSGNO

CHARACTER*1     STREAM_SOURCE

CHARACTER*1     U_FLAG

INTEGER*2       STN_NUMBER

REAL*4          LATITUDE

REAL*4          LONGITUDE

CHARACTER*1     Q_POS

CHARACTER*1     Q_DATE_TIME

CHARACTER*1     Q_RECORD

CHARACTER*8     UP_DATE

CHARACTER*12    BUL_TIME

CHARACTER*6     BUL_HEADER

CHARACTER*4     SOURCE_ID

CHARACTER*4     STREAM_IDENT

CHARACTER*4     QC_VERSION

CHARACTER*1     AVAIL

INTEGER*2       NO_PROF

INTEGER*2       NPARMS

INTEGER*2       SPARMS

INTEGER*2       NUM_HISTS

END STRUCTURE

STRUCTURE PROF(1:20)

INTEGER*2       NO_SEG

CHARACTER*4     PROF_TYPE

CHARACTER*1     DUP_FLAG

CHARACTER*1     DIGIT_CODE

CHARACTER*1     STANDARD

REAL*4          DEEP_DEPTH

END STRUCTURE

STRUCTURE SURFACE(1:20)

CHARACTER*4     PCODE

REAL*4          PARM

CHARACTER*1     Q_PARM

END STRUCTURE

STRUCTURE SURF_CODES(1:20)

CHARACTER*4     PCODE

CHARACTER*10    CPARM

CHARACTER*1     Q_PARM

END STRUCTURE

STRUCTURE HISTORY(1:100)

CHARACTER*2     IDENT_CODE

CHARACTER*4     PRC_CODE

CHARACTER*4     VERSION

INTEGER*4       PRC_DATE

CHARACTER*2     ACT_CODE

CHARACTER*4     ACT_PARM

REAL*4          AUX_ID

REAL*4          O_VALUE

END STRUCTURE

END STRUCTURE

RECORD /PR_STN/STAT

C       -------------------------------------------------------------

C...     PROFILE STRUCTURE

C       -------------------------------------------------------------

STRUCTURE /PR_PROFILE/

STRUCTURE FXD

CHARACTER*8     MKEY

INTEGER*4       ONE_DEG_SQ

CHARACTER*10    CR_NUMBER

CHARACTER*4     OBS_YEAR

CHARACTER*2     OBS_MONTH

CHARACTER*2     OBS_DAY

CHARACTER*4     OBS_TIME

CHARACTER*2     DATA_TYPE

INTEGER*4       IUMSGNO

CHARACTER*4     PROF_TYPE

CHARACTER*2     PROFILE_SEG

INTEGER*2       NO_DEPTHS

CHARACTER*1     D_P_CODE

END STRUCTURE

STRUCTURE PROF(1:1500)

REAL*4          DEPTH_PRESS

CHARACTER*1     DP_FLAG

REAL*4          PARM

CHARACTER*1     Q_PARM

END STRUCTURE

END STRUCTURE

RECORD /PR_PROFILE/PRF

C

OPEN(UNIT=1,STATUS='OLD',FORM='FORMATTED',

&   RECORDTYPE='VARIABLE',READONLY,RECL=25568)

C

OPEN(UNIT=2,STATUS='NEW',FORM='FORMATTED')

C

C   Use ILIKE to qualify whether or not a record should be printed.

C   If ILIKE = 1 it prints, otherwise it does not.

ILIKE = 1

NWR=1

NN=1

C

1       CONTINUE

READ(1,100,END=999) INSTR

100     FORMAT(A25568)

C

C   Read FXD structure

STAT.FXD.MKEY = INSTR(1:8)

READ(INSTR(9:16),101) STAT.FXD.ONE_DEG_SQ

101     FORMAT(I8)

STAT.FXD.CR_NUMBER = INSTR(17:26)

C*** Example 6

STAT.FXD.OBS_YEAR = INSTR(27:30)

STAT.FXD.OBS_MONTH = INSTR(31:32)

STAT.FXD.OBS_DAY = INSTR(33:34)

STAT.FXD.OBS_TIME = INSTR(35:38)

STAT.FXD.DATA_TYPE = INSTR(39:40)

READ(INSTR(41:52),102) STAT.FXD.IUMSGNO

102     FORMAT(I12)

STAT.FXD.STREAM_SOURCE = INSTR(53:53)

STAT.FXD.U_FLAG = INSTR(54:54)

READ(INSTR(55:62),103) STAT.FXD.STN_NUMBER

103     FORMAT(I8)

READ(INSTR(63:70),104)  STAT.FXD.LATITUDE

104     FORMAT(F8.4)

READ(INSTR(71:79),105) STAT.FXD.LONGITUDE

105     FORMAT(F9.4)

STAT.FXD.Q_POS = INSTR(80:80)

C*** Example 1

STAT.FXD.Q_DATE_TIME = INSTR(81:81)

STAT.FXD.Q_RECORD = INSTR(82:82)

STAT.FXD.UP_DATE = INSTR(83:90)

STAT.FXD.BUL_TIME = INSTR(91:102)

STAT.FXD.BUL_HEADER = INSTR(103:108)

STAT.FXD.SOURCE_ID = INSTR(109:112)

STAT.FXD.STREAM_IDENT = INSTR(113:116)

STAT.FXD.QC_VERSION = INSTR(117:120)

STAT.FXD.AVAIL = INSTR(121:121)

READ(INSTR(122:123),106) STAT.FXD.NO_PROF

106     FORMAT(I2)

READ(INSTR(124:125),106) STAT.FXD.NPARMS

READ(INSTR(126:127),106) STAT.FXD.SPARMS

READ(INSTR(128:130),107) STAT.FXD.NUM_HISTS

107     FORMAT(I3)

C

C       Read PROF structure

ISTART = 131

C*** Example 2

DO I=1,STAT.FXD.NO_PROF

IST = ISTART + 14*(I-1)

READ(INSTR(IST:IST+1),106) STAT.PROF(I).NO_SEG

STAT.PROF(I).PROF_TYPE = INSTR(IST+2:IST+5)

STAT.PROF(I).DUP_FLAG = INSTR(IST+6:IST+6)

STAT.PROF(I).DIGIT_CODE = INSTR(IST+7:IST+7)

STAT.PROF(I).STANDARD = INSTR(IST+8:IST+8)

READ(INSTR(IST+9:IST+13),108) STAT.PROF(I).DEEP_DEPTH

108       FORMAT(F5.0)

ENDDO

C

C   Read SURFACE structure

ISTART = ISTART + 14*(STAT.FXD.NO_PROF)

DO I=1,STAT.FXD.NPARMS

IST = ISTART + 15*(I-1)

STAT.SURFACE(I).PCODE = INSTR(IST:IST+3)

READ(INSTR(IST+4:IST+13),109) STAT.SURFACE(I).PARM

109       FORMAT(F10.3)

STAT.SURFACE(I).Q_PARM = INSTR(14:14)

ENDDO

C

C   Read SURF_CODES structure

ISTART = ISTART + 15*(STAT.FXD.NPARMS)

C*** Example 3-1

DO I=1,STAT.FXD.SPARMS

IST = ISTART + 15*(I-1)

STAT.SURF_CODES(I).PCODE = INSTR(IST:IST+3)

STAT.SURF_CODES(I).CPARM = INSTR(IST+4:IST+13)

STAT.SURF_CODES(I).Q_PARM = INSTR(14:14)

ENDDO

C

C   Read HISTORY structure

ISTART = ISTART + 15*(STAT.FXD.SPARMS)

DO I=1,STAT.FXD.NUM_HISTS

IST = ISTART + 42*(I-1)

STAT.HISTORY(I).IDENT_CODE = INSTR(IST:IST+1)

C*** Example 3-2

STAT.HISTORY(I).PRC_CODE = INSTR(IST+2:IST+5)

STAT.HISTORY(I).VERSION = INSTR(IST+6:IST+9)

READ(INSTR(IST+10:IST+17),103) STAT.HISTORY(I).PRC_DATE

STAT.HISTORY(I).ACT_CODE = INSTR(IST+18:IST+19)

STAT.HISTORY(I).ACT_PARM = INSTR(IST+20:IST+23)

READ(INSTR(IST+24:IST+31),110) STAT.HISTORY(I).AUX_ID

110       FORMAT(F8.3)

READ(INSTR(IST+32:IST+41),111) STAT.HISTORY(I).O_VALUE

111       FORMAT(F10.5)

ENDDO

C

IF(ILIKE.EQ.1) THEN

WRITE (6,4002) NWR

4002    FORMAT (///'***********************',I10/)

WRITE (6,4001) STAT.FXD.MKEY,STAT.FXD.IUMSGNO,

&   STAT.FXD.STREAM_SOURCE,STAT.FXD.U_FLAG

4001    FORMAT (' MKEY ',a10,'   IUMSGNO ',i10,'   STREAM_SOURCE ',a5,

&   '   UFLAG ',a5)

C

WRITE (6,4003) STAT.FXD.ONE_DEG_SQ,STAT.FXD.CR_NUMBER,

&   STAT.FXD.OBS_YEAR,STAT.FXD.OBS_MONTH,STAT.FXD.OBS_DAY,

&   STAT.FXD.OBS_TIME

4003    FORMAT (' ONE_DEG_SQ',1X,I6,3X,'CR_NUMBER',1X,A14,2X,'OBS_DATE',

&   1X,A4,2A2,3X,'OBS_TIME',1X,A4)

C

WRITE (6,4004) STAT.FXD.DATA_TYPE,STAT.FXD.STN_NUMBER,

&   STAT.FXD.LATITUDE,STAT.FXD.LONGITUDE

4004    FORMAT (' DATA_TYPE',1X,A2,3X,' STN_NUMBER ',I5,3X,' LATITUDE',

&   F10.4,3X,'LONGITUDE',F10.4)

C

WRITE (6,4005) STAT.FXD.Q_POS,STAT.FXD.Q_DATE_TIME,

&   STAT.FXD.Q_RECORD,STAT.FXD.UP_DATE

4005    FORMAT (' Q_POS ',A1,3X,'Q_DATE_TIME ',A1,3X,'Q_RECORD ',A1,3X,

&   'UP_DATE ',A8)

C

WRITE (6,4011) STAT.FXD.BUL_TIME,STAT.FXD.BUL_HEADER,

&   STAT.FXD.SOURCE_ID,STAT.FXD.STREAM_IDENT,STAT.FXD.QC_VERSION,

&   STAT.FXD.AVAIL

4011    FORMAT (' BUL_TIME',1X,A12,2X,'BUL_HEADER',1X,A6,2X,

&   'SOURCE_ID',1X,A4,2X,'STREAM_IDENT',1X,A4/' QC_VERSION ',A4,3X,

&   'DATA_AVAIL ',A1)

C

WRITE (6,4006) STAT.FXD.NO_PROF,

&   (STAT.PROF(I).NO_SEG,STAT.PROF(I).PROF_TYPE,

&   STAT.PROF(I).DUP_FLAG,STAT.PROF(I).DIGIT_CODE,

&   STAT.PROF(I).STANDARD,STAT.PROF(I).DEEP_DEPTH,

&   I=1,STAT.FXD.NO_PROF)

4006    FORMAT (/' VECTOR OF ',I2,' PROFILE DESCRIPTORS - NO_SEG,'

&   ' PROF_TYPE, DUP_FLAG,'/'   DIGIT_CODE, STANDARD, DEEP_DEPTH'/

&   (I5,1X,A4,1X,A1,1X,A1,1X,A1,F8.1,'.'))

C

WRITE (6,4007)  STAT.FXD.NPARMS,

&   (STAT.SURFACE(I).PCODE,STAT.SURFACE(I).PARM,

&   STAT.SURFACE(I).Q_PARM,

&   I=1,STAT.FXD.NPARMS)

4007    FORMAT (/' VECTOR OF ',I2,' STATION LEVEL PARAMETERS - PCODE,'

&   ' PARM, Q_PARM'/3(3X,A4,1X,F10.4,3X,A1))

C

WRITE (6,4014) STAT.FXD.SPARMS,

&   (STAT.SURF_CODES(I).PCODE,STAT.SURF_CODES(I).CPARM,

&   STAT.SURF_CODES(I).Q_PARM,

&   I=1,STAT.FXD.SPARMS)

4014    FORMAT (/' VECTOR OF ',I2,' STATION CHARACTER FIELDS - ',

&   'SRFC_CODE, SRFC_PARM, SRFC_Q_PARM'/3(3X,A4,1X,A10,1X,A1))

C

WRITE (6,4010) STAT.FXD.NUM_HISTS,

&   (STAT.HISTORY(I).IDENT_CODE,STAT.HISTORY(I).PRC_CODE,

&   STAT.HISTORY(I).VERSION,STAT.HISTORY(I).PRC_DATE,

&   STAT.HISTORY(I).ACT_CODE,STAT.HISTORY(I).ACT_PARM,

&   STAT.HISTORY(I).AUX_ID,STAT.HISTORY(I).O_VALUE,

&   I=1,STAT.FXD.NUM_HISTS)

4010    FORMAT (//' VECTOR OF ',I3,' HISTORY RECORDS'/

&   ' - IDENT_CODE, PRC_CODE, VERSION, PRC_DATE, ACT_CODE,

&   ACT_PARM, AUX_ID, ORIG_VAL'/

&   (1X,A2,1X,A4,2X,A4,2X,I8,2X,A2,2X,A4,2X,F9.3,2X,F9.3))

NWR=NWR+1

ENDIF

C

C  Count the number of profile segments to read

NO_PRF = 0

DO I = 1,STAT.FXD.NO_PROF

NO_PRF = NO_PRF + STAT.PROF(I).NO_SEG

ENDDO

C

C  Read the profile segments

DO J=1,NO_PRF

READ(1,100,END=999) INSTR

C  Read FXD structure

PRF.FXD.MKEY = INSTR(1:8)

READ(INSTR(9:16),101) PRF.FXD.ONE_DEG_SQ

PRF.FXD.CR_NUMBER = INSTR(17:26)

PRF.FXD.OBS_YEAR = INSTR(27:30)

PRF.FXD.OBS_MONTH = INSTR(31:32)

PRF.FXD.OBS_DAY = INSTR(33:34)

PRF.FXD.OBS_TIME = INSTR(35:38)

PRF.FXD.DATA_TYPE = INSTR(39:40)

READ(INSTR(41:52),102) PRF.FXD.IUMSGNO

PRF.FXD.PROF_TYPE = INSTR(53:56)

C*** Example 4.1

PRF.FXD.PROFILE_SEG = INSTR(57:58)

READ(INSTR(59:62),112) PRF.FXD.NO_DEPTHS

112       FORMAT(I4)

PRF.FXD.D_P_CODE = INSTR(63:63)

C

C   Read PROF structure

ISTART = 64

DO I=1,PRF.FXD.NO_DEPTHS

IST = ISTART + 17*(I-1)

C*** Example 4.2

READ(INSTR(IST:IST+5),113) PRF.PROF(I).DEPTH_PRESS

113         FORMAT(F6.1)

PRF.PROF(I).DP_FLAG = INSTR(IST+6:IST+6)

READ(INSTR(IST+8:IST+16),114) PRF.PROF(I).PARM

114         FORMAT(F9.3)

PRF.PROF(I).Q_PARM = INSTR(IST+17:IST+17)

ENDDO

C

C  Write the profile segments

IF(ILIKE.EQ.1) THEN

WRITE (6,5001) PRF.FXD.MKEY,PRF.FXD.IUMSGNO

5001    FORMAT (//' MKEY ',a10,'   IUMSGNO ',i10)

C

WRITE (6,5003) PRF.FXD.ONE_DEG_SQ,PRF.FXD.CR_NUMBER,

&   PRF.FXD.OBS_YEAR,PRF.FXD.OBS_MONTH,PRF.FXD.OBS_DAY,

&   PRF.FXD.OBS_TIME

5003    FORMAT (' ONE_DEG_SQ',1X,I6,3X,'CR_NUMBER',1X,A14,2X,'OBS_DATE',

&   1X,A4,2A2,3X,'OBS_TIME',1X,A4)

C

WRITE (6,5004) PRF.FXD.DATA_TYPE,PRF.FXD.PROF_TYPE,

&   PRF.FXD.PROFILE_SEG

5004    FORMAT (' DATA_TYPE',1X,A2,3X,' PROF_TYPE ',A4,3X,

&   ' PROFILE_SEG ',A2)

C

WRITE (6,5012) PRF.FXD.NO_DEPTHS,PRF.FXD.D_P_CODE,

&   (PRF.PROF(I).DEPTH_PRESS,PRF.PROF(I).DP_FLAG,

&   PRF.PROF(I).PARM,PRF.PROF(I).Q_PARM,

&   I=1,PRF.FXD.NO_DEPTHS)

5012    FORMAT (/' NO_DEPTHS ',I5,3X,'D-P-CODE ',A1/

&   ' - DEPTH_PRESS, DP_FLAG, PROF_PARM, ',

&   'PROF_Q_PARM'/3(5X,F8.1,1X,A1,F10.4,1X,A1))

ENDIF

ENDDO

C

NN=NN+1

GO TO 1

C

999     CONTINUE

PRINT 190,NN-1,NWR-1

190     FORMAT(' ',2I10,' STATIONS WERE READ AND OUTPUT')

STOP

END

Writing data to tables.

PROGRAM OCPROC_TO_TABLES

C       Reads binary format and writes contents to separate tables

C       for each structure

CHARACTER STNNO*4, AKEY*17, CRN*10, DATETIME*10, OTIME*5, PD*10

CHARACTER PDC*8, UDATE*10, STR*1, COMMA*1, STR1*5000

DIMENSION STR(5000)

EQUIVALENCE (STR1,STR)

C

DICTIONARY 'CDD$TOP.APPLICATION.OCEAN.PROCESS_STN_REC/LIST'

RECORD /PR_STN/STAT

DICTIONARY 'CDD$TOP.APPLICATION.OCEAN.PROCESS_PROFILE_REC/LIST'

RECORD /PR_PROFILE/PRF

C

OPEN(UNIT=1,STATUS='OLD',FORM='UNFORMATTED',

&   RECORDTYPE='VARIABLE',READONLY,IOSTAT=IOS)

IF(IOS.NE.0)OPEN(UNIT=1,STATUS='OLD',FORM='UNFORMATTED',

&   READONLY,RECORDTYPE='VARIABLE',ORGANIZATION='INDEXED')

C

OPEN(UNIT=11,STATUS='NEW',FORM='FORMATTED',RECL=10000)

OPEN(UNIT=12,STATUS='NEW',FORM='FORMATTED')

OPEN(UNIT=13,STATUS='NEW',FORM='FORMATTED')

OPEN(UNIT=14,STATUS='NEW',FORM='FORMATTED')

OPEN(UNIT=15,STATUS='NEW',FORM='FORMATTED')

OPEN(UNIT=16,STATUS='NEW',FORM='FORMATTED')

C

COMMA = ','

NWR=1

NWRITE1 = 0

NWRITE2 = 0

NWRITE3 = 0

NWRITE4 = 0

NWRITE5 = 0

NWRITE6 = 0

NN=1

C

1       CONTINUE

READ(1,END=999) STAT.FXD,

&     (STAT.PROF(I),I=1,STAT.FXD.NO_PROF),

&     (STAT.SURFACE(J),J=1,STAT.FXD.NPARMS),

&     (STAT.SURF_CODES(K),K=1,STAT.FXD.SPARMS),

&     (STAT.HISTORY(L),L=1,STAT.FXD.NUM_HISTS)

C

WRITE(STNNO,4000) STAT.FXD.STN_NUMBER

4000    FORMAT(I4.4)

IF(STAT.FXD.CR_NUMBER(10:10).EQ.' ') THEN

CRN = ' '//STAT.FXD.CR_NUMBER(1:9)

ELSE

CRN = STAT.FXD.CR_NUMBER

ENDIF

AKEY = CRN//'-'//STNNO//STAT.FXD.DATA_TYPE

DATETIME = STAT.FXD.OBS_MONTH//'/'//STAT.FXD.OBS_DAY//

&   '/'//STAT.FXD.OBS_YEAR

OTIME = STAT.FXD.OBS_TIME(1:2)//':'//STAT.FXD.OBS_TIME(3:4)

UDATE = STAT.FXD.UP_DATE(5:6)//'/'//STAT.FXD.UP_DATE(7:8)//

&   '/'//STAT.FXD.UP_DATE(1:4)

C

WRITE (STR1,4011) AKEY, COMMA, STAT.FXD.ONE_DEG_SQ,

&   COMMA, STAT.FXD.CR_NUMBER, COMMA,

&   STAT.FXD.STN_NUMBER, COMMA, DATETIME, COMMA,

&   STAT.FXD.OBS_MONTH, COMMA,

&   OTIME, COMMA, STAT.FXD.DATA_TYPE, COMMA,

&   STAT.FXD.LATITUDE, COMMA, STAT.FXD.LONGITUDE, COMMA,

&   STAT.FXD.Q_POS, COMMA, STAT.FXD.Q_DATE_TIME, COMMA,

&   STAT.FXD.Q_RECORD, COMMA, UDATE, COMMA, STAT.FXD.BUL_TIME,

&   COMMA, STAT.FXD.BUL_HEADER, COMMA,

&   STAT.FXD.SOURCE_ID, COMMA, STAT.FXD.STREAM_IDENT, COMMA,

&   STAT.FXD.QC_VERSION, COMMA, STAT.FXD.AVAIL

4011    FORMAT (A20,A1,I7,A1,A11,A1,I4.4,A1,A11,A1,A3,A1,A6,A1,A3,

&   A1,F10.4,A1,F10.4,6A2,A1,A11,

&   A1,A13,A1,A7,3(A1,A5),2A2)

CALL SQUEEZE(STR, NS)

WRITE(11,4100) (STR(I),I=1,NS)

4100    FORMAT(5000A1)

NWRITE1 = NWRITE1 + 1

C

DO I=1,STAT.FXD.NPARMS

WRITE (STR1,4013)  AKEY, COMMA,

&   STAT.SURFACE(I).PCODE, COMMA, STAT.SURFACE(I).PARM, COMMA,

&   STAT.SURFACE(I).Q_PARM

4013    FORMAT (A17,A1,A4,A1,F10.4,2A1)

CALL SQUEEZE(STR, NS)

WRITE(13,4100) (STR(II),II=1,NS)

NWRITE3 = NWRITE3 + 1

ENDDO

C

DO I=1,STAT.FXD.SPARMS

WRITE (STR1,4014) AKEY, COMMA,

&   STAT.SURF_CODES(I).PCODE, COMMA, STAT.SURF_CODES(I).CPARM,

&   COMMA, STAT.SURF_CODES(I).Q_PARM

4014    FORMAT (A17,A1,A4,A1,A10,2A1)

CALL SQUEEZE(STR, NS)

WRITE(14,4100) (STR(II),II=1,NS)

NWRITE4 = NWRITE4 + 1

ENDDO

C

DO I=1,STAT.FXD.NUM_HISTS

WRITE(PDC,4002) STAT.HISTORY(I).PRC_DATE

4002    FORMAT(I8)

PD = PDC(5:6)//'/'//PDC(7:8)//'/'//PDC(1:4)

WRITE (STR1,4015) AKEY, COMMA, I, COMMA,

&   STAT.HISTORY(I).IDENT_CODE, COMMA, STAT.HISTORY(I).PRC_CODE,

&   COMMA, STAT.HISTORY(I).VERSION, COMMA, PD, COMMA,

&   STAT.HISTORY(I).ACT_CODE, COMMA, STAT.HISTORY(I).ACT_PARM,

&   COMMA, STAT.HISTORY(I).AUX_ID, COMMA,

&   STAT.HISTORY(I).O_VALUE

CALL SQUEEZE(STR, NS)

WRITE(15,4100) (STR(II),II=1,NS)

NWRITE5 = NWRITE5 + 1

ENDDO

4015    FORMAT (A17,A1,I3.3,2A2,A1,A4,A1,A4,A1,A12,2A2,A1,A4,

&   A1,F9.3,A1,F9.3)

NWR=NWR+1

C

C  Count the number of profile segments to read

NO_PRF = 0

DO 20 I = 1,STAT.FXD.NO_PROF

NO_PRF = NO_PRF + STAT.PROF(I).NO_SEG

20      CONTINUE

C

C  Read the profile segments

DO 50 J=1,NO_PRF

READ(1,END=999) PRF.FXD,

&   (PRF.PROF(I),I=1,PRF.FXD.NO_DEPTHS)

C

C  Write the profile segments

DO I=1,PRF.FXD.NO_DEPTHS

WRITE (STR1,4016) AKEY, COMMA, PRF.FXD.PROF_TYPE, COMMA,

&   PRF.PROF(I).DEPTH_PRESS, COMMA, PRF.PROF(I).DP_FLAG, COMMA,

&   PRF.PROF(I).PARM, COMMA, PRF.PROF(I).Q_PARM

4016    FORMAT (A17,A1,A5,A1,F10.4,2A2,A1,F10.4,2A2)

CALL SQUEEZE(STR, NS)

WRITE(16,4100) (STR(II),II=1,NS)

NWRITE6 = NWRITE6 + 1

ENDDO

C

50      CONTINUE

C

DO I=1,STAT.FXD.NO_PROF

WRITE (STR1,4012) AKEY, COMMA,

&   STAT.PROF(I).PROF_TYPE, COMMA,

&   STAT.PROF(I).DUP_FLAG, COMMA, STAT.PROF(I).DIGIT_CODE, COMMA,

&   STAT.PROF(I).STANDARD, COMMA, PRF.FXD.D_P_CODE, COMMA,

&   STAT.PROF(I).DEEP_DEPTH

4012    FORMAT (A17,A1,A4,2A1,2A1,4A2,A1,F8.1)

CALL SQUEEZE(STR, NS)

WRITE(12,4100) (STR(II),II=1,NS)

NWRITE2 = NWRITE2 + 1

ENDDO

C

NN=NN+1

c       IF(NN.LE.100) GO TO 1

GO TO 1

C

999     CONTINUE

PRINT 190,NN-1,NWR-1

190     FORMAT(' ',2I10,' STATIONS WERE READ AND OUTPUT')

PRINT 198, NWRITE1, NWRITE2, NWRITE3, NWRITE4, NWRITE5, NWRITE6

198     FORMAT(' Number of rows written:',/,

&   ' Station header: ',I20,/,

&   ' Profile info:   ',I20,/,

&   ' Surface info:   ',I20,/,

&   ' SCodes info:    ',I20,/,

&   ' History:        ',I20,/,

&   ' Profile data:   ',I20)

STOP

END

C

SUBROUTINE SQUEEZE(STR, NS)

C   Squeeze out imbedded blanks and remove trailing blanks and commas

CHARACTER STR*1

DIMENSION STR(3300)

C

IDFLG = 0

NSIZE = 3300

NS = NSIZE

DO I=1,NSIZE

J = 1 + NSIZE - I

IF(IDFLG.EQ.0) THEN

IF(STR(J).EQ.' ') THEN

NS = NS - 1

ELSE

IDFLG = 1

ENDIF

ELSE

IF(STR(J).EQ.' ') THEN

DO IJK = J,NS-1

STR(IJK) = STR(IJK+1)

ENDDO

NS = NS - 1

ENDIF

ENDIF

ENDDO

C

RETURN

END