UK HYDROGRAPHIC OFFICE

BROKEN SERIAL DATA RECORD FORMAT (BROKEN SERD)

NUMBER OF	SIGNIFICANT

CHARACTERS	CHARACTERS	FIELD

1	1/1	CARD NUMBER (1)

1	2/2	RECORD TYPE (MAIN = 2, CONTINUATION = 3)

2	3/4	SEQUENTIAL RECORD NUMBER (MAIN = 01,CONTINUATION = 02)

1	5/5	DATA IDENTIFIER

START OF COMMON HEADER - REPEATED IN CONTINUATION RECORD

1	6/6	DATA USE CODE

2	7/8	FILE CODE

3	9/11	MARSDEN SQUARE

2	12/13	DEGREE SQUARE

3	14/16	LATITUDE - DEGREES  (NEGATIVE CHARACTER = SOUTH)

4	17/20	LATITUDE - TENTHS OF MINUTES(NEGATIVE CHARACTER = 			SOUTH)

4	21/24	LONGITUDE - DEGREES(NEGATIVE CHARACTER = WEST)

4	25/28	LONGITUDE - TENTHS OF MINUTES (NEGATIVE CHARACTER = 			WEST)

1	29/29	QUADRANT  (ICES CODE AS OPPOSED TO WMO)

1	30/30	POSITION DETERMINATION ACCURACY

1	31/31	POSITION ACCURACY CODE

12	32/43	ADDITIONAL POSITION REFERENCE

End of card number 1 : 44 chars including data sensitive mark (ascii 012).

There is only one of these cards per observation.

1	1/1	CARD NUMBER (2)

2	2/3	ARCHIVE YEAR

4	4/7	YEAR

2	8/9	MONTH

2	10/11	DAY

2	12/13	HOUR	}  GREENWICH

2	14/15	MINUTE	}  MEAN TIME

2	16/17	COUNTRY CODE (ICES)

2	18/19	SHIP NUMBER

1	20/20	SHIP NUMBER CODE

8	21/28	ORIGINATORS CRUISE NUMBER

6	29/34	STATION NUMBER

3	35/37	INSTITUTE NUMBER

1	38/38	INSTITUTE NUMBER CODE

1	39/39	SPACE (IF LAND CHECKED 1 IS INSERTED)

3	40/42	NUMBER OF DEPTH LEVELS

End of card number 2 : 43 chars including data sensitive mark (ascii 012).

There is only one of these cards per observation.

END OF COMMON HEADER

THE FOLLOWING FIELDS ARE IN MAIN RECORD HEADER ONLY

NUMBER OF	SIGNIFICANT

CHARACTERS	CHARACTERS	FIELD

1	1/1	CARD NUMBER (3)

5	2/6	DEPTH TO SEABED (METRES)

4	7/10	MINIMUM DEPTH

4	11/14	MAXIMUM DEPTH

3	15/17	DEPTH		}

1	18/18	SPACE		}  CORRECTIONS

3	19/21	TEMPERATURE	}  (+VE OR -VE

4	22/25	SALINITY		}  ALWAYS USED).

3	26/28	SOUND VELOCITY	}

1	29/29	UNITS

2	30/31	INSTRUMENT CODE

1	32/32	DATA TYPE CODE

1	33/33	DATA MODE CODE

1	34/34	METHOD

End of card number 3 : 35 chars including data sensitive mark (ascii 012).

There is only one of these cards per observation.

1	1/1	CARD NUMBER (4)

2	2/3	WIND DIRECTION

2	4/5	WIND SPEED (KNOTS)

4	6/9	DRY AIR TEMP  } DEGREES AND TENTHS CELSIUS

4	10/13	WET AIR TEMP  } WITH +VE, 0 OR -VE PREFIX

1	14/14	WEATHER

1	15/15	CLOUD

1	16/16	SEA STATE

2	17/18	WAVE PERIOD (SECONDS)

2	19/20	WAVE HEIGHT (HALF METRE UNITS)

5	21/25	ATMOSPHERIC PRESSURE (MILLIBARS AND TENTHS)

2	26/27	WATER COLOUR (FOREL-ULE SCALE)

2	28/29	WATER TRANSPARENCY (METRES)

1	30/30	SALINITY SCALE CODE

11	31/41	FILLER

End of card number 4 : 42 chars including data sensitive mark (ascii o12).

There is only one of these cards.

1	1/1	CARD NUMBER (5)

70	2/71	COMMENT

End of card number 5 : 72 chars including data sensitive mark (ascii 012).

There may be from zero to nine of these cards.

END OF HEADER

DATA FIELDS

1	1/1	CARD NUMBER (6)

1	2/2	DEPTH INDICATOR

4	3/6	DEPTH (METRES)

1	7/7	DEPTH QUALITY

4	8/11	TEMPERATURE (DEGREES CELSIUS TO HUNDREDTHS)

1	12/12	TEMPERATURE QUALITY

5	13/17	SALINITY (PPT TO THOUSANDTHS)

1	18/18	SALINITY QUALITY

5	19/23	SOUND VELOCITY (METRES/SEC TO TENTHS)

1	24/24	SOUND VELOCITY QUALITY

1	25/25	SOUND VELOCITY CODE

End of card number 6 : 26 chars including data sensitive mark (ascii 012).

There may be from 2 to 49 of these cards in a main record and from 1 to 50

In a continuation record.

/db/pc_backups /formats/bsrd.wpd