Brooks McCall Status Report - Cruise 12 - Day 2 - 12 July 2010

Complied by: Alison Lane, Ecosystem Management and Associates Inc. (for BP)

Site conditions

Calm seas with wave height only approximately 1 to 2 feet and light winds throughout the day, although winds picked up slightly around midday and conditions became slightly choppier as the day progressed.

There was heavy vessel traffic around the wellhead. This limited our ability to work close to the wellhead to the south west of the well, where we expected the strongest plume signal to be found.

At 1400 hours, just after retrieval of our third cast, sampling was suspended due to a directive for all vessels to exit the area a minimum distance of 10 nautical miles from the wellhead to allow for work to be conducted by the seismic vessel Topaz.

Air Quality

VOC and benzene monitoring was conducted while we were in areas of heavy surface oiling to the north of the wellhead by Paul Bradley, Industrial Hygiene Specialist with Bureau Veritas. Maximum VOC reading at any site was 2.9 ppm. One crew member, Larry Lane, was fitted with a passive diffusion monitor prior to commencing the first cast (0630 hours) until the completion of deck work for the day (1630 hours).

Equipment

The noise in the dissolved oxygen signal from the Seabird was not seen in any of the three casts today.

Experiments were conducted by taking samples for dissolved oxygen immediately after the sample array surfaced and comparing the results to samples taken a short time later after other water sampling was completed. It was found that the samples warmed by up to 3C during the additional approximately 15 minutes (noting that the same sample had increased in temperature by 4C between collection at depth and arriving on the surface). In general the DO concentration appeared to decrease slightly and percentage saturation increased slightly as samples “aged” over the period between arriving on the surface and when the sample collection from the Niskin bottles was completed.

Due to being run out of the area early in the day, there was time for the sampling array to be deployed and all Niskin bottles triggered at a depth of 10 metres to enable Entrix personnel to run matrix samples. This test was conducted at a distance of 15km from the wellhead.

Water samples taken at 10 metres during the matrix spike cast were used to cross check the reported DO concentrations using the Seabird, Extech and YSI sensors as sampling from a shallow depth meant there would be less temperature variability in the samples. It was found that the readings from the Seabird and the YSI (optical) agreed to within 0.02mg/L, while the Extech varied substantially from the other two instruments, with readings up to 2mg/L higher using the Extech.

Findings

The following definition has been adopted to provide common terminology on subsurface oil:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."

A total of 3 CTD casts were completed, as summarized below and shown in the figure (over page).

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM123 | 3 km SW | Weak | 1120m – 1200m | Two fluorescence peaks – one weak between 1120m and 1160m and a second very weak peak at 1190m. No DO decrease associated with peaks. Fresh oil, broken silver sheen. |
| BM124 | 3.5 km SW | Weak | 1030m – 1200m | There was three small fluorescence peaks evident at this site. One weak at 1180m, one weak at 1140m and one very weak at 1035m. There was no decrease in DO apparent during the cast. Fresh oil coming to surface and continuous rainbow sheen. |
| BM125 | 4 km SW | Moderate | 1000m – 1190m | Three distinct peaks across an almost 200m depth range. The strongest of these (1140m) was moderate although the other two peaks were weak. No DO decrease apparent at depth of fluorescence peak. Occasional sheen and fragments of emulsified oil. |

BM123 was located 3 km to the south west of the wellhead. There was broken, silver surface sheen and signs of fresh oil surfacing around the vessel. There were two small peaks in fluorescence between 1120m and 1200m, although LISST analysis did not detect an increase in small particles at this station. DO minimum was between 420m and 550m depth.

BM124 was located 3.5 km to the south west of the wellhead, continuing on the bearing from the wellhead out to B123, where a weak signal was detected. Continuous rainbow sheen was present around the vessel. There were three points at which the fluorescence signal peaked slightly, centered at 1035m, 1140m and 1180m, although there was no decrease in DO evident associated with any of these peaks. LISST analysis did not detect an increase in small particles at this station. DO minimum was at approximately 440m depth. An equipment blank was run at this site.

BM125 was located 4 km to the south west of the wellhead, continuing the trajectory from the wellhead to BM123 and BM124. There were three peaks observed in the fluorescence, across a 200 meter depth profile. The strongest of these was at 1140 meters and it was noted that the signal at this depth showed an increase between the time the CTD was lowered and as it was being raised (approximately 20 minutes) suggesting that the plume at this site is a more dynamic that observed at earlier sites. The DO minimum was between 400m and 500m depth. This was the only site where the LISST analysis detected an increase in small particles at depths consistent with the peak in the in-situ fluorescence signal during the cast.

Toxicity Testing

Rotox tests started today for sample stations BM123 – 125, with result due to be reported on 13 July 2010. The test from stations BM121 and BM122 will not be read until late tonight due to the extended sampling period yesterday and so will be reported along with stations BM123 – 125 tomorrow. Results from toxicity testing on stations BM118 – BM120 are included with today’s deliverables. No significant mortality relative to controls was observed in any of the tests for these stations.

LISST and Fluorometry

The reports from the on-board LISST and fluorometry for both 11 July 2010 and 12 July 2010 are included with today’s deliverables

Summary

Day 2 of the Brooks McCall Cruise 12 sampled between 3km and 4km to the south west of the wellhead before sampling was ceased as we were directed to leave the work area. A weak plume signal was detected at sites 3km and 3.5km from the wellhead, with the signal increasing in strength and vertical extent with increased distance from the wellhead. A moderate signal was encountered at 4km from the wellhead. Changes in the fluorescence signal between deploying and retrieving the CTD through the same depth in the water column suggests that there is some heterogeneity in the subsurface concentrations of oil at this distance from the wellhead.

Crew Manifest

| OPEL, Chris | Master | TDI BROOKS |
| --- | --- | --- |
| MAJNI, Geno | Chief Mate | TDI BROOKS |
| KAMAL, Wasib | Chief Engineer | TDI BROOKS |
| McCLUSKEY, Ben | Oiler | TDI BROOKS |
| FELIX, Frank | A/B | TDI BROOKS |
| MAJNI, Ann | OS | TDI BROOKS |
| NARCISIS, Paul | Cook | TDI BROOKS |
| MACDONALD, Ian | Crew Chief | TDI BROOKS |
| BROOKS, Jimmy | Winch Operator | TDI BROOKS |
| LANE, Larry | Deck | TDI BROOKS |
| LANE, Alison | Chief Scientist | EM&A |
| HILL, Stephanie | Data Manager | CSS |
| ESSEX, Laura | Rototox testing | EM&A |
| CLARK, Paul | CTD Tech | GERG/TEXAS A&M |
| MA, Xiaowei | LISST and fluorometry | DFO |
| THAMER, Peter | LISST and fluorometry | DFO |
| MOLINARO, George | Water sampling | ENTRIX |
| BROWN, Robyn | Water sampling | ENTRIX |
| Name to be confirmed | Water sampling | ENTRIX |
| BRADLEY, Paul | Industrial Hygienist | BV |
| CONLEE, Don | Meteorologist | TEXAS A&M |
| Name to be confirmed | Meteorologist | TEXAS A&M |
| SIMMONS, Zeke | Water sampling | NOAA |
| KROOME, Janice | EPA representative | EPA |

***