Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 10, 2010

Water samples were collected at five stations for particle size distribution measurements using the LISST-100X particle counter. A total of 67 LISST samples were analyzed, including duplicates. Samples at depths of elevated fluorescence or other significance were selected from the CTD trace for fluorescence intensity ratio measurements and analyzed using a Quantech Life Sciences fixed wavelength fluorometer.

| Station | Latitude | Longitude |
| --- | --- | --- |
| BM169 | 28 11.5499 | 89 50.6773 |
| BM170 | 27 49.6767 | 88 35.4426 |
| BM171 | 27 75.3553 | 88 71.7067 |

All 3 stations showed low small particle concentrations at all depths with a very slight elevation observed in the surface (0.5m) samples. Although a DO sag (no correcponding fluorescence peak) was seen ~1100m at station BM169, little evidence of elevated small particle concentrations was observed.

Fluorescence intensity ratios for all three stations were uniformly low.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations BM169 to BM171.