2 August 2010 R/V Ferrel Status Report, Cruise 4 – Day 3

Complied by Tony Parkin, Chief Scientist

1.)	Site Conditions, Daily Activities and Sampling Locations

The Ferrel spent the day approximately 40 miles south west of the well head conducting CTD casts to locate and sample what possibly is the remnant of the dispersed oil plume as indicated by a dip in dissolved oxygen levels towards the bottom of the water column. Stations previously sampled by the Brooks McCall, BM151, 150 and 149 were re-occupied and a total of three casts were conducted. The results of the CTD cast are:

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BP-TN08-SS01 (BM151) | 40 miles SW | None | N/A | DO dip of @0.3 mg/L from 1100m to 1160m (unclear if we reached the base of the anomaly). A second small dip on DO was detected at 460m and was also sampled |
| BP-TN08-SS02 (BM150) | 40 miles SW | None | N/A | DO dip of @0.4 mg/L from 1150m to 1340m. |
| BP-TN08-SS03 (BM149) | 40 miles SW | None | N/A | Do dip of @0.5 mg/L from 1139m to 1239m |

2.)	Air Quality

The Industrial Hygienist conducted air monitoring measurements analyzing for CO, H2S, LEL, O2 and VOC’s. Samples were collected in the work area during sample collection and hourly throughout the day. Data collected showed no air quality concerns.

3.)	Science Crew

| Name | First Name | Affiliation | Role |
| --- | --- | --- | --- |
| Parkin | Tony | EM&A | Chief Scientist |
| Ford | Alex | En Sci | Assistant Scientist |
| Jessich | Michael | Entrix | Water Sampler |
| Owens | Brandon | Entrix | Water Sampler |
| Smith | Craig | C&C | Party Chief Team |
| Daigle | Brad | C&C | Oceanography Team |
| Burke | Emily | C&C | Oceanography Team |
| McLeod | Alex | C&C | Oceanography Team |
| Fortney | Julien | LBNL | Microbiologist |
| Pecino | Yvette | LBNL | Microbiologist |
| Delano | Louise | NOAA | Data Manager |
| Bradley | Paul | Bureau Veritas | IH |