22 August 2010 - Brooks McCall Status Report - Cruise 19 - Day 1

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Day 1 of Cruise 19 continued tracking the subsurface plume signals as part of Operation Clean Sweep.  The day was spent working a 45km radial transect SW of the wellhead. Sampling was delayed by 90 minutes at the start of the day after a hydraulic hose on the winch ruptured and needed replacing. Weather remained good throughout the day with 10-15 knot winds and 2-4ft seas.  No surface oiling was observed.  As with recent cruises, on-board DO analysis was undertaken using the Hach modified Winkler titration method.

The 4 CTD casts completed today are summarized in the table, and shown in the figure, below.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM182 | Bearing 255o, 45 km | No signal | - | A very small (~0.1mg/L) uniform depression in DO between 1000-1060m, extending close to the seabed. |
| BM183 | Bearing 240o, 45 km | No signal | - | A very slight (<0.1mg/L) uniform depression in DO between 990m and 1050m. |
| BM184 | Bearing 225o, 45 km | No signal | - | A slight (<0.2mg/L) uneven depression in DO between 1050m and 1150m with small peaks at 1080, 1100, and 1120m. |
| BM185 | Bearing 210o, 45 km | No signal | - | A very slight (0-0.1mg/L) uniform depression in DO between 1080m and 1120m. |

There was no change detected in the background fluorometry signal, and no indication of oil in the LISST or fluorometry data at any of the sites sampled. The only signal of the residual plume was a slight deviation in DO compared to background concentrations. Without reference to the depths that the subsurface plume has previously been found at, these DO depressions would be very difficult to distinguish from background variance.

A total of 20 samples were analyzed by Hach modified Winkler methods today. DO data will be submitted with the daily deliverables.

Rototox tests were started for BM182, BM183, and BM184.  Results will be reported on 23 August 2010.

| Brooks McCall Science Team, Cruise 19 |  |
| --- | --- |
| Stevens, Leigh | EM&A : Chief Scientist |
| Essex, Laura | EM&A : Toxicity testing |
| Hill, Stephanie | NOAA :Data Manager |
| Susdorf, Gary | ENTRIX : Sample collection |
| Ascough, Rich | ENTRIX : Sample collection |
| Robinson, Brian | DFO, Canada : LISST/Fluorometry |
| Wohlgeschaffen, Gary | DFO, Canada : LISST/Fluorometry |
| Groen, Arie | Shaw Environmental : Winkler DO |
| Lindquist, Alan | USEPA : Observer |

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."