Daily Report:  Tracking the Plume of Dispersed Oil using Particle Size Distribution Measurements and Fluorescence Intensity Ratios

August 23, 2010

Water samples were collected at four stations for particle size distribution measurements using the LISST-100X particle counter. A total of 44 LISST samples were analyzed, including duplicates. Samples at depths of elevated fluorescence or other significance were selected from the CTD trace for fluorescence intensity ratio measurements and analyzed using a Quantech Life Sciences fixed wavelength fluorometer.

Figure 1 presents the small droplet (∑ 2.5 - 60µm) particle size data and fluorescence intensity ratios for stations BM186 through BM189.  The station locations were:

| Station | Latitude | Longitude |
| --- | --- | --- |
| BM186 | N 28 21.0654 | E -88 30.1050 |
| BM187 | N 28 13.3161 | E -88 32.8179 |
| BM188 | N 28 16.9056 | E -88 41.6040 |
| BM189 | N 28 22.2602 | E -88 48.8969 |

All 4 stations showed higher small particle concentrations at the surface than at depth.

Small particle concentrations at all stations were uniformly low in the deeper water.  There was no evidence of a subsurface plume based on the CTD fluorometer trace at any of the four stations.

Fluorescence intensity ratios for the four stations were all less than 1.2.

Figure 1: Average small particle concentrations and fluorescence intensity ratios as a function of depth for stations BM186 to BM189.