23 August 2010 - Brooks McCall Status Report - Cruise 19 - Day 3

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Day 3 of Cruise 19 continued tracking the subsurface plume signals as part of Operation Clean Sweep.  The day was spent working along Transect 1, ~75km-90km SW of the wellhead. Weather was good throughout the day with light winds and flat calm seas.  No surface oiling was observed.  As with recent cruises, on-board DO analysis was undertaken using the Hach modified Winkler titration method.

The 4 CTD casts completed today are summarized in the table, and shown in the figure, below.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM190 - T1S1 | Bearing 225o, 75 km | No signal | - | A very slight depression in DO distinguishable from background between 1070m and 1160m, peaking at 0.1mg/L at 1155m. |
| BM191 - T1S2 | Bearing 217o, 78 km | Trace | 1150m | A trace of fluorescence apparent at 1150m (0.05 mg/m3 above background) associated with a relatively strong DO signal. DO was depressed between 1030-1120m, mostly 0.1mg/L with a 30-40m peak to 0.6mg/L at 1150m. |
| BM192 - T1S3 | Bearing 210o, 85 km | No signal | - | A very slight (0.1mg/L) uniform depression in DO between 1000m and 1090m. |
| BM193 - T1S4 | Bearing 200o, 92 km | No signal | - | No depression in DO distinguishable from background. |

There was no indication of oil in the LISST or fluorometry data at any of the sites sampled, including at BM191 (T1S3) where a faint change was observed in the background fluorometry signal.  The change was not sufficiently strong to be regarded a weak signal, our lowest category for rating plume strength.  The deviation in DO compared to background concentrations was strongest at BM191 (T1S3) where it was present in a narrow 30m deep band and ~0.6mg/l below background.  Elsewhere, the signal of the residual plume was limited to a small depression of ~0.1mg/l.

A total of 20 samples were analyzed by Hach modified Winkler methods today. DO data have been submitted with the daily deliverables.  The 60 Hach results from Cruise 19 had a correlation of 0.99 with the in situ Seabird DO sensor.  This demonstrates that the Seabird DO meter continues to provide a very reliable in situ measure of DO.

Rototox tests were completed for BM186, BM187, and BM188.  No toxicity was apparent.

| Brooks McCall Science Team, Cruise 19 |  |
| --- | --- |
| Stevens, Leigh | EM&A : Chief Scientist |
| Essex, Laura | EM&A : Toxicity testing |
| Hill, Stephanie | NOAA :Data Manager |
| Susdorf, Gary | ENTRIX : Sample collection |
| Ascough, Rich | ENTRIX : Sample collection |
| Robinson, Brian | DFO, Canada : LISST/Fluorometry |
| Wohlgeschaffen, Gary | DFO, Canada : LISST/Fluorometry |
| Groen, Arie | Shaw Environmental : Winkler DO |
| Lindquist, Alan | USEPA : Observer |

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."