09 September 2010 – Ocean Veritas Status Report – Cruise 19 – Day 2

Compiled by: Richard Bowen, Metoc, (BP Rep)

Table 1 – Science Team Roster for 09 September 2010

| Name | Affiliation | Role |
| --- | --- | --- |
| Richard Bowen | Metoc (BP Rep) | Chief Scientist |
| Dan Pisegna | Fugro GEOS | Chief Scientist / Party Chief |
| Jed Campbell | EPA | Observer |
| Bill Greenwood | Entrix | Sampling Technician |
| Jeremy Burns | Entrix (Mactec) | Sampling Technician |
| Connor Kobeski | Entrix | Sampling Technician |
| Kristina Lauenstein | NOAA | Data Management |
| Mark Bushnell | Tellus | Party Chief |
| Ward Bekins | Fugro GEOS | Oceanography Team Field Technician |
| Hannuman Bull | Fugro GEOS | Oceanography Team Field Technician |
| Brian Moore | Fugro Chance | Navigation |
| Ed Morren | Shaw | DO Winkler Method |

Today’s sampling plan, as directed by the NOAA Subsurface Monitoring Unit for 09 September 2010, was to continue south from T16S15 until two consecutive no signals are found. Then move to T18S18 and continue north along T18.

Today’s weather conditions are summarized in Table 2, below.

Table 2 – Summary of the Weather for 09 September 2010

| Time | Wind | Sea | Weather |
| --- | --- | --- | --- |
| 08:00 | Light airs | 1-2 feet | Broken Clouds on Horizon |
| 13:10 | ESE 3-5 knots | 1-3 feet | 1012 mb; 85oF, Clear |
| 17:00 | < 5 knots | 1-2 feet | Partly Sunny |

The cast for station OV245 was conducted 294 kilometers SW of the wellhead to a depth of 1700 meters.  There was no fluorescence signal but a depression in dissolved oxygen was observed at 1088 meters (~0.7 mg/l), ranging from 994 meters to 1425 meters. 

We continued south along the transect and conducted the cast for station OV246, 300 kilometers SW to a depth of 1700 meters.  There was no fluorescence signal but a depression in dissolved oxygen was observed at 1077 meters (~0.6 mg/l), ranging from 906 meters to 1439 meters.

We continued south along the transect and conducted the cast for station OV247, 307 kilometers SW to a depth of 1700 meters.  There was no fluorescence signal but a depression in dissolved oxygen was observed at 1147 meters (~0.6 mg/l) and 1067m (~0.3 mg/l), ranging from 1044 meters to 1265 meters.

We continued south along the transect and conducted the cast for station OV248, 311 kilometers SW to a depth of 1604 meters.  There was no fluorescence signal and no depression in dissolved oxygen.

Our last cast of the day was conducted 318 kilometers away from the wellhead at station OV249 to a depth of 1700 meters.  There was no fluorescence signal and no depression in dissolved oxygen.

The locations and results of the CTD casts conducted today are summarized in Table 3 and Figure 1, below.

Table 3 – Station Summary for 09 September 2010

| Station | Sampling Location and Position at Deployment | Fluorescence Signal | Signal Depth | Dissolved Oxygen Depression | Depression Depth |
| --- | --- | --- | --- | --- | --- |
| OV245 | T16S15 Lat: 26.912627 Long: -90.524656 | No Signal | - | ~0.7 mg/l | 1088 m (994 m – 1425 m) |
| OV246 | T16S16 Lat: 26.825124 Long: -90.502394 | No Signal | - | ~0.6 mg/l | 1077 m (906 m – 1439 m) |
| OV247 | T16S17 Lat: 26.736991 Long: -90.480764 | No Signal | - | ~0.6 mg/l | 1147 m (1044 m – 1265 m) |
| OV248 | TS16S18 Lat: 26.649042 Long: -90.459023 | No Signal | - | No depression | - |
| OV249 | TS16S19 Lat: 26.561277 Long: -90.437761 | No Signal | - | No depression | - |

Figure 1 – Sampling Effort for 09 September 2010

No surface oil sheen was observed in any of the samples collected today.  Comparisons of DO concentrations (CTD’s SBE 43 oxygen sensor versus the hand-held YSI ProODO optic probe) are included in today’s deliverables.  The correlation between the two remains excellent. The results from the Hach Winkler titration method also shows excellent correlation with the DO results obtained by CTD and YSI optical probe.