10 September 2010 – Ocean Veritas Status Report – Cruise 19 – Day 3

Compiled by: Richard Bowen, Metoc, (BP Rep)

Table 1 – Science Team Roster for 10 September 2010

| Name | Affiliation | Role |
| --- | --- | --- |
| Richard Bowen | Metoc (BP Rep) | Chief Scientist |
| Dan Pisegna | Fugro GEOS | Chief Scientist / Party Chief |
| Jed Campbell | EPA | Observer |
| Bill Greenwood | Entrix | Sampling Technician |
| Jeremy Burns | Entrix (Mactec) | Sampling Technician |
| Connor Kobeski | Entrix | Sampling Technician |
| Kristina Lauenstein | NOAA | Data Management |
| Mark Bushnell | Tellus | Party Chief |
| Ward Bekins | Fugro GEOS | Oceanography Team Field Technician |
| Hannuman Bull | Fugro GEOS | Oceanography Team Field Technician |
| Brian Moore | Fugro Chance | Navigation |
| Ed Morren | Shaw | DO Winkler Method |

Today’s sampling plan, as directed by the NOAA Subsurface Monitoring Unit for 10 September 2010, was to start at T18S19 and continue northwards along T18.

Today’s weather conditions are summarized in Table 2, below.

Table 2 – Summary of the Weather for 10 September 2010

| Time | Wind | Sea | Weather |
| --- | --- | --- | --- |
| 09:45 | E 5 knots | 1 foot | Broken Clouds, 85oF, 1012 mb |
| 13:10 | 4-5 knots vairable | 1 foot | Partly sunny |
| 17:00 | Calm | 1 foot | Partly sunny |

The cast for station OV250 was conducted 342 kilometers SW of the wellhead to a depth of 1700 meters.  There was no fluorescence signal but a depression in dissolved oxygen was observed at 1115 meters (~0.4 mg/l), ranging from 1022 meters to 1333 meters. 

We continued north along transect 18 and conducted the cast for station OV251, 337 kilometers SW to a depth of 1700 meters.  There was no fluorescence signal but a depression in dissolved oxygen was observed at 1114 meters (~0.25 mg/l), ranging from 1064 meters to 1247 meters.

We continued north along transect 18 and conducted the cast for station OV252, 331 kilometers SW to a depth of 1700 meters.  There was no fluorescence signal and no depression in dissolved oxygen.

Our fourth and last cast of the day was conducted 326 kilometers away from the wellhead at station OV253 to a depth of 1700 meters.  There was no fluorescence signal but a depression in dissolved oxygen was observed at 1245 meters (~0.1 mg/l), ranging from 1227 meters to 1270 meters.

The locations and results of the CTD casts conducted today are summarized in Table 3 and Figure 1, below.

Table 3 – Station Summary for 10 September 2010

| Station | Sampling Location and Position at Deployment | Fluorescence Signal | Signal Depth | Dissolved Oxygen Depression | Depression Depth |
| --- | --- | --- | --- | --- | --- |
| OV250 | T18S19 Lat: 26.501087 Long: -90.730542 | No Signal | - | ~0.4 mg/l | 1115 m (1022 m – 1333 m) |
| OV251 | T18S18 Lat: 26.589927 Long: -90.752307 | No Signal | - | ~0.25 mg/l | 1114 m (1064 m – 1247 m) |
| OV252 | T18S17 Lat: 26.676740 Long: -90.774545 | No Signal | - | No depression | - |
| OV253 | TS18S16 Lat: 26.764994 Long: -90.797508 | No Signal | - | ~0.1 mg/l | 1245 m (1227 m – 1270 m) |

Figure 1 – Sampling Effort for 10 September 2010

No surface oil sheen was observed in any of the samples collected today.  Comparisons of DO concentrations (CTD’s SBE 43 oxygen sensor versus the hand-held YSI ProODO optic probe) are included in today’s deliverables.  The correlation between the two remains excellent. The results from the Hach Winkler titration method also shows excellent correlation with the DO results obtained by CTD and YSI optical probe.