| Seabed Sampling Operations |  |
| --- | --- |
| Vessel | R/V Gyre |
| Summary Report Number | 15 |
| Operating equipment | MEGA Corer (12 core unit) |
| Date | 12 October 2010 |
| Completed casts (24 hr) | 5 MEGA Corer |
| Report compiled by | Nick Morley/Jason Schitoskey |

Seabed Sampling Locations

| S/V Gyre Sampling Effort | Map Date: 12 OCT 2010 Created by: Nick Morley |
| --- | --- |
| Stn ID Latitude Longitude Depth D007S; 28.08652; 88.51685; 2052. D008S; 27.88764; 88.62671; 1968. D013S; 27.65434; 88.6377; 1774. D006S; 28.3429; 88.1398; 2127. D003S; 28.12528; 88.0719; 2329. | Sediment Sampling Stations Using Bowers & Connelly MEGA Corer ● Sampled prior to Cruise 5 ● Sampled Cruise 5, before 12th Oct 2010 ● Sampled on 12th October 2010 |

All cores were processed and stored in accordance with P1470_RN2448_GY_Rev1. Samples were prepared and stored for hydrocarbon, trace metal, BTEX, grainsize, total inorganic carbon, total organic carbon, meiofauna, macrofauna and microbiology testing and analysis onshore.

| Station D007S Lat: 28.08652 Long: -88.51685 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None. Olfactible contamination: None Description: 28 cm firm grey clay overlaid with 4-5 cm soft olive brown clay topped by 3 cm brown ooze. Some lamination observed in core. Fish scales observed on surface. | Internal Diameter of Core: 10 cms |
| --- | --- |

| Station D006S Lat: 28.342903 Long: -88.139802 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None. Olfactible contamination: None Description: 25 cm firm gray clay with some lamination, overlaid with 9 cm soft brown clay/silt topped by 2 cm brown ooze. | Internal Diameter of Core: 10 cms |
| --- | --- |

| Station D013S Lat: 27.65434 Long: -88.6377 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None. Olfactible contamination: None Description: Bottom 4-5 cm – very soft clay with some sand size particles 5 to 10 cm Firm to hard interleaved red and olive green clay, with some dark grey mottling 10 to 14 cm, gray clay with very dark-black (possibly residue of anoxic material) interleaved, occasional inclusions of lighter gray, very soft clay, about 1 cm in diameter 14 to 25 cm Soft brown silty clay 25-38 cm Soft brown ooze | Internal Diameter of Core: 10 cms |
| --- | --- |

| Station D008S Lat: 27.88764 Long: -88.62761 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None. Olfactible contamination: None Description: 27 cm firm gray brown clay overlaid with 7 cm soft brown clay/silt topped by 3 cm brown ooze. Some lamination observed in core. Fish scales observed on surface. | Internal Diameter of Core: 10 cms |
| --- | --- |

| Station D003S Lat: 28.125280 Long: -88.07190 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None. Olfactible contamination: None Description: 31 cm firm gray clay overlaid with 5 cm soft gray brown clay/silt topped by 2 cm brown ooze. Some lamination observed in core. | Internal Diameter of Core: 10 cms |
| --- | --- |