| Seabed Sampling Operations |  |
| --- | --- |
| Vessel | R/V Gyre |
| Summary Report Number | 17 |
| Operating equipment | MEGA Corer (12 core unit) |
| Date | 17 October 2010 |
| Completed casts (24 hr) | 6 MEGA Corer |
| Report compiled by | Nick Morley/Jason Schitoskey |

Seabed Sampling Locations

| S/V Gyre Sampling Effort | Map Date: 12 OCT 2010 Created by: Nick Morley |
| --- | --- |
| Stn ID Latitude Longitude Depth D042S; 28.742533; -88.370579; -1490. D040S*; 28.742278; -88.362225; -1511. D038SW*; 28.740428; -88.368094; -1517. D034S*; 28.734676; -88.362361; -1537. D031S; 28.731650; -88.358827; -1563. D044S; 28.744821; -88.374275; -1489. * Station coordinates after re-siting. | Sediment Sampling Stations Using Bowers & Connelly MEGA Corer ● Sampled prior to Cruise 5 ● Sampled Cruise 5, before 17th Oct 2010 ● Sampled on 17th October 2010 |

All cores were processed and stored in accordance with P1470_RN2448_GY_Rev1. Samples were prepared and stored for hydrocarbon, trace metal, BTEX, grainsize, total inorganic carbon, total organic carbon, meiofauna, macrofauna and microbiology testing and analysis onshore.

| Station D044S Lat: 28. Long: -88. Cores recovered: 12 of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None. Olfactible contamination: None Description: 31 cm firm grey clay overlaid with 3 cm soft olive gray clay topped by 3 cm brown ooze. | Internal Diameter of Core: 10 cms |
| --- | --- |

| Station D038SW Lat: 28. Long: -88. Cores recovered: 12 out of 12 Supernatant water Visible contamination Slight sheen Olfactible contamination: Oil present Sediment Visible contamination: Oil present/reducing (black) Olfactible contamination: Oil and sulphide smell particularly in immediate surface layer Description: Deeper section of core similar to previous cores, however this was overlaid by non-conformant deposits. Bottom 17 cm, firm gray clay overlaid by 7 cm soft olive gray/brown silty clay typical of the region. Above this was a further 7 cm of soft to firm gray clay, 4 cm of very soft gray to black clay and 3 cm of oily black ooze. It appears likely that the bottom two sections were natural sediments overlain by a sequence of anthropogenic deposits. | Interngal Diameter of Core: 10 cms |
| --- | --- |

| StationD040S Lat: 28. Long: -88. Cores recovered: 10 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: Small amount of oil present Olfactible contamination: Slight oil/sulphide smell Description: 28 cm of firm gray clay, overlain by 7 cm of soft olive gray clay under 3 cm brown to black ooze. There was considerable biological activity (worm tubes on top of the sediment and wormholes in the upper section | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD031S Lat: 28. Long: -88. Cores recovered: 11 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 26 cm of firm gray clay, overlain by 7 cm dark gray-brown silty clay under 3 cm brown to black ooze. One core from cast contained organism (nudibranch?) on sediment surface/ in supernatant water | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD034S Lat: 28. Long: -88. Cores recovered: 12 out of 12 Supernatant water Visible contamination: Slight sheen Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 27 cm of firm gray clay, overlain by 9 cm olive/gray clay under 3 cm brown to black ooze. | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD042S Lat: 28. Long: -88. Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: Some oily appearing material in surface layer Olfactible contamination: Slight sulphide odour Description: 26 cm of firm gray clay, overlain by 9 cm soft olive/gray clay under 3 cm dark gray/brown ooze. | Internal Diameter of Core: 10 cms |
| --- | --- |