| Seabed Sampling Operations |  |
| --- | --- |
| Vessel | R/V Gyre |
| Summary Report Number | 18 |
| Operating equipment | MEGA Corer (12 core unit) |
| Date | 18 October 2010 |
| Completed casts (24 hr) | 5 (+1 19 Oct 2010) MEGA Corer |
| Report compiled by | Nick Morley/Jason Schitoskey |

Seabed Sampling Locations

| S/V Gyre Sampling Effort | Map Date: 12 OCT 2010 Created by: Nick Morley |
| --- | --- |
| Stn ID Latitude Longitude Depth D021S; 28.7031; -88.3609; 1623. D012S; 28.672461; -88.234098; 1820. D010S; 28.5702; -88.3235; 1885. D004S; 28.581028; -87.88576; 2304. D002S; 28.557153; -87.760735; 2390. D068S*; 28.7108; -88.7483; 1168. * Sampled 19th October. | Sediment Sampling Stations Using Bowers & Connelly MEGA Corer ● Sampled prior to Cruise 5 ● Sampled Cruise 5, before 18th Oct 2010 ● Sampled on 18th October 2010 ● Sampled on 19th October 2010 |

All cores were processed and stored in accordance with P1470_RN2448_GY_Rev1. Samples were prepared and stored for hydrocarbon, trace metal, BTEX, grainsize, total inorganic carbon, total organic carbon, meiofauna, macrofauna and microbiology testing and analysis onshore.

| StationD021S Lat: 28. 7031 Long: -88. 3609 Cores recovered: 11 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 30 cm of firm gray clay, overlain by 5 cm soft olive/gray clay under 2 cm dark gray/brown ooze. Some tube worms in top of sediment with holes throughout | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD012S Lat: 28.672461 Long: -88.234098 Cores recovered: 11 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: Small amount of oil present Olfactible contamination: Slight oil/sulphide smell Description: 33 cm of firm gray clay, overlain by 6 cm of soft dark gray/brown silty clay under 3 cm brown ooze. There were worm holes throughout the core . In addition an organism (20 cm – scale indicated by hand), tentatively identified as a nudibranch, was found in one of the cores at this site | Internal Diameter of Core: 10 cms |
| --- | --- |

| Station D010S Lat: 28.5702 Long: -88. 3235 Cores recovered: 9 out of 12 Supernatant water Visible contamination None Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 31 cm firm gray clay overlain by 5 cm soft dark gray-brown silty clay and 3 cm brown ooze. Worm holes present throughout core, some lamination evident | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD004S Lat: 28.581028 Long: -88.88576 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 26 cm of firm gray clay, overlain by 7 cm olive/gray clay under 3 cm brown ooze. Large (1cm diameter) bore hole (?) in upper clay section | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD002S/FFS37 Lat: 28.557153 Long: -87.760735 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 29 cm of firm gray clay, overlain by 7 cm dark gray-brown silty clay under 3 cm brown ooze. | Internal Diameter of Core: 10 cms |
| --- | --- |

| StationD068S Lat: 28.7108 Long: -88.748 Sample date 19th October 2010 Cores recovered: 12 out of 12 Supernatant water Visible contamination: None Olfactible contamination: None Sediment Visible contamination: None Olfactible contamination: None Description: 26 cm of firm gray clay, overlain by 4 cm dark olive brown silty clay under 3 cm dark brown to gray brown ooze. | Internal Diameter of Core: 10 cms |
| --- | --- |