ORNL/CDIAC-84

NDP-052

TOTAL CARBON DIOXIDE, HYDROGRAPHIC, AND NITRATE

MEASUREMENTS IN THE SOUTHWEST PACIFIC DURING AUSTRAL AUTUMN,

1990: RESULTS FROM NOAA/PMEL CGC-90 CRUISE

Contributed by

Marilyn F. Lamb and Richard A. Feely

Pacific Marine Environmental Laboratory

Seattle, Washington

and

Lloyd Moore and Donald K. Atwood

Atlantic Oceanographic and Meteorological Laboratory

Miami, Florida

Prepared by Alexander Kozyr

Carbon Dioxide Information Analysis Center

Oak Ridge National Laboratory

Oak Ridge, Tennessee

Environmental Sciences Division

Publication No. 4420

Date Published:  September 1995

Prepared for the

Global Change Research Program

Environmental Sciences Division

Office of Health and Environmental Research

U.S. Department of Energy

Budget Activity Number KP 05 02 00 0

Prepared by the

Carbon Dioxide Information Analysis Center

OAK RIDGE NATIONAL LABORATORY

Oak Ridge, Tennessee 37831-6335

managed by

LOCKHEED MARTIN ENERGY SYSTEMS, INC.

for the

U.S. DEPARTMENT OF ENERGY

under contract DE-AC05-84OR21400

ABSTRACT

Lamb, M. F., R. A. Feely, L. Moore, and D. K. Atwood.  1995.  Total Carbon Dioxide,

Hydrographic, and Nitrate Measurements in the Southwest Pacific during Austral Autumn,

1990:  Results from NOAA/PMEL CGC-90 Cruise. ORNL/CDIAC-84, NDP-052.  Carbon

Dioxide Information Analysis Center, Oak Ridge National Laboratory, Oak Ridge, Tennessee. doi: 10.3334/CDIAC/otg.ndp052

In support of the National Oceanic and Atmospheric Administration (NOAA) Climate and

Global Change (C&GC) Program, Pacific Marine Environmental Laboratory (PMEL) scientists

have been measuring the growing burden of greenhouse gases in the thermocline waters of the

Pacific Ocean since 1980.  Collection of data at a series of hydrographic stations along longitude

170  W during austral autumn of 1990 was designed to enhance understanding of the increase in

the column burden of chlorofluorocarbons and carbon dioxide in the thermocline waters since the

last expedition in 1984.

This document presents the procedures and methods used to obtain total carbon dioxide

(TCO2), hydrographic, and nitrate data during the NOAA/PMEL research vessel (R/V) Malcolm

Baldrige CGC-90 Cruise.  Data were collected along two legs; sampling for Leg 1 began along

170 W from 15 S to 60 S, then angled northwest toward New Zealand across the Western

Boundary Current.  Leg 2 included a reoccupation of some stations between 30 S and 15 S on

170 W and measurements from 15 S to 5 N along 170 W.  Along the cruise track 68 CTD

stations were occupied for collection of chemical and hydrographic data.  The following data

report summarizes the TCO2, salinity, temperature, and nitrate measurements from 63 stations.

In addition, potential density and potential temperature were calculated from the measured

variables.

The TCO2 concentration in seawater samples was measured using a coulometric/extraction

system (Models 5011 and 5030, respectively) originated by Ken Johnson (Johnson et al. 1985,

1987).  Throughout the cruise, the accuracy was determined to be within 0.15%, and the precision

was within 0.12%.

The NOAA/PMEL R/V Malcolm Baldrige CGC-90 Cruise data set is available without

charge as a numeric data package (NDP) from the Carbon Dioxide Information Analysis Center.

The NDP consists of two oceanographic data files, two FORTRAN 77 data retrieval routine files,

a  documentation  file, and this printed documentation, which describes the contents and format of all

files as well as the procedures and methods used to obtain the data.

Keywords:   carbon dioxide; nitrate; hydrographic measurements; carbon cycle; Pacific Ocean

PART 1:

OVERVIEW

1. BACKGROUND INFORMATION

Human activity is rapidly changing the composition of the earth's atmosphere, causing the

greenhouse warming effect from excess carbon dioxide (CO2) and other trace gas species such as

chlorofluorocarbons, methane, and nitrous oxide.  Combined, these gases play a critical role in

controlling the earth's climate due to the increased trapping of outgoing infrared radiation.  This

mechanism has a large potential for significantly altering the world's climate.

Of all the anthropogenic CO2 that has been released into the atmosphere, only about half still

remains there; it is the  missing  CO2 for which the global ocean is considered to be a dominant

sink.  Understanding the assimilation process is critical in determining the moderating role the

oceans will play in delaying and damping the greenhouse warming predicted in the coming

decades.  Our goal is to help provide quantitative answers to the ways in which the oceans

regulate and assimilate the excess CO2, so that we can better predict the ocean's role in the natural

climate cycle.

In response to these concerns, the Pacific Marine Environmental Laboratory (PMEL)

conducted Cruise CGC-90 to the southwest Pacific onboard the research vessel (R/V) Malcolm

Baldrige under the sponsorship of the Climate and Global Change Program(C&GC) of the

National Oceanic and Atmospheric Administration (NOAA).  Chemical and hydrographic data

from 68 CTD stations were collected along the cruise track.  Several tracers were

measured during the cruise, including chlorofluorocarbons, helium, tritium, total carbon dioxide

(TCO2), 13C, pH, nutrients, salinities and oxygens.  The following data report summarizes the

TCO2, salinity, temperature, and nitrate data from 63 stations of this cruise.

2. DESCRIPTION OF THE EXPEDITION.

2.1 R/V Malcolm Baldrige Technical Details

The R/V Malcolm Baldrige (previously Researcher r103) is owned by the U.S. Department

of Commerce and operated by NOAA.  The basic features of the vessel are described below.

Port of registration:   Miami, Florida

Call sign:              WTER

Operator:               NOAA

Launched:               October 1968

Delivered:              June 1970

Commissioned:           October 1970

Basic Dimensions:

length                  84.8 m

beam                    15.5 m

draft                   5.6 m

gross tons              2802

power                   3200 hp

maximum speed           15.0 knots

cruise speed            13.0 knots

Personnel:              crew: 36; scientists: 14

Navigation Equipment:   Radar Loran SatNav Gyro DopLog

Hull:                   welded steel (ice-strengthened)

Cranes:                 Stern, Midships

2.2 NOAA/PMEL CGC-90 Cruise Information

The following is the cruise information:

Ship Name:   Malcolm Baldrige

Cruise/Leg:  CGC-90/1,2

Location:    Southwest Pacific Ocean

Dates:       February 22 - April 16, 1990

List of Participants:

Chief Scientist:           David Wisegarver, NOAA/PMEL

Project Manager:           Richard Feely

Total CO2:                 Marilyn Lamb-Roberts

Paulette Murphy

CTD:                       Linda Mangum

Kristy McTaggart

Marguerite McCarty

Dana Greeley

Jeff Benson

Salinity:                  Survey Department of NOAA R/V Malcolm Baldrige

Nitrate:                   Lloyd Moore

Don Atwood

Computer Support:          Cathy Cosca

Dan Lee

Doug Wilson

2.3 Brief Cruise Summary

The cruise track for Leg 1 of CGC-90 started at 15 S/170 W and proceeded south along

the meridional line; after reaching 60 S, it angled northwest, crossing the Western Boundary

Current and ending in New Zealand.   The cruise track for Leg 2 crossed the Kermadec Trench,

included a reoccupation of selected stations between 30 S and 15 S on 170 W, and additional

stations along the meridional line crossing the equator, to 5 N (Table A-1 in Appendix A).

Stations 2 through 7 were sampled using a 24-position rosette package equipped with 10-L

Niskin bottles, and a Neil Brown (Mark III) CTD sampling system.  While occupying Station

8, the CTD cable parted, and the equipment package was lost.  Subsequently, the remaining

stations (Stations 9 through 68) were sampled using a spare 12-position rosette with 10-L Niskin

bottles and Neil Brown (Mark III) CTD sampling system.  Multiple casts were performed at these

stations to ensure high density sample profiles.  Occasionally while at the most extreme latitudes,

weather deteriorated and prevented the occupation of some stations.  Several stations sampled

during Leg 1 were reoccupied on Leg 2 for data quality checks.

Two moorings were retrieved, and two deployed at the equator.

3. DESCRIPTION OF VARIABLES AND METHODS

Each station consisted of the lowering of a CTD/rosette package, upon which 10-L standard

Niskin bottles were suspended.  Pressure, temperature, and CTD salinities for the tabulated data

were taken from the calibrated CTD data.  The discrete water samples were drawn from the

Niskin bottles upon retrieval of the rosette on deck.  Samples were collected for analyses of

oxygen, chlorofluorocarbons (CFCs), helium, tritium (Leg 1 only), TCO2, 13C, pH (Leg 1 only),

nutrients, and salinity.  In addition, underway partial pressure of CO2 (pCO2) was measured

throughout the cruise.  This report addresses the temperature, salinity, TCO2, and nitrate data.

CTD data were collected using a Neil Brown (Mark III) Instrument System.  Pressure,

temperature, and conductivity were recorded on the downtrace, and the discrete water samples

were collected on the upcast with an electronically fired rosette sampler.  The bottle salinity

samples were analyzed with a Guideline Autosal, which was calibrated at the beginning of each

day's run with a vial of Wormley standard seawater, and again after each case of samples was

analyzed with another vial.  An accuracy of 0.002 Practical Salinity Scale (PSS) and a precision

of 0.001 PSS were achieved.  A more detailed description of the CTD data processing has been

published as a NOAA Data Report (McTaggart et al, 1993).

Nutrient analyses included nitrate, silicate, and phosphate.  Due to accuracy and precision

problems for silicate and phosphate, only nitrate data are included in this report.

Nitrate samples were collected from each Niskin bottle in aged 60-mL linear polyethylene

bottles and analyzed for dissolved inorganic nitrate (NO3--N).  Analyses were performed on

samples from all CTD casts with a five-channel Technicon Auto-Analyzer (AA-II) aboard ship.

The analytical procedures and methodologies used in the analysis of nitrate are similar to those

described by Armstrong et al. (1967), with modifications described in Atlas et al. (1971) and in

a Technicon Corporation (1977) technical report.  The detection limit for nitrate was 0.39 umol/kg

with a standard deviation of  0.1 umol/kg.  The precision of duplicate measurements was  0.25%,

full scale.  The accuracy was assumed to be 1% because no absolute standards were available.

Nitrate measurements are reported in umol/kg at 1 atm and an assumed laboratory temperature of

25 C.  Calibration standards ranged between 0 and 30 umol/kg; therefore, only samples within that

concentration range are reported in Appendix B and data file.

Upon retrieval of the CTD-rosette package on deck, samples for TCO2 were collected in 500-mL

glass-stoppered bottles and poisoned with 0.5 mL of saturated HgCl2 solution to decrease

bacterial oxidation of organic matter prior to analysis.  The samples were analyzed immediately

when possible, but always within 24 h after collection.

The coulometric technique for TCO2 analyses in seawater was originated by Ken Johnson

(Johnson et al. 1985, 1987).  UIC, Inc. supplied the coulometic/extraction system (Models 5011

and 5030, respectively).  The following changes were made to the standard extraction system:

1. Both the pipet and seawater sample bottle were jacketed (the sample bottle was placed in a

jacketed beaker) and were connected to a circulating bath set at 25 C.  2. Ultra-pure N2 was used

for both carrier gas and sample delivery; prior to hook-up to glassware, the N2 was sent through

an in-line NaOH (Malcosorb) scrubber to remove any CO2.  3. The condensing column was

connected to a circulating bath filled with antifreeze and run at 1 C.  4. An ORBO (activated Si

gel from Supelco) tube was placed in-line between the glassware and titration cell to eliminate any

excess moisture.  5. Standardization of instrumentation was modified (as described later in this

section).  The computer interfaced to the system was a Zenith ZBF-2339-BK.  The program

supplied by UIC was modified significantly, to accommodate our particular needs.  In coulometric

analysis of TCO2, all carbonate species in seawater (CO2(aq), H2CO3, HCO3- and CO32-) are

converted to CO2 by addition of excess acid.  The evolved CO2 is then moved into the titration

cell by N2 carrier gas where it is titrated potentiometrically by reacting quantitatively with

ethanolamine to form hydroxyethyl carbamic acid; this is titrated with OH- ions electrogenerated

by the reduction of H2O at a platinum cathode:

CO2 + HO(CH2)2NH2 -- HO(CH2)2NHCOO- + H+

Ag(s) -- Ag+ + e-

H2O + e- -- 0.5H2(g) + OH-

H+ + OH- -- H20

The equivalence point is detected photometrically with thymolphthalein as an indicator.  The

cell solution is blue at the equivalence point of 10.5 pH and colorless at pH 9.3 after the addition

of CO2 in aqueous solutions.  CO2 drives down the pH and raises percent transmittance.  As the

acid is titrated, pH increases (hence, the blue color returns) and percent transmittance decreases,

thus causing the titration current to pass from high to low to zero as the equivalence point is

approached and sensed by the optical detector.  The CO2 level is calculated based on the quantity

of electricity required to reach the equivalence point and the time of passage.  The entire sequence

takes from 8 to 11 min.

The volume of the pipet was ~ 50 mL, and was calibrated in the laboratory before and after

the cruise.  The pipet was cleaned by drawing a 25% solution of NaOH into the cell and allowing

it to soak overnight.  This eliminated any organic film inside the pipet and ensured a clean

delivery.  Pipet calibrations were conducted to be within the measured pipet temperature range

during the cruise (24.5 C 25.5 C) utilizing a circulating bath.  Milli-Q water was drawn into the

pipet in the exact manner that a liquid standard or seawater sample was handled.  The water was

delivered by N2 (flow rate is 200 mL/min) into a tared, ground-glass stoppered mixing flask, and

drained for an additional 5 s (monitored by a stop-watch) to allow the droplets of water to be

delivered; the flask was then immediately stoppered and weighed on a Mettler AE240 balance.

Approximately 15-20 samples were collected per experiment.  The following references were used

to correct the weighings:  1. Volume Properties of Ordinary Water.  2. Reductions of Weighings

in Air to Vacuo for Brass Weights and a Water Density of 1.00.  Density of air used was 0.0012.

3.  Temperature Correction for Glass Volumetric Apparatus.

The corrected volumes were then linearly regressed with temperature, and a calibration curve

was established (typical r2 = 0.80).

Schott-Duran glass 500-mL bottles were annealed at 450 C for 1 h, then cleaned in a

dishwasher with commercial grade dishwashing detergent.  Prior to collection of samples, the solid

ground-glass stoppers were coated with Type M Apiezon grease.

Acid used to convert carbonate species to CO2 was a 1:10 solution of Baker reagent grade

H3PO4.  All coulometric chemicals (cathode solution, anode solution, and KI) were purchased from

UIC, Inc.

Liquid standards were made up in a 0.7 M solution of KCl.  The standards were treated just

as a seawater sample and were delivered through the pipet under the same conditions.  The

standard used was Na2CO3 (Ultrex, Lot 935113); the KCl was reagent grade from Mallinkrodt.

The Na2CO3 was prepared in the laboratory by baking at 260 270 C for 0.5 h, and desiccated

overnight.  The standards were weighed into pre cleaned ground-glass stoppered vials with weights

ranging from 0.20 to 0.25 g.  They were immediately stored in an evacuated desiccator with fresh

Si gel until prepared.  The KCl was baked in a muffle furnace for 0.5 h at 260-270 C, and cooled

in a desiccator overnight.

The KCl solution and liquid standards were prepared in the following manner:  Milli-Q water

was boiled in a 3-L boiling flask for 20 min to drive off CO2, then cooled overnight with a NaOH

column attached to the neck of the flask.  A glove-box was purged with ultra-pure N2 for about

20 min; in the glove box, the KCl was mixed with the CO2-free water in a clean 2-L volumetric.

Half of this solution was stored in a 1-L sample bottle with siphon tube and clamp and was used

to determine blank values for the KCl (see following discussion).  The other half was used to

make the Na2CO3 in a 1-L volumetric.  After the standard equilibrated, it was poured into a 1-L

bottle with siphon tube and clamp.  This work was performed in an N2 environment in the glove

box.

The KCl solution was analyzed to determine a mean blank for the standard.  Being careful

not to expose the KCl solution to the atmosphere, it was drawn into the pipet in the same way as

a sample.  When handled in this way, the KCl blank was very constant, usually with a mean of

around 6.0 +/- 0.3 ug C for an individual batch and an  over the cruise  mean of 6 +/- 1 ug C.

The standards yielded a mean calibration factor of 99.6588% +/- 0.0600 (n = 25).

A Certified Reference Material (CRM) was prepared and bottled by Dr. Andrew Dickson of

the Scripps Institution of Oceanography (SIO).  The TCO2 concentration of the CRM was

determined to be 2020 +/- 0.009 umol/kg by manometric technique in the laboratory of Dr. Charles

Keeling of the SIO.  Bottles of the CRM were taken on the cruise and analyzed frequently to

determine the accuracy and precision of the coulometric method; the results are reported in

Table 1.  Throughout the cruise, the accuracy was determined to be within 0.15% and the

precision was within 0.12%.  Replicates analyzed at three different stations throughout the cruise

yielded a precision of +/- 0.05%.  The reported TCO2 data have been corrected to reflect the

difference in accuracy to the CRM; the correction applied is +3 umol/kg TCO2.

Samples were analyzed with the same method as standards, that is, 4.5 mLs of acid were

dispensed into the reaction vessel, and 2-3 minutes were allowed to pass to purge CO2 from the

acid.  Following that, the pipet was rinsed twice with the sample, and the third fill was isolated

and used for analysis.  The sample was emptied into the reaction vessel, and allowed to drain for

an additional 5 seconds (monitored via stopwatch) to allow droplets of water to be delivered.

Table 1.  Certified Reference Material (batch 1) analyzed for total CO2 during

R/V Malcolm Baldrige CGC-90 Cruise

----------------------

Date          TCO2

(umol/kg)

----------------------

24 Feb. 1990    2012

25 Feb. 1990    2024

27 Feb. 1990    2014

2 Mar. 1990    2015

5 Mar. 1990    2012

5 Mar. 1990    2016

6 Mar. 1990    2019

7 Mar. 1990    2019

8 Mar. 1990    2019

10 Mar. 1990    2015

11 Mar. 1990    2016

12 Mar. 1990    2016

13 Mar. 1990    2017

15 Mar. 1990    2017

16 Mar. 1990    2017

20 Mar. 1990    2017

29 Mar. 1990    2017

30 Mar. 1990    2014

2 Apr. 1990    2018

3 Apr. 1990    2014

5 Apr. 1990    2016

6 Apr. 1990    2017

7 Apr. 1990    2018

7 Apr. 1990    2018

11 Apr. 1990    2020

12 Apr. 1990    2017

--------------------

Mean    2017

Sta. Dev     2.5

--------------------

Sigma-t and sigma-theta were calculated using standard UNESCO algorithms (Fofonoff and

Millard 1983), and the CTD measured in situ temperature and bottle salinities.  When no bottle

salinities were available or when they were defined as a questionable or unacceptable

measurements, CTD salinities were used in the calculation.

4. DATA CHECKS AND PROCESSING PERFORMED BY CDIAC

An important part of the NDP process at the Carbon Dioxide Information Analysis Center

(CDIAC) involves the quality assurance (QA) of data before distribution.  Data received at

CDIAC are rarely in a condition that would permit immediate distribution, regardless of the

source.  To guarantee data of the highest possible quality, CDIAC conducts extensive QA reviews.

Reviews involve examining the data for completeness, reasonableness, and accuracy.  Although

they have common objectives, these reviews are tailored to each data set, often requiring extensive

programming efforts.  In short, the QA process is a critical component in the value-added concept

of supplying accurate, usable data for researchers.

The following summarizes the data QA checks and processing performed by CDIAC on the

data obtained during the NOAA/PMEL R/V Malcolm Baldrige CGC-90 Cruise in the Southwest

Pacific.

1. These data were provided to CDIAC as two ASCII-formatted files and accompanying printed

documentation (NOAA Data Report ERL PMEL-42) (Lamb et al. 1993).  A FORTRAN 77

retrieval code was written and used to reformat the original files.

2. To check for obvious outliers all data were plotted by use of a PLOTNEST.C program

written by Stewart C. Sutherland, of the Lamont-Doherty Earth Observatory.  The program

plots a series of nested profiles, using the station number as an offset; the first station is

defined at the beginning, and subsequent stations are offset by a fixed interval.

Several outliers were identified and flagged after consultation with the principal investigators.

3. To generate a section profile plot of TCO2 concentrations along the 170 W, the

PLOTSECT.C program written by Stewart C. Sutherland, LDEO was used.

4. To identify  noisy  data and possible systematic methodological errors, property-property

plots for all parameters were generated, carefully examined, and compared with

plots from previous expeditions in the Southwest Pacific.

5. To identify possible instrumentation drifts and methodological errors, the data

intercomparison for reoccupied stations was provided.

6. All variables were checked for values exceeding physical limits, such as sampling depth

values that are greater than the given bottom depths.

7. Station locations (latitudes and longitudes) and sampling dates were examined for consistency

with maps and with cruise information supplied by Lamb et al. (1993).

8. The designation for missing values, given as "-99.00"  in the original files, was changed to

"-999.90".

5. HOW TO OBTAIN THE DATA AND DOCUMENTATION

This data base is available upon request in machine-readable form, free-of-charge from

CDIAC.  CDIAC will also distribute subsets of the data base as needed.  It can be acquired on

9-track magnetic tape; 8-mm tape; 150-mB, quarter-inch tape cartridge; IBM-formatted floppy

diskettes; or from CDIAC's anonymous File Transfer Protocol (FTP) area via Internet (see FTP

address below).  Requests should include any specific media instructions (i.e., 1600 or 6250 BPI,

labeled or nonlabeled, ASCII or EBCDIC characters, and variable- or fixed-length records; 3.5-

or 5.25-inch floppy diskettes, high or low density; and 8200 or 8500 format 8-mm tape) required

by the user to access the data.  Magnetic tape requests not accompanied by specific instructions

will be filled on 9-track, 6250 BPI, standard-labeled tapes with EBCDIC characters.  Requests

should be addressed to:

Carbon Dioxide Information Analysis Center

Oak Ridge National Laboratory

Post Office Box 2008

Oak Ridge, Tennessee 37831-6335

U.S.A.

Telephone:             (615) 574-0390 or (615) 574-3645

Fax:                   (615) 574-2232

Electronic Mail:       INTERNET: CDIAC@ORNL.GOV

The data files can be also acquired from CDIAC's anonymous FTP account via Internet:

FTP to cdiac.esd.ornl.gov (128.219.24.36)

Enter "ftp" or "anonymous" as the userid

Enter your electronic mail address as the password (e.g., alex@alex.esd.ornl.gov )

Change to the directory  "/pub/ndp052"

Acquire the files using the FTP "get" or "mget" command

or World Wide Web URL:

http://cdiac.esd.ornl.gov/cdiac/

6. REFERENCES

Armstrong, F. A. J., C. R. Stearns, and J. D. H. Strickland.  1967.  The measurement of upwelling

and subsequent biological processes by means of the Technicon Auto-Analyzer and associated

equipment.  Deep-Sea Res. 14:381-89.

Atlas, E. L., J. C. Callaway, R. D. Tomlinson, L. I. Gordon, L. Barstow, and P. K. Park.  1971.

A Practical Manual for Use of the Technicon Autoanalyzer Nutrient Analysis, revised.

Oregon State University Technical Report 215, Reference No. 71-22.

Fofonoff, N. P., and R. C. Millard, Jr.  1983.  Algorithms for computation of fundamental

properties of seawater.  UNESCO Technical Paper.

Johnson, K. M., A. E. King, and J. McN. Sieburth.  1985.  Coulometric TCO2 analyses for marine

studies; an introduction.  Mar. Chem. 16:61-82.

Johnson, K. M., P. J. Williams, L. Brandstrom, and J. McN. Sieburth.  1987.  Coulometric total

carbon analysis for marine studies:  automation and calibration.  Mar. Chem. 21:117-33.

McTaggart K. E., D. Wilson, and L. J. Mangum.  1993.  CTD Measurements Collected on a

Climate and Global Change Cruise along 170 N during February April 1990.  NOAA Data

Report.  URL-PMEL-44.  Pacific Marine Environmental Laboratory, Seattle, Wash.

Lamb M. F., R. A. Feely, L. Moore, and D. K. Atwood.  1993.  Total CO2 and Nitrate

Measurements In the Southwest Pacific During Austral Autumn, 1990.  NOAA Data Report

ERL PMEL-42.  Pacific Marine Environmental Laboratory, Seattle, Wash.

Technicon Industrial Systems.  1977.  Nitrate and nitrite in water and seawater.  Technicon Auto-

Analyzer II, Industrial Method No. 158 71W/A. Technicon Instrument Corporation,

Tarrytown, N.Y.

UNESCO Technical Papers in Marine Science.  1991.  Reference Materials for Oceanic Carbon

Dioxide Measurements 60.

PART 2:

CONTENT AND FORMAT OF DATA FILES

7. FILE DESCRIPTIONS

This section describes the content and format of each of the five files that comprise this NDP

(Table 2).  Because CDIAC distributes the data set in several ways (e.g., via anonymous FTP,

floppy diskette,  and on 9-track magnetic tape), each of the five files is referenced by both an

ASCII file name, which is given in lower-case, bold-faced type (e.g., ndp052.txt), and a file number.

The remainder of this section describes (or lists, where appropriate) the contents of each file.  The

files are discussed in the order in which they appear on the magnetic tapes.

Table 2.  Content, size, and format of data files

------------------------------------------------------------------------------------

File number, name,                Logical     File size    Block     Record

and description                   records     in bytes     size      length

------------------------------------------------------------------------------------

1. ndp052.txt:                             954        50,859      8,000       80

a detailed description

of the cruise network, the

two FORTRAN 77 data

retrieval routines, and the

two oceanographic data

files

2. stainv.for:                          30         1,062      8,000       80

a FORTRAN 77 data retrieval

routine to read and print

cgc90sta.inv (File 4)

3. cgc90dat.for:                        36         1,368      8,000       80

a FORTRAN 77 data retrieval

routine to read and print

cgc90.dat (File 5)

4. cgc90sta.inv:                        63         3,906      4,100       41

a listing of the station locations,

sampling dates, and sounding

bottom depths for each station

5. cgc90.dat:                         1,224       134,640    16,000      160

hydrographic, carbon dioxide,

and nitrate data from 63 stations.

_____        ______

Total                              2,307       191,835

-----------------------------------------------------------------------------------

7.1 ndp052.txt (File 1)

This file contains a detailed description of the data set,  the two FORTRAN 77 data retrieval

routines, and the two oceanographic data files.  It exists primarily for the benefit of individuals

who acquire this database as machine-readable data files from CDIAC.

7.2 stainv.for (File 2)

This file contains a FORTRAN 77 data retrieval routine to read and print cgc90sta.inv (File 4).

The following is a listing of this program.  For additional information regarding variable

definitions, variable lengths, variable types, units, and codes, please see the description for the

cgc90sta.inv file.

c****************************************************************

c* This is a FORTRAN 77 retrieval code to read and print the    *

c* file named "cgc90sta.inv" (File 4)                           *

c****************************************************************

INTEGER  sta, cast, dep

REAL lat, lon

CHARACTER date*8, time*4

OPEN (unit=1, file='cgc90sta.inv')

OPEN (unit=2, file='pmel90sta.inv')

write (2, 5)

5     format (2X,'STANBR',1X,'CASTNBR',1X,'LATITUDE',1X,

1 'LONGITUDE',6X,'DATE',4X,'TIME',3X,'DEPTH',/,18X,

2 'DEC.DEG',3X,'DEC.DEG',5X,'D/M/Y',5X,'GMT',7X,'M',/)

7     CONTINUE

read (1, 10, end=999) sta, cast, lat, lon, date, time,

1 dep

10    format (6X, I2, 5X, I3, 2X, F7.3, 2X, F8.3, 2X, A8, 4X,

1 A4, 4X, I4)

write (2, 20) sta, cast, lat, lon, date, time,

1 dep

20    format (6X, I2, 5X, I3, 2X, F7.3, 2X, F8.3, 2X, A8, 4X,

1 A4, 4X, I4)

GOTO 7

999   close(unit=1)

close(unit=2)

stop

end

7.3 cgc90dat.for (File 3)

This file contains a FORTRAN 77 data retrieval routine to read and print cgc90.dat (File 5).

The following is a listing of this program. For additional information regarding variable

definitions, variable lengths, variable types, units, and codes, please see the description for the

cgc90.dat file.

c****************************************************************

c*  This is a FORTRAN 77 data retrieval code to read and format *

c*  the file named "cgc90.dat" (File 5)                         *

c****************************************************************

INTEGER  sta, samp, qcfl

REAL pres, temp, theta, ctdsal, botsal, sigma, sigmat

REAL no3, tco2

OPEN (unit=1, file='cgc90.dat')

OPEN (unit=2, file='pmel90.dat')

write (2, 5)

5     format (3X,'STANBR',3X,'SAMPID',3X,'CTDPRS',2X,'CTDTEMP',4X,

1 'THETA',3X,'CTDSAL',3X,'BOTSAL',4X,'SIGMA',2X,'SIGMA-T',2X,

2 'NITRATE',4X,'TCARB',3X'QC_FLAG',/,23X'DBAR',3X,'DEG(C)',

3 3X,'DEG(C)',6X,'PSS',6X,'PSS',4X,'THETA',11X,'UMOL/KG',2X,

4 'UMOL/KG',/,56X,'*******',20X,'*******',2X,'*******')

7     CONTINUE

read (1, 10,end=999) sta, samp, pres, temp, theta, ctdsal,

1 botsal, sigma, sigmat, no3, tco2, qcfl

10    format (7X,I2,4X,I5,3X,F6.1,3X,F6.3,3X,F6.3,3X,F6.3,1X,F8.3,

1 3X,F6.3,3X,F6.3,1X,F8.3,2X,F7.1,7X,I3)

write (2, 20) sta, samp, pres, temp, theta, ctdsal,

1 botsal, sigma, sigmat, no3, tco2, qcfl

20    format (7X,I2,4X,I5,3X,F6.1,3X,F6.3,3X,F6.3,3X,F6.3,1X,F8.3,

1 3X,F6.3,3X,F6.3,1X,F8.3,2X,F7.1,7X,I3)

GOTO 7

999    close(unit=1)

close(unit=2)

stop

end

7.4 cgc90sta.inv (File 4)

This file provides station inventory information for each of 63 stations occupied during the

R/V Malcolm Baldrige CGC-90 Cruise.  There is one entry for each station. Each line contains

a section number, cast number, latitude, longitude, sampling date (day/month/year), sampling time,

and sounding depth of the station.  The file is sorted by station number and can be read using the

following FORTRAN 77 code (contained in stainv.for, which is File 2):

INTEGER  sta, cast, dep

REAL lat, lon

CHARACTER date*8, time*4

read (1, 10, end=999) sta, cast, lat, lon, date, time,

1 dep

10    format (6X, I2, 5X, I3, 2X, F7.3, 2X, F8.3, 2X, A8, 4X,

1 A4, 4X, I4)

Stated in tabular form, the contents include the following:

-------------------------------------------------------------------------------

Variable        Variable         Starting       Ending

Variable                  type            width           column        column

-------------------------------------------------------------------------------

sta                      Numeric            2                7             8

cast                     Numeric            3               14            16

lat                      Numeric            7               19            25

lon                      Numeric            8               28            35

date                    Character           8               38            45

time                    Character           4               50            53

dep                      Numeric            4               58            61

------------------------------------------------------------------------------

where

sta    is the station number;

cast   is the cast number;

lat    is the latitude of the station (in decimal degrees).

Stations in the Southern Hemisphere have negative latitudes;

lon    is the longitude of the station (in decimal degrees).

Stations in the Western Hemisphere have negative longitudes

date   is the date the station was sampled (day/month/year);

time   is the time the station was sampled (Greenwich Mean Time);

dep    is the sounding depth of the station (in m).

7.5 cgc90.dat (File 5)

This file provides hydrographic, carbon dioxide, and nitrate data for the 63 stations occupied

during R/V Malcolm Baldrige CGC-90 Cruise in the Southwest Pacific.  Each line of the file

consists of a station number, sample ID, CTD pressure, CTD temperature, potential temperature,

CTD salinity, bottle salinity, sigma-t, sigma-theta, nitrate, total CO2, and data quality flags.  The

file is sorted by station number and pressure and can be read using the following FORTRAN 77

code (contained in cgc90dat.for, which is File 3):

INTEGER  sta, samp, qcfl

REAL pres, temp, theta, ctdsal, botsal, sigma, sigmat

REAL no3, tco2

read (1, 10,end=999) sta, samp, pres, temp, theta, ctdsal,

1 botsal, sigma, sigmat, no3, tco2, qcfl

10    format (7X,I2,4X,I5,3X,F6.1,3X,F6.3,3X,F6.3,3X,F6.3,1X,F8.3,

1 3X,F6.3,3X,F6.3,1X,F8.3,2X,F7.1,7X,I3)

Stated in tabular form, the contents include the following:

Variable        Variable         Starting       Ending

Variable                  type            width           column        column

sta                      Numeric            2                8             9

samp                     Numeric            5               14            18

pres                     Numeric            6               22            27

temp                     Numeric            6               31            36

theta                    Numeric            6               40            45

ctdsal                   Numeric            6               49            54

botsal                   Numeric            8               56            63

sigma                    Numeric            6               67            72

sigmat                   Numeric            6               76            81

no3                      Numeric            8               83            90

tco2                     Numeric            7               93            99

qcfl                     Numeric            3              107           109

where

sta     is the station number;

samp    is the sample number;

pres    is the CTD pressure (in dbar);

temp    is the CTD temperature (in deg. C);

theta   is the potential temperature (in deg. C);

ctdsal  is the CTD salinity (in PSS);

botsal* is the bottle salinity (in PSS);

sigma   is sigma-theta (in sigma units);

sigmat  is sigma-t (in sigma units);

no3*    is the nitrate concentration (in umol/kg);

tco2*   is the total carbon dioxide concentration (in umol/kg);

qcfl    is an 3-digit variable that contains data quality flag codes for parameters flagged

by an asterisk in the output file.

Quality flags definitions:

2            =  Acceptable measurement;

3            =  Questionable measurement;

5            =  Not reported;

9            =  Sample not drown for this measurement from this bottle.

8. VERIFICATION OF DATA TRANSPORT

The data files contained in this numeric data package can be read by using the FORTRAN

77 data retrieval programs provided.  Users should visually examine each data file to verify that

the data were correctly transported to their systems.  To facilitate the visual inspection process,

partial listings of each data file are provided in Tables 3 and 4.  Each of these tables contains the

first and last five lines of a data file.

Table 3.  Partial listing of  cgc90sta.inv  (File 4)

First five lines of the file:

2       2  -15.001  -170.005  24/02/90    0151    4817

3       3  -16.472  -169.993  24/02/90    1238    5073

4       4  -18.003  -170.007  24/02/90    2149    4929

5       7  -20.012  -169.993  25/02/90    2049    5320

7      10  -21.992  -169.997  26/02/90    1300    4839

Last five lines of the file:

64     105    0.000  -170.001  11/04/90    1400    5612

65     106    0.500  -170.005  11/04/90    1837    5285

66     107    1.001  -170.005  11/04/90    2145    5316

67     108    2.005  -170.015  12/04/90    0308    5357

68     109    5.001  -170.010  12/04/90    1655    7161

Table 4.  Partial listing of  cgc90.dat  (File 5)

First five lines of the file:

2      222      3.0   28.197   28.196   35.525   35.527   22.729

22.729 -999.900   -999.9       259

2      110      3.6   28.214   28.213   35.531 -999.900   22.727

22.726    0.391   1954.7       522

2      109     18.2   28.152   28.147   35.531 -999.900   22.748

22.746 -999.900   2047.9       553

2      108     39.0   28.043   28.034   35.522   35.527   22.782

22.779    0.391   1966.5       223

2      219     47.7   27.715   27.704   35.521   35.513   22.879

22.876    0.391   -999.9       229

Last five lines of the file:

68    10945    196.6   14.442   14.413   34.595   34.613   25.801

25.795   18.641   2124.9       222

68    10953    295.9    9.985    9.951   34.668   34.661   26.696

26.690 -999.900   2222.9       252

68    10943    496.6    7.978    7.927   34.611 -999.900   26.979

26.972 -999.900   2263.0       552

68    10942    695.1    6.208    6.145   34.558   34.553   27.181

27.173 -999.900   2292.0       253

68    10941    996.9    4.493    4.413   34.568   34.564   27.396

27.387 -999.900   2310.1       252

APPENDIX A

STATION INVENTORY

This appendix lists station inventory information for the 63 sites occupied during R/V

Malcolm Baldrige CGC-90 Cruise in the Southwest Pacific.  The meanings of the column

headings in Table A-1 are as follows.

STANBR       is the station number;

CASTNBR      is the cast number;

LATITUDE     is the latitude of the station (in decimal degrees). Stations in the Southern

Hemisphere have negative latitudes;

LONGITUDE    is the longitude of the station (in decimal degrees). Stations in the Western

Hemisphere have negative longitudes;

DATE         is the sampling date (day/month/year);

TIME         is the sampling time [Greenwich Mean Time (GMT)];

DEPTH        is the sounding bottom depth of each station (in m).

Table A.1.  Station  inventory information for the 63 sites occupied during

R/V Malcolm Baldrige CGC-90 Cruise

STANBR CASTNBR LATITUDE LONGITUDE      DATE    TIME   DEPTH

DEC.DEG   DEC.DEG     D/M/Y     GMT       M

2     1-2  -15.001  -170.005  24/02/90    0151    4817

3       3  -16.472  -169.993  24/02/90    1238    5073

4       4  -18.003  -170.007  24/02/90    2149    4929

5     5-7  -20.012  -169.993  25/02/90    2049    5320

7    9-10  -21.992  -169.997  26/02/90    1300    4839

9   11-13  -25.022  -170.015  27/02/90    1837    5712

10   14-15  -27.053  -170.015  28/02/90    0952    5316

11   16-18  -30.005  -170.043  01/03/90    0441    5429

12   19-21  -32.553  -170.052  02/03/90    0030    5568

13   22-24  -35.023  -170.010  02/03/90    2115    5225

14   25-26  -37.543  -170.037  03/03/90    1252    5170

15   27-29  -40.028  -170.028  04/03/90    0706    4626

16   30-31  -40.968  -170.483  04/03/90    1908    4323

17      32  -41.490  -170.723  05/03/90    0041    3984

18      33  -41.982  -170.983  05/03/90    0554    2974

19   34-35  -42.478  -171.208  05/03/90    1225    1857

20      36  -43.502  -170.853  05/03/90    1902    2904

21      37  -43.985  -170.693  06/03/90    0054    4473

22      38  -44.370  -170.328  06/03/90    0641    5108

23   39-41  -46.045  -170.001  06/03/90    2207    5190

24      42  -47.007  -170.013  07/03/90    0538    5252

25   43-44  -48.022  -169.915  07/03/90    1509    5294

26   45-47  -50.067  -170.070  08/03/90    0844    5279

27   48-49  -51.967  -169.985  09/03/90    0027    5054

28   50-51  -56.768  -170.068  10/03/90    0612    4822

29   52-53  -60.010  -169.883  11/03/90    0502    4139

30   54-55  -55.997  -174.168  12/03/90    2212    4970

31   56-57  -53.948  -176.158  13/03/90    1304    5289

32      58  -50.505  -179.395  15/03/90    0620    4448

33      59  -49.492  -179.745  15/03/90    1420    2012

34      60  -49.725  -179.998  16/03/90    0724    3111

35      61  -49.848  -179.878  16/03/90    1146    4030

36      62  -50.483  -179.357  18/03/90    0704    4458

38      65  -34.648  -178.637  28/03/90    0948    6556

39      66  -32.497  -178.313  28/03/90    2141    5091

40      67  -32.510  -178.523  29/03/90    0237    4249

41   68-69  -32.488  -178.767  29/03/90    1000    3003

42      70  -32.483  -178.502  29/03/90    1257    4211

43      71  -32.493  -178.297  29/03/90    1554    5004

44   72-73  -32.510  -177.998  30/03/90    0029    6005

45   74-75  -32.490  -175.502  30/03/90    1642    5556

46      76  -32.480  -171.478  31/03/90    1104    5259

47      77  -30.000  -170.007  01/04/90    0233    5532

48   78-80  -25.020  -170.030  02/04/90    0857    5834

49   81-82  -22.507  -170.008  03/04/90    0042    5645

50   83-85  -20.025  -170.013  03/04/90    1926    5502

51      86  -17.492  -170.005  04/04/90    0841    4848

52   87-89  -15.005  -170.007  05/04/90    0438    4903

53      90  -11.440  -169.608  05/04/90    2237    5216

54      91  -10.102  -169.503  06/04/90    0700    5332

55   92-93  -10.092  -170.000  06/04/90    1546    5260

56      94  -10.088  -170.248  06/04/90    2053    5141

57      95   -9.492  -170.213  07/04/90    0238    4515

58   96-98   -5.013  -170.020  08/04/90    0526    5511

59      99   -2.005  -170.007  08/04/90    2116    5214

60     100   -0.995  -170.020  09/04/90    0244    5435

61     101   -0.498  -170.007  09/04/90    0605    5698

62 102-104    0.000  -170.020  09/04/90    0931    5342

64     105    0.000  -170.001  11/04/90    1400    5612

65     106    0.500  -170.005  11/04/90    1837    5285

66     107    1.001  -170.005  11/04/90    2145    5316

67     108    2.005  -170.015  12/04/90    0308    5357

68     109    5.001  -170.010  12/04/90    1655    7161