| Groupe hydrographique et océanographique de l’Atlantique |  |
| --- | --- |

Followed by IETA Pierre-Antoine Dumont

Tél. +33 (0) 2 98 14 05 51

Mail : pierre-antoine.dumont@shom.fr

| Object | : | « SHOMPAC 2019 » cruise onboard R/V L’ATALANTE |
| --- | --- | --- |
| Reference | : | Consent letter - 19 June, 2019 – United States Department of State, Bureau of Oceans and International Environmental and Scientific Affairs |
| Attachment | : | 1 appendix. |

# TASK.

This report presents the bathymetric and hydrologic data surveyed in waters under the jurisdiction of the United States of America by Shom (French Hydrographic Office) onboard R/V L’ATALANTE during “SHOMPAC 2019” cruise from July 3rd to 4th , 2019. The scientific objectives of the cruise took part in French EEZ. Only en-route survey was carried out inside American EEZ.

# references.

All data are referenced to WGS84 datum, in geographical coordinates. Vertical reference is the mean sea level.

# OCEANOGRAPHIC DATA.

The configuration for each recorded data is available in log files or in the header of each data file.

Along track data was acquired using:

- A hull mounted thermosalinometer

- Two Vessel-Mounted ADCP at 38 and 150 kHz.

- XBT probes

- Multibeam echosounders.

# HYDROGRAPHIC DATA.

## Data acquisition.

The survey was conducted using Kongsberg EM122 multibeam echosounder. Survey localization is given in appendix 1.

## Data processing.

Processing was done using CARIS HIPS&SIPS 9.1.

### Sound velocity correction.

Sound velocity profiles were regularly sampled (at least every 6 hours) using XBT probes. Data was corrected from sound velocity profile effects.

### Localization and attitude.

Positioning and attitude was supplied by an inertial navigation system PHINS combined with GNSS system receiving correction from C-nav system.

Precision of the ship localization is estimated at 1.00 m at 95%.

### Tide correction.

No tide corrections have been applied to the soundings.

### Lever arms.

Data has been corrected from the lever arms of all the sensors.

## ACCURACY

The sounding accuracy in meter at 95% (where D = depth in meter) is better than:

Horizontal: 1 + 1.3%D

Vertical: 2 + 1.2%D

# DATA PROVIDED.

      - The present report

- Oceanographic data  :

  - XBT probes in EDF format (MK21 export data file readable by a word processor)

  - Hull mounted thermosalinometer in ASCII format

  - Vessel-Mounted ADCP in ASCII Format with theirs configuration in ASCII

- Bathymetric data :

  - Soundings in ASCII files (longitude latitude depth)

Chief scientist Denis Créach,

Director of groupe hydrographique et océanographique de l’Atlantique

Director of the campaign

APPENDIX: SURVEYED AREA

Figure 1 : MBES data acquired during SHOMPAC 2019 inside American EEZ

Background: Shom chart 6880 – INT606