A. Station Locations and Descriptions

Station 1. (Latitude 2119.453’N, Longitude 15758.206’W)

North side of entrance channel to Pearl Harbor, adjacent to a now unused discharge pipeline from Iroquois Point sewage treatment plant.  This station is the most exposed to oceanic conditions, with characteristics of a coral reef environment.  A shallow shoreline bench about 0.5 m deep lies along a calcareous sand beach and rises from the adjacent channel of about 10 m depth.  The primary substrata are consolidated calcareous submerged beach rock, reef with a low coverage of live corals, and intermittent coral boulders and cobbles.  The site is frequently exposed to short period waves generated by northeast trade winds and shows characteristics of a windward reef environment.  It also is directly exposed to large storm waves from the south generated by local Kona storms.  A variety of reef fish are present.  Benthic fauna are dominated by sponges, tunicates, bryozoans and macroalgae, with a few reef corals.

Station 2. (Latitude 2120.828, Longitude 15758.677’W)

North side of West Loch entrance channel about 600 m SE of Kekaa Point, on the western shore of Waipio Peninsula.  The substratum is consolidated limestone, within medium to fine calcareous white sand areas on the shore and channel sides of the hard substrata.  Bottom depths range from 6 m outside of the hard substratum to 1-3 m inshore.  Many abandoned wooden pilings provide habitat for wood borers and fouling organisms.  This is one of the two sites within the harbor where numerous reef corals occur.  A single colony of Porites compressa (Plate 4) and small Leptastrea purpurea colonies occur in shallow areas.  Many live Pocillopora damicornis corals (Plate 5)  occur along the edge of the hard substratum at approximately 4 m depth.

Station 3. (Latitude 2121.802’N, Longitude 15758.555’W)

Shoreline of Walker Bay, on the west shore of Waipio Peninsula, about half way up West Loch.  Substratum is fine, loose silt and mud sediment, with abundant mangroves along a calcareous shoreline bench.  Macrofauna growing in and on sponges occur only on mangrove roots and on debris in shallow water offshore.  Water is highly turbid and sediment laden; depth is 0.5 m Oysters are abundant on mangrove prop roots.

Station 4. (Latitude 2121.831’N, Longitude 15801.386’W)

West bay at head of West Loch near Pearl Harbor National Wildlife Refuge, 100 m offshore of mangroves.  Substratum is the remains of a metal hull of boat wreck covered with a heavy growth of oysters and sponges.  Depth 0.5-1.0 m; water highly turbid and sediment laden.

Station 5. (Latitude 2122.469’N, Longitude 15800.742’W)

In mangrove area at head of West Loch at mouth of Waikele stream.  Environment similar to Station 3, with substratum mostly deep, soft, mud-silt sediments and intermittent sponges. Water is highly turbid and sediment laden, depth is 0.5 m.  Large oysters are very abundant on mangrove prop roots.  Numerous shells of the Japanese little-neck clam Venerupis (Ruditapes) phillippinarum were found in the sediments.  None of these were alive, but the shells were intact and appeared to be recently formed.

Station 6. (Latitude 2120.835’N, Longitude 15758.025’W)

West side of channel entrance just northwest of Drydock Number 4 and Hospital Point and south of the presently unused thermal outfall from the Naval Station Power Plant Number 3.  Substrata are concrete wall shoreline and a calcareous bench and slope ranging from 1 m depth to a flat sand bottom at 6 m, and concrete walls and pilings of the drydock.  Macrofauna varies from reef coral and associated organisms (Plate 4) on the bench to a very dense coverage of a suspension feeding fouling community on walls and pilings (Cover Plate) of the drydock, especially chaetopterid polychaete worms (Plate 6) and sponges, bryozoans and tunicates (Plates 7-9).  Water turbidity was generally low, with underwater visibility of ca. 5 m.  An oil sheen was often observed on the water at this station, and the shoreline showed tar deposits from previous oil exposures.  Macrofauna varies from reef coral and associated organisms (Plate 4) on the bench to a very dense coverage of a suspension feeding fouling community on the drydockwalls and pilings, especially chaetopterid polychaete worms (Plate 6) and sponges, bryozoans and tunicates (Plates 7-9) and the introduced gorgonian Carijoa (=Testeo) riissei (Plate 10).

Station 7. (Latitude 2121.595’N, Longitude 15758.555’W)

Shallow bench area in Middle Loch Channel along the west side of Waipio Peninsula and across from Ford Island.  Bench is approximately 10 m wide and covered with a dense growth of the red

macroalgae Gracilaria salicornia which forms a substratum for macroinvertebrates.  At edge of the bench depth increases to 2-3 m to a flat, coarse sand bottom with abundant coral rubble.

Station 8. (Latitude 2122.498’N, Longitude 15758.658’W)

On west side of Waiawa Peninsula on Middle Loch, at the former Pan American Clipper Landing Dock.  Two sites were sampled, one at the shoreline which has a dense coverage of Gracilaria salicornia on a shallow bench similar to Station 7.  The other site was on concrete and wood pilings offshore of the dock, from the intertidal zone down to 5 m depth.

Station 9. (Latitude 2123.197’N, Longitude 15759.440’W)

At the head of Middle Loch in the vicinity of the floating drydock USS Machinist, which was brought to Pearl Harbor from the Philippines in 1992.  Samples were taken from the steel hull of the Machinist itself from the shallow subtidal to the bottom of the hull at 8 m depth, and from nearby wooden pilings from the intertidal to 4 m depth.

Station 10. (Latitude 2121.457’N, Longitude 15757.190’W)

East of the repair basin in Southeast Loch on pilings of Dock B-21, adjacent to Pearl Harbor Navy Shipyard.  This is the site most exposed to Navy and industrial operations, with considerable ship traffic, hull cleaning and ship maintenance being conducted in the area. Despite this high industrial use of the area, a very abundant fouling fauna was noted on all hard surfaces present.  A light sheen of oil was noted on the water at the time of sampling.  Sampling was conducted on wooden and concrete dock pilings from the shallow subtidal down to 6 m, and in the sediment at 8 m.

Station 11. (Latitude 2121.687’N, Longitude 15757.064’W)

North side of the entrance to Southeast Loch, on concrete pilings, the concrete dock wall and a fossil oyster reef at the bottom of the concrete wall.  Samples were taken from 0.5 to 5 m.  Despite the proximity to the industrial operations of Southeast Loch and the distance of this area into Pearl Harbor, water clarity was relatively high and a few Pocillopora damicornis and Leptastrea purpurea reef corals were found.

Station 12. (Latitude 2122.263’N, Longitude 15757.038’W)

Northeast of Ford Island and the USS Arizona Memorial and just northwest of Mokunui Island, near the terminus of the new Ford Island bridge.  Substratum is clay, compacted to the consistency of rock but still capable of being broken apart by hand, and outcroppings of calcareous beach rock and reef.  The compacted clay supported a high density of polychaetes that could be seen when the clay was broken apart.  An encrusted anchor chain provided additional hard substratum.  The bottom ranges from a nearshore flat area of 1 m depth to a rugose vertical face extending down to 5 m which provided habitat for abundant fishes, especially the yellow fin surgeonfish, Acanthurus xanthopterus.

Station 13. (Latitude 2122.154’N, Longitude 15757.639’W)

On the northwest side of Ford Island, on concrete dock pilings near the USS Utah Memorial along the north channel into East Loch.  Samples were taken from the intertidal to 7 m depth on the concrete pilings.  Underwater visibility ranged from about 3 m near the surface to 5 m near the bottom.

Station 14. (Latitude 2123.064’N, Longitude 15757.601’W)

On the sheet piling separating the intake and discharge zones for cooling water used for the Hawaiian Electric Waiau Generating Station at the head of East loch on its northwest side.  Samples were taken from the discharge side near the end of the sheet piling from the intertidal to the base of the piling at 2 m, about 500 m from the thermal effluent discharge point where the temperature is approximately 3C above ambient. More observations were made next to the discharge point, where temperatures are about 5C above ambient.  Fish are very abundant next to the discharge, where the substratum is a massive sponge reef which covers the entire bottom along the sheet piling side of the discharge.  Sponges also abound on the sheet piling along its entire length, along with dense populations of the anemone Aiptastia puchella, hydroids and bryozoans.

Station 15. (Latitude 2122.282’N, Longitude 15756.138’W)

Rainbow Bay Marina, at the northeast head of East Loch.  The substratum is a shallow, gently sloping intertidal to subtidal zone composed of calcareous rock and rubblewith a thin sediment cover, and soft sediments dominating further offshore.  Both substrata were dominated by a moderately heavy growth of a fine filamentous green algae (cf. Chloradesmis caespitosa), and the soft sediments offshore also have intermittent patches of high coverage of the branching leafy green macroalga Caulerpa sertularoides.  Two areas were sampled, one at the boat launching ramp from the shoreline to the shallow subtidal at 0.5, the other from the surfaces of floating buoys and dock floats of the Marina’s piers and docks.