Foreign/NODC Data Export Format         02/05/2001

Format of data:

write (20,801) irec , xlat , xlon , idate , itime , nlev , npar ,

&                 icode,ifirst , last , ibathy, dsnum

write(20,802)dbdb5,land,q_dtg,alls99,allt99,

&  sbad,tbad,sspike,tspike,deep

write(20,803)lxhdr(1),lxhdr(2),lxhdr(3),lxhdr(4),pophld(4),pophld(5)

801 FORMAT(i9,2f8.2,i8,i6,2i4,4i5,1x,i7)

802 FORMAT(10(i1,1x))

803 FORMAT(6i9)

DO 300 i = 1 , nlev

IF ( npar.EQ.2 ) WRITE (20,99009) depth, temp

IF ( npar.EQ.3 ) WRITE (20,99010) depth, temp, sal

IF ( npar.EQ.4 ) WRITE (20,99011) depth, temp, sal,ss

300  CONTINUE

99009 FORMAT (2f8.2)

99010 FORMAT (3f8.2)

99011 FORMAT (4f8.2)

DISTRIBUTION STATEMENT A: APPROVED FOR PUBLIC RELEASE

DISTRIBUTION IS UNLIMITED.

Definition of variables:

header line 1:

irec    = sequential number

xlat	= latitude in decimal degrees   ( Hemisphere Convention: + = N/E

xlon	= longitude in decimal degrees                           - = S/W )

idate	= date  (YYYYMMDD)

itime	= time  (HHMMSS)

nlev	= number of levels (data cycles) in profile

npar	= number of parameters in profile

2 = depth,temperature

3 = depth,temperature,salinity

4 = depth,temperature,salinity,sound speed or

depth,-99.,-99.,sound speed

icode	= instrument code

Codes:

0 unknown instrument

1 Message data (regardless of instrument)

2 Mechanical Bathythermograph (MBT)

3 Selected Level BT (SBT)

4 *Seabird CTD Time Series

9 Ship deployed AXBT

10 Some unknown electronic temperature depth instrument

11 Expendable bathythermograph

12 Air deployed expendable bathythermograph

14 Helicopter deployed expendable bathythermograph (HXBT)

15 Expendable sound velocity profiler (XSV)

16 sound velocimeter

18 Time Series XBT (TSXBT)

25 Hydrocast; bottles and reversing thermometers

30 Some unknown electronic salinity, temperature, depth instruments

31 Salinity, temperature and depth probe (STD)

32 Low-resolution STD from NODC

33 Conductivity, temperature, depth probe (CTD)

34 CTD with bottles, reversing thermometers

35 CTD time series (YO-YO)

36 Sound velocity, salinity, temp, and depth (SVSTD)

37 Sippican XCTD

38 Seabird CTD SBE-19 Seacat profiler

39 Temperature, salinity microstructure profiler

40 Some unknown current profile instrument

41 Seabird CTD SBE-911 deep ocean

42 Seabird SBE-25

43 Falmouth CTD

60 Some unknown optical profile instruments

75 NAVOCEANO RTDHS (GOODS) PALACE FLOATS

ifirst 	= depth of first measurement in profile

last   	= depth of last measurement in profile

ibathy	= ocean floor depth

dsnum		= sequential dataset number

header line 2: flags (0 or 1)

dbdb5	= ocean floor depth not measured, taken from dbdb5

land	= profile over land

q_dtg	= questionable date/time

alls99	= all salinity values missing

allt99	= all temperature values missing

sbad	= salinity value out of acceptable range

tbad	= temperature value out of acceptable range

sspike	= salinity spike detected (opposite gradients of 2 ppt/m or greater)

tspike	= temperature spike detected (opposite gradients of 2 degC/m or greater)

deep	= profile too deep for location

header line 3: envelop comparison results

lxhdr(1)    = Levitus temperature (0 or 9 digit number)

lxhdr(2)    = Levitus salinity    (0 or 9 digit number)

lxhdr(3)    = GDEM temperature    (0 or 9 digit number)

lxhdr(4)    = *GDEM salinity      (0 or 9 digit number)

pophld(4)   = *representative moods profile temperature (0 or 9 digit number)

pophld(5)   = *representative moods profile salinity    (0 or 9 digit number)

--------------------------------------------------------------

*not available at present time

9 digit number field breakdown:

digits 1-2 	program version (x10)

digit    3 	indicates that profile extended past the envelop (0 or 1)

digits 4-5 	standard deviation used to expand envelop (x10)

digits 6-8 	percent of profile outside envelop used for comparison

digit    9 	not used at present time

example of field breakdown:

101501000

10		= program version 1.0

1		= profile extended beyond envelop

50		= standard deviation of 5.0 used for envelop

100    	= percent of profile outside of envelop

0   	= not used at present time

--------------------------------------------------------------

data:

depth	= depth in meters

temp	= temperature in degrees Celcius

sal	= salinity

ss   	= sound speed in meters/sec