ACNAV SETUP AND OPERATION

A. GENERAL

No discussion can cover all the possible pitfalls and quirks

that would cause poor or no navigation. The operator must have a

basic understanding of how long baseline navigation works, and be

able to recognize when the raw data either collected by the ASP

or processed by the PC is no good and take corrective action. The

best source of information is the ACNAV manual. While it is often

confusing, it is the only information available.

Problems may arise in any of the hardware components. If any

one of the items has a problem, no navigation usually results.

Lets look at the basic interrogation cycle. The program first

determines the ship position, then the fish position. On the ship

cycle, the ASP sends out a 7.5 on the 00 seconds. This signal is

amplified by the UQC and transmitted through whichever transducer

is connected on the patch panel above the ASP. The return signals

from the transponders are received through the same ducer,

amplified by the UQC, filtered and displayed on the ASP. On the

fish cycle, usually 30 seconds later, the ship travel times are

read by the PC and the fix calculated for the ship. Note that

there is a 30 second delay in the calculation.  At 30 seconds

after the minute, the fish cycle is initiated with the output of

7.0 Khz. The fish hears this and replies at 7.5 Khz, triggering

the transponders. On the next ship cycle, the fish cycle times

are read by the PC and a fish fix calculated. A good ship fix is

required to get a good fish fix.  The error due to ship movement

during the 30 second cycle time is minimal when towing a fish

because of slow ship speeds. Careful observation of the received

travel times is the best trouble shooter. Are you getting a poor

travel time?  Maybe you have to change baselines, change ducers,

move the ship. Are the times received the ones being used by the

PC?  Maybe you are gateing out the good time because a minimum or

maximum time is set in the PC (see "Q" commands). Is the

calculated depth of the fish about right. If not, do you have the

approximate depth of the fish correct ("Q" command DF or +/-)?

If the program stops, there are a couple on common problems. Is

the printer still on-line? Is the plotter still in paper hold?

Did someone lean on the plotter standby button? If the system

locks up, there is usually little choice but to reboot. It may be

wise to occasionally close and open a new data file in case of

problems. A data disk will hold about 18 hours of fixes.

B. SOFTWARE SETUP

The most difficult part of long baseline navigation is getting

the net deployed, surveyed, and the proper initialization file

made. See XPONDER.DOC for info on deploying transponders. A good

initialization file, and subsequent accurate fixes, depends on a

good survey and "crunch" of the data. The ACNAV SETUP block

diagram summarizes the inter-relationship of the various

programs. The first step is transponder deployment. This should

be done during daylight hours so the streaming can be monitored

and the transponder picked up if there is a problem. On the Mid-

Atlantic Ridge, a GPS window should be available to get fixes

during deployment and survey to tie the net to the real world and

recover the transponders when required.

A good sound velocity profile is required for accurate fixes.

This may come from XBT data, standard profiles used for the area,

or measured depth/temperature data from over-the-side instruments

such as a CTD. For the first iteration on an init file, the

estimated baseline lengths and depth gathered during deployment

should be used. Use BOWDITCH, VOL 2, page 125-126 (Length of a

degree of lat/lon-located on the bridge) for converting LAT/LON

to meters.

Now to the software. If depth/temp data is available, it can be

converted to depth/velocity using the basic program SOUNDVEL.BAS.

Boot GWBASIC (on disk in d:) and run SOUNDVEL. Unless there is

something very unusual about the area salinity, the error from

the Pacific salinity should be minimal. Ignore it for simplicity.

Be sure the printer is on. You will end up with a printout to

manually input the sound velocity profile in RAYBD.EXE. ZVEL.BAS

will output a file that can be used in RAYBD, but I suggest you

don't bother with this complication. It is shown on the block

diagram. Run RAYBD.EXE (located on the D: disk). Follow the

prompts and refer to the ACNAV manual if you have problems. Use

the file extension .COE (.COEficient) in the output file name.

NOTE: MOST OF THESE PROGRAMS WANT RESPONSES IN CAPS, SO USE CAPS

LOCK.

Once you have the sound velocity coefficients from RAYBD, run

ACNIT.EXE. Follow the prompts and use the RAYBD output when asked

for sound velocity input. Use the file extension .NIT

(iNITialization) on the output. Use the FISH.NIT file as an input

and modify the transponder parameters, and plotter parameters as

required. Refer to the ACNAV manual for details.

With this preliminary initialization file, you are ready to track

the ship while collecting survey points. The fixes will not be

accurate, but good enough to know where you are going. Save the

times on a data disk for later replay. See the file HOWTOUSE.DOC

for specifics on running ACNAV.EXE. See the ACNAV manual for

general information. Try to run the survey during a GPS period to

get the best real world position of the net. The GPS data should

be getting logged on the AII MicroVax, and can be retrieved

during post cruise data reduction. The TRANSLAT utility can be

used to convert the final X,Y,s to LAT/LON. Run the survey

pattern shown in the manual and on the SURVEY TRACKS sketch. When

the survey is complete run SWURV.EXE (3 xpndr net) or SURV2.EXE

(2 xpndr net) to "crunch", or get the proper depth/baseline

lengths of the transponders. The programs require good,

simultaneous, travel times from all transponders on each ship

cycle used. Usually 3-6 sets of travel times at each location

should be used.

Re-run ACNIT and modify the FISH.NIT file based on the SWURV

output data. The survey data can be re-run with this new FISH.NIT

in ACNAV to get an accurate replay of the survey track. The

replay may indicate that the net should be rotated to match

actual ship track. Re-run ACNIT and revise the Baseline Angle

(BA).

To further refine the baseline angle (BA), re-run the survey data

using the new FISH.NIT and record x,y positions that are coincident

in time with good GPS (or other good) lat/lon data. With this

information in hand, run the ORIGIN.EXE program. Read the file

ORIGIN.DOC for program instructions. The output of this file will

give the real world position of the x,y specified and also the

net rotation from true north. The baseline angle can be corrected

in the FISH.NIT if required.

Good Luck, Dudley Foster 9/30/89