HARDWARE SETUP FOR ACOUSTIC NAVIGATION  (Standard ALNAV Setup)

------------------------               --------------------------

|  Unit Selector       |               | Acoustic Control Panel |

------------------------               |     (ACP)              |

|                      |               --------------------------

|  Benthos 455         |               | Drake Receiver         |

|                      |               --------------------------

|  Acoustic Signal     |               |     Clock              |

|    Processor         |               --------------------------

|                      |               |  Marker Receiver       |

|    (ASP)             |               --------------------------

|                      |               |     Mixer              |

|                      |               --------------------------

|                      |               |      NEL               |

|                      |               --------------------------

|                      |               |     UQC                |

------------------------               --------------------------

1. UQC

a)Function  NORMAL

b)Range     FAR

c)Gain      50%

2. ACP in ASP position

3. Unit Selector

a)Gain 60%

b)STANDARD position

c)Selector BENTHOS

4. ASP (Benthos 455)

a) power ON

b) Control REMOTE (if using LOCAL, see 455 manual for Setup)

c) Mode RUN

d) Transmit REPETITIVE

e) Output XDUCER 2

Notes:

1.  A red dot has been placed at most required switch positions.

2.  Turn off UQC when done interrogating transponders to preserve

transponder batteries.