THESE ARE THE INSTRUCTIONS TO REPLAY ALVIN ACNAV DATA DISKS.

THE PROGRAM DISK YOU WILL PUT IN DRIVE A SHOULD HAVE THE FOLLOWING

FILES:

COMMAND.COM

ACNAV.EXE

AND ONE OR MORE XXXXXX.NIT, XXX.SUB OR XXX.FSH FILES.

NEWER DATA DISKS MAY HAVE THE INIT FILE INCLUDED ON THE DATA DISK

LABELED AS SUB.NIT OR FISH.NIT.

1. PUT ACNAV/INIT DISK IN A:

2. PUT DXXXXX.DAT DISK IN B:

3. REBOOT COMPUTER: ALT/CONT/DEL OR ACNAV AT A> PROMPT.

4. NAME OF INIT FILE?--A:NAME.NIT OR B:SUB.NIT

5. TRAVEL TIME INPUT FROM?--B:DXXXXX.DAT

6. RECOMPUTE FIXES?--C

7. LOG OUTPUT DEVICE?--PRN OR NUL FOR NO PRINTED OUTPUT

8. NAME OF NAV OUTPUT FILE--NUL

9. EDIT PLOT PARAMETERS:

IF  PLOTTER HOOKED UP (SUPPORTS HP7475A AND HP7220C) TYPE OK

AND PUSH RETURN. AT "PLOT NET?"--Y

IF NO PLOTTER:  TYPE:  PS -1 AND PRESS RETURN. THEN TYPE: OK

AND PRESS RETURN.

10. AT CC: TYPE GO AND RETURN

THE  FOLLOWING  IS A LIST OF "Q" COMMANDS TO MODIFY  THE  REPLAY.

THESE  ARE SELECTED BY PRESSING Q WHILE THE PLAYBACK IS  RUNNING.

(THIS GIVES YOU THE "CC:" PROMPT)

THEY  ALLOW  THE  OUTPUT  TO BE  MODIFIED  AS  INDICATED  BY  THE

ASSOCIATED FUNCTION.  FOR CORRECT SOLUTIONS FISH (SUB) DEPTH MUST

BE INPUT FROM THE KEYBOARD (DFXXXX OR +,-).

1 October 1988

COMMAND  DEFAULT  FUNCTION                    IBM Q.CMD

+       100      ADD 100M TO ESTIMATED DEPTH OF FISH/SUB

-       100      SUBTRACT 100M FROM ESTIMATED DEPTH

AL                PLOT ALL VEHICLES

BL       1        SETS BASELINE; BL1(AB) BL2(AC) BL3(BC).

CM                ADDS A COMMENT TO UPPER LEFT CORNER OF PLOT

CO                INPUT NEW PLOT CENTER (x,y)

CW       ON       SET SHIP CW OF BASELINE (NEED Y3OF OR RG0 FIRST)

DF       100      SET ESTIMATED DEPTH OF FISH/SUB

DP       OFF      PRINT REDUNDANT FISH/SUB POSITIONS

DS                SET DEPTH OF SHIPS DUCER

EF                PUT END-OF-FILE ON NAV OUTPUT FILE OR START A NEW FILE

EP n,x,y          SET ESTIMATED POSITION OF DEVICE n

EX                STOW PEN, END FILE & TERMINATE PROGRAM

F3       ON       USE THIRD XPONDER TO RESOLVE FISH/SUB POSIT

FF       OFF      ON, FISH ON OPPOSITE SIDE OF B/L FROM SHIP

FH       OFF      IF ON, CHOOSES FISH/SUB POSITION CLOSEST TO SUB POSITION. IF

OFF, CHOOSES FISH/SUB POSITION CLOSEST TO ESTIMATED DEPTH

FI                PLOT FISH ONLY

FR       OFF      ON, USE SHIP POSITION TO SOLVE SURFACE BOUNCE

GA       9        SET GAIN OF XPNDR "A"

GB       9         "    "   "   "   "B"

GC       9         "    "   "   "   "C"

GF       9         "    "   " FISH

GN       9        SET GAIN OF FREQUENCY n1 TO n2 (GN 7,6 SETS CH 7 GAIN TO 6)

GO                ENTER "GO" MODE

GS       9        SET GAIN OF SHIP FREQUENCY

HR       OFF      PRINT HORIZ RANGES

ID                ENTER IDLE MODE (DOES NOT CLOSE FILE)

IN                EDIT FREQUENCY PARAMETERS (GAINS, INTERVALS, FREQS, ETC)

LP       ON       TURNS PRINTER OUTPUT OFF AND ON

MA       ON       MOVE PLOT OUT OF WAY BETWEEN PLOTS

MF                SETS MINIMUM FISH DEPTH TO x, AND MINIMUM TRAVEL TIME FOR

CHANNEL 4 TO A CORRESPONDING VALUE

MT                SET MINIMUM TRAVEL TIME FOR CH n (MT 4,2.5 SUB OR FISH

SET AT 2.5 SEC) 1 =A, 2 = B, 3 = C, 4 = FISH/SUB

NV       ON       TURN PLOTTER OFF OR ON

PS                INPUT NEW PLOT SCALE

RG       0        SET REGION NUMBER ("0" FOR "OFF" OR 1 TO 7)

SF                SET SPEED FILTER (SF 3,5 EXAMPLE SUB SPEED LIMIT TO

5 KNOTS) SHIP = 1, FISH = 2, SUB = 3, CONVERTED TO

M/SEC FOR FUNCT KEY F1 & F2 OPTION (-1 DISABLES)

SH                PLOT SHIP ONLY

SR       OFF      PRINT SLANT RANGES

SU                PLOT SUB ONLY

T1       OFF      PRINT ONE-WAY TRAVEL TIMES

T2       OFF      PRINT TWO-WAY TRAVEL TIMES

TE                SAME AS "GO" BUT NO NAV OUTPUT FILE

TG                SET NEW TARGET POSITION

VT       ON       CONTROLS USE OF TRAVEL TIME CORRECTION FOR SHIP

XT       15 SEC   SET MAXIMUM TRAVEL TIME FOR CHANNEL n. (CHANNELS SAME AS MT)

Y3       ON       USE THIRD XPNDR TO DETERMINE SHIP POSITION

FUNCT SW F3       PLOTTER OFF AND ON

FUNCT SW F10      DEBUG MODE

IF YOU RUN INTO DIFFICULTIES, PLEASE CALL OUR OFFICE (617)548-1400 X2273 AND

ASK FOR DUDLEY FOSTER.

############################################################################