Instructions to run MAKELAT program

This program translates ACNAV x,y positions into Lat/Lon or Lat/lon

into ACNAV x,y positions.  To setup the program, a coincident lat/lon

and x,y must be known, as well as any rotation of the x,y grid from

true north.

Note that the program uses north and west positions as positive values,

south and east as negative.  Since positive x values in the x,y grid

are east of the 0x,0y origin, they should be entered as negative values.

Definitions:

NET ORIGIN: This can be any point that has a coincident x,y and lat/lon.

For convenience, this could be a transponder location, or a

calculated lat/lon of 0x,0y.

NET ROTATION: This is the rotation of the x,y grid from true north, NOT

the baseline bearing used in ACNAV. Ideally, by this point the

net has been rotated to true north in ACNAV, and this value would

be 0.00

ORIGIN OFFSET: This is the x,y of the point used as the NET ORIGIN.

Remember to enter as a negative number if x is east of x0,y0.

To run the program:

1. At > type, MAKELAT <cr>

2. Enter data as prompted, keeping in mind the above definitions.

3. When using option 1, remember to enter x values as negative if

they are east of x0,y0.

##############################################