HP PLOTTER INFORMATION

04/20/93

Alvin users wishing to replay and plot ACNAV data may find the following

information usefull to set up their HP7220 or HP7475 plotters.

The ACNAV program relies on hard wire hand-shaking to communicate with

the plotter rather than software setup. The result is that a standard

RS-232 cable from the IBM to the plotter will not work.  The correct cable

for use with both plotters is as follows:

Plotter		Computer                Computer

25 pin male  	      9 pin  Female	      25 pin Female

2			2   RxD			   3

3			3   TxD			   2

7			5   Grd			   7

20			1   Carrier Detect	   8

6   Data Set Ready	   6

8   Clear to Send	   5

CAUTION - Note the 2/3 swap required for the 25->25 pin cable.

The setups that work for the Alvin systems are as follows:

Computer

Comm2 set for 2400,N,8,1

HP7220					HP7475A

Cable to modem port			Cable to RS232 port

2400 baud				B1=0

conf/test to OFF			B2=0

bypass/norm to NORM			B3=0

parity to OFF				B4=1

even/odd to EVEN			A3=1

half/full to HALF			US=1

hard/wire to MODEM			YD=0

S1=0

S2=0

In case of trouble, both model plotters have self test features which

can be run as follows:

7220C	Connect the modem and terminal ports together using a male/male

straight through interface cable (pins 1->7 & 20 seem to be used).

Turn the power on.

Place the CONF TEST dip switch in the ON position.

Test should draw 7 diagonal lines and end with most of the front

panel lights on - including the ERROR light.

Reset the plotter by returning the test dip switch to OFF.

7475A	Load paper and hold down the P1 and P2 buttons while turning on

the power.

The test program plots a fairly complicated image using all the

pens and ends with the error light off.

#