R/V Brooks McCall

Sampling Plan

- Discrete dissolved oxygen will be collected first, and delivered to the laboratory for analysis.  Dissolved oxygen samples will be collected according to standard practice and will be analyzed immediately using a LaMotte Dissolved Oxygen Water Quality Test Kit #5860. XTech DO700 Handheld Probe dissolved oxygen analyses will also be performed.

- Two pre-acidified 40-ml glass vials are filled to the top (no headspace) for volatile organics analyses (VOA)

- One 12-oz plastic cup will be filled and delivered for LISST particle size distribution.  The LISST samples may be used to inform additional sampling efforts.

- The remaining sample volume from this cup will be delivered for preservation at 4 °C and subsequent UV-fluorescence measurement.  Paul Kepkay will take custody of these samples.

- A 1 liter certified clean amber glass jar will be used to collect a one liter sample for total polycyclic aromatic hydrocarbon (TPH) analysis.  These samples will be stored at 4 °C and one inch (1”) of headspace will be left.

- One additional 12-oz plastic cup will be collected and handled according to the procedure outlined in steps 3 and 4 above. This sample will be used as a means of determining whether there is gradation within the Niskin bottle which would affect the quantity and distribution of any oil collected within the Niskin bottle.