06-06-10_Brooks McCall_Status Report

Complied by Cynthia Cooksey (NOAA)

Communications with shore remain much improved thanks to stable internet connectivity.  Thunderstorms moved through the area throughout the day.

We remain ready to conduct an Offshore VOC sampling transect (per Chris Pfeifer – VOC sampling plan).

A total of 4 CTD casts were completed today.

The first station occupied was BM68, a continuation of the S transect started 6/5/2010.  Station BM68 was located 10km south of the well head.  No significant fluorescence spike was detected at any depth.  The surface oil sheen at this station was nearly continuous and milky white in places.

Continuing our exploration of the southeastern quadrant we located station BM69 5 km SSE of the wellhead.  No significant fluorescence spike was detected at any depth.  However, there was some interesting noise in the fluorescence data in the top 100m.  The surface oil sheen at this station was nearly continuous and milky white in places.

Moving around the 5 km buffer our next station, BM70, was located 5km SE of the wellhead.  No significant fluorescence spike was detected at any depth.  The surface oil sheen at this station was nearly continuous and milky white in places.

We attempted to located the sub surface plume by moving closer to the source along the SE transect.  Station BM71 was situated 1.5 km SE of the well head.  No significant fluorescence spike was detected at any depth.  The surface oil sheen at this station was nearly continuous and milky white in places.

Rototox tests were started today for samples BM68 through BM71, and results will be reported on the evening of 6-07-10.

I will attach a few photos of the surface to highlight the change in coloration of the oil sheen.