06-07-10_Brooks McCall_Status Report

Complied by Cynthia Cooksey (NOAA)

We completed an Offshore VOC sampling transect (per Chris Pfeifer – VOC sampling plan) today.  Collected air and surface water samples at 11 stations along a SW transect from the Discoverer Enterprise.

A total of 3 CTD casts were completed today.

The first station occupied was BM72, located 1.5 km NW of the well head.  No significant fluorescence spike was detected at any depth.  A very weak signal was detected from ~890 -1010m with some small reductions in DO.  The surface oil sheen at this station was thick and continuous.

Not finding asignificant sub-surface plume in the NW quadrant we relocated to the SW quadrant.  Station BM73 was located 1.5km SSW of the wellhead.  No significant fluorescence spike was detected at any depth.  However, there was some weak signals in the fluorescence data from 500-1000m.  The surface oil sheen at this station was thick and continuous.

Our third station of the day, BM74 was located 7.5 km SW of the wellhead near the end of the Offshore VOC transect.  We did detect a weak sub-surface plume from 1100-1150m.

Rototox tests were started today for samples BM72 through BM73, and results will be reported on the evening of 6-08-10.  Station BM67 and BM69 were re-run today due to insufficient rotifers per well on the first attempt.