06-11-10_Brooks McCall_Status Report, Cruise 7 - Day 1

Complied by: Leigh Stevens, Wriggle Coastal Management (for BP)

A number of science staff rotated off RV Brooks McCall following completion of Cruise 6, and some familiarization of new staff with their roles was undertaken prior to, and as part of, the first sample deployment of the day.  The new staff have all done a fantastic job today.

Internet communication has been intermittent due to overloading of the shipboard network.

Weather conditions continue to be ideal, with winds of approximately 5 kts from the NE and calm seas. Overhead conditions were sunny with very hot working conditions on the sampling deck. Several other vessels were working in the near vicinity booming oil for in situ burns.

A total of 4 CTD casts were completed on Day 1 as summarized below and shown in the figure (over page).

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM75 | 5 km WSW ~235o | None | - | Clear surface waters, no sheen. |
| BM76 | 5 km WSW ~250o | Weak, but distinctive | 1080-1120m | Clear surface waters, light sheen, occ. small (<10cm) emulsified patties. |
| BM77 | 5 km WNW ~265o | Weak, but distinctive | 1100-1150m | Light patchy sheen, occasional emulsified oil patches. |
| BM78 | 5 km WNW ~280o | Weak, but distinctive | 1120-1150m | Light sheen, patches of emulsified oil. |

Based on the results of the previous cruise by the RV Ocean Veritas, it was decided to initiate sampling 5km WSW of the wellhead. The first station occupied was BM75, at 06.30h.  No significant fluorescence spike was detected at any depth. A very slight DO drop observed around 910m was sampled but had no corresponding fluorescence signal.

In order to try and find the edge of the plume, BM76 was located a further 1km to the West on the 5km arc from the wellhead at 09.30h. The plume was detected with a weak but clear fluorescence spike at 1080-1120.  Again a slight drop in DO was noted and sampled at 910m, again with no corresponding fluorescence signal.

To determine the width of the plume BM77 was located a further 1km to the West on the 5km arc from the wellhead (located just N of West) at 12.12h.  A definite, but weak hit, was detected at 1100-1150m. A slight drop in DO was also present around 900m and sampled. Again no corresponding fluorescence signal was present.

A fourth site was established a further 1km to the West on the 5km arc from the wellhead (located N of West) at 14.40h. Again a definite, but weak hit, was detected at 1120-1150m.  No significant reductions in DO were associated with any of the fluorescence spikes detected at any of the sites.

The results indicate a weak plume at least 2 km wide is currently located 5 km due west of the wellhead.  Sampling tomorrow will continue to look for the boundary on the NW edge of the plume.

Rototox tests were started today for samples BM75, 76, and 77.  Results will be reported on the evening of 12 June, 2010. BM78 was not initiated because of poor incubation results and will be run on 12 June 2010.

Air quality was monitored during each on-deck sample collection period using a portable VOC meter.  No VOCs were detected (readings all 0 ppm).  The primary sampler was also fitted with a passive diffusion monitor for the majority of the sampling day (9h 12min exposure period).

At the suggestion of the onboard EPA representatives, a freshwater reagent blank was collected and submitted as part of QA/QC procedures. This will be repeated daily as either a reagent or equipment blank.