06-12-10_Brooks McCall_Status Report, Cruise 7 - Day 2

Complied by: Leigh Stevens, Wriggle Coastal Management (for BP)

Weather conditions continue to be ideal, with variable winds of 5 kts from the NE and calm, glassy seas. Overhead conditions were again sunny with very hot working conditions on the sampling deck. A large fleet of vessels were working to the north booming oil and conducting in situ burns.

4 CTD casts were completed on Day 2 and are summarized below and in the figure (over page).

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM79 | 5 km NW ~310o | Weak, narrow | 1140-1160m | Clear surface waters, no sheen. |
| BM80 | 2.5 km NW ~310o | Weak, but distinctive | 1150m | Clear surface waters, very light and sparse sheen, no surface oil |
| BM81 | 2.5 km W ~260o | Weak, but distinctive | 1110-1170m | Clear surface waters, no surface oil, no sheen |
| BM82 | 1 km W ~275o | Weak, 2 distinctive signals | 1040-1060m 1120-1170m | Extensive metallic sheen, occasional emulsified oil patches |

BM79, the first station of Day 2 was deployed at 06.30h on a continuation of yesterdays sampling line heading in a NW arc 5km from the wellhead.  A weak fluorescence spike was detected at ~1500m, and extended over ~20 vertical metres. Slight drops in DO observed at 940m and 980m were sampled, the lower one with a very weak fluorescence signal.  The weak signal, combined with the previous days results indicated this station was near the edge of the plume.

BM80 was located on a direct line between BM79 and the wellhead on a 2.5km arc from the wellhead at 09.00h. The plume was positively identified, peaking at 1150m, and was present across a relatively narrow depth band (~25m).  A small fluorescence signal associated with a small DO drop was sampled at 940m.  The strength and width of the signal indicated it was within the main plume.

BM81 was located at 11.30h approximately 1.5km south of the previous station, on a direct line between BM76 and the wellhead on the 2.5km arc.  The plume was positively identified, peaking at 1140m, and was evident across a 60m depth band. No DO reduction was apparent. Above this, several smaller fluorescence and DO spikes were apparent between 880 and 960m. All were sampled. While still considered a weak signal, the data indicate sampling was within the main plume, and was the strongest signal obtained so far on Cruise 7.

BM82 was located approx. 1km West of the wellhead in an effort to measure the plume near the source. A clear fluorescence signal (stronger than BM81) was measured at 2 depths: a peak at 1050 (20m wide), and a peak at 1140 (50m wide). The 1050m peak was associated with a slight DO decrease, but the lower peak was not. At this location an equipment blank was sampled by filling 1 Niskin sampler near the seabed, returning it to the vessel, draining it, flushing the Niskin with fresh drinking water, refilling it with drinking water and then drawing a sample as usual.

The results of day 1 and 2 show a fluorescence signal from the subsurface plume is measurable, albeit it relatively weak, and extends at least 5km from the wellhead.  It is over 3km wide 5km from the well (but not considered likely to be much wider based on the declining signal encountered toward the margin).  It is over 1.5 km wide 5km from the well.  The strongest signals were returned 1km west of the wellhead.

Sampling tomorrow will seek to locate the NW outer extent of the plume and address data gaps in the NW quadrant.

Rotox tests were started today for samples BM78 through BM82. Results will be reported on the evening of 13 June, 2010. Results for completed tests BM75 and BM76 showed no toxicity.

Air quality was again monitored during on-deck sample collection using a portable VOC meter.  No VOCs were detected (readings all 0 ppm).  The primary sampler was again fitted with a passive diffusion monitor for the majority of the sampling day (8h 10min exposure period).