06-13-10_Brooks McCall_Status Report, Cruise 7 - Day 3

Complied by: Leigh Stevens, Wriggle Coastal Management (for BP)

Weather conditions continue to be ideal, with no appreciable breeze, and flat calm, glassy seas. Overhead conditions were again sunny with very hot working conditions.  Sampling was concentrated in the NW quadrant along the 5km mark based on the westerly subsurface current predictions provided by Shejun Fan (Fugro Global Environmental & Ocean Sciences). 5 CTD casts were completed and weak, but clear, fluorescence signals were detected at 2 distinct depths (1050m and 1150m) at all sites.  See summary below and in the figure (over page).

Overall, Day 3 results showed distinct, albeit it relatively weak, fluorescence signals from the subsurface plume. It was at least 3km wide, 5km from the well.  Although sampling over the past 3 days suggests the plume could be as wide as 7km, the weak signal obtained on Day 1 at BM78 indicated the plume was between this site and BM75.  It appears more likely that the weak small eddies reported as being near the wellhead have pushed the plume northward over the following days, and that this is reflected in the sampling results on Days 2 and 3.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM83 | 6 km W ~275o | Very weak, narrow | 1080m | Clear surface waters, no sheen. |
| BM84 | 5 km NW ~315o | Weak, 2 distinctive signals | 1040-1060m 1120-1160m | Clear surface waters, very light and sparse sheen, no surface oil |
| BM85 | 5km NW ~325o | Weak, 2 distinctive signals | 1045-1055m 1090-1150m | Clear surface waters, no sheen. |
| BM86 | 5 km NNW ~335o | Weak, 2 distinctive signals | 1030-1050m 1130-1160m | Clear surface waters, no sheen. |
| BM87 | 5 km N ~355o | Weak, 2 distinctive signals, occasional spikes and noise near surface. | 1040-1060m 1140-1160m | Dull metallic sheen. No other surface oil. |

BM83 (deployed at 06.30h), 6km from the wellhead on a SSW heading (through the centre of the plume tracked the previous day).  A very weak fluorescence signal just above background was present at ~1080m, and extended ~20 vertical metres. The weak signal, combined with the previous days results, suggest this station is the outer plume edge.

BM84 was located on the 5km arc NW from the wellhead, 1km north of BM79. A clear but weak fluorescence signal was measured at 2 depths: a peak at 1050 (20m wide), and a peak at 1140 (40m wide). The 1050m peak was associated with a DO decrease, but the lower peak was not. At this location an equipment blank was sampled at 200m on the upcast.

BM85 was located 1km north of BM84 on the same 5km arc from the wellhead. Again a clear but weak fluorescence signal was measured at 2 depths: a very small narrow peak at 1050 (10m wide), and a wider signal peak at 1150 (60m wide). The 1050m peak was associated with a relatively strong DO decrease, with a weaker one at the lower peak. A small fluorescence spike was sampled at 1100m along with the stronger signals.

BM86 was located 1km north of BM85, 5km from the wellhead. Again, a clear but weak fluorescence signal was measured at 2 depths: a small narrow peak at 1050 (~20m wide), and a wider signal peak at 1140 (~30m wide). The 1050m peak was associated with a relatively strong DO decrease, with a weaker one at the lower peak. A small fluorescence spike was sampled at 910m along with the stronger signals.

BM87 was located 1km north of BM85, 5km NNW from the wellhead. Yet again, a clear but weak fluorescence signal was measured at 2 depths: a small narrow peak at 1050 (~20m wide), and a wider signal peak at 1140 (~30m wide). The 1050m peak was associated with a relatively strong DO decrease, with a weaker one at the lower peak. A small fluorescence spike was sampled at 910m along with the stronger signals.

A review of the fluorometry data from Days 1 and 2 showed the fluorescence intensity ratio (measured between 340 and 450nm) indicated oil encountered on 12 June 2010 was not as well dispersed as the previous day.  The subsea dispersant report received overnight indicated dispersant operations were interrupted for 12h50min over the sampling period. The results suggest the reduction in dispersant use had a detectable impact on the measured plume character. At the same time, the Rototox test results for samples BM77, BM78, and BM79 collected late on 11 and early on12 June 2010 showed a definite response (24-58% toxicity) at depths where the plume was encountered. Control survival was 100% for these tests. Again this correlates with the interruption in dispersant use and is the first time elevated mortalities have been observed. Unfortunately the timing of the undispersed oil release is unknown at this time so further interpretation is difficult.  Samples BM80-82, collected close to the wellhead later on 12 June 2010, showed no toxicity.

A retest of sample BM77 has been initiated to recheck the initial findings (started today along with BM83 and BM85), and will be and will be reported on 14 June 2010. Samples BM84 and BM86 will be reported on the evening of 15 June and BM87 on 16 June, 2010.

Based on the null results of previous air quality testing a decision was made to monitor on-deck sample collection using the portable VOC meter only when oil was present on the water or odours were apparent. No monitoring was undertaken on Day 3. The primary sampler was again fitted with a passive diffusion monitor for the majority of the sampling day (10h42 min exposure period).