19 June 2010 Brooks McCall_Status Report, Cruise 8 - Day 3

Complied by: Leigh Stevens, Wriggle Coastal Management (for BP)

Weather conditions remain calm with a light breezes blowing predominantly from the northwest.  Sampling began in the northeast along the 10km arc from the wellhead, proceeding through north to the northwest.  Two further samples were collected in the NW, 5km and 3km from the wellhead later in the day.  The aim was to delineate the outer extent, and the NW edge, of the plume.  Because the surface plume has been above the predicted subsurface plume for most of Cruise 8, sampling locations have been located so as to avoid interference with the surface operations and to minimize exposure of the sampling team to oil vapors from fresh surface oil.

On Day 3, a total of 5 CTD casts were completed, as summarized below and shown in the figure (over page).

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM95 | 10 km NE | Very weak | 1100m | Very weak fluorescence signal at 1100m with signal noise between 850-950m detected on the upcast. Slight DO drop at 900m and 1100m. |
| BM96 | 10 km N | No signal | - | Small DO drop at 880m. Shallower site than elsewhere (bottom at ~1000m). |
| BM97 | 10 km NW | No signal | - | Small DO drops at 900m and 1000m. |
| BM98 | 5 km NW | Weak signal | 1140 | Weak but distinct signal, corresponding to DO drop at 1140m. DO drop also present at 1025m not associated with fluorescence signal |
| BM99 | 3 km NNW | No signal | - | Small DO drops at 1025m and 1150m. |

BM95, located in the same general area that the Cape Hatteras reported a plume from yesterday, showed a very weak fluorescence signal at 1100m (20m wide) associated with a slight DO drop (50m wide). A DO drop was also apparent between 875-925m that was not associated with a clear fluorescence signal, although signal noise suggested the possible presence of oil across this depth. The presence of a strong DO signal relative to the very weak fluorescence may represent the residual trace of a subsurface plume in this area.  However, there is little to suggest that the plume is concentrated in this location based on the previous days sampling which showed no evidence of a plume between this site and the wellhead. It is possible an eddy may have peeled from the subsurface plume in the north and drifted to the northeast to give these results.

Site BM96 showed no fluorescence signal associated with the slight DO drop at 880m. The Rachel Bordelon sampling nearby radioed that they had also not encountered the subsurface plume in this location. This site is shallower than most of the other locations sampled and the Seabird sampler was placed on the bottom at 1031m.  Bottom samples indicated no evidence of a plume or concentrations of oil at this depth, which is shallower than where the subsurface plume is usually encountered.

Site BM97 showed no fluorescence signal associated with slight DO drops at 900m and 1000m. DO was variable between 100 and 1050m.  The results from sites BM96 and BM97 indicate the plume was not present 10km north and northwest of the wellhead

BM98, 5km NW of the well, showed a small fluorescence signal and associated DO drop indicating the presence of a weak subsurface plume at this location.  It was proposed to sample 5km due north of the wellhead but because of in situ burning and heavy surface oiling, the site was positioned 3 km NW instead.  No fluorescence or DO signals indicating a plume were found at this location.

The consistent pattern emerging during this voyage is the presence of a slight DO drop at depths where previous voyages were detecting subsurface oil.  As much of the current voyage sampled outside of areas where the subsurface plume was predicted to be located (primarily avoid working in the surface oil plume for health and safety reasons), the absence of a strong fluorescence signal is not surprising.  However, the continued presence of a small DO sag at these depths may reflect the ongoing biological response from oil-degrading bacteria in the absence of a concentrated oil plume being present.

Rototox data submitted for samples BM87-90 last night (after the daily report was submitted) showed no significant toxicity.  Rototox tests were started today for samples BM91-96, with results due to be reported on 20 June, 2010. Tests for BM97- BM99 will be started tomorrow, with reporting due on 21 June 2010. The results will be submitted via email when they are completed.

VOC monitoring continues to be undertaken regularly on all three decks of the vessel.  Screening is undertaken whenever surface oil is encountered, and specific measurements are collected and recorded at each site prior to allowing the sampling team on deck to collect samples.  If an elevated VOC level is detected, staff are kept inside and the vessel relocates to a safe working area. During sampling one VOC meter is left running continually on deck, with another located in the laboratory where most staff are working.  No elevated VOC levels were detected during the days sampling.