18 June 2010 Brooks McCall_Status Report, Cruise 8 - Day 2

Complied by: Leigh Stevens, Wriggle Coastal Management (for BP)

Weather conditions remain ideal, with negligible winds and calm seas.  Proposed sampling 5km north and northwest of the Discovery Enterprise was abandoned due to high VOC readings at the north station, and heavy vessel traffic and surface oiling in the north-northwest quadrant.  Instead, sampling began in the north-northeast, the first cast deployed at 07.15h, 5km from the wellhead.  Based on feedback from NOAA SMU that the Cape Hatteras was detecting the plume 5nm E-NE of the well, we decided to compliment their efforts by sampling the NE quadrant between the Cape Hatteras and the wellhead to see if we could detect the plume in this area.

A total of 4 CTD casts were completed, as summarized below and shown in the figure (over page). None of the casts had a fluorescence signal detected that indicated the presence of a subsurface plume in the northeast quadrant 5km from the wellhead, nor 7km due north of the wellhead.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM91 | 5 km NNE | No signal | - | Slight DO drop at 1000m. Sparse light sheen on surface. |
| BM92 | 5 km NE | No signal | - | Very light sheen on surface. |
| BM93 | 5 km ENE | No signal | - | Thick windrows of fresh oil. |
| BM94 | 7 km N | No signal | - | Relatively heavy surface oiling. |

One sample bottle failed to fire during sampling at BM90 at the 1450m depth site, although sufficient sample was collected from the paired sample for biological and toxicity testing.  The trigger was checked and no fault detected. No further problems were encountered.

BM91 showed no fluorescence or DO signals indicating a plume. BM92 showed a slight DO decrease at depths of 950m (width=10m), and 1050-1100m. Noise in the fluorescence signal was also present but very slight. BM93 showed a slight DO decrease at depths of 860m (width=50m), 1050m (10m), and 1120m (20m).  BM94 showed a very similar pattern, slight DO decreases at 1020m (width=50m), and 1150m (10m). Neither BM93 or BM94 had any appreciable fluorescence signal. As all the slight DO drops were at depths where the plume has been encountered previously, the signal in the DO may reflect the biological response from oil-degrading bacteria in the absence of a concentrated oil plume being present.

Based on the early morning report that the Cape Hatteras was detecting the plume 5nm E-NE of the well, the absence of the plume in the locations we sampled today, which were between their location and the wellhead, is surprising.  A review of our combined results for the day, and of any other vessels sampling in the area, may help in the interpretation of possible plume movement.

Sampling tomorrow will start 10km NE to see if the plume can be located beyond the areas sampled today, before seeking to delineate the plume in the northwest around the 5km mark.

Rototox tests were started for samples BM91-94 and results will be reported on 19 June, 2010.

Due to the late arrival of fresh Rototox tests, the start time for the tests mean results will be completed late today for samples BM87 – 90. The results will be submitted via email when they are completed, and commented on in tomorrow’s daily report.

VOC monitoring continues to be undertaken at each site prior to commencement of sampling, and continually during drawing of samples following retrieval.  VOC alarms were triggered north of the wellhead on the 5km arc this morning while transiting through the area.  Staff and crew remained inside until the readings returned to safe levels, and proposed sampling locations in this area were relocated to the NE.