Brooks McCall Status Report - Cruise 12 - Day 1 - 11 July 2010

Complied by: Alison Lane, Ecosystem Management and Associates Inc. (for BP)

Site conditions

We experienced calm seas with wave height only approximately 1 to 2 feet and very light winds throughout the day, although winds increased gradually during the afternoon and conditions became slightly choppier as the day progressed. As a result of the exceptional conditions we were able to complete 5 casts.

There was heavy vessel traffic around the wellhead, limiting our options for site selection. SIMOPS indicated a preference that we work at least 3 km from the wellhead due to the other operations going on at the site.

Air Quality

VOC and benzene monitoring was conducted while we were in areas of heavy surface oiling to the north of the wellhead by Paul Bradley, Industrial Hygiene Specialist with Bureau Veritas. Maximum VOC reading at any site was 0.9 ppm. One crew member, Janice Kroome, was fitted with a passive diffusion monitor prior to commencing the first cast (0630 hours) until the completion of deck work for the day (2020 hours).

Equipment

The pump for the CTD had been replaced between voyages to see if this got rid of the noise occasionally seen in the DO signal, however on the descent of three of the casts some noise was still seen in the DO signal over a short depth. We are reviewing options for trying to correct this while on this cruise but suspect we will have to switch out the CTD unit on our return to port.

Findings

The following definition has been adopted to provide common terminology on subsurface oil:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."

A total of 5 CTD casts were completed, as summarized below and shown in the figure (over page). Note that LISST results will be presented and discussed in an update submitted on 12 July 2010.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM118 | 3 km W | None |  | There was no fluorescence signal or decrease in DO apparent during the cast. Small patches of emulsified oil. |
| BM119 | 3 km S | None |  | There was no fluorescence signal or decrease in DO apparent during the cast. Continuous rainbow sheen. |
| BM120 | 3 km SW | Very weak | 1190 - 1210 | Slight increase in fluorescence signal over narrow band around 1200m, no reduction in DO evident at this depth. Almost no observed oil at surface. |
| BM121 | 3 km WSW | None |  | There was no fluorescence signal or decrease in DO apparent during the cast. Occasional small amounts of light sheen < 1% cover. |
| BM122 | 4 km WS | None |  | There was no fluorescence signal or decrease in DO apparent during the cast. Occasional small amounts of light sheen < 1% cover. |

BM118 was located 3 km to the west of the wellhead. The initial sample site selected was not accessible due to heavy vessel traffic close to the position, requiring us to relocate to more open water. Small patches (5 – 10cm diameter) of emulsified oil were present on the water surface but did not cover even 1% of the water surface. No fluorescence signal or DO decrease at the depths where the plume is generally encountered. DO minimum was between 420m and 560m depth.

BM119 was located 3 km to the south of the wellhead. Continuous rainbow sheen was present around the vessel. Although there was an odour of oil, it was not strong. No increase in fluorescence signal or decrease in DO was observed the depths where the plume is generally encountered. DO minimum was between 450m and 550m depth. An equipment blank was run at this site.

BM120 was located 3 km to the south west of the wellhead. A very weak spike (0.5 mg/m^3) was detected at approximately 1205 m. Coincident with this depth there was distortion of the DO signal during the cast, although this was not apparent as the CTD was retrieved and it did not appear that there was a true dip in DO related to the fluorescence spike. The DO minimum was between 450m and 600m depth.

BM121 was located 3 km to the south west of the wellhead. No increase in fluorescence signal or decrease in DO was observed the depths where the plume is generally encountered. DO minimum was between 440m and 520m depth.

BM 122 was located 4 km to the south west of the wellhead, continuing the trajectory from the wellhead through site BM120 where a weak signal had been observed. The recent data from Ocean Veritas showed an increased signal strength relative to background at some of the more distant sites samples and as we were unable to sample closer due to vessel activities it seemed worth a try. No increase in fluorescence signal or decrease in DO was observed the depths where the plume is generally encountered. DO minimum was between 450m and 550m depth. An equipment blank was run at this site.

Toxicity Testing

Rotox tests started today for sample stations BM118 – 122, with result due to be reported on 12 July 2010.

Summary

Day 1 of the Brooks McCall Cruise 12 sampled between 3 and 4 kilometres to the west and south west of the wellhead. Evidence of the subsurface plume was only detected at one site (BM120).

Crew Manifest

| OPEL, Chris | Master | TDI BROOKS |
| --- | --- | --- |
| MAJNI, Geno | Chief Mate | TDI BROOKS |
| KAMAL, Wasib | Chief Engineer | TDI BROOKS |
| McCLUSKEY, Ben | Oiler | TDI BROOKS |
| FELIX, Frank | A/B | TDI BROOKS |
| MAJNI, Ann | OS | TDI BROOKS |
| NARCISIS, Paul | Cook | TDI BROOKS |
| MACDONALD, Ian | Crew Chief | TDI BROOKS |
| BROOKS, Jimmy | Winch Operator | TDI BROOKS |
| LANE, Larry | Deck | TDI BROOKS |
| LANE, Alison | Chief Scientist | EM&A |
| HILL, Stephanie | Data Manager | CSS |
| ESSEX, Laura | Rototox testing | EM&A |
| CLARK, Paul | CTD Tech | GERG/TEXAS A&M |
| MA, Xiaowei | LISST and fluoremetry | DFO |
| THAMER, Peter | LISST and fluorometry | DFO |
| MOLINARO, George | Water sampling | ENTRIX |
| BROWN, Robyn | Water sampling | ENTRIX |
| Name to be confirmed | Water sampling | ENTRIX |
| BRADLEY, Paul | Industrial Hygienist | BV |
| CONLEE, Don | Meteorologist | TEXAS A&M |
| Name to be confirmed | Meteorologist | TEXAS A&M |
| SIMMONS, Zeke | Water sampling | NOAA |
| KROOME, Janice | EPA representative | EPA |

***