Brooks McCall Status Report - Cruise 13 - Day 2 - 18 July 2010

Complied by: Alison Lane, Ecosystem Management and Associates Inc. (for BP)

Site conditions

Seas gentle with 1 to 2 foot wave height and light winds all day. Seismic survey work all day prevented access closer than approximately 7 nm to the wellhead.

Air Quality

VOC monitoring was conducted at regular intervals at each sample site throughout the day by Bob Miller, Industrial Hygiene Specialist with Bureau Veritas. VOC maximum, recorded at BM135 was 0.7 ppm.

Equipment

The CTD continued to give us problems today, despite several hours of cabling work last night. On both the casts today the pressure sensor signal gave anomalous readings, and although the fluorometry signal appeared unaffected, the depth of the system could not be verified. At the first site we were able to trigger some Niskins. clearly below, within and above the plume before the problem appeared. On the second site initially no Niskins were triggered as the problem appeared during the descent. Significant rewiring and replacement of parts was done throughout the day. This enabled us to resample the second station and trigger the Niskin bottles.

Findings

The following definition has been adopted to provide common terminology on subsurface oil:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."

Due to the CTD woes only two casts were completed, as summarized below and shown in the figure (over page).

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM135 | 14 km SSW | Weak | 1200m – 1230m | One very weak signal centered at 1215m. DO decrease with the peak in fluorescence. Occasional broken light sheen. |
| BM136 | 14 km SSW | Very weak | 1200m – 1240m | One very slight peak in fluorescence at 1230m but with a clear DO decrease associated with the peak in fluorescence. No surface oil observed. |

BM135 was located 14km to the south west of the wellhead approximately 2 km from the last station sampled yesterday where a weak plume signal was detected. There were only very occasional small traces of surface sheen at the site. There was a single weak peak in fluorescence between 1200m and 1230m. LISST analysis showed a very slight increase in small particles at this depth. DO minimum was between 500m and 600m depth.

BM136 was located 14km to the south south west of the wellhead, approximately 1 km south east of site BM136 to compensate for the small number of samples collected at that site. There was no surface oil observed at the station. There was a single very weak peak in fluorescence between 1200m and 1250m with a decrease in DO at the same depth. On the initial cast at this site CTD problems meant that samples could not be collected. After repairs to the CTD we returned to the station and used the fluorescence data from the earlier cast to complete the sampling program aborted earlier. Please note that the problems with the CTD recording mean that only information from the repeated cast are attached to this report and the in-situ fluorescence data is not available. LISST analysis did not detect an increase in small particles at this depth. DO minimum was between 400m and 600m depth.

Toxicity Testing

Rotox tests started today for sample stations BM135 to BM136 with results due to be reported on 19 July 2010.

Tests were completed today for stations BM131 to BM132. No significant mortality was observed in any test.

LISST and Fluorometry

The LISST analysis demonstrated an slight increase in small particles in the depths associated with the in-situ fluorescence peak at stations BM135 but not at BM136. The fluorescence ratio intensity at Stations BM133 and BM134 were elevated. Today’s LISST and fluorometry report is attached in the daily deliverables for today.

Summary

Day 2 of the Brooks McCall Cruise 12 sampled 14 kilometers to the south south west of the wellhead and we were plagued with equipment issues although still managed two successful casts and sample collection.

The choice of locations was based on data from the yesterday, the Fugro modeling and limitations to areas closer to the site as a result of seismic vessel operations. A very weak or weak plume signal was detected at both sites sampled today. The maximum vertical extent of the plume at any of the sample sites was 40 meters. The lowest recorded DO reading using the handheld Extech probe was 3.85 mg/L in the DO minimum at 500 meters at site BM136.

Crew Manifest

| OPEL, Chris | Master | TDI BROOKS |
| --- | --- | --- |
| MAJNI, Geno | Chief Mate | TDI BROOKS |
| KAMAL, Wasib | Chief Engineer | TDI BROOKS |
| McCLUSKEY, Ben | Oiler | TDI BROOKS |
| FELIX, Frank | A/B | TDI BROOKS |
| MAJNI, Ann | OS | TDI BROOKS |
| NARCISIS, Paul | Cook | TDI BROOKS |
| MACDONALD, Ian | Crew Chief | TDI BROOKS |
| GREEN, Billy | Winch Operator | TDI BROOKS |
| LANE, Larry | Deck | TDI BROOKS |
| LANE, Alison | Chief Scientist | EM&A |
| HILL, Stephanie | Data Manager | CSS |
| ESSEX, Laura | Rototox testing | EM&A |
| CLARK, Paul | CTD Tech | GERG/TEXAS A&M |
| KEPKAY, Paul | LISST and fluorometry | DFO |
| RYAN, Scott | LISST and fluorometry | DFO |
| SUSDORF, Gary | Water sampling | ENTRIX |
| McTHENIA, Andy | Water sampling | ENTRIX |
| WANNER, Brandon | Water sampling | ENTRIX |
| MILLER, Robert | Industrial Hygienist | BV |
| TODD, Brandi | EPA representative | EPA |

***