29 July 2010 - Brooks McCall Status Report - Cruise 13 - Day 1

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Because of limited sampling undertaken near the well recently (due to seismic survey work and weather constraints), the strategy for Day 1 was to sample a 2km radius from the wellhead to determine if any subsurface oil was present following capping of the well, which has stopped the flow of oil into the area.

Sampling was undertaken in fine sunny weather with 1-2 ft seas and a 5-10 knot northerly breeze.  No surface oiling was observed. A total of 4 CTD casts were completed, as summarized below and shown in the figure (over page). Sampling commenced in the west, circling anti-clockwise through south, east and north.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM141 | 2.0 km W | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM142 | 2.0 km S | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM143 | 2.0 km E | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM144 | 2.0 km N | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |

The above results, and LISST and flurometry data from the days sampling, showed no indication of subsurface oil near the well. There were also no indications of any depression in DO concentrations at depths where subsurface oil has previously been detected.

Water samples collected today were also tested using automated titration methods in accordance with Dispersant Monitoring and Assessment Directive – Addendum 4.  A total of 20 samples were collected and analyzed, 12 samples from deeper than 1000m and 8 samples from shallower depths.  This initial data, submitted with the daily data summary, indicate no major deviations between the in situ Seabird DO sensor measurements and those obtained using automated titration methods on samples brought to the surface. The handheld meters (membrane and optical) showed similar trends and, while less in agreement with the other measures, showed no major deviations between each other.  Further testing will be undertaken on the remaining 2 days of the cruise to compare the different measurement methods.

VOC monitoring has been undertaken regularly throughout the day by Adam Kueker, Bureau Veritas Industrial Hygiene Specialist.  No VOCs were detected above background.

Rotox tests were started for samples BM141-144 with results due to be reported on 20 June, 2010.

Other issues: A new data manager, new samplers, combined with recent changes in the numbering system, resulted in early confusion before the correct numbering sequences were determined.  Also, a single sample bottle was inadvertently left unfilled on cast 2 at the 500m depth (SW-20100729-BM15-12).  A note has been added to the data spreadsheet identifying this.

It is proposed to continue sampling near the wellhead on Day 2 to establish a robust baseline around the well before further work commences on the proposed static kill on Saturday.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."