30 July 2010 - Brooks McCall Status Report - Cruise 15 - Day 2

Complied by: Leigh Stevens, Ecosystem Management and Associates (for BP)

Sampling continued from yesterday in the vicinity of the wellhead to determine if any subsurface oil was present following capping of the well. The strategy for Day 2 was to sample a 5km radius from the wellhead at locations between the sites sampled yesterday.  The assistance of SIMOPS in facilitating access to areas near the well over the past two days is much appreciated.

Weather for the day was overcast with 2-3 ft seas and a 10-15 knot NNW breeze dying down in the afternoon. No surface oiling was observed. A total of 4 CTD casts were completed, as summarized below and shown in the figure (over page). Sampling commenced in the SW, then SE, NW and NE.

| Station | Position from wellhead | Fluorescence signal | Signal depth | Comment |
| --- | --- | --- | --- | --- |
| BM145 | 5.0 km SW | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM146 | 5.0 km SE | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM147 | 5.0 km NW | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |
| BM148 | 5.0 km NE | No signal | - | No signals in either the fluorescence or DO profiles indicating any evidence of oil in the water column. |

As with yesterday, the above results and LISST and fluorometry data from the days sampling showed no indication of subsurface oil near the well. There were also no indications of any depression in DO concentrations at depths where subsurface oil has previously been detected.

Water samples were again collected and tested using automated titration methods in accordance with Dispersant Monitoring and Assessment Directive – Addendum 4.  20 samples were collected and analyzed, 12 samples from deeper than 1000m and 8 samples from shallower depths. As with yesterdays initial data, no major deviations were apparent between the in situ Seabird DO sensor measurements and those obtained using automated titration methods. Handheld DO meters (membrane and optical) showed similar trends although better concordance today following the daily recalibration. Again, these measures showed no major deviations between each other. Membranes on 2 separate handheld meters failed today and were replaced. Measurements were completed using a spare pre-prepared probe and membrane. Further testing will be undertaken on the final day of the cruise to compare the different measurement methods.

VOC monitoring has been undertaken regularly throughout the day by Adam Kueker, Bureau Veritas Industrial Hygiene Specialist.  No VOCs were detected above background.

Rotox tests were started for samples BM145-148 (results due to be reported on 31 July 2010), and testing was completed for samples BM141-144.  No toxicity was apparent in any samples.

Tomorrow it is proposed to sample approximately 30nm SW of the wellhead in areas where the ASA SIMAP computer model suggests a dilute subsurface oil signal may remain.

The following definition has been adopted to provide common terminology on how subsurface oil is referred to:

Oil Plume: "Concentration of oil (above background) in the water column that appears to be part of a larger pattern of dispersed oil based on real-time fluorometry and LISST particle data analysis."